| Radiocommunication Study Groups Third Meeting of Working Party 5D Seoul, Korea, 8-15 October 2008 |  |
| --- | --- |
| Source: Revision 2 to Document 5D/TEMP/109 |  |
|  | 30 October 2008 |
|  | English only |
| Working Party 5D |  |
| LIAISON STATEMENT TO EXTERNAL ORGANIZATIONS |  |
| PARAMETERS OF IMT RADIO INTERFACE TECHNOLOGIES FOLLOWING wrc-07 |  |

Following the liaison statement that ITU-R Working Party (WP) 5D sent to the external organizations at its Dubai meeting, June 2008, WP 5D has considered the associated replies (documents 5D/244, 5D/249, 5D/254, 5D/292, 5D/298 and 5D/302) and has compiled the information received from some SDOs and other input contributions regarding the parameters of radio interfaces technologies.

The compiled information is attached to this document. WP 5D draws the attention of the external organizations to the fact that some parameters are missing in the compiled tables for some technologies. While noting that certain parameters have already been requested and not provided, WP 5D reiterates its goal to have the tables as complete as possible, and also seeks confirmation/refinement of the values already shown in the tables.

WP 5D thanks the SDOs for their cooperation and will be happy to further communicate on this subject.

| Contact: | Colin Langtry Counsellor, ITU-R SG5 | Tel: +41 22 730 6178 Email: colin.langtry@itu.int |
| --- | --- | --- |

Attachments: 1

Attachment

Preliminary IMT parameters for the band 790-862 MHz

PART N°1: Parameters for UMTS (WCDMA), LTE (OFDMA) and WiMAX (OFDMA)

TABLE 1

Parameters for UMTS (WCDMA), LTE (OFDMA) and WiMAX (OFDMA)

| No/No | Parameter | Base station | Mobile station | Base station | Mobile station | Base station | Mobile station |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TECHNOLOGIES |  | UMTS (WCDMA) |  | LTE (OFDMA) |  | Mobile WiMAX (OFDMA) |  |
| 1. | Class of emission |  |  |  |  | For 5 MHz, 5M00D7WEW For 7 MHz, 7M00D7WEW For 10 MHz, 10M0D7WEW |  |
| 2. | Modulation parameters | QPSK 16-QAM 64-QAM | QPSK 16-QAM | QPSK 16-QAM 64-QAM | QPSK 16-QAM 64-QAM | QPSK 1/2 QPSK 3/4 16-QAM 1/2 16-QAM 3/4 64-QAM is optional : 64-QAM 1/2 64-QAM 2/3 64-QAM 3/4 64-QAM 5/6 | QPSK ½ QPSK 3/4 16-QAM 1/2 16-QAM ¾ |
| 3. | Duplex mode | FDD |  | FDD |  | FDD/TDD |  |
| 4. | Spectral mask of signals, including | - | - | - | - | The final WiMAX Forum emission masks for equipment in these frequency ranges remain under development. An example of regulatory unwanted emission requirements can be found in the FCC regulations Sections 27.53 and 90.543 for the 700 MHz band (698-806 MHz). These provide an example of the target levels that will be taken into consideration during development of the final WiMAX Forum emission masks. |  |
| 4.1 | -3 dB radiation bandwidth | - | - | - | - |  |  |
| 4.2 | -30 dB radiation bandwidth | - | - | - | - |  |  |
| 4.3 | -60 dB radiation bandwidth | - | - | - | - |  |  |
| 5. | Maximum spectral power density, dB(mW/Hz) | -22,8 | -41,8 | -23,5 | -43,5 | 5 MHz: -19,6 7 MHz: -21,1 10 MHz: -22,6 See Note | -29 |
| 6. | Signal bandwidth (MHz) | 3,84 | 3,84 | 1.08, 2.7, 4.5, 9, 13.5 and 18 | 1.08, 2.7, 4.5, 9, 13.5 and 18 |  |  |
| 7. | Transmitter e.i.r.p. (dBm) | 55 | 21 | 55 | 23 | 63,6 | 25 |
| 8. | Typical height of the transmitting antenna (m) | 20 to 30 | 1,5 | 20 to 30 | 1,5 | 15 to 32 | 1.5 |
| 9. | Transmitting antenna type (sectorized/omnidirectional) | 3 sectors | omni | 3 sectors | omni | sectorized | omnidirectional |
| 10. | Transmitting antenna gain, dBi | 15 | 0 | 15 | 0 | 15.6 | 0 |
| 11. | Feeder loss (dB) | 3 | 0 | 3 | 0 |  |  |
| 12. | Antenna pattern width, degrees |  |  |  |  |  |  |
| 12.1 | - in a horizontal plane (at 3 dB) | 65° | NA | 65° | NA | 68° | Omni |
| 12.2 | - in a vertical plane | ITU-R F.1336-2 | NA | ITU-R F.1336-2 | NA | 13° | 40° |
| 12.3 | - antenna downtilt | 3° | NA | 3° | NA |  |  |
| 13. | Relative level of side lobes |  |  |  |  | -20 dB See Note |  |
| 14. | Channel bandwidth (MHz) | 5 | 5 | 1.4, 3, 5, 10, 15, 20 | 1.4, 3, 5, 10, 15, 20 | 5, 7, 10 | 5, 7, 10 |
| 15. | Power control range (dB) | >18 | 75 | - | 63 |  |  |
| 16. | density of the equipments (number per km2) | TBD | TBD | TBD | TBD | TBD | TBD |
| 17. | density of the equipments (number per km2) operating at co-frequency | TBD | TBD | TBD | TBD | TBD | TBD |
| 18. | Polarization | TBD | TBD | TBD | TBD | TBD | TBD |
| 19. | Capacity Criteria, including capacity per cell |  |  |  |  |  |  |
| 20. | Frequency Reuse Factor |  |  |  |  |  |  |
| 21. | Receiver thermal noise |  |  |  |  |  |  |
| 22. | Reference sensitivity |  |  |  |  |  |  |
| 23. | Receiver blocking response |  |  |  |  |  |  |

NOTE 1 – This table contains only parameters relevant to IMT emissions (i.e. when the IMT system is the source of potential interference). Further parameters (e.g. receiver performance) are relevant for the case where the IMT system is the victim of interference.

NOTE 2 – The density of the mobile terminals (number per km2), operating at the same frequency at the same time and in the same geographical area, as well as the average transmit power of terminals are also important parameters when studying potential interference from IMT mobile terminals particularly in the case of stochastic analysis and should therefore be considered in the coexistence studies.

TABLE 2

Complementary information about WiMAX (OFDMA)

| Frequency range MS (MHz) | Frequency range BS (MHz) | Profile name | Channel freq step (kHz) | Channel bandwidth (MHz) | FFT size | Duplex mode | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 698-862 | 698-862 | 7D | 250 | 5 | 512 | TDD | The bandwidths are applicable to both the MS and BS. |
|  |  |  |  | 7 | 1024 |  |  |
|  |  |  |  | 10 | 1024 |  |  |
|  |  |  |  | 2×5 | 512 | FDD |  |
|  |  |  |  | 2×7 | 1024 |  |  |
|  |  |  |  | 2×10 | 1024 |  |  |

PART N°2: Parameters for cdma2000, High Rate Packet Data (HRPD), and Ultra Mobile Broadband (UMB)

The values for requested system parameters are presented in Table 3. Please note that some parameters relating to UMB in this section (Table 3 and 4) are near completion in 3GPP2 and will be provided in future. Table 4 shows the mobile station and access terminal maximum transmit power as currently specified in the 3GPP2 specifications.  Additional information relating to the other band classes in Table 5 will be provided in the future.  It is anticipated that the maximum power and emission specifications for Band Classes 18 and 19 will be identical to those for Band Classes 7 and 10.

TABLE 3

cdma2000, High Rate Packet Data (HRPD), and Ultra Mobile Broadband (UMB)

| No/No | Parameter |  | Base station | Mobile station | Base station | Mobile station | Base station | Mobile station |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TECHNOLOGIES |  |  | cdma2000 |  | HRPD |  | UMB |  |
| 1. | Class of emission |  |  |  |  |  |  |  |
| 2. | Modulation parameters | Forward link or downlink | Data modulation: BPSK; QPSK, 8-PSK, 16-QAM Spreading modulation: QPSK |  | Data modulation: QPSK, 8-PSK, 16-QAM, and 64-QAM Spreading modulation: QPSK |  | Data modulation: QPSK, 8-PSK, 16-QAM, and 64-QAM |  |
|  |  | Reverse link or uplink | Data modulation: 64-ary Orthogonal Modulation, BPSK; QPSK, 8-PSK Spreading modulation: HPSK |  | Data modulation: BPSK; QPSK, 8-PSK Spreading modulation: HPSK |  | Data modulation: QPSK, 8-PSK, 16-QAM, and 64-QAM Spreading modulation: QPSK (CDMA control segment) |  |
| 3. | Duplex mode |  |  |  |  |  |  |  |
| 4. | Spectral mask of signals, including |  | See the Recommenda-tions ITU-R M.1580-2 and M.1581-2 for cdma-MC” | See the Recommenda-tions ITU-R M.1580-2 and M.1581-2 for cdma-MC | See the future revision of Recommenda-tions ITU-R M.1580-2 and M.1581-2 for cdma-MC | See the future revision of Recommenda-tions ITU-R M.1580-2 and M.1581-2 for cdma-MC | To be provided | To be provided |
| 4.1 | -3 dB radiation bandwidth |  |  |  |  |  |  |  |
| 4.2 | -30 dB radiation bandwidth |  |  |  |  |  |  |  |
| 4.3 | -60 dB radiation bandwidth |  |  |  |  |  |  |  |
| 5. | Maximum spectral power density, dB(mW/Hz) |  |  |  |  |  |  |  |
| 6. | Signal bandwidth (MHz) |  |  |  |  |  |  |  |
| 7. | Transmitter e.i.r.p. (dBm) |  | Per Licence | See Table 4 | Per Licence | See Table 4 | To be provided | To be provided |
| 8. | Typical height of the transmitting antenna (m) |  |  |  |  |  |  |  |
| 9. | Transmitting antenna type (sectorized/omnidirectional) |  |  |  |  |  |  |  |
| 10. | Transmitting antenna gain, dBi |  |  |  |  |  |  |  |
| 11. | Feeder loss (dB) |  |  |  |  |  |  |  |
| 12. | Antenna pattern width, degrees |  |  |  |  |  |  |  |
| 12.1 | - in a horizontal plane (at 3 dB) |  |  |  |  |  |  |  |
| 12.2 | - in a vertical plane |  |  |  |  |  |  |  |
| 12.3 | - antenna downtilt |  |  |  |  |  |  |  |
| 13. | Relative level of side lobes |  |  |  |  |  |  |  |
| 14. | Channel bandwidth (MHz) Channel bandwidth(s), some systems being capable of multiple bandwidths |  | It has a chip rate of N= 1.2288 Mchip/s (currently, N = 1 and 3 are specified) Carrier spacing is 1.23 MHz for band Class 0; 1.25 MHz for all other Band Classes |  | M carriers of 1.2288 Mchip/s can be aggregated in a single or multi-carrier operation, where M  1, 2, 3, 4, …, 15. Carriers in a multi- carrier operation do not have to be contiguous and their spacing is multiple of 30, 50, 25, 20, or 12.5 kHz depending on the Band Class |  | Native bandwidth can fit deployment of 1.25 to 20 MHz (0.768 + N  0.1536 MHz, N  0, …, 123). Also, multi-carrier configurations can be used to aggregate two or more carriers of possibly different native bandwidths and operating in non-contiguous spectral allocation |  |
| 15. | Power control range (dB) |  | Open loop and Closed loop (800, 400, 200, 50, 25, 12.5 Hz update rate) with power control steps: 0.25  N, N  dB |  | Open loop and Closed loop (600 and 150 Hz update rate) with power control steps: 0.5  N, N  dB |  | Open loop and Closed loop (Once every N data frames (0.911ms), N  4, 8, 16, 32) with power control steps: 0.25  N, N  dB |  |
| 16. | density of the equipments (number per km2) |  |  |  |  |  |  |  |
| 17. | density of the equipments (number per km2) operating at co-frequency |  |  |  |  |  |  |  |
| 18. | Polarization |  |  |  |  |  |  |  |
| 19. | Capacity Criteria, including capacity per cell |  | Capacity in a cellular system can be measured in terms of simultaneous voice users per cell, data throughput per cell, etc. The actual capacity is dependent on the assumptions made about system configuration, loading, quality, and fairness, among other things. The actual capacity of the system is not part of the cdma2000 specifications. |  |  |  |  |  |
| 20. | Frequency Reuse Factor |  | Frequency reuse of one is supported. |  |  |  |  |  |

| 21. | Receiver thermal noise | Band Class 0, 2, 3, 5, 7, 9, 10, 11, 12 | -105 dBm/1.23 MHz (8 dB NF) | -103 dBm/1.23 MHz (10 dB NF) | -105 dBm/1.23 MHz (8 dB NF) | -103 dBm/1.23 MHz (10 dB NF) | To be provided | To be provided |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Band Class 1,4,6,8 | -105 dBm/1.23 MHz (8 dB NF) |  | -105 dBm/1.23 MHz (8 dB NF) |  | To be provided |  |
| 22. | Reference sensitivity | Band Class 0, 2, 3, 5, 7, 9, 10, 11, 12 | -122 dBm/1.23 MHz 9600 bps | -104 dBm/1.23 MHz (9600 bps) Traffic/cell -15.6 dB | -122 dBm/1.23 MHz 9600 bps | -105.5 dBm/1.23 MHz (307 kbps in 2 slots) | To be provided | To be provided |
|  |  | Band Class 1,4,6,8 | -122 dBm/1.23 MHz 9600 bps |  | -122 dBm/1.23 MHz 9600 bps |  | To be provided |  |
| 23. | Receiver blocking response applies to Band Class 6 only |  | 100 dB for 3dB link loss. Band Class 6 only | 45 dB at ± 5 MHz for degradation of PER spec from 0.5% to 10% and 3 dB link loss 57 dB at ± 7.5 MHz for degradation of PER spec from 0.5% to 10% and 3 dB link loss | 100 dB for 3 dB link loss. Band Class 6 only | 46 dB at ± 5 MHz for degradation of PER spec from 0.5% to 10% 58 dB at ± 7.5 MHz for degradation of PER spec from 0.5% to 10% | To be provided | To be provided |

## Deployment parameters

The deployment parameters, part of Table 3, are typically not included in the cdma2000 specifications.

–	Transmitting antenna type (directional/omnidirectional/adaptive antenna)

–	Antenna gain

–	Antenna height

–	Antenna pattern (vertical and horizontal)

–	Antenna downtilt

–	Feeder loss

–	Polarization

–	Cell coverage radius

–	Minimum coupling loss (MCL).

Some of the relevant information can be found in the Report ITU-R M.2039.  However, 3GPP2 plans to update some of the outdated information in the current version of Recommendation ITU-R M.2039 in the future.

TABLE 4

Maximum power for cdma2000 mobile stations and HRPD access terminals

| Band Class | Mobile Station Class / Access Terminal Class | Radiating measurement | Lower limit | Upper limit |
| --- | --- | --- | --- | --- |
| 0, 3, 9, and 12 | Class I | e.r.p. | 1 dBW (1.25 W) | 8 dBW (6.3 W) |
|  | Class II | e.r.p. | -3 dBW (0.5 W) | 4 dBW (2.5 W) |
|  | Class III | e.r.p. | -7 dBW (0.2 W) | 0 dBW (1.0 W) |
| 1, 4, 8, 14, and 15 | Class I | e.i.r.p. | -2 dBW (0.63 W) | 3 dBW (2.0 W) |
|  | Class II | e.i.r.p. | -7 dBW (0.2 W) | 0 dBW (1.0 W) |
|  | Class III | e.i.r.p. | -12 dBW (63 mW) | -3 dBW (0.5 W) |
|  | Class IV | e.i.r.p. | -17 dBW (20 mW) | -6 dBW (0.25 W) |
|  | Class V | e.i.r.p. | -22 dBW (6.3 mW) | -9 dBW (0.13 W) |
| 6 (outside Japan) | Class I | e.i.r.p. | -2 dBW (0.63 W) | 3 dBW (2.0 W) |
|  | Class II | e.i.r.p. | -7 dBW (0.2 W) | 0 dBW (1.0 W) |
|  | Class III | e.i.r.p. | -12 dBW (63 mW) | -3 dBW (0.5 W) |
|  | Class IV | e.i.r.p. | -17 dBW (20 mW) | -6 dBW (0.25 W) |
|  | Class V | e.i.r.p. | -22 dBW (6.3 mW) | -9 dBW (0.13 W) |
| 6 (in Japan) | Class I and II | e.i.r.p. | -10 dBW (0.1 W) | -6 dBW (0.25 W) |
|  | Class III | e.i.r.p. | -12 dBW (63 mW) | -6 dBW (0.25 W) |
|  | Class IV | e.i.r.p. | -17 dBW (20 mW) | -6 dBW (0.25 W) |
|  | Class V | e.i.r.p. | -22 dBW (6.3 mW) | -9 dBW (0.13 W) |
| 2 | Class II | e.r.p. | 1 dBW (1.25 W) | 8 dBW (6.3 W) |
|  | Class III | e.r.p. | -3 dBW (0.5 W) | 4 dBW (2.5 W) |
|  | Class IV | e.r.p. | -7 dBW (0.2 W) | 0 dBW (1.0 W) |
| 5 and 11 | Class I | e.r.p. | 3 dBW (2.0 W) | 10 dBW (10 W) |
|  | Class II | e.r.p. | -2 dBW (0.63 W) | 5 dBW (3.2 W) |
|  | Class III | e.r.p. | -7 dBW (0.2 W) | 0 dBW (1.0 W) |
|  | Class IV | e.r.p. | -12 dBW (63 mW) | -5 dBW (320 mW) |
| 7 and 10 | Class I | e.r.p. | -3 dBW (0.5 W) | 4 dBW (2.5 W) |
|  | Class II | e.r.p. | -7 dBW (0.2 W) | 0 dBW (1.0 W) |

Table 5 maps the band classes in Table 4 to the actual frequencies.  Note that the frequency range of 790-862 MHz is covered by Band Classes 0, 3, 10 and 18.

Table 5

Band Class designations

| Band Class | Usage | Transmit Frequency Band (MHz) |  |
| --- | --- | --- | --- |
|  |  | Mobile Station | Base Station |
| 0 | 800 MHz cellular band | 815-849 | 860-894 |
| 1 | 1.8 to 2.0 GHz PCS band | 1850-1910 | 1930-1990 |
| 2 | 872 to 960 MHz TACS band | 872-888 890-915 | 917-933 935-960 |
| 3 | 832 to 925 MHz JTACS band | 889-889 893-901 915-925 | 832-834 838-846 860-870 |
| 4 | 1.75 to 1.87 GHz Korean PCS band | 1750-1780 | 1840-1870 |
| 5 | 450 MHz NMT band | 410-420, 450-460, and 479-483.5 | 420-430, 460-470, and 489-493.5 |
| 6 | 2 GHz IMT-2000 band | 1920-1980 | 2110-2170 |
| 7 | 700 MHz band | 776-788 | 746-758 |
| 8 | 1 800 MHz band | 1710-1785 | 1805-1880 |
| 9 | 900 MHz band | 880-915 | 925-960 |
| 10 | Secondary 800 MHz band | 806-824 896-901 | 851-869 935-940 |
| 11 | 400 MHz European PAMR band | 410-420, 450-460, and 479-483.5 | 420-430, 460-470, and 489-493.5 |
| 12 | 800 MHz PAMR band | 870-876 | 915-921 |
| 13 | 2.5 GHz IMT-2000 extension band | 2500-2570 | 2620-2690 |
| 14 | US PCS 1.9 GHz band | 1850-1915 | 1930-1995 |
| 15 | AWS band | 1710-1755 | 2110-2155 |
| 16 | US 2.5 GHz band | 2502-2568 | 2624-2690 |
| 17 | US 2.5 GHz forward link only band | Not specified | 2624-2690 |
| 18 | 700 MHz public safety band | 787-799 | 757-769 |
| 19 | Lower 700 MHz band | 698-716 | 728-746 |
| NOTE – The frequency ranges listed are the full range of each Band Class. Many of these have band subclasses that may limit the actual frequency range for a particular deployment. See 3GPP2 specification C.S0057-C for details. |  |  |  |

________________

Note that the FDD profile of mobile WiMAX is currently not part the IMT-2000 interfaces. However, there are some on-going requests to include it.

See document: 3GPP 25 104 v 8.4.0

See document: 3GPP 25 101 v 8.4.0

See document: 3GPP 36 104 v 8.3.0, see tables of section 6.6.3

See document: 3GPP 36 101 v 8.3.0, see Table 6.6.2.1.1-1 (General E-UTRA spectrum emission mask)

US Code of Federal Regulations, Title 47, FCC Rules Parts 27 and 90

Based on a bandwidth of 3,84 MHz

This value corresponds to a 4,5 MHz bandwidth, noting that other measurement bandwidth are possible: 1.08 MHz, 2.7 MHz,  9.0 MHz, 13.5 MHz and 18 MHz.

These are average spectral power density values over the channel bandwidth.

This value assumes the mobile station is transmitting on one subchannel out of a possible 30 (15) in a 10 (5) MHz channel bandwidth.  For subchannelisation the active (data and pilot) sub-carriers are grouped into subsets of sub-carriers

This value includes the effect of multiple transmit antennas

A front to back ratio of 25 dB should be assumed.

This figure refers to the block size

See document 3GPP 36104-830, Table 6.3.2.1-1 (E-UTRA BS total power dynamic range)

Note that the FDD profile of mobile WiMAX is currently not part the IMT-2000 interfaces. However, there are some on-going requests to include it.

Note that these recommendations are currently being reviewed by WP 5D

This figure refers to the block size