ANNEX 
Harmonised frequency bands and technical parameters for short-range devices

| Type of short-range device | Frequency band | Power limit / field strength limit / power density limit | Additional parameters / spectrum access and mitigation requirements | Other usage restrictions | Implementation deadline |
| --- | --- | --- | --- | --- | --- |
| Non-specific short-range devices | 6765 - 6795 kHz | 42 dBµA/m at 10 metres |  |  | 1 October 2008 |
|  | 13,553 - 13,567 MHz | 42 dBµA/m at 10 metres |  |  | 1 October 2008 |
|  | 26,957 - 27,283 MHz | 10 mW effective radiated power (e.r.p.), which corresponds to 42 dBµA/m at 10 metres |  | Video applications are excluded | 1 June 2007 |
|  | 40,660 - 40,700 MHz | 10 mW e.r.p. |  | Video applications are excluded | 1 June 2007 |

| Non-specific short-range devices (cont.) | 433,050 - 434,040 MHz | 1 mW e.r.p. and -13dBm/10 kHz power density for bandwidth modulation larger than 250 kHz |  | Audio and voice signals, and video applications, are excluded | 1 October 2008 |
| --- | --- | --- | --- | --- | --- |
|  |  | 10 mW e.r.p. | Duty cycle: 10% | Audio and voice signals, and video applications, are excluded | 1 June 2007 |
|  | 434,040 - 434,79011 MHz | 1 mW e.r.p. and -13dBm/10 kHz power density for bandwidth modulation larger than 250 kHz |  | Audio and voice signals, and video applications, are excluded | 1 October 2008 |
|  |  | 10 mW e.r.p. | Duty cycle12: 10% | Audio and voice signals, and video applications, are excluded | 1 June 2007 |
|  |  |  | Duty cycle12: 100% subject to channel spacing up to 25 kHz | Audio and voice signals, and video applications, are excluded | 1 October 2008 |

| Non-specific short-range devices (cont.) | 863,000 - 868,000 MHz | 25 mW e.r.p. | Techniques to access spectrum and mitigate interference that provide at least equivalent performance to the techniques described in harmonised standards adopted under Directive 1999/5/EC must be used. Alternatively a duty cycle12 of 0,1% may also be used | Audio and voice signals, and video applications, are excluded | 1 October 2008 |
| --- | --- | --- | --- | --- | --- |
|  | 868,000 - 868,60011 MHz | 25 mW e.r.p. | Techniques to access spectrum and mitigate interference that provide at least equivalent performance to the techniques described in harmonised standards adopted under Directive 1999/5/EC must be used. Alternatively a duty cycle12 of 1% may also be used | Video applications are excluded | 1 October 2008 |
|  |  | 25 mW e.r.p. | Techniques to access spectrum and mitigate interference that provide at least equivalent performance to the techniques described in harmonised standards adopted under Directive 1999/5/EC must be used. Alternatively a duty cycle12 of 0,1% may also be used | Audio and voice signals, and video applications, are excluded | 1 October 2008 |

| Non-specific short-range devices (cont.) | 868,700 - 869,20011 MHz | 25 mW e.r.p. | Techniques to access spectrum and mitigate interference that provide at least equivalent performance to the techniques described in harmonised standards adopted under Directive 1999/5/EC must be used. Alternatively a duty cycle12 of 0,1% may also be used | Video applications are excluded | 1 October 2008 |
| --- | --- | --- | --- | --- | --- |
|  |  | 25 mW e.r.p. | Techniques to access spectrum and mitigate interference that provide at least equivalent performance to the techniques described in harmonised standards adopted under Directive 1999/5/EC must be used. Alternatively a duty cycle12 of 0,1% may also be used | Audio and voice signals, and video applications, are excluded | 1 October 2008 |

| Non-specific short-range devices (cont.) | 869,400 - 869,65011 MHz | 500 mW e.r.p. | Techniques to access spectrum and mitigate interference that provide at least equivalent performance to the techniques described in harmonised standards adopted under Directive 1999/5/EC must be used. Alternatively a duty cycle12 of 10 % may also be used Channel spacing must be 25 kHz, except that the whole band may also be used as a single channel for high-speed data transmission | Video applications are excluded | 1 October 2008 |
| --- | --- | --- | --- | --- | --- |
|  |  | 25 mW e.r.p. | Techniques to access spectrum and mitigate interference that provide at least equivalent performance to the techniques described in harmonised standards adopted under Directive 1999/5/EC must be used. Alternatively a duty cycle12 of 0,1% may also be used | Audio and voice signals, and video applications, are excluded | 1 October 2008 |
|  | 869,700 - 870,00011 MHz | 5 mW e.r.p. | Voice applications allowed with advanced mitigation techniques | Audio and video applications are excluded | 1 June 2007 |
|  |  | 25 mW e.r.p. | Techniques to access spectrum and mitigate interference that provide at least equivalent performance to the techniques described in harmonised standards adopted under Directive 1999/5/EC must be used. Alternatively a duty cycle12 of 0,1% may also be used | Audio and voice signals, and video applications, are excluded | 1 October 2008 |

| Non-specific short-range devices (cont.) | 2400 - 2483,5 MHz | 10 mW equivalent isotropic radiated power (e.i.r.p.) |  |  | 1 June 2007 |
| --- | --- | --- | --- | --- | --- |
|  | 5725 - 5875 MHz | 25 mW e.i.r.p. |  |  | 1 June 2007 |
|  | 24,150 - 24,250 GHz | 100 mW e.i.r.p. |  |  | 1 October 2008 |
|  | 61,0 - 61,5 GHz | 100 mW e.i.r.p. |  |  | 1 October 2008 |
| Wideband data transmission systems | 2400 - 2483,5 MHz | 100 mW e.i.r.p. and 100 mW/100 kHz e.i.r.p. density applies when frequency hopping modulation is used, 10 mW/MHz e.i.r.p. density applies when other types of modulation are used | Techniques to access spectrum and mitigate interference that provide at least equivalent performance to the techniques described in harmonised standards adopted under Directive 1999/5/EC must be used. |  | 1 November 2009 |
|  | 57,0 – 66,011 GHz | 40 dBm e.i.r.p. and 13 dBm/MHz e.i.r.p. density |  | Outdoor applications are excluded | 1 November 2009 |
|  |  | 25 dBm e.i.r.p. and -2 dBm/MHz e.i.r.p. density |  | Fixed outdoor installations are excluded | 1 November 2009 |

| Alarm systems | 868,600 - 868,700 MHz | 10 mW e.r.p. | Channel spacing: 25 kHz The whole frequency band may also be used as a single channel for high-speed data transmission Duty cycle12: 1.0% |  | 1 October 2008 |
| --- | --- | --- | --- | --- | --- |
|  | 869,250 - 869,300 MHz | 10 mW e.r.p. | Channel spacing: 25 kHz Duty cycle12: 0.1% |  | 1 June 2007 |
|  | 869,300 – 869,400 MHz | 10 mW e.r.p. | Channel spacing: 25 kHz Duty cycle12: 1.0% |  | 1 October 2008 |
|  | 869,650 - 869,700 MHz | 25 mW e.r.p. | Channel spacing: 25 kHz Duty cycle12 : 10% |  | 1 June 2007 |
| Social alarms | 869,200 – 869,250 MHz | 10 mW e.r.p. | Channel spacing: 25 kHz Duty cycle12: 0.1% |  | 1 June 2007 |

| Inductive applications | 20,050 - 59,750 kHz | 72 dBµA/m at 10 metres |  |  | 1 June 2007 |
| --- | --- | --- | --- | --- | --- |
|  | 59,750 - 60,250 kHz | 42 dBµA/m at 10 metres |  |  | 1 June 2007 |
|  | 60,250 - 70,000 kHz | 69 dBµA/m at 10 metres |  |  | 1 June 2007 |
|  | 70 - 119 kHz | 42 dBµA/m at 10 metres |  |  | 1 June 2007 |
|  | 119 - 127 kHz | 66 dBµA/m at 10 metres |  |  | 1 June 2007 |
|  | 127 - 140 kHz | 42 dBµA/m at 10 metres |  |  | 1 October 2008 |
|  | 140 - 148,5 kHz | 37,7 dBµA/m at 10 metres |  |  | 1 October 2008 |
|  | 148,5 - 5000 kHz In the specific bands mentioned below, higher field strengths and additional usage restrictions apply: | -15 dBµA/m at 10 metres in any bandwidth of 10 kHz Furthermore the total field strength is -5 dBμA/m at 10 m for systems operating at bandwidths larger than 10 kHz |  |  | 1 October 2008 |

| Inductive applications (cont.) | 400 - 600 kHz | -8 dBµA/m at 10 metres |  | This set of usage conditions applies to RFID only | 1 October 2008 |
| --- | --- | --- | --- | --- | --- |
|  | 3155 - 3400 kHz | 13,5 dBµA/m at 10 metres |  |  | 1 October 2008 |
|  | 5000 - 30000 kHz In the specific bands mentioned below, higher field strengths and additional usage restrictions apply: | -20 dBµA/m at 10 metres in any bandwidth of 10 kHz Furthermore the total field strength is -5 dBμA/m at 10 m for systems operating at bandwidths larger than 10 kHz |  |  | 1 October 2008 |
|  | 6765 - 6795 kHz | 42 dBµA/m at 10 metres |  |  | 1 June 2007 |
|  | 7400 - 8800 kHz | 9 dBµA/m at 10 metres |  |  | 1 October 2008 |
|  | 10200 - 11000 kHz | 9 dBµA/m at 10 metres |  |  | 1 October 2008 |

| Inductive applications (cont.) | 13553 - 13567 kHz | 42 dBµA/m at 10 metres |  |  | 1 June 2007 |
| --- | --- | --- | --- | --- | --- |
|  |  | 60 dBµA/m at 10 metres |  | This set of usage conditions applies to RFID15 and EAS only | 1 October 2008 |
|  | 26957 - 27283 kHz | 42 dBµA/m at 10 metres |  |  | 1 October 2008 |
| Active medical implants | 9 - 315 kHz | 30 dBµA/m at 10m | Duty cycle12: 10% |  | 1 October 2008 |
|  | 402 - 405 MHz | 25 µW e.r.p. | Channel spacing: 25 kHz Individual transmitters may combine adjacent channels for increased bandwidth up to 300 kHz. Other techniques to access spectrum or mitigate interference, including bandwidths greater than 300 kHz, can be used provided they result at least in an equivalent performance to the techniques described in harmonised standards adopted under Directive 1999/5/EC to ensure compatible operation with the other users and in particular with meteorological radiosondes. |  | 1 November 2009 |

| Wireless audio applications | 87,5 - 108,0 MHz | 50 nW e.r.p. | Channel spacing up to 200 kHz |  | 1 October 2008 |
| --- | --- | --- | --- | --- | --- |
|  | 863 - 865 MHz | 10 mW e.r.p. |  |  | 1 June 2007 |
| Radio determination applications | 2400 - 2483,5 MHz | 25 mW e.i.r.p. |  |  | 1 November 2009 |
|  | 17,1 – 17,3 GHz | 26 dBm e.i.r.p. | Techniques to access spectrum and mitigate interference that provide at least equivalent performance to the techniques described in harmonised standards adopted under Directive 1999/5/EC must be used. | This set of usage conditions applies to ground based systems only | 1 November 2009 |
| Tank Level Probing Radar | 4,5 - 7,0 GHz | 24 dBm e.i.r.p. |  |  | 1 November 2009 |
|  | 8,5 - 10,6 GHz | 30 dBm e.i.r.p.21 |  |  | 1 November 2009 |
|  | 24,05 - 27,0 GHz | 43 dBm e.i.r.p.21 |  |  | 1 November 2009 |
|  | 57,0 - 64,0 GHz | 43 dBm e.i.r.p.21 |  |  | 1 November 2009 |
|  | 75,0 – 85,0 GHz | 43 dBm e.i.r.p.21 |  |  | 1 November 2009 |

| Model Control | 26990 - 27000 kHz | 100 mW e.r.p. |  |  | 1 November 2009 |
| --- | --- | --- | --- | --- | --- |
|  | 27040 - 27050 kHz | 100 mW e.r.p. |  |  | 1 November 2009 |
|  | 27090 - 27100 kHz | 100 mW e.r.p. |  |  | 1 November 2009 |
|  | 27140 - 27150 kHz | 100 mW e.r.p. |  |  | 1 November 2009 |
|  | 27190 - 27200 kHz | 100 mW e.r.p. |  |  | 1 November 2009 |
| Radio Frequency Identification (RFID) | 2446 - 2454 MHz | 100 mW e.i.r.p. |  |  | 1 November 2009 |

Member States must allow the usage of spectrum up to the power, field strength or power density given in this table. In conformity with Article 3(3) of Decision 2006/771/EC, they may impose less restrictive conditions, i.e. allow the use of spectrum with higher power, field strength or power density.

Member States may only impose these ‘additional parameters / spectrum access and mitigation requirements’, and may not add other parameters or spectrum access and mitigation requirements. Less restrictive conditions within the meaning of Article 3(3) of Decision 2006/771/EC mean that Member States may completely omit the parameters / spectrum access and mitigation requirements in a given cell or allow higher values.

Member States may only impose these ‘other usage restrictions’, and may not add additional usage restrictions. As less restrictive conditions may be introduced within the meaning of Article 3(3) of Decision 2006/771/EC, Member States may omit one or all of these restrictions.

This category is available for any type of application which fulfils the technical conditions (typical uses are telemetry, telecommand, alarms, data in general and other similar applications).

For this frequency band Member States must make all the alternative sets of usage conditions possible.

‘Duty cycle’ means the ratio of time during any one-hour period when equipment is actively transmitting. Less restrictive conditions within the meaning of Article 3(3) of Decision 2006/771/EC mean that Member States may allow a higher value for ‘Duty cycle’.

Social alarm devices are used to assist elderly or disabled people when they are in distress.

This category covers, for example, devices for car immobilisation, animal identification, alarm systems, cable detection, waste management, personal identification, wireless voice links, access control, proximity sensors, anti-theft systems, including RF anti-theft induction systems, data transfer to handheld devices, automatic article identification, wireless control systems and automatic road tolling.

This category covers inductive applications used for Radio Frequency Identification (RFID).

This category covers inductive applications used for Electronic Article Surveillance (EAS).

This category covers the radio part of active implantable medical devices, as defined in Council Directive 90/385/EEC of 20 June 1990 on the approximation of the laws of the Member States relating to active implantable medical devices and their peripherals (OJ L 189, 20.7.1990, p. 17).

Applications for wireless audio systems, including: cordless loudspeakers; cordless headphones; cordless headphones for portable use, e.g. portable CD, cassette or radio devices carried on a person; cordless headphones for use in a vehicle, for example for use with a radio or mobile telephone, etc.; in-ear monitoring, for use at concerts or other stage productions.

This category covers applications used for determining the position, velocity and/or other characteristics of an object, or for obtaining information relating to these parameters.

Tank Level Probing Radars (TLPR) are a specific type of radiodetermination application, which are used for tank level measurements and are installed in metallic or reinforced concrete tanks, or similar structures made of material with comparable attenuation characteristics. The purpose of the tank is to contain a substance.

The power limit applies inside a closed tank and corresponds with a spectral density of -41,3 dBm/MHz e.i.r.p. outside a 500 litre test tank.

This category covers applications used to control the movement of models (principally miniature representations of vehicles) in the air, on land or over or under the water surface.