| Radiocommunication Study Groups |  |
| --- | --- |
|  | IEEE L802.16-11/0012d1 |
|  | ***DRAFT*** |
| Received: xxxxx 2011 | Document 5D/xxxx |
|  | April 2011 |
|  | English only VOCABULARY |
| IEEE |  |
| VOCABULARY TERMS and abbreviations RELATED TO IEEE 802.16 |  |

# 1	Background

IEEE has received the liaison statement (Revision 1 to Document 5D/TEMP/423) of 19 October 2010 from ITU-R Working Party 5D (WP 5D) regarding the Revision of Recommendation ITU-R M.1224 with an expanded scope to include IMT-Advanced.

Prior to WP 5D Meeting #9, IEEE had submitted 5D/814 (“Vocabulary terms related to IEEE 802.16”) supplying terms and definitions included in IEEE Std 802.16-2009, in its amendment IEEE Std 802.16j, and in IEEE Draft P802.16m/D8.

We recognize that the liaison statement indicates a request for abbreviations as well as definitions. Accordingly, this contribution provides the additional information. Also, it updates the prior contribution by including definitions from IEEE Draft P802.16m/D12 to replace the information submitted earlier from IEEE Draft P802.16m/D8. Note that P802.16m/D12 is a draft amendment of IEEE Std 802.16 specifying the WirelessMAN-Advanced radio interface technology that was approved for IMT-Advanced.

# 2	Content

Annex 1 provides a list of abbreviations included in IEEE Std 802.16-2009, in its amendment IEEE Std 802.16j, and in IEEE draft P802.16m/D12.

Annex 2 provides a list of terms and definitions included in IEEE Draft P802.16m/D12.

# 3	Proposal

We propose that the abbreviations in Annex 1 be considered for inclusion in the draft revision of Recommendation ITU-R M.1224.

We propose that Annex 2 be considered as an update of Annex 3 in 5D/814. We propose that the terms and definitions in Annexes 1 and 2 of 5D/814, and also Annex 2 of the current contribution, be considered for inclusion in the draft revision of Recommendation ITU-R M.1224.

Annex 1: Abbreviations from IEEE Std 802.16-2009, IEEE Std 802.16j, and IEEE draft P802.16m/D12

IEEE Std 802.16-2009:

3-DES	triple data encryption standard

AAS	adaptive antenna system

ACM	account management

AES	advanced encryption standard

AGC	automatic gain control

AK	authorization key

AKID	authorization key identifier

AMC	adaptive modulation and coding

ARQ	automatic repeat request

ASA	authentication and service authorization

ASR	anchor switch reporting

ATDD	adaptive time division duplexing

ATM	asynchronous transfer mode

BCC	block convolutional code

BE	best effort

BER	bit error ratio

BPSK	binary phase shift keying

BR	bandwidth request

BS	base station

BSN	block sequence number

BTC	block turbo code

BW	bandwidth

BWA	broadband wireless access

BWAA	bandwidth allocation/access

C/I	carrier-to-interference ratio

C/N	carrier-to-noise ratio

CA	certification authority

CBC	cipher block chaining

CBC-MAC	cipher block chaining message authentication code

CC	confirmation code

CCH	control subchannel

CCI	co-channel interference

CCM	CTR mode with CBC-MAC

CCS	common channel signaling

CCV	clock comparison value

CDMA	code division multiple access

ChID	channel identifier

CID	connection identifier

CINR	carrier-to-interference-and-noise ratio

CIR	channel impulse response

CLP	cell loss priority

CMAC	cipher-based message authentication code

CP	cyclic prefix

CPS	common part sublayer

CQI	channel quality information

CQICH	channel quality information channel

CRC	cyclic redundancy check

CS	convergence sublayer

CSCF	centralized scheduling configuration

CSCH	centralized scheduling

CSIT	channel state information at the transmitter

CTC	convolutional turbo code

CTR	counter mode encryption

DAMA	demand assigned multiple access

DARS	digital audio radio satellite

dBi	decibels of gain relative to the 0 dB gain of a free-space isotropic radiator

dBm	decibels relative to 1 mW

DCD	downlink channel descriptor

DES	data encryption standard

DFS	dynamic frequency selection

DHCP	Dynamic Host Configuration Protocol

DIUC	downlink interval usage code

DL	downlink

DLFP	downlink frame prefix

DSA	dynamic service addition

DSC	dynamic service change

DSCH	distributed scheduling

DSCP	differentiated services codepoint

DSD	dynamic service deletion

DSx	dynamic service addition, change, or deletion

D-TDOA	Downlink Time Difference Of Arrival

EAP	extensible authentication protocol

EC	encryption control

ECB	electronic code book

ECRTP	a IP-header-compression CS PDU format (IETF RFC 3545)

EDE	encrypt-decrypt-encrypt

EESS	earth exploratory satellite system

EIK	EAP Integrity Key

EIRP	effective isotropic radiated power

EKS	encryption key sequence

EVM	error vector magnitude

FBSS	fast base station switching

FC	fragmentation control

FCAPS	Fault Management, Configuration Management, Account Management, Performance Management, Security Management

FCH	frame control header

FDD	frequency division duplex or duplexing

FEC	forward error correction

FFSH	fast-feedback allocation subheader

FFT	fast Fourier transform

FHDC	frequency hopping diversity coding

FPC	fast power control

FSH	fragmentation subheader

FSN	fragment sequence number

FSS	fixed satellite service

FUSC	full usage of subchannels

GPCS	Generic Packet Convergence Sublayer

GF	galois field

GKEK	group key encryption key

GMSH	grant management subheader

GPS	global positioning system

GS	guard symbol

GTEK	group traffic encryption key

HCS	header check sequence

HEC	header error check

H-FDD	half-duplex frequency division duplex

HMAC	hashed message authentication code

HO	handover

HT	header type

HUMAN	high-speed unlicensed metropolitan area network

I	inphase

IANA	internet assigned numbers authority

IE	information element

IFFT	inverse fast Fourier transform

IMM	idle mode management

IP	Internet Protocol

IV	initialization vector

IWF	interworking function

KEK	key encryption key

LAN	local area network

LBS	location based services

LDPC	low-density parity check

LFSR	linear feedback shift register

LLC	logical link control

LOS	line-of-sight

LSB	least significant bit

MAC	medium access control layer

MAK	MBS authorization key

MAN	metropolitan area network

MBS	multicast and broadcast service

MCID	multicast CID

MCS	modulation coding scheme

MDHO	macro diversity handover

MDS	multipoint distribution service

MGTEK	MBS group traffic encryption key

MIB	management information base

MIC	message integrity check

MIH	media independent handover

MIHF	MIH Function

MIMO	multiple input multiple output

MMDS	multichannel multipoint distribution service

MPEG	moving pictures experts group

MS	mobile station

MSB	most significant bit

MSK	master session key

NAI	network access identifier

NAS	network access server

NCFG	network configuration

NCMS	network control and management system

NCMS(BS)	network control and management system at the BS side (network side)

NCMS(SS/MS)	network control and management system at the SS/MS side

NEM	network entry management

NENT	network entry

NLOS	non-line-of-sight

NNI	network-to-network interface (or network node interface)

NRM	network reference model

nrtPS	non-real-time polling service

NSP	network service provider

OFDM	orthogonal frequency division multiplexing

OFDMA	orthogonal frequency division multiple access

OID	object identifier

PAK	primary authorization key

PAPR	peak to average power ratio

PBR	piggyback request

PDU	protocol data unit

PER	packet error ratio

PHS	payload header suppression

PHSF	payload header suppression field

PHSI	payload header suppression index

PHSM	payload header suppression mask

PHSS	payload header suppression size

PHSV	payload header suppression valid

PHY	physical layer

PKM	privacy key management

PM	poll-me bit

PMD	physical medium dependent

PMK	pairwise master key

PMP	point-to-multipoint

PN	packet number

PPP	Point-to-Point Protocol

PRBS	pseudo-random binary sequence

PS	physical slot

PSC	power saving class

PSH	packing subheader

PTI	payload type indicator

PtP	point to point

PUSC	partial usage of subchannels

PUSC-ASCA	partial usage of subchannels – adjacent subcarrier allocation

PVC	permanent virtual circuit

Q	quadrature

QAM	quadrature amplitude modulation

QoS	quality of service

QPSK	quadrature phase-shift keying

REQ	request

RLAN	radio local access network

RNG	ranging

ROHC	an IP-header-compression CS PDU format (IETF RFC 3095)

RRA	radio resource agent

RRC	radio resource controller

RRM	radio resource management

RS	Reed–Solomon

RSP	response

RSS	receive signal strength

RSSI	receive signal strength indicator

RTG	receive/transmit transition gap

rtPS	real-time polling service

Rx	receive

RxDS	receiver delay spread clearing interval

SA	security association

SAID	security association identifier

SAP	service access point

SAR	synthetic aperture radar

SC	single carrier

SDMA	spatial division multiple access

SDU	service data unit

SF	service flow

SFM	service flow management

SFID	service flow identifier

SHA	secure hash algorithm

SI	slip indicator

SIQ	service information query

SM	spatial multiplexing

SN	sequence number

SNMP	Simple Network Management Protocol

SNR	signal-to-noise ratio

SS	subscriber station

SSID	subscriber station identification (MAC address)

SSM	subscriber station management

SSTG	subscriber station transition gap

STC	space time coding

STTD	space time transmit diversity

SVC	switched virtual circuit

TCS	transmission convergence sublayer

TCM	trellis coded modulation

TCP	Transmission Control Protocol

TDD	time division duplex or duplexing

TDM	time division multiplexing

TDMA	time division multiple access

TDOA	time difference of arrival

TEK	traffic encryption key

TFTP	Trivial File Transfer Protocol

TLV	type/length/value

TTG	transmit/receive transition gap

TUSC	tile usage of subchannels

Tx	transmit (abbreviation not used as verb)

UCD	uplink channel descriptor

UDP	User Datagram Protocol

UEP	unequal error protection

UGS	unsolicited grant service

UIUC	uplink interval usage code

UL	uplink

UNI	user-to-network interface (or user-network interface)

U-NII	unlicensed national information infrastructure

UTC	universal coordinated time

U-TDOA	uplink time difference of arrival

UW	unique word

VC	virtual channel

VCI	virtual channel identifier

VLAN	virtual local area network

VP	virtual path

VPI	virtual path identifier

WirelessMAN	Wireless Metropolitan Area Networks

WirelessHUMAN	Wireless High-speed Unlicensed Metropolitan Area Networks

WLAN	wireless local area network

XOR	exclusive-or

IEEE Std 802.16j:

AC	authentication control

FRS	fixed relay station

HR	handover ranging

IR	initial ranging

IS	infrastructure station

MRS	mobile relay station

MR-BS	multihop relay base station

RS	relay station

RTD	round trip delay

R-ACK	relay ACK

R-DL	relay downlink

R-FCH	relay zone frame control header

R-MAP	relay zone MAP

R-RTI	relay receive/transmit transition interval

R-TTI	relay transmit/receive transition interval

R-UL	relay uplink

R-Zone	relay zone

TDU	tunnel data unit

P802.16m/D12:

AAI	advanced air interface

ABS	advanced base station

aGPS	adaptive grant polling service

A-MAP	advanced MAP

AMS	advanced mobile station

AOA	angle of arrival

A-Preamble	advanced preamble

ARS	advanced relay station

ASN	access service network

ASN.1	abstract syntax notation

CDR	conjugate data repetition

CL	closed-loop

CLRU	contiguous LRU

CMI	codebook matrix index

CRID	context retention identifier

CRT	context retention timer

CRU	contiguous resource unit

CRV	CoRe version

CoRe	constellation re-arrangement

CSG	closed subscriber group

CSM	collaborative spatial multiplexing

DCAS	downlink CRU allocation size

DCR	deregistration with context retention

DL	downlink

DLRU	distributed LRU

DSAC	downlink subband allocation count

DRU	distributed resource unit

EBB	established before break

EH	extended header

FFR	fractional frequency reuse

FID	flow identifier

FMT UL	feedback mini-tile

FP	frequency partition

FPEH	fragmentation and packing extended header

FPC	frequency partition configuration

FPCT	frequency partition count

FPS	frequency partition size

FPSC	frequency partition subband count

GRA	group resource allocation

GMH	generic MAC header

HARQ	hybrid ARQ

HE	horizontal encoding

HMT	UL HARQ mini-tiles

ICV	integrity check value

IE	information element

IR	incremental redundancy

ISL	interference sensitivity level

LBS	location based services

LDM	low duty mode

LRU	logical resource unit

MCEH	MAC control extended header

MCS	modulation and coding scheme

MEF	MIMO encoder format

MEH	 multiplexing extended header

MLRU	minimum A-MAP logical resource unit

MU	multi-user

N/A	not applicable

NIP	normalized interference power

NLRU	miniband LRU

NS/EP	National Security/Emergency Preparedness

NS-RCH	non-synchronized ranging channel

OL	open-loop

OSG	open subscriber group

PA	persistent allocation

PA-Preamble	primary advanced preamble

PFBCH	UL primary fast feedback channel

PGID 	paging-group identifier

PMI	preferred matrix index

PPRU	permuted physical resource unit

PRU	physical resource unit

P-SFH	primary superframe header

PSI	pilot stream index

RA-ID	random access identifier

RCH 	ranging channel

RCP 	ranging cyclic prefix

RD	relative delay

RFMT	reordered UL feedback mini-tile

RHMT	reordered UL HARQ mini-tile

RP	ranging preamble

RTD	round trip delay

RU	resource unit

S-ABS	serving ABS

SAC	subband allocation count

SA-Preamble	secondary advanced preamble

Sc    	sub-carrier

SFBC	space-frequency block code

SFBCH	UL secondary fast feedback channel

SFH	superframe header

SLRU	subband LRU

SOHO	small office home office

SON	self organizing networks

SP	S-SFH sub-packet

SPID	subpacket ID

SPMH	short-packet MAC header

S-RCH 	synchronized ranging channel

S-SFH 	secondary superframe header

STC	space-time coding

STID 	station identifier

SU	single-user

T-ABS	target ABS

TOA	time of arrival

TSTID	temporary STID

UCAS	uplink CRU allocation size

UFPC	uplink frequency partition configuration

UL	uplink

USAC	uplink subband allocation count

VE	vertical encoding

Annex 2: Terms and Definitions from IEEE Draft P802.16m/D12

3.97	AAI subframe: A structured data sequence of predefined duration used by the Advanced Air Interface specification.

3.98	advanced base station (ABS): A base station that supports the Advanced Air Interface protocol defined in Clause 16.

3.99	advanced mobile station (AMS): A subscriber station capable of performing the 12.5 WirelessMAN-OFDMA TDD Release 1 subset of mobile station (MS) features and functions additionally implementing the Advanced Air Interface protocol defined in Clause 16.

3.100	advanced relay station (ARS): A relay station that supports the Advanced Air Interface protocol defined in Clause 16.

3.101	superframe: A structured data sequence of fixed duration used by the Advanced Air Interface specifications. A superframe is comprised of four frames.

3.102	primary carrier: An OFDMA carrier on which an ABS and an AMS/MS exchange traffic and full PHY/MAC control information defined in the Advanced Air Interface specification. Further, the primary carrier is used for control functions for proper AMS/MS operation, such as network entry. Each AMS shall have only one  carrier it considers to be its primary carrier in a cell.

3.103	secondary carrier: An OFDMA carrier that an AMS may use to exchange traffic with an ABS, based on allocation commands and rules received over the primary carrier of that ABS. The secondary carrier may also include control signaling to support multi-carrier operation.

3.104	fully configured carrier: A carrier for which all control channels including synchronization, broadcast, multicast and unicast control signaling are configured. Further, information and parameters regarding multi-carrier operation and the other carriers can also be included in the control channels.

3.105	partially configured carrier: A downlink only carrier configured with control channels to support downlink transmission.

3.106	physical resource unit (PRU): The basic resource allocation unit that consists of 18 adjacent subcarriers in consecutive symbols in the same AAI subframe.

3.107	distributed resource unit (DRU): The resource allocation unit of the same size as the PRU that has undergone the subband partitioning and miniband permutation, assigned to distributed allocation and will be submitted to the subcarrier permutation in DL and tile permutation in UL.

3.108	contiguous resource unit (CRU): The resource allocation unit of the same size as the PRU that has undergone the subband partitioning and miniband permutation, assigned to contiguous allocation and will bypass subcarrier permutation in DL and tile permutation in UL. Also known as a localized resource unit.

3.109	logical resource unit (LRU): the generic name of logical units for distributed and localized resource allocations.

3.110	transmission time interval (TTI): The duration of the transmission of the physical layer encoded packet over the radio air interface and is equal to an integer number of AAI subframes. The default TTI is 1 AAI subframe.

3.111	MIMO layer: An information path fed to the MIMO encoder as an input. A MIMO layer represents one channel coding block.

3.112	MIMO stream: Each information path encoded by the MIMO encoder that is passed to the precoder

3.113	horizontal encoding: Indicates transmitting multiple MIMO layers over multiple antennas. The number of MIMO layers is more than 1. The number of MIMO streams is same as the number of MIMO layers in this case.

3.114	vertical encoding: Indicates transmitting a single MIMO layer over multiple antennas. The number of MIMO layers is always 1.

3.115	multi-layer encoding: The number of MIMO streams is same as the number of MIMO layers in this case.

3.116	resource unit: A granular unit in frequency and time, described by the number of OFDMA subcarriers and OFDMA symbols

3.117	single user MIMO (SU-MIMO): A MIMO transmission scheme in which a single MS is scheduled in one RU

3.118	multi-user MIMO (MU-MIMO): A MIMO transmission scheme in which multiple MSs are scheduled in one RU, by virtue of spatial separation of the transmitted signals

3.119	Time-division transmit and receive (TTR) relaying: a relay mechanism where transmission to subordinate station(s) and reception from the superordinate station, or transmission to the superordinate station and reception from subordinate station(s) is separated in time.

3.120	AAI (DL/UL) Access Zone: An integer multiple of subframes located in the MZone of the ABS frame or ARS frame, where an ABS or ARS transmit to the AMSs or receive from AMSs.

3.121	AAI (DL/UL) Relay Zone: An integer multiple of subframes located in the MZone of the ABS frame, where an ABS transmit to the ARSs and/or AMSs or receive from ARSs and AMSs, or ARS frame, where an ARS transmit to the ABS or receive from ABS.

3.122	ARS transmit/receive transition gap (ARSTTG): The minimum transmit-to-receive turnaround gap required at an ARS. ARS-TTG is measured from the time of the last sample of the transmitted burst to the first sample of the received burst at the antenna port of the ARS.

3.123	ARS receive/transmit transition gap (ARSRTG): The minimum receive-to-transmit turnaround gap required at an ARS. ARS-RTG is measured from the time of the last sample of the received burst to the first sample of the transmitted burst at the antenna port of the ARS.

3.124	relative delay (RD): The delay of neighbor DL signals relative to the serving/attached BS.

3.125	round trip delay (RTD): The time required for a signal or packet to transfer from a MS to a BS and back again.

3.126	Macro hotzone ABS: An ABS with smaller transmission power/cell size than that of macro ABS's, typically deployed by service provider and operated on service provider backhaul.

3.127	frame index: The frame order within a Superframe (i.e. the 1st, 2nd, 3rd, or 4th frame of Superframe).

3.128	Femto ABS: an ABS with low transmit power, typically installed by a subscriber in the home, SOHO, or enterprise to provide the access to closed or open group of users as configured by the subscriber and/or the access provider. A Femto ABS is typically connected to the service providers network via a broadband connection.

3.129	OSG Femto ABS: a femto ABS accessible to any AMS.

3.130	closed subscriber group (CSG): a set of subscribers authorized by the Femto ABS owner or the network service provider, for accessing CSG femto ABS.

3.131	CSG Femto ABS: CSG-Closed or CSG-Open Femto ABS.

3.132	CSG-Closed Femto ABS: a femto ABS accessible only to the AMSs, which are in its CSG(s), except for emergency services. AMSs which are not the members of the CSG(s), should not try to access CSG-Closed Femto ABSs.

3.133	CSG-Open Femto ABS: a femto ABS primarily accessible to the AMSs that belong to its CSG(s), while other AMSs, outside CSG(s), may also access such Femto ABS, and will be served at lower priority. CSG-Open Femto ABS will provide service to such AMSs as long as the QoS of AMSs in its CSG(s) is not compromised.

3.134	WirelessMAN-OFDMA R1 Reference System: a network compliant with the WirelessMAN-OFDMA capabilities as specified in subclause 12.5  WirelessMAN-OFDMA TDD Release 1

3.135	WirelessMAN-OFDMA  Advanced  co-existing  System: an ABS and/or AMS that also implements LZone functionality compliant with 12.5 WirelessMAN-OFDMA TDD Release 1

3.136	R1 MS: A mobile station compliant with the WirelessMAN-OFDMA R1 Reference System

3.137	R1 BS: A base station compliant with the WirelessMAN-OFDMA R1 Reference System

3.138	LZone: A positive integer number of consecutive subframes during which an ABS communicates with RSs or R1 MSs, and where an ARS or an RS communicates with one or more R1 MSs.

3.139	MZone: A positive integer number of consecutive subframes during which an ABS communicates with one or more ARSs or AMSs, and where an ARS communicates with one or more ARSs or AMSs.

3.140	Mixed Mode ABS: An ABS with an operating Lzone and operating Mzone.

3.141	default service flow: A service flow which is established automatically without DSA procedure after successful registration procedure. QoS parameters for the default service flow are predefined.

3.142	Simultaneous transmit and receive (STR) relaying: a relay mechanism where transmission to subordinate station(s) and reception from the superordinate station, and transmission to the superordinate station and reception from subordinate station(s) are performed simultaneously.

3.143	Macro ABS: an ABS with high transmit power. A Macro ABS is directly connected to the service providers network.

3.144	Single Radio MS: A multimode MS/AMS that operates with only a single transmitting radio and with one or more receiving radios at any given time.

3.145	Dual Radio MS: A multimode MS/AMS that can have both radios (transmitting and receiving) active at the same time. A Dual Radio MS/AMS can simultaneous transmit and receive on both radios.  A Dual Radio MS/AMS may behave as a Single Radio MS by operating in Single Radio Mode

3.146	Multi Radio MS: A multimode MS/AMS that can have multiple radios (transmitting and receiving) active at the same time. A Multi Radio MS/AMS can simultaneous transmit and receive on multiple radios.  A Multi Radio MS/AMS may behave as a Single Radio MS by operating in Single Radio Mode

3.147	Frame Number: In WirelessMAN-OFDMA, the frame number is a 24-bit number transmitted in every frame. Frame numbers are not necesseraly synchronized across base stations. In WirelessMAN-Advanced air interface, the frame number is obtained by concatenating the 12-bit superframe number (transmitted in every superframe) and the 2-bit frame index. Superframe numbers are synchronized across base stations.