| Radiocommunication Study Groups |  |
| --- | --- |
|  | IEEE L802.16-11/0042 IEEE L802.16-18/0067r1 |
|  | ***DRAFT*** |
| Received: xxxxx 2011 | Document 5D/xxxx |
|  | July 2011 |
|  | English only VOCABULARY |
| Institute of Electrical and Electronics Engineers, Inc. |  |
| Vocabulary Terms and Abbreviations related to IEEE 802.16 in Preliminary Draft Revision of Recommendation ITU-R M.1224 |  |

# 1	Background

IEEE has considered the liaison statement (5D/TEMP/520, of July 2011) from ITU-R Working Party 5D (WP 5D) requesting review of the Preliminary Draft Revision of Recommendation ITU-R M.1224 (5D/TEMP/521). That draft incorporates some materials previously contributed by IEEE (5D/814 and 5D/930).

Accordingly, IEEE has reviewed this PDNR and provides its comments, particularly regarding the materials related to IEEE standards.

# 2	Comments

We note that the materials based on IEEE contributions are contained in 4.1.2.1 (“Terms related to WirelessMAN-Advanced”) and 4.2.2.1 (“Abbreviations Terms related to WirelessMAN-Advanced).

Regarding 4.1.2.1, we observe that definitions are organized into four sections:

- Terms from IEEE Std 802.16-2009

- Terms from IEEE Std 802.16j-2009

- Terms from IEEE P802.16m/D8

- Terms and Definitions from IEEE Draft P802.16m/D12

We have three high-level comments this structure:

(1) While we acknowledge that the material was submitted to ITU in separate sections, we believe that the WirelessMAN-Advanced materials would be more effectively presented to the reader as a single alphabetized section.

(2) The fourth section arose from 5D/930, which indicated that the material from IEEE Draft P802.16m/D12 was intended to replace material from the earlier draft IEEE Draft P802.16m/D8. Therefore, we suggest the deletion of the section regarding “Terms from IEEE P802.16m/D8.”

(3) The materials from IEEE Draft P802.16m/D12 have themselves become obsolete with the final approval of IEEE Std 802.16m. Therefore, we suggest that this material be updated to the final form as well.

Regarding 4.2.2.1, we observe that abbreviations are also organized into sections:

(1) IEEE Std 802.16-2009:

(2) IEEE Std 802.16j:

(3) P802.16m/D12:

As before, we believe that a single integrated and alphabetized WirelessMAN-Advanced section would be more effective for the reader.

# 3	Proposal

We propose that Annex 1 be considered as an update of section 4.1.2.1 of the PDNR.

We propose that Annex 2 be considered as an update of section 4.2.2.1 of the PDNR.

Annex 1: Update of 4.1.2.1

4.1.2.1 Terms related to WirelessMAN-Advanced

AAI (DL/UL) Access Zone: An integer multiple of subframes located in the MZone of the ABS frame or ARS frame, where an ABS or ARS transmit to the AMSs or receive from AMSs.

AAI (DL/UL) Relay Zone: An integer multiple of subframes located in the MZone of the ABS frame, where an ABS transmit to the ARSs and/or AMSs or receive from ARSs and AMSs, or ARS frame, where an ARS transmit to the ABS or receive from ABS.

AAI subframe: A structured data sequence of predefined duration used by the Advanced Air Interface specification.

Access link: A radio link between an MR-BS or RS and an MS, or between an MR-BS or RS and a subordinate RS during network entry. The access link is either an uplink or downlink.

Access RS: A relay station that serves as an access station.

Access station: A station that provides a point of access into the network for an MS or RS. An access station can be a base station (BS), relay station (RS), or multihop relay BS (MR-BS).

Active base station (BS): A BS that is informed of the mobile station (MS) capabilities, security parameters, service flows, and full medium access control layer (MAC) context information. For macro diversity handover (MDHO), the MS transmits/receives data to/from all active BSs in the diversity set.

Adaptive antenna system (AAS): An array of antennas and associated signal processing that together is able to change its antenna radiation pattern dynamically to adjust to noise environment, interference and multipath.

Adaptive modulation: A system’s ability to communicate with another system using multiple burst profiles and a system’s ability to subsequently communicate with multiple systems using different burst profiles.

Adjacent subcarrier allocation: A permutation where the subcarriers are located adjacent to each other.

advanced base station (ABS): A base station that supports the WirelessMAN-Advanced Air Interface.

advanced mobile station (AMS): A subscriber station capable of performing the Wireless-MAN OFDMA TDD Release 1 subset of mobile station (MS) features and functions and additionally implementing the WirelessMAN-Advanced Air Interface.

advanced relay station (ARS): A relay station that supports the WirelessMAN-Advanced Air Interface.

Anchor base station: For macro diversity handover (MDHO) or fast BS switching (FBSS) supporting mobile stations (MSs), a BS where the MS is registered, is synchronized, performs ranging, and monitors the downlink (DL) for control information. For FBSS supporting MSs, the anchor BS is the serving BS that is designated to transmit/receive data to/from the MS at a given frame.

ARS receive/transmit transition gap (ARSRTG): The minimum receive-to-transmit turnaround gap required at an ARS. ARS-RTG is measured from the time of the last sample of the received burst to the first sample of the transmitted burst at the antenna port of the ARS.

ARS transmit/receive transition gap (ARSTTG): The minimum transmit-to-receive turnaround gap required at an ARS. ARS-TTG is measured from the time of the last sample of the transmitted burst to the first sample of the received burst at the antenna port of the ARS.

Authenticator: Authenticator functionality is part of AAA Services, which is included in the NCMS. An authenticator is an entity at one end of a point-to-point link that facilitates authentication of a supplicant (MS) attached to the other end of that link. It can enforce authentication before allowing access to services that are accessible to the supplicant. The authenticator incorporates an AAA client functionality that enables it to communicate with AAA backend infrastructure (AAA based Authentication Server). The AAA server provides the Authenticator with authentication and authorization services over AAA protocols. The authenticator function contains a Key Distributor and may also include a Key Receiver function.

Automatic repeat request (ARQ) block: A distinct unit of data that is carried on an ARQ-enabled connection. Such a unit is assigned a sequence number and is managed as a distinct entity by the ARQ state machines. Block size is a parameter negotiated during connection establishment.

Backbone network: A communication mechanism by which two or more base stations (BSs) communicate to each other. It may also include communication with other networks. The method of communication for backbone networks is outside the scope of this standard.

Bandwidth stealing: The use, by a subscriber station (SS), of a portion of the bandwidth allocated in response to a bandwidth request (BR) for a connection to send a BR or data for any of its connections.

Base station (BS) receive/transmit transition gap (RTG): A gap between the last sample of the uplink (UL) burst and the first sample of the subsequent downlink (DL) burst at the antenna port of the BS in a time division duplex (TDD) transceiver. This gap allows time for the BS to switch from receive (Rx) to transmit (Tx) mode. During this gap, the BS is not transmitting modulated data but simply allowing the BS transmitter carrier to ramp up and the Tx/Rx antenna switch to actuate. Not applicable for frequency division duplex (FDD) systems.

Base station (BS) transmit/receive transition gap (TTG): A gap between the last sample of the downlink (DL) burst and the first sample of the subsequent uplink (UL) burst at the antenna port of the BS in a time division duplex (TDD) transceiver. This gap allows time for the BS to switch from transmit (Tx) to receive (Rx) mode. During this gap, the BS is not transmitting modulated data but simply allowing the BS transmitter carrier to ramp down, the Tx/Rx antenna switch to actuate, and the BS receiver section to activate. Not applicable for frequency division duplex (FDD) systems.

Base station (BS): A generalized equipment set providing connectivity, management, and control of the subscriber station (SS). See also: active base station (BS), anchor base station (BS), neighbour base station (BS), serving base station (BS), target base station (BS).

Basic connection: Connection that is established during subscriber station (SS) initial ranging and used to transport delay-intolerant medium access control layer (MAC) management messages.

Broadband wireless access (BWA): Wireless access in which the connection(s) capabilities are broadband.

Broadband: Having instantaneous bandwidths greater than around 1 MHz and supporting data rates greater than about 1.5 Mb/s.

Broadcast connection: The management connection used by the base station (BS) to send medium access control layer (MAC) management messages on a downlink (DL) to all subscriber stations (SSs). The broadcast connection is identified by a well-known connection identifier (CID). A fragmentable broadcast connection is a connection that allows fragmentation of broadcast MAC management messages.

Burst profile: Set of parameters that describe the uplink (UL) or downlink (DL) transmission properties associated with an interval usage code. Each profile contains parameters such as modulation type, forward error correction (FEC) type, preamble length, guard times, etc. See also: interval usage code.

Centralized scheduling: A mode of operation applicable to multihop relay where a multihop relay BS (MR-BS) determines the bandwidth allocations and generates the corresponding MAPs [or dictates the information used by relay stations (RSs) to generate their MAPs] for all access and relay links in the MR-cell.

Channel identifier (ChID): An identifier used to distinguish between multiple uplink (UL) channels, all of which are associated with the same downlink (DL) channel.

closed subscriber group (CSG): A set of subscribers authorized by the Femto ABS owner or the network service provider, for accessing CSG Femto ABS.

Concatenation: The act of combining multiple medium access control layer (MAC) protocol data units (PDUs) into a single physical layer (PHY) service data unit (SDU).

Connection identifier (CID): A 16-bit value that identifies a transport connection or an uplink (UL)/ downlink (DL) pair of associated management connections [i.e., belonging to the same subscriber station (SS)] to equivalent peers in the medium access control layer (MAC) of the base station (BS) and SS. The CID address space is common (i.e., shared) between UL and DL and partitioned among the different types of connections. Security associations (SAs) also exist between keying material and CIDs. See also: connection.

Connection: A unidirectional mapping between base station (BS) and subscriber station (SS) medium access control layer (MAC) peers. Connections are identified by a connection identifier (CID). The MAC defines two kinds of connections: management connections and transport connections. See also: connection identifier (CID).

contiguous resource unit (CRU): The resource allocation unit of the same size as the PRU that has undergone the subband partitioning and miniband permutation, assigned to contiguous allocation and will bypass subcarrier permutation in DL and tile permutation in UL. Syn: localized resource unit.

CSG Femto ABS: CSG-Closed or CSG-Open Femto ABS.

CSG-Closed Femto ABS: A Femto ABS accessible only to the AMSs, which are in its CSG(s), except for emergency services. AMSs that are not the members of the CSG(s) should not try to access CSGClosed Femto ABSs.

CSG-Open Femto ABS: A Femto ABS primarily accessible to the AMSs that belong to its CSG(s), while other AMSs, outside CSG(s), may also access such Femto ABS, and will be served at lower priority. CSG-Open Femto ABS will provide service to such AMSs as long as the QoS of AMSs in its CSG(s) is not compromised.

DC subcarrier: In an orthogonal frequency division multiplexing (OFDM) or orthogonal frequency division multiple access (OFDMA) signal, the subcarrier whose frequency would be equal to the radio frequency (RF) center frequency of the station.

default service flow: A service flow that is established automatically without DSA procedure after successful registration procedure. QoS parameters for the default service flow are predefined. IEEE

distributed resource unit (DRU): The resource allocation unit of the same size as the PRU that has undergone the subband partitioning and miniband permutation, assigned to distributed allocation and will be submitted to the subcarrier permutation in DL and tile permutation in UL.

Distributed scheduling: A mode of operation applicable to multihop relay where the MR-BS and each RS in the MR-cell (with or without information from the MR-BS) determine the bandwidth allocations and generate the corresponding MAPs for the access link to/from their subordinate SSs and/or relay links to/ from their subordinate RSs.

Diversity set: A list of active base stations (BSs) to the mobile station (MS). The diversity set is managed by the MS and BSs and is applicable to macro diversity handover (MDHO) and fast BS switching (FBSS).

DL access zone: A portion of the DL subframe in the MR-BS/RS frame used for MR-BS/RS to MS or RS (except TTR RS in TDD mode) transmission. The DL access zone may consist of the entire downlink subframe, depending on the method used to separate the transmissions on the access and relay links.

DL relay zone: A portion of the DL subframe in the MR-BS/RS frame used for MR-BS/RS to RS transmission. A frame may have no DL relay zone, depending on the method used to separate the transmissions on the access and relay links.

Downlink (DL): The direction from the base station (BS) to the subscriber station (SS).

Downlink burst transition gap (DLBTG): The gap included on the trailing edge of each allocated downlink (DL) burst so that ramp-down can occur and delay-spread can clear receivers.

Downlink channel descriptor (DCD): A medium access control layer (MAC) message that describes the physical layer (PHY) characteristics of a downlink (DL) channel.

Downlink interval usage code (DIUC): An interval usage code specific to a downlink (DL). See also: interval usage code.

Downlink map (DL-MAP): A medium access control layer (MAC) message that defines burst start times for both time division multiplex and time division multiple access (TDMA) by a subscriber station (SS) on the downlink (DL).

dual radio MS: A multimode MS/AMS that can have both radios (transmitting and receiving) active at the same time. A dual radio MS/AMS can simultaneous transmit and receive on both radios. A dual radio MS/AMS may behave as a single radio MS by operating in single radio mode.

Dynamic frequency selection (DFS): The ability of a system to switch to different physical radio frequency (RF) channels based on channel measurement criteria to conform to particular regulatory requirements.

Dynamic service: The set of messages and protocols that allow the base station (BS) and subscriber station (SS) to add, modify, or delete the characteristics of a service flow.

Fast base station switching (FBSS): Base station (BS) switching that utilizes a fast switching mechanism to improve link quality. The mobile station (MS) is only transmitting/receiving data to/from one of the active BS (anchor BS) at any given frame. The anchor BS can change from frame to frame depending on the BS selection scheme.

Femto ABS: An ABS with low transmit power, typically installed by a subscriber in the home, SOHO, or enterprise to provide the access to closed or open group of users as configured by the subscriber and/or the access provider. A Femto ABS is typically connected to the service providers network via a broadband connection.

Fixed wireless access: Wireless access application in which the locations of the base station (BS) and subscriber station (SS) are fixed in location during operation.

frame index: The frame order within a superframe (i.e., the 1st, 2nd, 3rd, or 4th frame of superframe).

frame number: In WirelessMAN-OFDMA, the frame number is a 24-bit number transmitted in every frame. Frame numbers are not necessarily synchronized across base stations. In WirelessMAN-Advanced Air Interface, the frame number is obtained by concatenating the 12-bit superframe number (transmitted in every superframe) and the 2-bit frame index. Superframe numbers are synchronized across base stations.

Frame: A structured data sequence of fixed duration used by some physical layer (PHY) specifications. A frame may contain both an uplink (UL) subframe and a downlink (DL) subframe.

Frequency assignment (FA) index: A network-specific logical FA index assignment. FA index assignment is used in combination with operator-specific configuration information provided to the mobile station (MS) in a method outside the scope of this standard.

Frequency assignment (FA): A logical assignment of downlink (DL) center frequency and channel bandwidth programmed to the base station (BS).

Frequency division duplex (FDD): A duplex scheme in which uplink (UL) and downlink (DL) transmissions use different frequencies but are typically simultaneous.

Frequency offset index: An index number identifying a particular subcarrier in an orthogonal frequency division multiplexing (OFDM) or orthogonal frequency division multiple access (OFDMA) signal, which is related to its subcarrier index. Frequency offset indices may be positive or negative.

Group key encryption key (GKEK): A random number generated by the base station (BS) or a network entity [e.g., an authentication and service authorization (ASA) server] used to encrypt the group traffic encryption keys (GTEKs) sent in broadcast messages by the BS to mobile stations (MSs) in the same multicast group.

Handover (HO): The process in which a mobile station (MS) migrates from the air-interface provided by one base station (BS) to the air-interface provided by another BS. A break-before-make HO is where service with the target BS starts after a disconnection of service with the previous serving BS. A makebefore-break HO is where service with the target BS starts before disconnection of the service with the previous serving BS.

horizontal encoding: Indicates transmitting multiple MIMO layers over multiple antennas. The number of MIMO layers is more than 1. The number of MIMO streams is same as the number of MIMO layers in this case.

Infrastructure station: An MR-BS or RS.

Initial ranging connection: A management connection used by the subscriber station (SS) and the base station (BS) during the initial ranging process. The initial ranging connection is identified by a well-known connection identifier (CID). This CID is defined as a constant value within the protocol since an SS has no addressing information available until the initial ranging process is complete.

Intermediate RS: A relay station that is located on a path between an MR-BS and an access RS.

Interval usage code: A code identifying a particular burst profile that can be used by a downlink (DL) or uplink (UL) transmission interval.

Location Based Services (LBS): Services that are based on location data of the MS and/or BS in a network of IEEE 802.16 devices. Examples in location sensitized applications, emergency call origination tracking, equipment tracking etc.

logical resource unit (LRU): The generic name of logical units for distributed and localized resource allocations.

LZone: A positive integer number of consecutive subframes during which an ABS communicates with RSs or R1 MSs, and where an ARS or an RS communicates with one or more R1 MSs.

Macro ABS: An ABS with high transmit power. A Macro ABS is directly connected to the service providers network.

Macro diversity handover (MDHO): The process in which an mobile station (MS) migrates from the air-interface provided by one or more base stations (BSs) to the air-interface provided by one or more other BSs. This process is accomplished in the downlink (DL) by having two or more BSs transmitting the same medium access control layer (MAC) or physical layer (PHY) protocol data unit (PDU) to the MS so that diversity combining can be performed by the MS. In the uplink (UL), it is accomplished by having two or more BSs receiving (demodulating, decoding) the same PDU from the MS so that diversity combining of the received PDU can be performed among the BSs.

Macro hot-zone ABS: An ABS with smaller transmission power/cell size than that of macro ABSs, typically deployed by service provider and operated on service provider backhaul.

Management connection: A connection used for transporting medium access control layer (MAC) management messages or standards-based messages required by the MAC. For MAC management messages, see also: basic connection, primary management connection, broadcast connection, initial ranging connection. For standards-based messages required by the MAC, see also: secondary management connection.

Management tunnel CID (MT-CID): An identifier taken from the connection identifier (CID) space managed by an MR-BS that uniquely identifies a management tunnel connection between the MR-BS and an access RS.

MIMO layer: An information path fed to the MIMO encoder as an input. A MIMO layer represents one channel coding block.

MIMO stream: Each information path encoded by the MIMO encoder that is passed to the precoder.

Minislot: A unit of uplink (UL) bandwidth allocation equivalent to n physical slots (PSs), where n = 2m and m is an integer ranging from 0 through 7.

mixed-mode ABS: An ABS with an operating Lzone and operating Mzone.

Mobile station (MS): A station in the mobile service intended to be used while in motion or during halts at unspecified points. An MS is always a subscriber station (SS) unless specifically excepted otherwise in this standard.

MR-BS frame: Frame structure for DL transmission/UL reception by MR-BS.

multi radio MS: A multimode MS/AMS that can have multiple radios (transmitting and receiving) active at the same time. A multi radio MS/AMS can simultaneous transmit and receive on multiple radios. A Multi Radio MS/AMS may behave as a Single Radio MS by operating in Single Radio Mode.

multi-layer encoding: The number of MIMO streams is same as the number of MIMO layers in this case.

multi-user MIMO (MU-MIMO): A MIMO transmission scheme in which multiple MSs are scheduled in one RU, by virtue of spatial separation of the transmitted signals.

Multicast polling group: A group of zero or more subscriber stations (SSs) that are assigned a multicast address for the purposes of polling.

Multihop relay base station (MR-BS): A generalized equipment set providing connectivity, management, and control of relay stations and subscriber stations. See also: base station (BS), relay station (RS).

Multiple input multiple output (MIMO): A system employing at least two transmit (Tx) antennas and at least two receive (Rx) antennas to improve the system capacity, coverage, or throughput.

MZone: A positive integer number of consecutive subframes during which an ABS communicates with one or more ARSs or AMSs, and where an ARS communicates with one or more ARSs or AMSs.

Neighbor base station (BS): For any mobile station (MS), a BS (other than the serving BS) whose downlink (DL) transmission can be received by the MS.

Non-transparent RS: A relay station that transmits DL frame-start preamble, FCH, MAP message(s) and channel descriptor (DCD/UCD) messages.

Operator ID: Operator ID is an identifier of the network provider. The Operator ID is contained in the Base Station ID.

Orderly power-down procedure: The procedure that the mobile station (MS) performs when powering down, for example, as directed by user input or as prompted by a automatic power-down mechanism.

OSG Femto ABS: A Femto ABS accessible to any AMS.

Packing: The act of combining multiple service data units (SDUs) from a higher layer into a single medium access control layer (MAC) protocol data unit (PDU).

Paging Controller: Paging controller is a unit that belongs to the idle mode services in the NCMS. The paging controller retains the MS state and operational parameters and/or administers paging activity for the MS while in idle mode.

partially configured carrier: A downlink only carrier configured with control channels to support downlink transmission.

Payload header suppression (PHS): The process of suppressing the repetitive portion of payload headers at the sender and restoring the headers at the receiver.

Payload Header Suppression field (PHSF): A string of bytes representing the header portion of a protocol data unit (PDU) in which one or more bytes are to be suppressed (i.e., a snapshot of the uncompressed PDU header inclusive of suppressed and unsuppressed bytes).

Payload header suppression index (PHSI): An 8-bit value that references the payload header suppression (PHS) rule.

Payload header suppression masks (PHSM): A bit mask indicating which bytes in the Payload Header Suppression field (PHSF) to suppress and which bytes to not suppress.

Payload header suppression size (PHSS): The length of the suppressed field in bytes. This value is equivalent to the number of bytes in the Payload Header Suppression field (PHSF) and also the number of valid bits in the payload header suppression mask (PHSM).

Payload header suppression valid (PHSV): A flag that tells the sending entity to verify all bytes that are to be suppressed.

physical resource unit (PRU): The basic resource allocation unit that consists of 18 adjacent subcarriers in consecutive symbols in the same AAI subframe.

Physical slot (PS): A unit of time, dependent on the physical layer (PHY) specification, for allocating bandwidth.

Point-to-point (PtP): A mode of operation whereby a link exists between two network entities.

primary carrier: An OFDMA carrier on which an ABS and an AMS/MS exchange traffic and full PHY/MAC control information defined in the Advanced Air Interface specification. Further, the primary carrier is used for control functions for proper AMS/MS operation, such as network entry. Each AMS shall have only one carrier it considers to be its primary carrier in a cell.

Primary management connection: A connection that is established during initial subscriber station (SS) ranging and used to transport delay-tolerant medium access control layer (MAC) management messages.

Privacy Key Management (PKM) Protocol: A client/server model between the base station (BS) and subscriber station (SS) that is used to secure distribution of keying material.

Protocol data unit (PDU): The data unit exchanged between peer entities of the same protocol layer.

Quality of service (QoS) parameter set: A parameter set associated with a service flow identifier (SFID). The contained traffic parameters define scheduling behavior of uplink (UL) or downlink (DL) flows associated with transport connections.

R1 BS: A base station compliant with the WirelessMAN-OFDMA R1 Reference System.

R1 MS: A mobile station compliant with the WirelessMAN-OFDMA R1 Reference System.

Radio frequency (RF) center frequency: The center of the frequency band in which a base station (BS) or subscriber station (SS) is intended to transmit.

relative delay (RD): The delay of neighbor DL signals relative to the serving/attached BS.

Relay link (R-link): A radio link between an MR-BS and an RS or between a pair of RSs. This can be a relay uplink or downlink.

Relay station (RS): A generalized equipment set, dependent on a multihop relay base station (MR-BS) providing connectivity, to other RSs or subscriber stations (SS). An RS may also provide management and control of subordinate RSs or SSs. The air interface between an RS and an SS is identical to the air interface between a BS and an SS. See also: multihop relay base station (MR-BS), base station (BS), subscriber station (SS).

Relay zone: A portion of a frame used for the relay link.

resource unit: A granular unit in frequency and time, described by the number of OFDMA subcarriers and OFDMA symbols

Round trip delay (RTD): The round trip delay time between communicating stations (i.e. such as between an RS and its superordinate station).

round trip delay (RTD): The time required for a signal or packet to transfer from a MS to a BS and back again.

RS frame: Frame structure for DL/UL transmission/reception by RS.

RS receive/transmit transition gap (RSRTG): The minimum receive-to-transmit turnaround gap required at an RS. RSRTG is measured from the time of the last sample of the received burst to the first sample of the transmitted burst at the antenna port of the RS.

RS transmit/receive transition gap (RSTTG): The minimum transmit-to-receive turnaround gap required at an RS. RSTTG is measured from the time of the last sample of the transmitted burst to the first sample of the received burst at the antenna port of the RS.

Scanning interval: A time period intended for the mobile station (MS) to monitor neighbor base stations (BSs) to determine the suitability of the BSs as targets for handover (HO).

Scheduling RS: A relay station that serves as a scheduling station; i.e., a non-transparent RS with unique BSID and operating in distributed scheduling mode.

Scheduling station: In centralized scheduling mode, the scheduling station is always the MR-BS. In distributed scheduling mode, the scheduling station of a given MS/RS is the first station along the route to the MR-BS that transmits MAPs; i.e., either a non-transparent RS or the MR-BS itself.

secondary carrier: An OFDMA carrier that an AMS may use to exchange traffic with an ABS, based on allocation commands and rules received over the primary carrier of that ABS. The secondary carrier may also include control signaling to support multicarrier operation.

Secondary management connection: A connection that may be established during subscriber station (SS) registration that is used to transport standards-based [e.g, Simple Network Management Protocol (SNMP), Dynamic Host Configuration Protocol (DHCP)] messages.

Security association (SA): The set of security information that a base station (BS) and one or more of its client subscriber stations (SSs) share in order to support secure communications. This shared information includes traffic encryption keys (TEKs) and cipher block chaining (CBC) initialization vectors (IVs).

Security association identifier (SAID): An identifier shared between the base station (BS) and subscriber station (SS) that uniquely identifies a security association (SA). The SAID is unique within MS. The uniqueness of this identifier shall be guaranteed by {MS MAC Address, SAID} pair.

Security zone (SZ): A group consisting of one or more RSs and the MR-BS that share key material for the protection of MAC management messages produced and processed by members of the group.

Security zone key (SZK): A group key shared by the MR-BS and a group of RSs within the same security zone. The SZK is a head of key hierarchy used to satisfy the security requirements such as integrity protection for MAC management messages within a defined security zone.

Service access point (SAP): The point in a protocol stack where the services of a lower layer are available to its next higher layer.

Service data unit (SDU): The data unit exchanged between two adjacent protocol layers. On the downward direction, it is the data unit received from the previous higher layer. On the upward direction, it is the data unit sent to the next higher layer.

Service flow (SF): A unidirectional flow of medium access control layer (MAC) service data units (SDUs) on a connection that is provided a particular quality of service (QoS).

Service flow identifier (SFID): A 32-bit quantity that uniquely identifies a service flow to the subscriber station (SS).

Serving base station (BS): For any mobile station (MS), the BS with which the MS has most recently completed registration at initial network-entry or during a handover (HO).

simultaneous transmit and receive (STR) relaying: A relay mechanism where transmission to subordinate station(s) and reception from the superordinate station, and transmission to the superordinate station and reception from subordinate station(s) are performed simultaneously.

Simultaneous transmit and receive (STR) relaying: A relay mechanism where transmission to subordinate station(s) and reception from the superordinate station, or transmission to the superordinate station and reception from subordinate station(s) are performed simultaneously.

single radio MS: A multimode MS/AMS that operates with only a single transmitting radio and with one or more receiving radios at any given time.

single user MIMO (SU-MIMO): A MIMO transmission scheme in which a single MS is scheduled in one RU.

STC layer: OFDMA Space Time Coding information-flow fed to the STC encoder as an input. The number of STC layers in a system with vertical encoding is one, while in horizontal encoding, it depends on the number of encoding/modulation paths. This term may be used interchangeably with the word layer when used in the context of OFDMA STC.

STC stream: OFDMA Space Time Coding information path encoded by the STC encoder that is passed to subcarrier mapping and sent through one antenna, or passed on to the beamformer. The number of STC streams in both vertical and horizontal encoding systems is the same as the number of output paths of the STC encoder. This term may be used interchangeably with the word stream when used in the context of OFDMA STC.

STR RS: A non-transparent relay station capable of performing STR relaying.

Subcarrier index: An index number identifying a particular used subcarrier in an orthogonal frequency division multiplexing (OFDM) or orthogonal frequency division multiple access (OFDMA) signal. Subcarrier indices are greater than or equal to zero.

Subscriber station (SS): A generalized equipment set providing connectivity between subscriber equipment and a base station (BS).

Subscriber station receive/transmit gap (SSRTG): The minimum receive-to-transmit turnaround gap. SSRTG is measured from the time of the last sample of the received burst to the first sample of the transmitted burst at the antenna port of the SS.

Subscriber station transmit/receive gap (SSTTG): The minimum transmit-to-receive turnaround gap. SSTTG is measured from the time of the last sample of the transmitted burst to the first sample of the received burst at the antenna port of the SS.

superframe: A structured data sequence of fixed duration used by the Advanced Air Interface specifications. A superframe is comprised of four frames.

Target base station (BS): The BS with which a mobile station (MS) intends to be registered at the end of a handover (HO).

Time division duplex (TDD): A duplex scheme where uplink (UL) and downlink (DL) transmissions occur at different times but may share the same frequency.

Time division multiple access (TDMA) burst: A contiguous portion of the uplink (UL) or downlink (DL) using physical layer (PHY) parameters, determined by the downlink interval usage code (DIUC) or uplink interval usage code (UIUC), that remain constant for the duration of the burst. TDMA bursts are separated by preambles and are separated by gaps in transmission if subsequent bursts are from different transmitters.

Time division multiplexing (TDM) burst: A contiguous portion of a TDM data stream using physical layer (PHY) parameters, determined by the downlink interval usage code (DIUC), that remain constant for the duration of the burst. TDM bursts are not separated by gaps or preambles.

time-division transmit and receive (TTR) relaying: A relay mechanism where transmission to subordinate station(s) and reception from the superordinate station, or transmission to the superordinate station and reception from subordinate station(s) is separated in time.

Time-division transmit and receive (TTR) relaying: A relay mechanism where transmission to subordinate station(s) and reception from the superordinate station, or transmission to the superordinate station and reception from subordinate station(s) is separated in time.

transmission time interval (TTI): The duration of the transmission of the physical layer encoded packet over the radio air interface and is equal to an integer number of AAI subframes. The default TTI is 1 AAI subframe. IEEE Std 802.16m-2011 AMENDMENT TO IEEE STD 802.16-2009 8

Transparent RS: A relay station that does not transmit DL frame-start preamble, FCH, MAP message(s) or channel descriptor (DCD/UCD) messages.

Transparent zone: A portion of the DL subframe in the MR-BS/RS frame for an RS operating in the transparent mode used for MR-BS/RS to MS transmission. A DL subframe may, or may not, have a transparent zone.

Transport connection identifier (CID): A unique identifier taken from the CID address space that uniquely identifies the transport connection. All user data traffic is carried on transport connections, even for service flows that implement connectionless protocols, such as Internet Protocol (IP). An active or admitted service flow [identified by a service flow identifier (SFID)] maps to a Transport CID assigned by the base station (BS).

Transport connection: A connection used to transport user data. It does not include any traffic over the basic, primary, or secondary management connections. A fragmentable transport connection is a connection that allows fragmentation of service data units (SDUs).

TTR RS: A non-transparent relay station that performs TTR relaying.

Tunnel CID (T-CID): An identifier taken from the connection identifier (CID) space that uniquely identifies a transport tunnel connection.

Turbo decoding: Iterative decoding, using soft inputs and soft outputs.

Type/length/value (TLV): A formatting scheme that adds a tag to each transmitted parameter containing the parameter type (and implicitly its encoding rules) and the length of the encoded parameter.

U Interface: The management and control interface that exists between the SS and the BS over the air interface.

UL access zone: A portion of the UL subframe in the MR-BS/RS frame used for MS or RS (except TTR RS in TDD mode) to MR-BS/RS transmission. A frame may have no UL access zone, or the UL access zone may consist of the entire uplink subframe, depending on the method used to separate the transmissions on the access and relay links.

UL relay zone: A portion of the UL subframe in the MR-BS/RS frame used for RS to MR-BS/RS transmission. A frame may have no UL relay zone, or the UL relay zone may consist of the entire uplink subframe, depending on the method used to separate the transmissions on the access and relay links.

Uplink (UL): The direction from a subscriber station (SS) to the base station (BS).

Uplink channel descriptor (UCD): A medium access control layer (MAC) message that describes the physical layer (PHY) characteristics of an uplink (UL).

Uplink interval usage code (UIUC): An interval usage code specific to an uplink (UL).

Uplink map (UL-MAP): A set of information that defines the entire access for a scheduling interval.

User data: Protocol data units (PDUs) of any protocol above a service-specific convergence sublayer (CS) received over the CS service access point (SAP).

vertical encoding: Indicates transmitting a single MIMO layer over multiple antennas. The number of MIMO layers is always 1.

Wireless access: End-user radio connection(s) to core networks.

WirelessMAN-OFDMA Advanced Co-existing System: An ABS and/or AMS that also implements LZone functionality compliant with WirelessMAN-OFDMA TDD Release 1.

WirelessMAN-OFDMA R1 Reference System: A network compliant with the WirelessMANOFDMA capabilities as specified in WirelessMAN-OFDMA TDD Release 1.

Annex 2: Update of 4.2.2.1

4.2.2.1: Abbreviations  related to WirelessMAN-Advanced

3-DES		triple data encryption standard

A-MAP		advanced MAP

A-Preamble		advanced preamble

AAI		Advanced

AAS		adaptive antenna system

ABS		advanced

AC		authentication control

ACM		account management

AES		advanced encryption standard

AGC		automatic gain control

AGMH		advanced generic MAC header

aGPS		adaptive grant polling service

AK		authorization key

AKID		authorization key identifier

AMC		adaptive modulation and coding

AMS		advanced mobile station

AOA		angle of arrival

ARQ		automatic repeat request

ARS		advanced relay station

ASA		authentication and service authorization

ASN		access service network

ASN.1		abstract syntax notation

ASR		anchor switch reporting

ATDD		adaptive time division duplexing

ATM		asynchronous transfer mode

BCC		block convolutional code

BE		best effort

BER		bit error ratio

BPSK		binary phase shift keying

BR		bandwidth request

BS		base station

BSN		block sequence number

BTC		block turbo code

BW		bandwidth

BWA		broadband wireless access

BWAA		bandwidth allocation/access

C/I		carrier-to-interference ratio

C/N		carrier-to-noise ratio

CA		certification authority

CBC		cipher block chaining

CBC-MAC		cipher block chaining message authentication code

CC		confirmation code

CCH		control sub channel

CCI		co-channel interference

CCM		CTR mode with CBC-MAC

CCS		common channel signaling

CCV		clock comparison value

CDMA		code division multiple access

CDR		conjugate data repetition

ChID		channel identifier

CID		connection identifier

CINR		carrier-to-interference-and-noise ratio

CIR		channel impulse response

CL		closed-loop

CLP		cell loss priority

CLRU		contiguous LRU

CMAC		cipher-based message authentication code

CMI		codebook matrix index

CoRe		constellation rearrangement

CP		cyclic prefix

CPS		common part sublayer

CQI		channel quality information

CQICH		channel quality information channel

CRC		cyclic redundancy check

CRID		context retention identifier

CRT		context retention timer

CRU		contiguous resource unit

CRV		CoRe version

CS		convergence sublayer

CSCF		centralized scheduling configuration

CSCH		centralized scheduling

CSG		closed subscriber group

CSIT		channel state information at the transmitter

CSM		collaborative spatial multiplexing

CTC		convolutional turbo code

CTR		counter mode encryption

D-TDOA		Downlink Time Difference Of Arrival

DAMA		demand assigned multiple accesses

DARS		digital audio radio satellite

dBi		decibels of gain relative to the 0 dB gain of a free-space isotropic radiator

dBm		decibels relative to 1 mW

DCAS		downlink CRU allocation size

DCD		downlink channel descriptor

DCR		deregistration with context retention

DES		data encryption standard

DFS		dynamic frequency selection

DHCP		Dynamic Host Configuration Protocol

DID		deregistration identifier

DIUC		downlink interval usage code

DL		downlink

DLFP		downlink frame prefix

DLRU		distributed LRU

DRU		distributed resource unit

DSA		dynamic service addition

DSAC		downlink subband allocation count

DSC		dynamic service change

DSCH		distributed scheduling

DSCP		differentiated services code point

DSD		dynamic service deletion

DSx		dynamic service addition, change, or deletion

E-MBS		multicast and broadcast service

EAP		extensible authentication protocol

EBB		established before break

EC		encryption control

ECB		electronic code book

ECRTP		a IP-header-compression CS PDU format (IETF RFC 3545)

EDE		encrypt-decrypt-encrypt

EESS		earth exploratory satellite system

EH		extended header

EIK		EAP Integrity Key

EIRP		effective isotropic radiated power

EKS		encryption key sequence

EVM		error vector magnitude

FBSS		fast base station switching

FC		fragmentation control

FCAPS		Fault Management, Configuration Management, Account Management, Performance Management, Security Management

FCH		frame control header

FDD		frequency division duplex or duplexing

FEC		forward error correction

FFR		fractional frequency reuse

FFSH		fast-feedback allocation subheader

FFT		fast Fourier transform

FHDC		frequency hopping diversity coding

FID		flow identifier

FMT		UL feedback mini-tile

FP		frequency partition

FPC		frequency partition configuration

FPC		fast power control

FPCT		frequency partition count

FPEH		fragmentation and packing extended header

FPS		frequency partition size

FPSC		frequency partition subband count

FRS		fixed relay station

FSH		fragmentation subheader

FSN		fragment sequence number

FSS		fixed satellite service

FUSC		full usage of subchannels

GF		galois field

GKEK		group key encryption key

GMH		generic MAC header

GMSH		grant management subheader

GPCS		Generic Packet Convergence Sublayer

GPS		global positioning system

GRA		group resource allocation

GS		guard symbol

GTEK		group traffic encryption key

H-FDD		half-duplex frequency division duplex

HARQ		hybrid ARQ

HCS		header check sequence

HE		horizontal encoding

HEC		header error check

HMAC		hashed message authentication code

HMT		UL HARQ mini-tiles

HO		handover

HR		handover ranging

HT		header type

HUMAN		high-speed unlicensed metropolitan area network

I		inphase

IANA		internet assigned numbers authority

ICV		integrity check value

IE		information element

IFFT		inverse fast Fourier transform

IMM		idle mode management

IP		Internet Protocol

IR		incremental redundancy

IR		initial ranging

IS		infrastructure station

ISL		interference sensitivity level

IV		initialization vector

IWF		interworking function

KEK		key encryption key

LAN		local area network

LBS		location based services

LDM		low-duty mode

LDPC		low-density parity check

LFSR		linear feedback shift register

LLC		logical link control

LOS		line-of-sight

LRU		logical resource unit

LSB		least significant bit

MAC		medium access control layer

MAK		MBS authorization key

MAN		metropolitan area network

MBS		multicast and broadcast service

MCEH		MAC control extended header

MCID		multicast CID

MCS		modulation coding scheme

MDHO		macro diversity handover

MDS		multipoint distribution service

MEF		MIMO encoder format

MEH		multiplexing extended header

MGTEK		MBS group traffic encryption key

MIB		management information base

MIC		message integrity check

MIH		media independent handover

MIHF		MIH Function

MIMO		multiple input multiple output

MLRU		minimum A-MAP logical resource unit

MMDS		multichannel multipoint distribution service

MPEG		moving pictures experts group

MR-BS		multihop relay base station

MRS		mobile relay station

MS		mobile station

MSB		most significant bit

MSK		master session key

MU		multi-user

N/A		not applicable

NAI		network access identifier

NAS		network access server

NCFG		network configuration

NCMS		network control and management system

NCMS(BS)		network control and management system at the BS side (network side)

NCMS(SS/MS)		network control and management system at the SS/MS side

NEM		network entry management

NENT		network entry

NIP		normalized interference power

NLOS		non-line-of-sight

NLRU		miniband LRU

NNI		network-to-network interface (or network node interface)

NRM		network reference model

nrtPS		non-real-time polling service

NS-RCH		non-synchronized ranging channel

NS/EP		National Security/Emergency Preparedness

NSP		network service provider

OFDM		orthogonal frequency division multiplexing

OFDMA		orthogonal frequency division multiple access

OID		object identifier

OL		open-loop

OSG		open subscriber group

P-SFH		primary superframe header

PA		persistent allocation

PA-Preamble		primary advanced preamble

PAK		primary authorization key

PAPR		peak to average power ratio

PBR		piggyback request

PDU		protocol data unit

PER		packet error ratio

PFBCH		UL primary fast feedback channel

PGID		paging-group identifier

PHS		payload header suppression

PHSF		payload header suppression field

PHSI		payload header suppression index

PHSM		payload header suppression mask

PHSS		payload header suppression size

PHSV		payload header suppression valid

PHY		physical layer

PKM		privacy key management

PM		poll-me bit

PMD		physical medium dependent

PMI		preferred matrix index

PMK		pairwise master key

PMP		point-to-multipoint

PN		packet number

PPP		Point-to-Point Protocol

PPRU		permuted physical resource unit

PRBS		pseudo-random binary sequence

PRU		physical resource unit

PS		physical slot

PSC		power saving class

PSH		packing subheader

PSI		pilot stream index

PTI		payload type indicator

PtP		point to point

PUSC		partial usage of subchannels

PUSC-ASCA		partial usage of subchannels – adjacent subcarrier allocation

PVC		permanent virtual circuit

Q		quadrature

QAM		quadrature amplitude modulation

QoS		quality of service

QPSK		quadrature phase-shift keying

R-ACK		relay ACK

R-DL		relay downlink

R-FCH		relay zone frame control header

R-MAP		relay zone MAP

R-RTI		relay receive/transmit transition interval

R-TTI		relay transmit/receive transition interval

R-UL		relay uplink

R-Zone		relay zone

RA-ID		random access identifier

RCH		ranging channel

RCP		ranging cyclic prefix

RD		relative delay

REQ		request

RFMT		reordered UL feedback mini-tile

RHMT		reordered UL HARQ mini-tile

RLAN		radio local access network

RNG		ranging

ROHC		an IP-header-compression CS PDU format (IETF RFC 3095)

RP		ranging preamble

RRA		radio resource agent

RRC		radio resource controller

RRM		radio resource management

RS		Reed–Solomon

RS		relay station

RSP		response

RSS		receive signal strength

RSSI		receive signal strength indicator

RTD		round trip delay

RTD		round trip delay

RTG		receive/transmit transition gap

rtPS		real-time polling service

RU		resource unit

Rx		receive

RxDS		receiver delay spread clearing interval

S-ABS		serving ABS

S-RCH		synchronized ranging channel

S-SFH		secondary superframe header

SA		security association

SA-Preamble		secondary advanced preamble

SAC		subband allocation count

SAID		security association identifier

SAP		service access point

SAR		synthetic aperture radar

SC		single carrier

Sc		subcarrier

SDMA		spatial division multiple access

SDU		service data unit

SF		service flow

SFBC		space-frequency block code

SFBCH		UL secondary fast feedback channel

SFH		superframe header

SFID		service flow identifier

SFM		service flow management

SHA		secure hash algorithm

SI		slip indicator

SIQ		service information query

SLRU		subband LRU

SM		spatial multiplexing

SN		sequence number

SNMP		Simple Network Management Protocol

SNR		signal-to-noise ratio

SOHO		small office home office

SON		self organizing networks

SP		S-SFH subpacket

SPID		subpacket ID

SPMH		short-packet MAC header

SS		subscriber station

SSID		subscriber station identification (MAC address)

SSM		subscriber station management

SSSCH		service-specific scheduling control header

SSTG		subscriber station transition gap

STC		space time coding

STID		station identifier

STTD		space time transmit diversity

SU		single-user

SVC		switched virtual circuit

T-ABS		target ABS

TCM		trellis coded modulation

TCP		Transmission Control Protocol

TCS		transmission convergence sublayer

TDD		time division duplex or duplexing

TDM		time division multiplexing

TDMA		time division multiple access

TDOA		time difference of arrival

TDU		tunnel data unit

TEK		traffic encryption key

TFTP		Trivial File Transfer Protocol

TLV		type/length/value

TOA		time of arrival

TSTID		temporary STID

TTG		transmit/receive transition gap

TUSC		tile usage of subchannels

Tx		transmit (abbreviation not used as verb)

U-NII		unlicensed national information infrastructure

U-TDOA		uplink time difference of arrival

UCAS		uplink CRU allocation size

UCD		uplink channel descriptor

UDP		User Datagram Protocol

UEP		unequal error protection

UFPC		uplink frequency partition configuration

UGS		unsolicited grant service

UIUC		uplink interval usage code

UL		uplink

UNI		user-to-network interface (or user-network interface)

USAC		uplink subband allocation count

UTC		universal coordinated time

UW		unique word

VC		virtual channel

VCI		virtual channel identifier

VE		vertical encoding

VLAN		virtual local area network

VP		virtual path

VPI		virtual path identifier

WLAN		wireless local area network

XOR		exclusive-or