#### Before the

FEDERAL COMMUNICATIONS COMMISSION

Washington, D.C. 20554

| In the Matter of Request by Progeny LMS, LLC, for waiver of certain multilateration location and monitoring service rules. |  | WT Docket No. 11-49 |
| --- | --- | --- |

Via the ECFS

Reply Comments Of IEEE 802

- IEEE 802 respectfully submits its Comments in the above-captioned Proceeding.

- IEEE 802, as a leading consensus-based industry standards body, produces standards for wireless networking devices, including wireless local area networks (“WLANs”), wireless personal area networks (“WPANs”), wireless metropolitan area networks (“Wireless MANs”), and wireless regional area networks (“WRANS”).  Included in our standards development activity is an emphasis on coexistence, which is the focus of our Wireless Coexistence working group. We appreciate the opportunity to provide these comments to the FCC.

introduction

- On January 27, 2012, Progeny LMS, LLC (Progeny) filed a report in WT Docket No. 11-49 seeking to demonstrate that its Multilateration Location and Monitoring Service (“M-LMS”) network does not cause unacceptable levels of interference to Part 15 devices in the 902-928 MHz band.

- IEEE 802 has a particular interest in the 902-928 MHz band, since the IEEE 802.11 Working Group (“802.11”) is developing an amendment to the base standard for sub-1 GHz operation of WLANs targeting this band, and the IEEE 802.15 Working Group (“802.15”) has now finalized an amendment to the IEEE 802.15.4 base standard for WPANs specifically targeted at Smart Grid applications in this band which is expected to be published in the near future. In addition, previous equipment designed to comply with IEEE 802.15.4 is widely deployed in the marketplace.

IEEE 802 believes that Progeny’s test report fails to establish that their M-LMS equipment does not interfere with part 15 devices

- IEEE 802 supports the comments of Itron and Landis+Gyr challenging the adequacy of the test plan and the conclusions drawn in the Progeny test report.

- We further believe that Progeny failed to engage the manufacturers of Part 15 equipment operating in the 902-928 MHz band in an effective dialog leading to a more effective interference test methodology, including more equipment types in the tests conducted.

CONCLUSION

- IEEE 802 encourages the Commission reject the conclusions drawn by Progeny in their test report.

Respectfully submitted,

/s/

Michael Lynch

Chair, IEEE 802.18 Radio Regulatory Technical Advisory Group 108 Brentwood Court Allen, TX 75013 972.814.4901 freqmgr@ieee.org

The IEEE Local and Metropolitan Area Networks Standards Committee (“IEEE 802” or the “LMSC”).

This document represents the views of IEEE 802. It does not necessarily represent the views of the IEEE as a whole or the IEEE Standards Association as a whole.