#### Before the

FEDERAL COMMUNICATIONS COMMISSION

Washington, D.C. 20554

| In the Matter of Amendment of the Commission‘s Rules with Regard to Commercial Operations in the 3550-3650 MHz Band. |  | GN Docket No. 12-354 |
| --- | --- | --- |

Via the ECFS

Comments Of IEEE 802

- IEEE 802 respectfully submits its Comments in the above-captioned Proceeding.

- IEEE 802, as a leading consensus-based industry standards body, produces standards for wireless networking devices, including wireless local area networks (“WLANs”), wireless personal area networks (“WPANs”), wireless metropolitan area networks (“Wireless MANs”), and wireless regional area networks (“WRANs”).  Included in our standards development activity is an emphasis on coexistence, which is the focus of our Wireless Coexistence working group. We appreciate the opportunity to provide these comments to the FCC.

introduction

- On December 12, 2012 the Commission issued a Notice of Proposed Rulemaking, under GT Docket 12-354, in which the Commission seeks comments related to spectrum sharing in the 3550-3650 MHz and the 3650-3700 MHz Band.

- IEEE 802 applauds the President’ s Council of Advisors on Science and Technology (PCAST) report promoting spectrum sharing and more efficient use of spectrum through new cognitive radio technologies, receiver standards and interference mitigation techniques.

- IEEE 802 applauds the Commission for adopting the recommendations of the PCAST report for finding novel ways to use the spectrum for commercial uses.

- IEEE 802 agrees with the FCC that the spectrum should not remain unused if there are radio technologies that can make use of the spectrum while preserving the established regulatory framework. Spectrum can be shared in frequency, time and space.

- IEEE 802 supports the FCC’ s Notice of Proposed Rule Making, to create a new Citizens Broadband Service in the 3550-3650 MHz band (3.5 GHz Band) which will promote advances that enable more efficient use of radio spectrum.

- We agree with the commission as stated in Paragraph 3 (hereinafter “Paragraph” followed by the paragraph number), that increased use of small cell network deployments can multiply wireless capacity within existing spectrum resources.

- IEEE 802 supports the establishment of a regulatory framework as suggested in Paragraph 7 permitting the opportunistic use of cognitive unlicensed and licensed devices in these Bands.  We also seek broadening the rulemaking to the lower Extended C Band from 3400 MHz to 3550 MHz, where similar Federal uses can enjoy the same protections that are described in GT-12-354.

IEEE 802 RecommendS A RIGOROUS ANALYSIS of Separations around fixed satellite service IS REQUIRED

[Peter Ecclesine proposes comments which recommend the creation of a more rigorous method for determining the exclusion zone from FSSS, and a re-examination of the size of the guard band to FSSS. Propose requiring FSSS license holders to improve the immunity of their sites from interference.]

- IEEE 802 observes that good fences make good neighbors.

- The FSSS industry has 20 years experience with a fence of moderately good conducting material around an FSS receiver site (e. g. http://www.panynj.gov/real-estate-development/teleport.html click on the ‘map’ tab) showing that receivers are protected from terrestrial interference. We propose requiring the 3700-4200 MHz band FSSS license holders to improve the immunity of their sites from interference. We note that nuclear power plant operators are required to have intrusion-proof security around their physical plants as a condition of operation.

- Our experience in the 3650-3700 MHz band indicates that if FSSS protection was by power control and OOBE and not spatial separation, the number of deployments would be at least double what it is today. The size of the FSS guard band below 3700 MHz is an issue requiring more rigorous analysis.

- .

- IEEE 802 has created multiple Standards for White Space Device (WSD) deployments in the TV Bands that can access the database service and based on parameters communicated from it, can change their radio parameters to utilize the spectrum at a given location and at a given time.

- IEEE 802 believes that technologies exist to enable real time and semi real time spectrum sharing, where such sharing can be enabled through the database (also referred to as the SAS in the NPRM)

IEEE 802 working groups have incorporated spectrum sensing mechanisms for spectrum sharing into some of their standards

- FCC in Paragraph 125 has asked if spectrum sensing techniques may be deployed for spectrum sharing and interference mitigation in these bands.

- The 802.22 Working Group on Wireless Regional Area Networks (RAN) has investigated a variety of spectrum sensing techniques such as cyclostationary approaches, higher order statistics, covariance based approaches, matched filter, energy detection etc. to detect and recognize the signals of interest in TV bands. The IEEE 802.22-2011™ standard has created a dedicated Annex that describes these techniques and provides detailed quantitative performance comparisons for various techniques using over-the-air TV Broadcast captured signals that were provided to the various participants.

- 802.22 investigated techniques that would allow detection of TV band signals, some 20 dB below the noise floor.  For this case, the use of a combination of techniques, such as energy (interference) detection, Receiver Signal Strength Indication (“RSSI”) monitoring, and more complex feature based techniques described above, were needed to detect and identify the signal.

IEEE 802 SUPPORTS the use of NOVEL INTERFERENCE MITIGATION TECHNIQUES SUCH AS SIGNAL BEACONS TO ENABLE SPECTRUM SHARING

- In Paragraph 149 and 152, FCC has asked if a beaconing technology can be applied for interference mitigation in these bands.

- IEEE recently authorized a revision project to add Advanced Beaconing capabilities to the IEEE Standard 802.22.1™-2010 to enable spectrum sharing in the 3550-3650 MHz band with existing radars and fixed satellite earth stations. This revision PAR was introduced to support the PCAST report promoting spectrum sharing and more efficient use of spectrum through new cognitive radio technologies and interference mitigation techniques.

- The NPRM proposes to use database service or a Spectrum Access System (SAS) driven operation which will enforce large exclusion zones along the US coastline to protect U.S. Navy coastal operations and other Department of Defense test and training areas. Such large exclusion zones will not allow majority of the large cities along the US coast to gain benefits from this spectrum.

- Some advanced beaconing approaches, such as the one developed in the IEEE Standard 802.22.1-2010 originally designed for interference protection of licensed wireless microphones may be used for these bands. Such a beaconing approach allows time-division spectrum sharing operation in real time.

- revenues.

CONCLUSION

- IEEE 802 applauds the President’ s Council of Advisors on Science and Technology (PCAST) report promoting spectrum sharing and more efficient use of spectrum through new cognitive radio technologies and interference mitigation techniques.

- IEEE 802 applauds the Commission for adopting the recommendations of the PCAST report to find novel ways to use the spectrum for commercial uses.

- IEEE 802 has and continues to develop technologies that will support the regulatory framework to be established in the 3550-3650 MHz bands.

Respectfully submitted,

/s/

Michael Lynch

Chair, IEEE 802.18 Radio Regulatory Technical Advisory Group 108 Brentwood Court Allen, TX 75013 972.814.4901 freqmgr@ieee.org

References

The IEEE Local and Metropolitan Area Networks Standards Committee (“IEEE 802” or the “LMSC”).

This document represents the views of IEEE 802. It does not necessarily represent the views of the IEEE as a whole or the IEEE Standards Association as a whole.