| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 5 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  | TD 74rev1 (GEN/5) |
|  |  | English only Original: English |
| Question(s): | 8/5 | Geneva, 29 January - 7 February 2013 |
| TD |  |  |
| Source: | Rapporteurs Q8/5 |  |
| Title: | 3rd revision of draft Recommendation K.mhn |  |

# Foreword

This document presents 3rd revision of draft Recommendation K.mhn.  Annex A gives revised version based on latest version of draft K.mhn(TD.1134rev1 (Geneva, 11-19, April, 2012).

Major modifications are as follows:

1) Title is modified to clarify its scope by request from SG9. New title is as follows

“Techniques to mitigate interference between radio devices and cable or equipment connected to wired broadband networks and ‘cable’ television networks”

2) Section 7.1.3.1 is revised, in order to clearly explain how to evaluate it

3) In section 7.1.5, mitigation techniques, that do not related to electromagnetic phenomena, such as software setting, error correction, etc, are added.

4) In section 7.2.1, mitigation measures are divided into two part related to cable types, i.e. balanced or unbalanced one.

5) As a result of comment from SG9,  Fig.3/K.mhn is revised. radio device checking is moved before installation check and added combined check after finishing these separated checks.

Remaining issues to be pointed out in previous meeting are as follows.

a) Definition of “television network” or “cable television network”

Definition does not find out in ITU terminology Database. Definition of “cable television” is in ITU-T Rec. J142

“Communications systems distributes broadcast and non-broadcast signals, as well as a multiplicity of satellite signals originating programming and other signals by means of coaxial cable and/or optical fibre.”

b) Definition of “radio device”

Definition does not find out in ITU terminology Database. In previous meeting, delegate pointed out that term “radio device” is not clear.

Annex A

3rd Draft Recommendation K.mhn – “Techniques to mitigate interference between radio devices and cable or equipment connected to wired broadband networks and cable television networks”

Summary

This Recommendation describes techniques to mitigate interference between radio devices used in home and cable or equipment connected to wired broadband networks and cable televisionnetworks. With the progress of the household appliances, many devices connected to home networks are widely used in home. This Recommendation aims to introduce the relevant EMC requirement to wired home network devices and feasible solutions to resolve the EMC problems caused by proximity use of the radio devices and other wired network equipment.

Keywords

Home network, Cables,  Interference, Wireless

Introduction

Along with the advances in radio communication technologies, possible interference between home-use radio devices and cable or equipment connected to wired broadband networks and cable television networks are new phenomena of interference between radio devices and telecommunication devices. Interference to cabling or devices falls under the responsibility to ITU-T whereas the interference between radio devices is under the responsibility of ITU-R. Such problems may occur in the field, especially in a home environment. This Recommendation gives guidance on how to solve these kinds of phenomena by taking measures to reduce the interference and to ensure normal operating condition of wired telecommunication equipment. Measures against radio devices are not treated in this Recommendation.

# 1	Scope

This recommendation gives guidance to identify interference problems in home networking, produced by radio devices and cable or equipment connected to wired broadband networks and cable television networks. It also presents appropriate measures to be applied to the equipment connected to wired broadband networks and TV networks for preventing the interference and procedure to address these troubles in the field. Moreover, necessary requirements for cable or equipment connected to wired broadband networks and TV networks related to emission and immunity are given by international standards, such as ITU-T Recommendations K.74, K.93, K.37, K.43, K.48, and IEC/CISPR 22.

This Recommendation presents procedures to identify the cause of interference, and mitigation measures to prevent interference.

Interference related to PLC is outside the scope of this Recommendation, because that phenomenon is already covered with Recommendation K.60. Moreover, mitigation techniques for the radio devices and interference between the radio devices are also outside the scope of this Recommendation.

# 2	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T K.37]	ITU-T Recommendation K.37 (99/02), Low and high frequency EMC mitigation techniques for telecommunication installations and systems – Basic EMC Recommendation

[ITU-T K.43]	ITU-T Recommendation K.43 (09/07), Immunity requirements for telecommunication network equipment

[ITU-T K.48]	ITU-T Recommendation K.48 (09/06), EMC requirements for telecommunication equipment – Product family Recommendation

[ITU-T K.74]	ITU-T Recommendation K.74 (09/08), EMC, resistibility and safety requirements for home network devices

[ITU-T K.93] ITU-T Recommendation K.93 (05/2012), Immunity of home network devices to

electromagnetic disturbances

[Mitigation Handbook] Mitigation measures for telecommunication installation (2008)

IEC CISPR publication 22(2008), Information technology equipment – Radio disturbance characteristics – Limits and methods of measurement.

IEC CISPR publication 24(2011), Information technology equipment – Immunity characteristics – Limits and methods of measurement.

# 3	Definitions

## 3.1	Terms defined elsewhere:

This Recommendation uses the following terms defined elsewhere:

3.1.1	home network device[K.74]: A home network device is an electronic/electric equipment whose primary function is the distribution of data within the home, between the network termination point and one or more terminal devices.

3.1.2 Shielding effectiveness [K.43]: For a given external source, the ratio of electric or magnetic field strength at a point before and after the placement of the shield in question.

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	Radio device: A device that uses radio signal to communicate with another device in home networks and does not require any physical wire for distributing and receiving data to another device.

3.2.2	Cable television networks: A cable television network is the network for distribution of television program, whereby a central operation provides programming to many television stations or pay TV providers by using coaxial cable or optical fibre

# check definition in other international standards (e.g. ITU-R stds)

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

Broadband cable: the cable connected to the wireline broadband telecommunication networks, such as: PSTN 2-line cables, LAN cables and so on.

AC 		Alternating Current

ADSL		Asymmetric Digital Subscriber Line

CMC		Common-Mode Choke coil

DC		Direct Current

DSL		Digital Subscriber Line

DVB		Digital Video Broadcasting

EM 		ElectroMagnetic

EMC		ElectroMagnetic Compatibility

EMI 		ElectroMagnetic Interference

HD		High Definition

NGN 		Next Generation Network

PC 		Personal Computer

PE 		Protective Earth

PLC		Power Line Cable

PSTN		Public Switched Telephone Network

RF 		Radio Frequency

RFI 		Radio Frequency Interface

STB 		Set Top Box

VoIP 		Voice over Internet Protocol

-To be added more terms that are used in this Recommendation

# 5	 Issues to be considered in this Recommendation

## 5.1 Background

With widespread use of a home network, many new or conventional radio devices could be used in the home and there is a possibility for new EMC problems in the fields. One concern for ITU-T is possible interference between radio/wireless devices such as IMT user equipment and cable or equipment connected to wired broadband networks and cable television networks. This may occur due to lack of proper installation, such as faulty connections, cabling, close proximity use of the devices, and so on. Therefore, part of this issue falls under EMC problems and requires necessary mitigation techniques. Key issues are to give methodologies for preventing possible interference between them and for providing appropriate measures to be applied to device connected to wired broadband networks and cable television networks.

For example, if the connection between a coaxial cable and its connecter does not have sufficient contact, electrically, then it may cause interference to wired broadcasting systems, or radio devices, vice-versa. Therefore, urgent issue is to take into consideration problems caused by wired broadband networks and cable television network installation.

## 5.2  EMC problems

Problems to be treated in this Recommendation are considered from an EMC viewpoint. Interference into radio devices falls under the responsibility in ITU-R. Possible cause of interference is installation failure, if devices and cables comply with relevant standards. The area covered with this Recommendation is given in Fig.1/K.mhn.

Fig.1/K.mhn  Area covered with this Recommendation

# 6	Relevant EMC Requirements

Devices considered in this Recommendation should comply with relevant international standards, such as K.74, CISPR 22, CISPR 24 and so on. If the devices do not comply with those standards, these shall be tested and checked.

# 7. 	Guidance to identify and address interference

## 7.1 Procedure to identify interference sources

### 7.1.1 Flow chart

The procedure to reduce interference is shown in Fig.2/K.mhn. An interference case is dependent on the surrounding conditions, i.e. how many devices in the surrounding, and how is an electromagnetic environment, how far from the antenna of radio devices and so on. Therefore, users of this Recommendation should take into account these points.

Fig.1/K.mhn Flow chart how to trouble shoots and reduce the interference case

No

### 7.1.2 Measurement methods

Fundamental measurement methods for measuring interference cases are given in the Mitigation Handbook. In case of interference between radio devices and a device and equipment connected to wired broadband telecommunication networks and TV networks, measurement equipment, such as vector spectrum analyser, digital demodulation receiver, communication analyzer, and so on, are help to understand phenomena, correctly.

Electromagnetic environment should be measured by following three steps.

1) Before measuring the electromagnetic environment, all devices that intentionally radiate electromagnetic waves should be turned off due to reducing intentional electromagnetic radiation. Then, electromagnetic environment is measured by appropriate measurement instruments.

2) After step 1 is carried out, all devices that are turned off in step 1shoud be turned on. Then all devices that are connected to wired broadband networks should be turned off, in order to reduce unintentional emission from a devices.

3) After step1 and 2 are carried out, all devices that may be related to interference should be turned on, and electromagnetic environment should be measured. After the measurement, the results obtained by above three steps shall be compared to find out the difference among them.

### 7.1.3 Evaluation of measured data

Several quantities obtained by the measurements, such as frequency, signal levels, demodulation data (eye –pattern, constellation diagram), and so on, shall be evaluated.

#### 7.1.3.1 Frequency and levels

At first, intentional radio signals in measured frequency band should be checked using standards and/or information of frequency allocation By comparing measured results with frequencies according to the radio frequency allocation table or reference publications, intentional radio signals can be identified and checked their levels to fulfill requirements. According to the results from above step 1, radiation signal from radio devices could be disappeared. However, if a signal exists at the same frequency that is used by the radio devices in spectrum, then that signal may consider as an unintentional emission from a cable or devices connected to the wired broadband networks, or an intentional radiation from the surrounding. Demodulation data of that signal could give useful information.

More measurement could be performed to get more detailed information from interference source.

#### 7.1.3.2 Demodulation data

Current radio devices are mainly using digital modulation techniques. Evaluate the demodulated data could get more information about the interference source. For example, if the signal comes from the radio device, transmission property of the device can be obtained. On the other hand, if the signal unintentionally emitted from the cable or device, then demodulation property cannot be obtained, if modulation is different between radio device and wired networks.   Therefore, it is important to check a modulation method that is used in radio device.

### 7.1.4 Finding out interference sources and part of emission or entry

Evaluating to the information obtained from measurement data and interference phenomena, an interference sources, an area and/or a location could be found out. After the source can find out, then a mechanism of the interference should be analyzed.

If the source of interference or entry point does not obtain from the information, then additional measurement should be carried out to identify them.

The following measures are helpful to distinguish the cause of trouble and find out the interference or disturbance source in the field:

1)	Using directional antenna,

2)	turning on and off the devices one by one,

3)	moving a cable or device connected to wired broadband network

4)	taking off/on a radio devices

Finding out and determining the source of interference is the most important issue to solve or reduce the interference problem.

If the interference source is found out, the interference phenomenon should be repeated to confirm the results and determine the coupling path of emission

### 7.1.5 Checking installation

After obtaining information on the electromagnetic environment in the surroundings, installation shall be checked. Points need to be checked are as follows.

1) Do devices or cables comply with relevant international or national standards??

2) Is installation of cable/or and device that are connected to broadband networks appropriate?  i.e. check the user manual of device specification of cable, or connector, etc

3) The connection point should be checked, e.g. does unstable or faulty connection occur at this point?

4) Does the cable or the shell of device have damage?

5) Is cabling/cable distribution suitable for using in home?

6) Does the radio device are working in normal operating condition?

7) Checking settings of software, protocol, error correction, such as FEC, place diversity techniques, and so on

## 7.2 Mitigation techniques to be applied to equipment connected to wired broadband networks and cable television networks

### 7.2.1 Measures to be applied to cables connected to wired broadband networks

Mitigation techniques for balanced cables are as follows

1) A shielded cable is used and connected to the earth both end or one side, if necessary;

2) A magnetic ring is attached on the cables or CMC is inserted into them;

3) A differential-mode filter is inserted into the cable11).

Note 1: Relationship between a frequency-band of transmission signal and of interference cause should be taking into consideration when  differential-mode filter is selected, in order to reduce influence of the transmission signal in the cable.

4) Distribution of the cable is changed or a length of the cable is changed.

For unbalanced cables, such as coaxial type, following techniques can be considered as mitigation measures.

5) Cables replace to double/triple shielded type;

6) A magnetic ring is attached on the cable;

7) Isolation transformer is inserted.

### 7.2.2 Measures to the devices connected to wired broadband networks

Mitigation measures for the device are as follows.

1) The devices are replaced from original to the other place;

2) An electromagnetic shield is given to the devices. Using a metal plate, a conductive paint, or a sheet included electromagnetic materials (e.g. carbon powder, ferrite powder, and so on) is the effective and easy method to increase shielding effect.

## 8 Procedure for selecting appropriate mitigation techniques

Mitigation measures to solve interference case are given in the Mitigation Handbook. Procedure to mitigate the interference is shown in Fig. 3/K.mhn.

8.1 Checking information obtained from the measurement or interference phenomena.

All information obtained from measurement and phenomena should be checked. Considerable information are as follows:

- timing of interference and malfunction status of interfered devices;

- working mode of interfered device and other electric devices;

- position of devices, layout of cables, distance among cables;

- frequency and level of measurement results;

- analysis and evaluation of measurement results.

8.2 Checking radio devices in normal operation, and installation

Before taking mitigation measures, normal operation of the radio devices should be checked. If radio device does not work normally, then users of the device will ask it to manufacturer. If it works as intended, then all installation, including cable, cabling, devices, connection between cable and cable, or cable and devices, should be checked.  If any faults, or mistaken are founded out, then service personnel or users would try to fix them.

After the checks and fixes, interference phenomena should be confirmed, again.

8.3 Adding appropriate mitigation measures

Mitigation measures are given in Mitigation Handbook. For the cable connected to broadband networks, Common-Mode Choke coil (CMC) and filters for differential-mode should be selected to reduce a effects of the interference. For the devices, shielding materials, such as metal plate, conductive paint, or sheet made of ferrite or conductive materials.

After mitigation has done, electromagnetic environment should be measured again, in order to check an effectiveness of the mitigation measures.

If the mitigation measure can not give a sufficient result, then additional measures should be taken.

Fig.3/K.mhn  Procedure to mitigate interference problem

Appendix I

Following section are given examples of the interference between radio devices and wired broadband services/TV services.

# I.1 Example 1: Interference between mobile phone and STB

| Recognizing trouble under interference and Obtaining information about the interference |  |
| --- | --- |
| Interfered device | Digital Video Broadcasting (DVB) Set Top Box (STB) device |
| Phenomena and timing | When the mobile phone made a phone call, the decoding of HDTV program may become not stable (blue screen for several seconds). When the phone call is end, the HDTV program is recovered automatically. The reason that it happened is close proximity use of mobile phone to STB. |
| Checking other possibilities | There is no hardware or software failure |
| Checking installation |  |
| Earthing and bonding | OK |
| Cabling | OK, and the cabling connected and around STB devices are checked and unified into parallel layout. |
| Checking EMC standard complied with |  |
| Both mobile phone and STB device are marked with certifications which are showing to fulfill the national and international EMC standards and radio standard. |  |
| Measuring the EM environment: When the mobile phone is made a phone call, there is transmission signal measured on 900MHz band or 1800MHz band, which is determined by the base station. |  |
| Evaluating results and finding the cause of trouble: The DVB STB was malfunction when the transmission signal was on 900MHz band. |  |
| Mitigation measures: Change the position of DVB STB device to the other corner of desk, put the mobile phone away from the DVB STB (at least 0.5 meters), and use the mobile phone away from the DVB STB, the distance is at least 0.5 to 1 meter and as far as possible. |  |

Bibliography

[b-ITU-T X.yyy]	ITU-T Recommendation X.yyy (date), Title

_______________