IEEE P802.18
Radio Regulatory Technical Advisory Group (RR-TAG)

| Proposed response to the Draft European Commission RSPG opinion on the development of 6G |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2023-08-04 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Dorothy Stanley | HP Enterprise Aruba |  | +1 630-363-1389 | dorothy.stanley@hpe.com |

This document contains the draft of a proposed IEEE 802 LMSC response to the European Commission Radio Spectrum Policy Group (RSPG) draft opinion on “The development of 6G and possible implications for spectrum needs and guidance on the rollout of future wireless broadband networks,” see https://radio-spectrum-policy-group.ec.europa.eu/system/files/2023-06/RSPG23-026final-draft_RSPG_Opinion_on_6G_development_with_Annexes.pdf

Electronic filing								         August 25, 2023

Re:  RSPG23-026: Draft opinion “The development of 6G and possible implications for spectrum needs and guidance on the rollout of future wireless broadband networks”

Dear Radio Spectrum Policy Group,

IEEE 802 LAN/MAN Standards Committee (LMSC) thanks the Radio Spectrum Policy Group (RSPG) for issuing the questionnaire on the role of radio spectrum policy to help combat climate change and for the opportunity to provide feedback on this topic.

IEEE 802 LMSC is a leading consensus-based standards developing organization, producing standards for wireless networking devices, including wireless local area networks (“WLANs”), wireless specialty networks (“WSNs”), wireless metropolitan area networks (“Wireless MANs”), and wireless regional area networks (“WRANs”).  We also produce both standards for wired Ethernet networks and standards for Time Sensitive Networks (“TSNs”), and technologies produced by implementers of our standards are critical for all networked applications today.

IEEE 802 LMSC is a committee of the IEEE Standards Association and Technical Activities, two of the Major Organizational Units of the Institute of Electrical and Electronics Engineers (IEEE). IEEE has about 400,000 members in over 160 countries.  IEEE’s core purpose is to foster technological innovation and excellence for the benefit of humanity.  In submitting this document, IEEE 802 LMSC acknowledges and respects that other components of IEEE Organizational Units may have perspectives that differ from, or compete with, those of IEEE 802 LMSC.  Therefore, this submission should not be construed as representing the views of IEEE as a whole.

IEEE 802 LMSC supports the RSPG’s effort in developing 6G spectrum roadmap and encourages the RSPG to consider license-exempt technologies, especially IEEE 802 technologies, as playing an important role in providing energy efficient wireless broadband connectivity to EU citizens. Please find below the responses of IEEE 802 LMSC on the draft opinion.

Response

Regarding the Draft opinion, the IEEE 802 LMSC sees some misconceptions in the underlying assumptions reflected in the report that we would like highlight and address. In doing so we would like to provide insights into the projected evolution of 802 wireless technologies as relevant for the 6G timeframe. Lastly, we would urge the RSPG to continue to evolve and emphasize the concept of technology neutrality in its positions.

Regarding the misconceptions in the underlying assumptions, our response focuses on the following three misconceptions:

- Technologies designed for license exempt usage (Like IEEE802.11/Wi-Fi) are less suitable for Enterprise use,

- License exempt technologies cannot deliver predictable QoS, and

- Spectrum needs for license exempt usage are already covered for the 6G timeframe.

Misconception: License exempt wireless technologies are less suited for Enterprise use

Chapter 5, section 5.1 states: “Use of unlicensed spectrum is mainly targeted towards consumers use and non-critical systems and is less suited for enterprise customers”.

This comment points to a fundamental misunderstanding on the European and global role of Wireless Technologies like IEEE 802.11/Wi-Fi and their critical role in enterprises today and in the foreseeable future, with further enhancements projected to roll out in the market between now and the year 2030.The enterprise WLAN equipment is a growing market and is a multi-billion Euro market. IDC publicly reported a Quarterly revenue market size of $2.8 BN (EUR 2.6 BN) for Q1 2023 and an annual market size of around $9.5 BN (EUR 8.7 BN) for calendar year 2022.

It is currently not uncommon that for new or refurbished carpeted office environments, Wi-Fi networking has fully replaced wireline Ethernet networks for device connectivity. In those environments all enterprise applications (voice/video conferencing, office productivity applications, access to server/cloud-based systems etc.) run over the Wi-Fi network. One European construction market segment customer for example has deployed a Software Defined-Branch solution including Wi-Fi for offices and for large and medium construction sites across EMEA. Currently approximately 1,400 APs are in use on approximately 200 sites, providing connectivity for office areas as well as for construction sites. Wi-Fi is critical for their business operation, and for the needs of the digitized construction site.

Below are a few publicly available examples where license exempt technologies are being used to support critical services in enterprise settings:

- Universities:

  - Cyprus University of Technology (https://www.cisco.com/c/dam/en/us/services/collateral/se/cyprus-case-study.pdf)

  - University of Michigan in the US (https://wifinowglobal.com/news-and-blog/university-of-michigan-deploys-colossal-Wi-Fi-6e-campus-network/,

  - Heilongjiang International University, China  (https://e.huawei.com/en/case-studies/enterprise-networking/2022/heilongjiang-international-university)

- Stadiums:

  - Ghelamco Stadium, Belgium (https://www.commscope.com/resources/case-studies/ghelamco-stadium/)

  - Chase Stadium in San Francisco, CA, (https://www.hpe.com/psnow/doc/a00115219enw)

- Container Shipping Terminals:

  - La Spezia Container Termina, Italy (https://www.cisco.com/c/en/us/products/wireless/ultra-reliable-wireless-backhaul/index.html#~customer-stories0

  - Port of Thessaloniki, Greece (https://www.cisco.com/c/en/us/about/case-studies-customer-success-stories/thessaloniki-port-authority.html?dtid=odicdc000509)

  - Malta Freeport Terminal (https://www.cisco.com/c/en/us/about/case-studies-customer-success-stories/malta-freeport-terminals.html)

- Public Transportation:

  - Cable Car Operation Cortina d’Ampezzo) (https://www.cisco.com/c/en/us/solutions/collateral/enterprise-networks/faloria-case-study.html0

- Manufacturing:

  - Mettis Aerospace, UK (https://wballiance.com/wireless-broadband-alliance-members-successfully-complete-first-phase-Wi-Fi-6-industry-4-0-trials-with-mettis-aerospace/)

  - AWL, Netherlands (https://www.arubanetworks.com/assets/cs/CS_AWL-Techniek_UK.pdf)

Another manufacturing applications example is data communication in automated logistics warehouses based on Wi-Fi using dozens to hundreds of APs per installation. One European customer has 1,700 Wi-Fi network systems in operation globally, and some of these systems are serving an area the size of 10+ football fields. Their customers are in the food retail, healthcare, fashion, industry, wholesale sector. Wi-Fi is mission-critical, with many operating on a 24x7 basis, thus dictating high availability and quality of service.

A European senior care provider (healthcare application) has a WLAN installation with approximately 8.000 AP in 30 locations, primarily used today for care and documentation devices and Voice over Wi-Fi. Applications are being added to the existing network to support patient tracking, alarming, electronic door locks and other use cases, all to be deployed over a single Wi-Fi infrastructure. Wi-Fi is mission-critical and requires high performance, scalability, and quality of service.

Globally and across Europe, in university, stadium, transportation, healthcare and manufacturing use cases, Wi-Fi networks using license-exempt spectrum are being used in “enterprise” applications to support and deliver mission-critical networks. Examples abound in which the deployed Wi-Fi network, supporting in many cases thousands of Access Points, is mission-critical and provides a high performance, scalable network with quality-of-service support.

Misconception ‘Licence exempt technologies cannot deliver predictable QoS’

Item 6 of the Opinion states: “Recognises, as for 5G, the role and need of license exempt or light-licensed spectrum for offloading some of the 6G traffic and to provide private and personal networks. This spectrum supports improved end-user connectivity, machine-to-machine and other applications, which do not require a predictable quality of service.”

In Section 6.3 ‘Role of license exempt spectrum use in 5G and 6G and spectrum issues’ it is stated: 
License exempt spectrum can be used to complement mobile networks in individually licensed spectrum, for example: 
• to offload indoor traffic through WLAN 
• in ad hoc cases of massive events (e.g. concerts, festivals, sports events) 
• to provide end-user connectivity, fulfilling non-critical industry needs (i.e. M2M, private,).

Use of license exempt spectrum for wireless broadband services does not guarantee a certain QoS provided to the customer due to its susceptibility to interference. Therefore, license-exempt 5G and 6G spectrum can be a solution for applications that do not need their own spectrum and for which individually licensed spectrum would be too expensive.”

In its writing, the underlying assumption for the RSPG appears to be that only through the use of licensed spectrum can predictable QoS be delivered and mission critical applications supported.

IEEE 802.11 standard based Wi-Fi products are highly suited for low power indoor networks that support QoS applications. With today’s technology, for a given scenario, implementations of the IEEE 802.11 standard achieve multi-Gbps throughout, sub-10 ms latency and packet losses lower than 0.1%.

For the Wi-Fi 6 generation of products, IEEE 802.11ax introduced a Trigger mechanism where the Wi-Fi AP can schedule uplink transmissions of a client, enabling predictability of access. Moreover, if a Wi-Fi 6 AP knows the QoS (throughput, latency) requirement of a client, the AP can schedule the clients accordingly. IEEE 802.11be (Wi-Fi 7), further enhances the ability to control the medium (e.g., through restricted target wake time service periods – rTWT) advertised in Beacon frames. These scheduling mechanisms introduced in IEEE 802.11ax and IEEE 802.11be work well to deliver predictable QoS in environments where the spectrum environment is controlled by a network manager, e.g., in industrial and manufacturing sites.

In November 2023, the IEEE 802.11 working group will begin work on the major Next Generation MAC/PHY standard (IEEE802.11bn), which is projected to be the basis for the Wi-Fi 8 generation of technologies, with a projected market entry in the 2028 timeframe. A key theme for IEEE 802.11bn is driving further improvements to the reliability (as measured in throughput, latency, and packet loss) of Wi-Fi networking. The frequency range covered by the new standard will be 1 – 7.25 GHz. In parallel, the IEEE 802.1 Working Group has completed and continues to develop TSN standards, components of which are applicable to wired and wireless networks. Work underway in the Internet Engineering Taks Force (IETF) in Deterministic Networking (including Reliable and Available Wireless (RAW) further supports Quality of Service driven wireless applications and networks.

As described, there is no inherent disadvantage for 802.11 standards-based technology for the delivery of predictable QoS or for support of mission critical systems. In cases where highly stringent QoS requirements need to be met (e.g., manufacturing robotics), it may come down to the amount of control that the network owner has over their wireless spectrum (either license exempt spectrum or lightly licensed spectrum).

Network evolution in the 6G timeframe

The IEEE 802 LMSC would like to point out that the most likely scenario in the 6G timeframe will be that the vast majority of QoS sensitive, mission critical wireless broadband data communication will be delivered through Fiber access coupled with lower power indoor wireless connectivity. The powerful combination of the multiple Gbps access speeds, low latency and ‘end to end’ power consumption benefits of Fiber connectivity plus the highly dense deployment scenario of low power indoor wireless (e.g., one network per room) should be a key public policy consideration when planning for the 6G future.

As is the case for 5G, 6G standards will likely include modes for indoor low power operation, and architectures of low power 6G small cells, connected to fiber access for in-building connectivity appear likely.  However, the predominant business model for licensed spectrum has long been based on macro cell deployments, which will still likely be the case in the 6G era, given the underlying Mobile Network Operator business economics.

Misconception: Spectrum needs for license exempt usage are already covered for the 6G timeframe

Section 6.3 of the DRAF RSPG opinion further states: “License exempt spectrum could be used as the last few meters link for the FTTH network at homes or even from mobile smartphones devices to other devices, but will always be relying on the fixed or mobile networks to provide end to end connectivity to the users. Due to the license exempt spectrum usage, limited coverage, limited power to provide the last few meters link, the spectrum needs for license exempt are covered already with the identified spectrum in Europe”.

The DRAFT RSPG does not provide any evidence upon which this assertion is made. Under the scenario whereby the vast majority of high throughput and application usage will be delivered through the combination of fiber and low power indoor wireless connectivity in the 6G timeframe, the available indoor wireless spectrum will likely become the constraining factor in providing the required level of service to European consumers and businesses alike.

A study by Intel (https://www.intel.com/content/dam/www/central-libraries/us/en/documents/spectrum-needs-Wi-Fi-7-whitepaper.pdf), concluded that at a minimum of 3 non-overlapping 320 MHz channels will be required to effectively support Wi-Fi 7 for emerging delay-sensitive residential, enterprise, and industrial applications  such as AR/VR and industrial IoT, and in dense deployment scenarios such as stadiums.

In addition, in the 6G timeframe fiber access speeds of higher than 5Gbps may become commonplace and at that point the capacity of low power indoor wireless connectivity networks in dense deployment environments (residential, enterprise, public spaces) will become the bottle neck in the overall end to end infrastructure, to a large extent due to an insufficient amount of license exempt spectrum being available for dense deployment scenarios.

Importance of Technology Neutrality

As described in this response the IEEE 802 LMSC points out that a highly likely scenario in the 6G timeframe will be that the vast majority of QoS sensitive, mission critical wireless broadband data communication will be delivered through the combination of fiber access coupled with lower power indoor wireless connectivity.  In addition, robust and high capacity outdoor 6G networks will be required most likely predominantly delivered by macro cell deployments of cellular networks, supporting critical mobility and outdoor usage scenarios. The 802 LMSC encourages the RSPG to fully embrace the concept of technology neutrality in its considerations going forward and continue to enable consumers and network owners to choose the mix of technologies that are best aligned with their needs on a ‘fit for purpose’ basis.

Conclusion

IEEE 802 LMSC thanks the RSPG for the opportunity to provide this submission and kindly requests the RSPG to take into account our responses in its decision towards the role of radio spectrum policy.

Respectfully submitted,

By: /ss/.

Paul Nikolich

IEEE 802 LAN/MAN Standards Committee Chairman

em: p.nikolich@ieee.org

References:

This document solely represents the views of IEEE 802 LMSC and does not necessarily represent a position of either the IEEE or the IEEE Standards Association.

Source https://www.idc.com/getdoc.jsp?containerId=prUS50864423#:~:text=NEEDHAM%2C%20Mass.%2C%20June%2013,Worldwide%20Quarterly%20Wireless%20LAN%20Tracker accessed 2023-08-04.