| Radiocommunication Study Groups |  |
| --- | --- |
| Received: Subject: Proposed modification to M.1450-5 | Document 5A/xx |
|  | 14 – 24 May 2024 |
|  | English only |
| Institute of Electrical and Electronics Engineers |  |
| Proposed modification to Annex 6 to Document 5A/ 837-E |  |

Source Information

IEEE 802 LAN/MAN Standards Committee (LMSC) respectfully submits this submission to ITU-R Working Party 5A (WP 5A). IEEE 802 is a committee of the IEEE Standards Association and Technical Activities, two of the Major Organizational Units of the Institute of Electrical and Electronics Engineers (IEEE). IEEE has about 400,000 members in over 160 countries. IEEE's core purpose is to foster technological innovation and excellence for the benefit of humanity. In submitting this document, IEEE 802 acknowledges and respects that other components of IEEE Organizational Units may have perspectives that differ from, or compete with, those of IEEE 802. Therefore, this submission should not be construed as representing the views of IEEE as a whole.

Introduction

This document proposes updates to the Annex 6 to Doc.5A/837 - Working document towards a preliminary draft revision of Recommendation ITU-R M.1450-5 - Characteristics of broadband radio local area networks. The proposed changes and embedded comments are indicated via the ‘track changes’ and highlighted in Turquoise Blue Color.

We applaud the efforts of the participants in WP 5A for undertaking this work and giving IEEE 802 the opportunity to contribute.

Discussion

Since the last revision of ITU-R M.1450-5 (2014), there have been a number of updates and comments to IEEE 802 standards that we would like to share with WP 5A for consideration.

Proposal

Incorporate the proposed updates below into ITU-R M.1450.

IEEE 802 proposes that the revision of Recommendation ITU-R M.1450-5 be completed during the May 2024 meeting of WP 5A and submitted to SG 5.

Contact:	LYNCH Michael			E-mail:	freqmgr@ieee.org

Incl.: Annex 1

| Radiocommunication Study Groups |  |
| --- | --- |
| Source: Document 5A/TEMP/340(Rev.1) | Annex 6 to Document 5A/837-E |
|  | 28 September 2023 |
|  | English only |
| Annex 6 to Working Party 5A Chair’s Report |  |
| PRELIMINARY DRAFT REVISION OF RECOMMENDATION ITU-R M.1450-5 |  |
| Characteristics of broadband radio local area networks |  |

(Questions ITU-R 212/5 and ITU-R 238/5)

(2000-2002-2003-2008-2010-2014)

Summary of the revision

This revision includes updated characteristics of broadband radio local area networks (RLANs).  Technical requirements applicable in certain administrations and/or regions were updated based on inputs from administrations. Abbreviations/Glossary (formerly Table 1) were inserted and Note 1 referring to “Table 1” was deleted to conform with the mandatory format of ITU-R Recommendations.

Scope

This Recommendation provides the characteristics of broadband radio local area networks (RLANs) including technical parameters, and information on RLAN standards and some operational characteristics. Basic characteristics of broadband RLANs and general information for their system design are also provided.

Abbreviations/Glossary

Access method	Scheme used to provide multiple access to a channel

AP	Access point

ARIB	Association of Radio Industries and Businesses

ATM	Asynchronous transfer mode

Bit rate	The rate of transfer of a bit of information from one network device to another

BPSK	Binary phase-shift keying

BRAN	Broadband Radio Access Networks (A technical committee of ETSI)

Channelization	Bandwidth of each channel and number of channels that can be contained in the RF bandwidth allocation

Channel Indexing	The frequency difference between adjacent channel centre frequencies

CSMA/CA	Carrier sensing multiple access with collision avoidance

DAA	Detect and avoid

DFS	Dynamic frequency selection

DFT	Discrete Fourier Transform

DFT-S OFDM	DFT-spread OFDM

DSSS	Direct sequence spread spectrum

e.i.r.p.	Equivalent isotropically radiated power

ENG	Electronic News Gathering

ETSI	European Telecommunications Standards Institute

Frequency band	Nominal operating spectrum of operation

FHSS	Frequency hopping spread spectrum

HIPERLAN2	High performance radio LAN 2

HiSWANa	High speed wireless access network – type a

HSWA	High speed wireless access

IEEE 	Institute of Electrical and Electronics Engineers

IETF	Internet Engineering Task Force

LAN	Local area network

LPI	Low power indoor

LBT	Listen before talk

MU	Medium utilisation

MMAC	Multimedia mobile access communication

Modulation	The method used to put information onto an RF carrier

MIMO	Multiple input multiple output

OFDM	Orthogonal frequency division multiplexing

OFDMA	Orthogonal frequency division multiple access

PSD	Power spectral density

PSTN 	Public switched telephone network

QAM 	Quadrature amplitude modulation

QoS	Quality of Service

QPSK	Quaternary phase-shift keying

RF	Radio frequency

RLAN	Radio local area network

RU	Resource unit

SSMA	Spread spectrum multiple access

Tx power	Transmitter power – RF power in Watts produced by the transmitter

TCP	Transmission control protocol

TDD 	Time division duplex

TDMA 	Time-division multiple access

TPC 	Transmit power control

TTA	Telecommunications Technology Association (A standards developing organization in the Republic of Korea)

VLP	Very low power

WATM	Wireless asynchronous transfer mode

The ITU Radiocommunication Assembly,

considering

a)	that broadband radio local area networks (RLANs) are widely used for fixed, semi-fixed (transportable) and portable computer equipment for a variety of broadband applications;

b)	that broadband RLANs are used for fixed, nomadic and mobile wireless access applications;

c)	that broadband RLAN standards currently being developed are compatible with current wired LAN standards;

d)	that it is desirable to establish guidelines for broadband RLANs in various frequency bands;

e)	that broadband RLANs should be implemented with careful consideration to compatibility with other radio applications,

recognizing

a)	that the operation of systems that implement the standards in this Recommendation must comply with the applicable provisions of the Radio Regulations in all cases;

[COMMENT: Annex 5 is not necessary. It quotes some elements from RR No. 5.150 and Resolution 229 which address the 2.4 GHz and 5 GHz bands. However, these bands have been covered by the current revision of Recommendation ITU-R M.1450 since 2014 without the need for such quotations. Also, other parts of the RR may be relevant and are not quoted in Annex 5, making this an incomplete list. The general need for all applications covered by Recommendation ITU-R M.1450 are clearly stated in the main body of the Recommendation and there is no need to try and reproduce certain elements in Annex 5.]

b)	that provision 0.4 of the Preamble of Radio Regulations provides that all stations, whatever their purpose, must be established and operated in such a manner as not to cause harmful interference to the radio services or communications of other Members or of recognized operating agencies, or of other duly authorized operating agencies which carry on a radio service, and which operate in accordance with the provisions of these Regulations (No. 197 of the Constitution);

c)	that the use of the frequency bands 5 150-5 250 MHz, 5 250-5 350 MHz and 5 470-5 725 MHz by RLAN’s is covered in Resolution 229 (Rev.WRC-19);

d)	that the frequency band 6 425-7 125 MHz is also used for the implementation of wireless access systems (WAS) including radio local area networks (RLANs) as stated in RR Nos. 5.6A12 and 5.6C12.
[Editor’s Note: The above RR footnote numbers should be updated once the Final Acts of WRC-23 are published (expected before the May meeting of WP5A).]

e)	that other characteristics of broadband WAS, including RLANs, is contained in Recommendations ITU-R F.1763, ITU-R M.1652, ITU-R M.1739 and ITU-R M.1801

noting

[COMMENT: Support deleting this reference to Report ITU-R F.2086]

a)	that Annex 2 provides information on how to obtain the standards described in Table 1 of Annex 1;

b)	that Annex 3 provides basic characteristics of broadband RLANs and general characteristics for deployment and system design;

[COMMENT: No need to call out Table 2 of Annex 3 specifically. The previous noting already refers to Annex 3 in its entirety.]

d)	that Table 3 in Annex 3 contains general technical requirements applicable in certain administrations and/or regions;

[COMMENT: These sharing parameters are not needed in Recommendation ITU-R M.1450. The specific parameters had been suggested for use in EESS studies in WP7C during the previous study cycle to assess whether EESS should be considered in the 4.2-4.4 GHz and 8.4-8.5 GHz bands, but they were never agreed in WP5A In the meantime, WRC-23 has agreed a WRC-27 agenda item (1.19) to consider EESS in the 4.2-4.4 GHz and 8.4-8.5 GHz bands and studies between EESS and RLANs in the 6 GHz band are no longer required. Therefore, instead of trying to modify the content of Annex 4 to make it acceptable to WP5A, this draft new annex can just be removed as no longer needed.]

f)	that other information on broadband WAS, including RLANs, is contained in Recommendations ITU-R F.1763, ITU-R M.1652, ITU-R M.1739 and ITU-R M.1801,

recommends

1	that for guidance on the characteristic of broadband RLAN systems standards, the broadband RLAN standards in Annex 1 should be used by administrations wishing to implement broadband RLANs based on their assessment of coexistence with incumbent service.

[COMMENT: Edits to the first sentence of this recommends are OK but the new second sentence is not appropriate. This is stating the obvious (see also recognizing b) above) and is not specifically related to the RLAN standards contained in this Recommendation but rather related to the licensing and/or authorizing of spectrum by administrations at national level, which is outside the scope of the ITU. Also, this Recommendation has existed since the year 2000 without the need for such an extra sentence in the recommends, and there is no need to add this now.]

[COMMENT: As for noting e) above, these sharing parameters are not needed in Recommendation ITU-R M.1450. The specific parameters had been suggested for use in EESS studies in WP7C during the previous study cycle to assess whether EESS should be considered in the 4.2-4.4 GHz and 8.4-8.5 GHz bands, but they were never agreed in WP5A In the meantime, WRC-23 has agreed a WRC-27 agenda item (1.19) to consider EESS in the 4.2-4.4 GHz and 8.4-8.5 GHz bands and studies between EESS and RLANs in the 6 GHz band are no longer required. Therefore, instead of trying to modify the content of Annex 4 to make it acceptable to WP5A, this draft new annex can just be removed as no longer needed.]

Annex 1

Broadband RLAN standards

TABLE 1-1

Characteristics including technical parameters associated with broadband RLAN standards: IEEE and ATIS

| Characteristics | IEEE Std 802.11-2020 (Clause 16, commonly known as 802.11b) | IEEE Std 802.11-2020 (Clause 17, commonly known as 802.11a(1)) | IEEE Std 802.11-2020 (Clause 18, commonly known as 802.11g(1)) | IEEE Std 802.11-2020 (Clause 17, Annex D and Annex E, commonly known as 802.11j) | IEEE Std 802.11-2020 (Clause 19, commonly known as 802.11n) | IEEE Std 802.11-2020- (Clause 20, commonly known as 802.11ad) | IEEE Std 802.11-2020 (Clause 21, commonly known as 802.11ac) | IEEE Std 802.11-2020 (Clause 23, commonly known as 802.11ah) | IEEE Std 802.11ax-2021 | IEEE Std 802.11ay-2021 | ATIS RLAN (*) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Access method | CSMA/ CA, SSMA | CSMA/CA |  |  |  | Scheduled, CSMA/CA | CSMA/CA | CSMA/CA | CSMA/CA, Trigger-based access and OFDMA | Scheduled, CSMA/CA | Scheduled, CSMA, TDMA/TDD |
| Modulation | CCK (8 complex chip spreading) | 64-QAM-OFDM 16-QAM-OFDM QPSK-OFDM BPSK-OFDM 52 subcarriers (see Fig. 1) | DSSS/CCK OFDM PBCC DSSS-OFDM | 64-QAM-OFDM 16-QAM-OFDM QPSK-OFDM BPSK-OFDM 52 subcarriers (see Fig. 1) | 64-QAM-OFDM 16-QAM-OFDM QPSK-OFDM BPSK-OFDM 56 subcarriers in 20 MHz 114 subcarriers in 40 MHz MIMO, 1-4 spatial streams | Single Carrier: DPSK, π/2-BPSK, π/2-QPSK, π/2-16QAM OFDM: 64-QAM, 16-QAM, QPSK, SQPSK 352 subcarriers | 256-QAM-OFDM 64-QAM-OFDM 16-QAM-OFDM QPSK-OFDM BPSK-OFDM 56 subcarriers in 20 MHz 114 subcarriers in 40 MHz 242 subcarriers in 80 MHz 484 subcarriers in 160 MHz and 80+80 MHz MIMO, 1-8 spatial streams | 256-QAM-OFDM 64-QAM-OFDM 16-QAM-OFDM QPSK-OFDM BPSK-OFDM 26 subcarriers in 1 MHz 56 subcarriers in 2 MHz 114 subcarriers in 4 MHz 242 subcarriers in 8 MHz 484 subcarriers in 16 MHz MIMO, 1-4 spatial streams | 1024-QAM 256-QAM-OFDM 64-QAM-OFDM 16-QAM-OFDM QPSK-OFDM BPSK-OFDM Non-OFDMA: 242 subcarriers/frequency segment in 20 MHz 484 subcarriers/frequency segment in 40 MHz 996 subcarriers/frequency segment in 80 and 80+80 MHz 1992 subcarriers/frequency segment in 160 MHz OFDMA RU Size: 26, 52, 106, 242, 484, 996, 1992 subcarriers/RU MIMO, 1-8 spatial streams | Single Carrier: DPSK, π/2-BPSK, π/2-QPSK, π/2-8-PSK, π/2-16QAM, π/2-64-QAM, π/2-64-NUC OFDM: DCM BPSK, DCM QPSK, 16-QAM, 64-QAM 355 subcarrriers in 2.16 GHz 773 subcarrriers in 4.32 GHz 1193 subcarrriers in 6.48 GHz 1611 subcarrriers in 8.64 GHz | OFDM: 256-QAM, 64-QAM, 16-QAM, QPSK MIMO 1-4 spatial streams DFT-S-OFDM: 256-QAM, 64-QAM, 16-QAM, QPSK, π/2-BPSK |

TABLE 1-1 (continued)

| Characteristics | IEEE Std 802.11-2020 (Clause 16, commonly known as 802.11b) | IEEE Std 802.11-2020 (Clause 17, commonly known as 802.11a(1)) | IEEE Std 802.11-2020 (Clause 18, commonly known as 802.11g(1)) | IEEE Std 802.11-2020 (Clause 17, Annex D and Annex E, commonly known as 802.11j) | IEEE Std 802.11-2020 (Clause 19, commonly known as 802.11n) | IEEE Std 802.11-2020 (Clause 20, commonly known as 802.11ad) | IEEE Std 802.11-2020 (Clause 21, commonly known as 802.11ac) | IEEE Std 802.11-2020 (Clause 23, commonly known as 802.11ah) | IEEE Std 802.11ax-2021 | IEEE Std 802.11ay-2021 | ATIS RLAN (*) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data rate | 1, 2, 5.5 and 11 Mbit/s | 6, 9, 12, 18, 24, 36, 48 and 54 Mbit/s | 1, 2, 5.5, 6, 9, 11, 12, 18, 22, 24, 33, 36, 48 and 54 Mbit/s | 3, 4.5, 6, 9, 12, 18, 24 and 27 Mbit/s for 10 MHz channel spacing 6, 9, 12, 18, 24, 36, 48 and 54 Mbit/s for 20 MHz channel spacing | From 6.5 to 288.9 Mbit/s for 20 MHz channel spacing From 6 to 600 Mbit/s for 40 MHz channel spacing | From 693.00 to 6756.75 Mbit/s | From 6.5 to 693.3 Mbit/s for 20 MHz channel spacing From 13.5 to 1 600 Mbit/s for 40 MHz channel spacing From 29.3 to 3 466.7 Mbit/s for 80 MHz channel spacing From 58.5 to 6 933.3 Mbit/s for 160 MHz and 80+80 MHz channel spacing | From 0.300 to 17.7778 Mbit/s for 1 MHz channel spacing From 0.650 to 34.6667 Mbit/s for 2 MHz channel spacing From 1.350 to 80.000 Mbit/s for 4 MHz channel spacing From 2.925 to 173.3333 Mbit/s for 8 MHz channel spacing From 5.850 to 346.6667 Mbit/s for 16 MHz channel spacing | From 0.4 to 117.6 Mbit/s for 26-tone RU From 0.8 to 235.3 Mbit/s for 52-tone RU From 1.6 to 500.0 Mbit/s for 106-tone RU From 3.6 to 1 147.1 Mbit/s for 242-tone RU and 20 MHz non-OFDMA channel spacing From 7.3 to 2 294.1 Mbit/s for 484-tone RU and non-OFDMA 40 MHz channel spacing From 15.3 to 4 803.9 Mbit/s for 996-tone RU and non-OFDMA 80 MHz channel spacing From 30.6 to 9 607.8 Mbit/s for 2×996-tone RU and 160 MHz and 80+80 MHz channel spacing | From 630.00 to 8 316.00 Mbit/s for 2.16 GHz From 1 376.25 to 18 166.50 Mbit/s for 3.32 GHz From 2 126.25 to 28 066.50 Mbit/s for 6.48 GHz From 2 872.50 to 37 917.00 Mbit/s for 8.64 GHz | Up to 453 Mbit/s for 20 MHz channel Up to 907 Mbit/s for 40 MHz channel Up to 1 386 Mbit/s for 60 MHz channel Up to 1 857 Mbit/s for 80 MHz channel |

TABLE 1-1 (continued)

| Characteristics | IEEE Std 802.11-2020 (Clause 16, commonly known as 802.11b) | IEEE Std 802.11-2020 (Clause 17, commonly known as 802.11a(1)) | IEEE Std 802.11-2020 (Clause 18, commonly known as 802.11g(1)) | IEEE Std 802.11-2020 (Clause 17, Annex D and Annex E, commonly known as 802.11j) | IEEE Std 802.11-2020 (Clause 19, commonly known as 802.11n) | IEEE Std 802.11-2020- (Clause 20, commonly known as 802.11ad) | IEEE Std 802.11-2020 (Clause 21, commonly known as 802.11ac) |  | IEEE Std 802.11-2020 (Clause 23, commonly known as 802.11ah) | IEEE Std 802.11ax-2021 |  | IEEE Std 802.11ay-2021 | ATIS RLAN (*) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [COMMENT: The frequency bands supported by the standards should be included explicitly (in the row below), not just as broad ranges, as has been the case ever since this Rec. was first created.] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Frequency band [COMMENT: The specific bands should be kept here instead of just a broad frequency range in the row above. This has been the case since the Rec. was first created. Could add some text after “frequency band” here like “supported by the standards …” as we had it in the Frequency Range row above.] | 2 400-2 483.5 MHz | 5 150-5 250 MHz(4) 5 250-5 350 MHz 5 470-5 725 MHz 5 725-5 825 MHz | 2 400-2 483.5 MHz |  | 4 940- 4 990 MHz(2) 5 030-5 091 MHz(2) 5 150-5 250 MHz(4) 5 250-5 350 MHz 5 470-5 725 MHz 5 725-5 825 MHz |  | 2 400-2 483.5 MHz 5 150-5 250 MHz(4) 5 250-5 350 MHz 5 470-5 725 MHz 5 725-5 825 MHz |  | 57-71 GHz |  | 5 150-5 250 MHz(4) 5 250-5 350 MHz 5 470-5 725 MHz 5 725-5 825 MHz |  |  | 755-787 MHz 779-787 MHz 863-868.6 MHz 902-928 MHz 916.5-927.5 MHz 917.5-923.5 MHz |  | 2 400-2 483.5 MHz 5 150-5 250 MHz 5 250-5 350 MHz(3) 5 470-5 725 MHz(3) 5 725-5 825 MHz 5 825-5 850 MHz 5 850-5 895 MHz 5925-7125 MHz 5945–6425 MHz |  |  | 57-71 GHz |  | 5 150-5 925 MHz |  |
| Channel indexing | 5 MHz |  |  |  | 5 MHz in 2.4 GHz 20 MHz in 5 GHz | 2 160 MHz | 20 MHz |  | 1 MHz | 20 MHz |  | 2 160 MHz | 20 MHz |
| Spectrum mask | 802.11b mask (Fig. 4) | OFDM mask (Fig. 1) |  |  | OFDM mask (Figs. 2A, 2B for 20 MHz and Figs. 3A, 3B for 40 MHz) | 802.11ad mask (Fig. 5) | OFDM mask (Fig. 2b for 20 MHz, Fig. 3b for 40 MHz, Fig. 3c for 80 MHz, Fig. 3d for 160 MHz, and Fig. 3e for 80+80 MHz) |  | 802.11ah mask (Fig. 6a for 1 MHz, Fig. 6b for 2 MHz, Figure 6c for 4 MHz, Fig. 6d for 8 MHz and Fig. 6e for 16 MHz) | Spectrum Mask (Fig. 7a for 20 MHz, Fig. 7b for 40 MHz, Fig. 7c for 80 MHz, Fig. 7d for 160 MHz and Fig. 7e for 80+80 MHz) |  | 802.11ay mask (Fig .8a for 2.16 GHz, Fig. 8b for 4.32 GHz, Fig. 8c for 6.48 GHz, Fig. 8d for 8.64 GHz and Fig. 8e for 2.16+2.16 GHz) Fig. 8f for 4.32+4.32 GHz) | Fig. 9A for 20 MHz, Fig. 9B for 40 MHz, Fig. 9C for 60 MHz, Fig. 9D for 80 MHz |
| Transmitter |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Interference mitigation | LBT | LBT/DFS/ TPC | LBT |  | LBT/DFS/TPC | Entergy Detect, Frequency, Time and Spatial sharing |  | LBT/DFS/TPC | Entergy Detect CCA, Frequency, Time and Spatial sharing |  | LBT/DFS/TPC/AFC* | Entergy Detect, Frequency, Time and Spatial sharing | LBT/DFS/TPC |
| Receiver |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sensitivity | Listed in Standard | Listed in Standard | Listed in Standard |  | Listed in Standard | Listed in Standard |  | Listed in Standard | Listed in Standard |  | Listed in Standard | Listed in Standard | Listed in Standard |
| Notes to Table 1-1 [Editor’s Note: Need to check which of the notes below are still relevant depending on the way forward for the “frequency band” row above] (1) Parameters for the physical layer are common between IEEE 802.11a and ARIB HiSWANa. (*) See ATIS.3GPP.37.213V1640 and related ATIS Standards. - TTA also issued RLAN standards as identical as the IEEE Std 801.11-2020, IEEE Std 802.11ax-2021 and IEEE Std 802.11ay-2021 listed in the table. |  |  |  |  |  |  |  |  |  |  |  |  |  |

TABLE 1-2

Characteristics including technical parameters associated with broadband RLAN standards: ETSI and ARIB

| Characteristics | ETSI EN 300 328 (0) | ETSI EN 301 893 (V2.1.1) (0) | ARIB HiSWANa, (1) | ETSI EN 302 567 (V2.2.1) (0) | ETSI EN 303 722 (V1.2.1) | ETSI EN 303 687 (V1.1.1) (0) |
| --- | --- | --- | --- | --- | --- | --- |
| Access method |  | See clause 4.2.7 of the standard | TDMA/TDD | See clause 4.2.5 of the standard | See clause 4.2.5 of the standard | See clause 4.3.6 of the standard |
| Modulation | No restriction on the type of modulation | Unspecified | 64-QAM-OFDM 16-QAM-OFDM QPSK-OFDM BPSK-OFDM 52 subcarriers (see Fig. 1) | Unspecified | Unspecified | Unspecified |
| Data rate |  | Unspecified | 6, 9, 12, 18, 27, 36 and 54 Mbit/s | Unspecified | Unspecified | Unspecified |
| [COMMENT: The frequency bands supported by the standards should be included explicitly (in the row below), not just as broad ranges, as has been the case ever since this Rec. was first created.] |  |  |  |  |  |  |
| Frequency band [COMMENT: The specific bands should be kept here instead of just a broad frequency range in the row above. This has been the case since the Rec. was first created. Could add some text after “frequency band” here like “supported by the standards …” as we had it in the Frequency Range row above.] | 2 400-2 483.5 MHz | 5 150-5 350 MHz and 5 470- 5 725 MHz(3) | 4 900 to 5 000 MHz (2) 5 150 to 5 250 MHz (4) (*) | 57-71 GHz | 57-71 GHz | 5 945-6 425 MHz |
| Channel indexing |  | 20 MHz | 20 MHz channel spacing 4 channels in 100 MHz | Unspecified | Unspecified | 20 MHz |
| Spectrum mask |  | Fig. 1x | OFDM mask (Fig. 1) | Fig. 2x | Fig. 3x | Fig. 1x (5) |
| Transmitter |  |  |  |  |  |  |
| Interference mitigation | DAA/LBT, DAA/non-LBT, MU | See standard | LBT | See standard | See standard | See standard |
| Receiver |  |  |  |  |  |  |
| Sensitivity |  | See standard |  | See standard | See standard | See standard |
| Notes to Table 1-2 [Editor’s Note: Need to check which of the notes below are still relevant depending on the way forward for the “frequency band” row above] (0) These Harmonized Standards (HS) are not technology standards, but rather are used to demonstrate that products, services, or processes comply with relevant EU legislation. (1) Parameters for the physical layer are common between IEEE 802.11a and ARIB HiSWANa. (2) See IEEE 802.11j-2004 and JAPAN MIC ordinance for Regulating Radio Equipment, Articles 49-20 and 49-21. (3) DFS rules apply in the 5 250-5 350 and 5 470-5 725 MHz bands in many administrations and administrations must be consulted. (4) Resolution 229 (Rev.WRC-19) applies. (5) Additional masks are specified for multi-channel use cases. (*) Pursuant to Resolution 229 (Rev.WRC-19) |  |  |  |  |  |  |

Figure 1a

OFDM transmit spectrum mask for 802.11a, 11g, 11j, and HiSWANa systems

NOTE 1 – The outer heavy line is the spectrum mask for 802.11a, 11g, 11j, HiSWANa and the inner thin line is the envelope spectrum of OFDM signals with 52 subcarriers.

NOTE 2 – The measurements shall be made using a 100 kHz resolution bandwidth and a 30 kHz video bandwidth.

NOTE 3 – In the case of the 10 MHz channel spacing in 802.11j, the frequency scale shall be half.

Figure 1b

Transmit spectrum mask for EN 301 893

NOTE – dBc is the spectral density relative to the maximum spectral power density of the transmitted signal.

FIGURE 2a

Transmit spectral mask for 20 MHz 802.11n transmission in 2.4 GHz band

NOTE – Maximum of −45 dBr and −53 dBm/MHz at 30 MHz frequency offset and above.

FIGURE 2b

Transmit spectral mask for a 20 MHz 802.11n transmission in 5 GHz band and
transmit spectral mask for 802.11ac

NOTE – For 802.11n, the maximum of −40 dBr and −53 dBm/MHz at 30 MHz frequency offset and above. For 802.11ac, the transmit spectrum shall not exceed the maximum of the transmit spectral mask and −53 dBm/MHz at any frequency offset.

FIGURE 3a

Transmit spectral mask for a 40 MHz 802.11n channel in 2.4 GHz band

NOTE – Maximum of −45 dBr and −56 dBm/MHz at 60 MHz frequency offset and above.

FIGURE 3b

Transmit spectral mask for a 40 MHz 802.11n channel in 5 GHz band and
transmit spectral mask for 802.11ac

NOTE – For 802.11n, maximum of −40 dBr and −56 dBm/MHz at 60 MHz frequency offset and above. For 802.11ac, the transmit spectrum shall not exceed the maximum of the transmit spectral mask and −56 dBm/MHz at any frequency offset.

FIGURE 3c

Transmit spectral mask for an 80 MHz 802.11ac channel

NOTE – The transmit spectrum shall not exceed the maximum of the transmit spectral mask and −59 dBm/MHz at any frequency offset.

FIGURE 3d

Transmit spectral mask for a 160 MHz 802.11ac channel

NOTE – The transmit spectrum shall not exceed the maximum of the transmit spectral mask and −59 dBm/MHz at any frequency offset.

FIGURE 3e

Transmit spectral mask for a 80+80 MHz 802.11ac channel

NOTE – The transmit spectrum shall not exceed the maximum of the transmit spectral mask and −59 dBm/MHz at any frequency offset.

Figure 4

Transmit spectrum mask for 802.11b

Figure 5

Transmit spectrum mask for 802.11ad

FIGURE 6a

Transmit spectrum mask for 1 MHz 802.11ah channel

FIGURE 6b

Transmit spectrum mask for 2 MHz 802.11ah channel

FIGURE 6c

Transmit spectrum mask for 4 MHz 802.11ah channel

FIGURE 6d

Transmit spectrum mask for 8 MHz 802.11ah channel

FIGURE 6e

Transmit spectrum mask for 16 MHz 802.11ah channel

FIGURE 7a

Transmit spectrum mask for 20 MHz 802.11ax channel

FIGURE 7b

Transmit spectrum mask for 40 MHz 802.11ax channel

FIGURE 7c

Transmit spectrum mask for 80 MHz 802.11ax channel

FIGURE 7d

Transmit spectrum mask for 160 MHz 802.11ax channel

FIGURE 7e

Transmit spectrum mask for 80+80 MHz 802.11ax channel

FIGURE 8a

Transmit spectrum mask for 2.16 GHz P802.11ay channel

FIGURE 8b

Transmit spectrum mask for 4.32 GHz P802.11ay channel

FIGURE 8c

Transmit spectrum mask for 6.48 GHz P802.11ay channel

FIGURE 8d

Transmit spectrum mask for 8.64 GHz P802.11ay channel

FIGURE 8e

Transmit spectrum mask for 2.16+2.16 GHz P802.11ay channel

FIGURE 8f

Transmit spectrum mask for 4.32+4.32 GHz P802.11ay channel

Figure 9a

Transmit spectrum mask for 20 MHz ATIS RLAN

NOTE – The spectrum emission mask is defined relative to the maximum power density in a 1 MHz measurement bandwidth within the channel bandwidth. The relative power of any transmitter emission shall not exceed the levels indicated by the mask or –30 dBm/MHz whichever is the greatest.

Figure 9b

Transmit spectrum mask for 40 MHz ATIS RLAN

NOTE – The spectrum emission mask is defined relative to the maximum power density in a 1 MHz measurement bandwidth within the channel bandwidth. The relative power of any transmitter emission shall not exceed the levels indicated by the mask or –30 dBm/MHz whichever is the greatest.

Figure 9c

Transmit spectrum mask for 60 MHz ATIS RLAN

NOTE – The spectrum emission mask is defined relative to the maximum power density in a 1 MHz measurement bandwidth within the channel bandwidth. The relative power of any transmitter emission shall not exceed the levels indicated by the mask or –30 dBm/MHz whichever is the greatest.

Figure 9d

Transmit spectrum mask for 80 MHz ATIS RLAN

NOTE – The spectrum emission mask is defined relative to the maximum power density in a 1 MHz measurement bandwidth within the channel bandwidth. The relative power of any transmitter emission shall not exceed the levels indicated by the mask or –30 dBm/MHz whichever is the greatest.

Figure 10a

Transmit Mask for EN 302 567

Figure 10b

Transmit Mask for EN 303 722

Annex 2

Obtaining additional information on RLAN standards

The ETSI EN 300 328, EN 301 893 and EN 302 567 standards can be downloaded from http://pda.etsi.org/pda/queryform.asp. In addition to these standards, the Hiperlan type 2 standards can still be downloaded from the above link.

The IEEE 802.11 standards can be downloaded from: http://standards.ieee.org.

IEEE 802.11 has developed a set of standards for RLANs, IEEE Std 802.11 – 2020, which has been adopted by ISO as ISO/IEC/IEEE 8802-11:2022. The medium access control (MAC) and physical characteristics for wireless local area networks (LANs) are specified in ISO/IEC/IEEE 8802-11:2022, which is part of a series of standards for local and metropolitan area networks. The medium access control unit in ISO/IEC/IEEE 8802-11:2022 is designed to support physical layer units as they may be adopted dependent on the availability of spectrum. Approved amendments to the IEEE Std 802.11-2020 base standard include IEEE Std 802.11ax-2021, IEEE Std 802.11ay-2021, IEEE Std 802.11ba-2021, IEEE Std 802.11az-2022, and IEEE 802.11bd-2022. .  -This ISO/IEC/IEEE 8802-11:2022 standard can be downloaded from https://ieeexplore.ieee.org/document/9930960.

The ATIS RLAN standards can be downloaded from: http://www.atis.org/3gpp-documents/Rel16

Annex 3

Basic characteristics of broadband RLANs
and information for deployment and system design

CONTENTS

1	Introduction

2	Mobility

3	Operational environment and considerations of interface

4	System architecture including fixed applications

5	Interference mitigation techniques under frequency sharing environments

6	General technical characteristics

# 1	Introduction

Broadband RLAN standards have been designed to allow compatibility with wired LANs such as IEEE 802.3-- and ATM at comparable data rates. Some broadband RLANs have been developed to be compatible with current wired LANs and are intended to function as a wireless extension of wired LANs using TCP/IP and ATM protocols.  License-exempt use of spectrum allocations globally further promoted development of broadband RLANs allowing many applications such as cellular offload, voice/video over RLAN, audio/video streaming, mobile hotspot, real-time gaming, AR/VR to be supported in various segments including enterprise and residential connectivity, health, education, retail, leisure/hospitality, smart cities, transportation, IoT and Industrial IoT.

Portability is a feature provided by broadband RLANs but not wired LANs. Laptop computers and palmtop are portable and have the ability, when connected to a wired LAN, to provide interactive services. However, when they are connected to wired LANs they are no longer portable. Broadband RLANs allow portable computing devices such as notebooks, tablets, smartphones and wearable devices to remain portable and operate at maximum potential.

Advanced applications such as cellular offload, voice/video over RLAN, audio/video streaming, mobile hotspot, real-time gaming, AR/VR require higher level of performance in some characteristics of RLAN such as throughput and latency. -

# 2	Mobility

Broadband RLANs may be either pseudo fixed as in the case of a desktop computer that may be transported from place to place or portable as in the case of battery operated notebooks, tablets, smartphones and wearable devices with integrated wireless LAN connectivity. In some RLAN technologies, the relative velocity between these devices and an RLAN wireless access point remains low. In warehousing applications, RLANs may be used to maintain contact with lift trucks at speeds of up to 6 m/s. RLAN devices are generally not designed to be used at automotive or higher speeds.

[COMMENT: The new text above is not needed since the point about portable use is already covered in the first paragraph of this section.]

[COMMENT: It is unclear where these use cases are applicable and why those are listed here specifically. Also, these use cases will very much depend on the regulatory framework for each country and no general conclusions can be drawn from the above text.]

# 3	Operational environment and considerations of interface

Broadband RLANs are predominantly deployed inside buildings, in offices, factories, warehouses, etc. For RLAN devices deployed inside buildings, emissions are attenuated by the structure. In order to better support the outdoor operations, WAS/RLAN has developed various features including longer Orthogonal Frequency Division Multiple (OFDM) symbol, preamble includes repeated Legacy Signal field (L-SIG), extended range preamble includes repeated High Efficiency Signal A field(HE-SIG-A), dual carrier modulation improves robustness in Data field.

[COMMENT: The last sentence of the above paragraph is not relevant for this section. Reference to Resolution 229 and the need for administrations to assess coexistence when considering the implementation of RLANs at national level are already covered above in the main body of the Recommendation.]

RLANs utilize low power levels because of the short distances inside buildings. Power spectral density requirements are based on the basic service area of a single RLAN. defined by a circle with a radius from 10 to 50 m. When larger networks are required, RLANS may be logically concatenated via bridge or router function to form larger networks without increasing their composite power spectral density.

One of the most useful RLAN features is the connection of mobile computer users to a wireless LAN network. In other words, a mobile user can be connected to his own LAN subnetwork anywhere within the RLAN service area. The service area may expand to other locations under different LAN subnetworks, enhancing the mobile user’s convenience.

There are several remote access network techniques to enable the RLAN service area to extend to other RLANs under different subnetworks. The Internet Engineering Task Force (IETF) has developed a number of the protocol standards on this subject.

To achieve the coverage areas specified above, it is assumed that RLANs require a peak power spectral density of e.g. approximately 10 mW/MHz in the 5 GHz operating frequency range (see Table 2). For data transmission, some standards use higher power spectral density for initialization and control the transmit power according to evaluation of the RF link quality. This technique is referred to as transmit power control (TPC). The required power spectral density is proportional to the square of the operating frequency. The large scale, average power spectral density will be substantially lower than the peak value. RLAN devices share the frequency spectrum on a time basis. Activity Factor will vary depending on the usage, in terms of application and period of the day.

Broadband RLAN devices are normally deployed in high-density configurations and may use an etiquette such as listen before talk and dynamic channel selection (referred to here as dynamic frequency selection, DFS) or TPC to facilitate spectrum sharing between devices.

# 4	System architecture including fixed applications

Broadband RLANs are often point-to-multipoint architecture. Point-to-multipoint applications commonly use omnidirectional, down-looking antennas. The multipoint architecture employs several system configurations:

–	point-to-multipoint centralized system (multiple devices connecting to a central device or access point via a radio interface);

–	point-to-multipoint non-centralized system (multiple devices communicating in a small area on an ad hoc basis);

–	RLAN technology is sometimes used to implement fixed applications, which provide point-to-multipoint (P-MP) or point-to-point (P-P) links, e.g. between buildings in a campus environment. P-MP systems usually adopt cellular deployment using frequency reuse schemes similar to mobile applications. Technical examples of such schemes are given in Report ITU-R F.2086 (see § 6.6). Point-to-point systems commonly use directional antennas that allow greater distance between devices with a narrow lobe angle. This allows band sharing via channel and spatial reuse with a minimum of interference with other applications;

–	RLAN technology is sometimes used for multipoint-to-multipoint (fixed and/or mobile mesh network topology, in which multiple nodes relay a message to its destination). Omnidirectional and/or directional antennas are used for links between the nodes of the mesh network. These links may use one or multiple RF channels. The mesh topology enhances the overall reliability of the network by enabling multiple redundant communications paths throughout the network. If one link fails for any reason (including the introduction of strong RF interference), the network automatically routes messages through alternate paths.

# 5	Interference mitigation techniques under frequency sharing environments

RLANs are generally intended to operate in unlicensed or license-exempt spectrum and must allow adjacent uncoordinated networks to coexist whilst providing high service quality to users. In the 5 GHz bands, sharing with primary services must also be possible. Whilst multiple access techniques might allow a single frequency channel to be used by several nodes, support of many users with high service quality requires that enough channels are available to ensure access to the radio resource is not limited through queuing, etc. One technique that achieves a flexible sharing of the radio resource is DFS.

Dynamic frequency selection (DFS) is an interference mitigation technique, usable in frequency sharing environments, that dynamically detects presence of signals from priority systems, such as radar systems, and avoids co-channel operation. In DFS all radio resources are available at all RLAN nodes. A node (usually a controller node or access point (AP)) can temporarily allocate a channel and the selection of a suitable channel is performed based on interference detected or certain quality criteria, e.g. received signal strength, C/I. To obtain relevant quality criteria both the mobile terminals and the access point make measurements at regular intervals and report this to the entity making the selection.

In the 5 250-5 350 MHz and 5 470-5 725 MHz bands, DFS must be implemented to ensure compatible operation with systems in the co-primary services, i.e. the radiolocation service.

DFS can also be implemented to ensure that all available frequency channels are utilized with equal probability. This maximizes the availability of a channel to node when it is ready to transmit, and it also ensures that the RF energy is spread uniformly over all channels when integrated over a large number of users. The latter effect facilitates sharing with other services that may be sensitive to the aggregated interference in any particular channel, such as satellite-borne receivers.

TPC is intended to reduce unnecessary device power consumption, but also aids in spectrum reuse by reducing the interference range of RLAN nodes, the RF link quality and/or extend the battery life.

Automated Frequency Coordination (AFC) is a geolocation aware spectrum use coordination system that uses databases of registered terrestrial incumbent services/links to determine frequency/channel availability and maximum permissible power level for RLAN Access Points based on their reported 3D locations. AFC System is intended to protect incumbent Fixed Services from harmful interference from RLAN license exempt operation. AFC Systems may be used to enforce exclusion zones around radio observatories for protection of Radio Astronomy Services. AFC System is used for Wi-Fi Standard Power mode at 6 GHz band.

Contention-Based Protocol (CBP) is a protocol that allows multiple wireless communication devices to use the same frequency/channel, without pre-coordination, by specifying the events that must occur when two or more transmitters attempt to simultaneously access the same channel and establishing rules by which various devices will have reasonable transmit opportunities to use the spectrum. Listen Before Talk protocol in IEEE 802.11 is one example of CBP.

[COMMENT: AFC and CBP are supported by the IEEE 802.11 standard and some regulatory bodies have already adopted and certified products and therefore the above text is relevant to this section and should be included here like other Interference mitigation techniques.]

Antenna Pointing Restriction is in form of a more stringent limit on maximum e.i.r.p. at angles above a specified elevation in degrees, relative to near horizon direction, for outdoor devices to protect satellite operations in the band.

Other restrictions such as indoor only, operation at lower transmit power, prohibition on RLAN operations on specific platforms such as oil platforms and abroad ships may be selectively required on specific frequency range and regions as other interference mitigation techniques.

[COMMENT: Suggest to remove the various text options below and instead administrations can add the relevant information for their countries in Table 2 below as appropriate, and refer to the information in this section as needed.]

# 6	General technical characteristics

Table 2 summarizes technical characteristics applicable to operation of RLANs in certain frequency bands and in certain geographic areas. Operation in the 5 150-5 250 MHz, 5 250-5 350 MHz and 5 470-5 725 MHz frequency bands are in accordance with Resolution 229 (Rev.WRC-19).

[COMMENT: Frequency band information should be listed in table in Annex 1 as has been the case in previous revisions of this Recommendation.]

TABLE 2

General technical requirements applicable in certain administrations and/or regions

| General band designation | Administration or region | Specific frequency band (MHz) | Transmitter output power (mW) (except as noted) | Antenna gain (dBi) | Other use conditions |
| --- | --- | --- | --- | --- | --- |
| 2.4 GHz band | USA | 2 400-2 483.5 | 1 000 | 0-6 dBi (Omni) |  |
|  | Canada | 2 400-2 483.5 | 4 W e.i.r.p. | N/A | Canada permits point-to-point systems in this band with e.i.r.p. > 4 W provided that the higher e.i.r.p. is achieved by employing higher gain antenna, but not higher transmitter output power. |
|  | CEPT | 2 400-2 483.5 | 100 mW (e.i.r.p.) | N/A | The transmitter output power requirement refers to ETSI EN 300 328 |
|  | China | 2 400-2 483.5 | 20 dBm (e.i.r.p. for integrated antenna gain < 10 dBi) 10 dBm/MHz (e.i.r.p. for Integrated antenna gain < 10 dBi) 27 dBm (e.i.r.p. for antenna gain >= 10 dBi) 17 dBm/MHz (e.i.r.p. for Integrated antenna gain >= 10 dBi) |  | Interference Avoidance mechanism is mandatory Additional out of band emission limit applies in order to protect the service in the adjacent band and in specific bands. |
|  | Japan | 2 471-2 497 2 400-2 483.5 | 10 mW/MHz 10 mW/MHz | 0-6 dBi (Omni) 0-6 dBi (Omni) | See Japan MIC ordinance for Regulating Radio Equipment, Articles 49-20, 49-20-2 and 49-21 for details. |
|  | Republic of Korea | 2 400-2 483.5 | 10 mW/MHz @ OBW 0.5-26 MHz 5 mW/MHz @ OBW 26-40 MHz 2.5 mW/MHz @ OBW 60-80 MHz | 6 dBi max 6 dBi max 6 dBi max | In the Republic of Korea, for RLANs operating in the 2.4 GHz band, for antenna gains greater than 6 dBi, some reduction in output power required. |
|  | Brazil | 2 400-2 483.5 | 1 000 | 0-6 dBi (Omni) | For antenna gains greater than 6 dBi, some reduction in output power is required. |
| 5 GHz band | USA | 5 150-5 250 5 250-5 350 5 470-5 725 5 725-5 850 5 850-5 895 | 50 2.5 mW/MHz 250 12.5 mW/MHz 250 12.5 mW/MHz 1 000 50.1 mW/MHz | 0-6 dBi (Omni) 0-6 dBi (Omni) 0-6 dBi (Omni) 0-6 dBi (Omni) | [COMMENT: Not applicable to the US due to reservation at WRC-19] In the U.S., providers deploying more than 1 000 outdoor access points in the 5 150-5 250 MHz band must notify the FCC and ensure that the maximum e.i.r.p. at any elevation angle above 30 degrees as measured from the horizon shall not exceed 125 mW. In the United States of America, for RLANs operating in the 5 GHz band, for antenna gains greater than 6 dBi, some reduction in output power required. See sections 15.407 and 15.247 of the FCC’s rules for details. |
|  | Canada | 5 150-5 250 5 250-5 350 5 470-5 725 5 725-5 850 | 200 mW e.i.r.p. 10 dBm/MHz e.i.r.p. 250 12.5 mW/MHz (11 dBm/MHz) 1 000 mW e.i.r.p. 250 12.5 mW/MHz (11 dBm/MHz) 1 000 mW e.i.r.p. 1 000 50.1 mW/MHz |  | See RSS-247 section 6.2 for the detailed rules on devices with maximum e.i.r.p. greater than 200 mW: https://ised-isde.canada.ca/site/spectrum-management-telecommunications/en/devices-and-equipment/radio-equipment-standards/radio-standards-specifications-rss/rss-247-digital-transmission-systems-dtss-frequency-hopping-systems-fhss-and-licence-exempt-local |
|  | CEPT | 5 150-5 250 5 250-5 350 5 470-5 725 | 200 mW (e.i.r.p.) 10 mW/MHz (e.i.r.p.) 200 mW (e.i.r.p.) 10 mW/MHz (e.i.r.p.) 1 000 mW (e.i.r.p.) 50 mW/MHz (e.i.r.p.) 200 mW (e.i.r.p.) 25mW (e.i.r.p.) | N/A | Operation in the 5 250-5 350 MHz band is limited to indoor use. See ECC Decision (04)08 https://docdb.cept.org/download/3450 and ECC Decision (20)01 https://docdb.cept.org/download/1448. 200 mW – limited to indoor usage. 25 mW – no fixed outdoor usage. |
| 5 GHz band | China | 5 150-5 350 | 23 dBm (e.i.r.p.) 10 dBm/MHz (e.i.r.p.) |  | Indoor use only (use within vehicle is prohibited). 5 250-5 350 MHz, TPC and DFS are mandatory. Interference Avoidance mechanism is mandatory Additional out of band emission limit applies in order to protect the service in the adjacent band and in specific bands. Resolution 229 (Rev.WRC-19) establishes the conditions under which WAS, including RLANs, may use the 5 150 5 250 MHz, 5 250-5 350 MHz and 5 470-5 725 MHz. DFS rules apply in the 5 250-5 350 MHz and 5 470-5 725 MHz bands in regions and administrations and must be consulted. |
|  |  | 5 725-5 850 | 33 dBm (e.i.r.p.) 19 dBm/MHz (e.i.r.p) |  | Interference Avoidance mechanism is mandatory Additional out of band emission limit applies in order to protect the service in the adjacent band and in specific bands |
|  | Japan | 4 900-5 000 5 150-5 250 5 250-5 350 5 470-5 730 | 250 mW 50 mW/MHz 1 W (e.i.r.p.) 200 mW (e.i.r.p.) 1 W (e.i.r.p.) | 13 N/A N/A N/A | 4 900-5 000 MHz is for fixed wireless access, registered. In Japan, registration is required for RLAN access points with a maximum e.i.r.p. greater than 200 mW, and operations inside automobiles are allowed with a maximum e.i.r.p. of 40 mW. For 5 150-5 250 MHz, registration is required for RLAN access points with a maximum e.i.r.p. greater than 200 mW, and operations inside automobiles are allowed with a maximum e.i.r.p. of 40 mW. Operation in the 5 250-5 350 MHz band is limited to indoor use |
|  | Republic of Korea | 5 150-5 350 & 5 470-5 850 | 10 mW/MHz @ OBW 0.5-20 MHz 5 mW/MHz @ OBW 20-40 MHz 2.5 mW/MHz @ OBW 40-80 MHz 1.25 mW/MHz @ OBW 80-160 MHz | 7 dBi max 7 dBi max 7 dBi max 7 dBi max | In the Republic of Korea, for the device using OBW 40 MHz in 5 230-5 250 MHz, power density is limited to 2.5 mW/MHz or less. In the Republic of Korea, for RLANs operating in 5 150-5 350 MHz, 5 470-5 850 MHz, for antenna gains greater than 7 dBi, some reduction in output power required. And in this case OBW can use maximum 160 MHz, power density is limited to 1.25 mW/MHz or less. |
|  | Brazil | 5 150-5 250 5 250-5 350 5 470-5 725 5.725-5.850 | 1 000 250 1 000 (e.i.r.p.) 1 000 | 6 6 N/A N/A | For 5 150-5 350, limited to indoor use only. |
| 6 GHz band | CEPT | 5 945-6 425 MHz | Low Power Indoor(LPI) 23 dBm (e.i.r.p.) 10 dBm /MHz (e.i.r.p.) Very Low Power (VLP) 14 dBm (e.i.r.p.) 1 dBm/MHz (e.i.r.p) | N/A | LPI equipment use is limited to indoor only use. No fixed outdoor use is allowed by VLP equipment. Narrowband VLP devices that operate in channels bandwidths below 20 MHz can operate at a higher e.i.r.p. density up to 10 dBm/MHz if they implement a frequency hopping mechanism based on at least 15 hop channels |
|  | Republic of Korea | 5 925-6 425 MHz 5 925-7 125 MHz 5 925-6 425 MHz | 14 dBm (e.i.r.p) & 1 dBm/MHz (VLP) 2 dBm/MHz (LPI) 2 dBm/MHz (only for subway train) | N/A | In the Republic of Korea, the band 5 925-6 425 MHz is not allowed for using in drone. And for device built in vehicle, only 6 085-6 425 MHz band is allowed. And it is applicable for both indoor and outdoor usage. In the Republic of Korea, the band 5 925-7 125 MHz is only applicable for indoor usage. This band is not allowed for using in moving objects, such as vehicle, aircraft, railway, ship and drone. In the Republic of Korea, the band 5 925-6 425 MHz is only applicable for inside subway train. |
|  | USA | 5 925-7 125 MHz 5 925-6 425 MHz 6 525-6 875 MHz | Low Power Indoor (LPI): 30 dBm (max. e.i.r.p.); 5 dBm/MHz and Client Connected to Low Power Access Point (AP): 24 dBm (max e.i.r.p.); –1 dBm/MHz Standard Power (SP) AP (AFC Controlled): 36 dBm (max. e.i.r.p.); 23 dBm/MHz and Client Connected to SP AP: 30 dBm (max e.i.r.p); 17 dBm/MHz SP AP (AFC Controlled): 36 dBm (max. e.i.r.p.); 23 dBm/MHz and Client Connected to SP AP: 30 dBm (max. e.i.r.p.); 17 dBm/MHz | N/A | The technical requirements are as of October 2021 and are under review. |
|  | Brazil | 5 925-7 125 | Indoor access point and subordinate access point 30 dBm (e.i.r.p.) 5 dBm/MHz (e.i.r.p.) Client equipment under control of indoor access point 24 dBm (e.i.r.p.) –1 dBm/MHz (e.i.r.p.) Very Low Power 17 dBm (e.i.r.p.) –5 dBm/MHz (e.i.r.p.) | N/A | Indoor access point and subordinate access point must be directly connected to the power grid |
|  | Japan |  | 5 925-6 425 |  | Low Power Indoor (LPI): 200 mW (e.i.r.p.) Very Low Power (VLP): 25 mW (e.i.r.p.) |  | N/A |  | LPI equipment use is limited to indoor only use. |  |
|  | Canada |  | 5 925 – 7 125 MHz 5 925 – 6 875 MHz |  | Low-power indoor access-points and indoor subordinate devices 30 dBm (e.i.r.p.) 5 dBm/MHz (e.i.r.p.) Low-power client devices 24 dBm (e.i.r.p.) −1 dBm/MHz (e.i.r.p.) Standard-power access points and fixed client devices (AFC Controlled) 36 dBm (e.i.r.p.) 23 dBm/MHz (e.i.r.p.) Standard client devices 30 dBm (e.i.r.p.) 17 dBm/MHz (e.i.r.p.) |  | N/A |  | The maximum e.i.r.p. for a device not enclosed by walls and a ceiling, measured at any elevation angle greater than 30 degrees above the horizon, shall not exceed 21 dBm over the 5 925-6 875 MHz frequency band. For standard client devices, the maximum power limits shall remain at least 6 dB below the power levels authorized for the associated standard-power access point. See RSS-248 for more details: https://ised-isde.canada.ca/site/spectrum-management-telecommunications/en/devices-and-equipment/radio-equipment-standards/radio-standards-specifications-rss/rss-248-radio-local-area-network-rlan-devices-operating-5925-7125-mhz-band |  |
|  | Mexico |  | 5925-6425 MHz |  | Low Power Indoor (LPI): 30 dBm (e.i.r.p.) 5 dBm /MHz Client Connected to Low Power Access Point: 24 dBm (max e.i.r.p.) −1 dBm/MHz Very Low Power (VLP): 14 dBm (e.i.r.p.) 1 dBm/MHz |  | N/A |  | LPI equipment: – Limited to indoor use. – Must have an integrated non-removable antenna and no possibility of connecting external antennas. – The maximum channel bandwidth allowed for transmission is 320 MHz. – The use in communications for unmanned aerial vehicle systems, on ships, oil platforms, automobiles, trains and airplanes is prohibited, unless its operation is within large aircraft that fly at altitudes of more than 10,000 feet. – The use of containment-based protocols is mandatory. – Access Points and Slave Access Points cannot be weather resistant, cannot use batteries, and the power supply must be connected directly to the electrical outlet. |  |
| 57-71 GHz | Europe |  | 57-71 GHz (C1) 57-71 GHz (C2) |  | 40 dBm (e.i.r.p.) 13 dBm/MHz (e.i.r.p.) 40 dBm (e.i.r.p.) 13 dBm/MHz (e.i.r.p.) |  | N/A Max conducted power 27 dBm |  | This refers to the highest power level of the transmitter power control range during the transmission burst if transmitter power control is implemented. Fixed outdoor installations are not allowed. See ERC Recommendation 70-03 Annex 3 (Table 3) entries c1 and c2 https://docdb.cept.org/download/25c41779-cd6e/Rec7003e.pdf. |  |
|  | Brazil | 57-71 GHz | 40 dBm (e.i.r.p.) average 43 dBm (e.i.r.p.) peak 13 dBm/MHz (e.i.r.p.) | N/A | Point-to-area indoor and outdoor |
| - |  |  |  |  |  |

[COMMENT: These sharing parameters are not needed in Recommendation ITU-R M.1450. The specific parameters had been suggested for use in EESS studies in WP7C during the previous study cycle to assess whether EESS should be considered in the 4.2-4.4 GHz and 8.4-8.5 GHz bands, but they were never agreed in WP5A In the meantime, WRC-23 has agreed a WRC-27 agenda item (1.19) to consider EESS in the 4.2-4.4 GHz and 8.4-8.5 GHz bands and studies between EESS and RLANs in the 6 GHz band are no longer required. Therefore, instead of trying to modify the content of Annex 4 to make it acceptable to WP5A, this draft new annex can just be removed as no longer needed.]

[COMMENT: Annex 5 is not necessary. It quotes some elements from RR No. 5.150 and Resolution 229 which address the 2.4 GHz and 5 GHz bands. However, these bands have been covered by the current revision of Recommendation ITU-R M.1450 since 2014 without the need for such quotations. Also, other parts of the RR may be relevant and are not quoted in Annex 5, making this an incomplete list. The general need for all applications covered by Recommendation ITU-R M.1450 are clearly stated in the main body of the Recommendation and there is no need to try and reproduce certain elements in Annex 5.]

____________________

This document solely represents the views of the IEEE 802 LAN/MAN Standards Committee and does not necessarily represent a position of either the IEEE or the IEEE Standards Association.

, Information technology – Telecommunications and information exchange between systems – Local and metropolitan area networks – Specific requirements – Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications.