IEEE P802.19
Wireless Coexistence

| Parameters for simulation of Wireless Coexistence in the US and Canada 3.65GHz band |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-04-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Paul Piggin | NextWave Broadband | 12670 High Bluff Drive San Diego CA 92130 USA | 1 858 480 3100 | ppiggin @ nextwave.com |

Abstract

This document provides a set of assumptions and simulation parameters for use in modelling wireless coexistence in the 3.65GHz band for US and Canada.

Notice: This document has been prepared to assist IEEE 802.19. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.19.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the TAG of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Shellhammer@ieee.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.19 TAG. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

## Environmental factors

A description to be included of the expected deployment scenario(s) for both 802.16 and 802.11 devices. How do these factors relate to terrain and topology, and how does this impact the large-scale and small-scale fading model?

### Large-scale fading

Based on SUI model

Shadow margin

Multiple breakpoint model/statistical.

Building penetration losses.

### Small-scale fading

Based on SUI model

Ricean K factor

Doppler profile

Delay spread

>> Fade margin (SUI model)

## Device parameters

### General

Nominal maximum antenna gain and radiation pattern

Base Station:

Fixed Station:

Mobile Station:

Portable Station:

Cabling loss:

Base Station: 2dB (802.16), 2dB (802.11)

Fixed Station: 2dB (802.16), 2dB (802.11)

Mobile Station: 0dB (802.16), 0dB (802.11)

Portable Station: 0dB (802.16), 0dB (802.11)

Noise Figure/Implementation loss:

Base Station: 5dB (802.16), 5dB (802.11)

Fixed Station:

Mobile Station:

Portable Station:

EIRP based on maximum limits of Power Spectral Density (47 CFR 90, Subpart Z)

5 MHz:

Base/Fixed stations = 5W (37dBm) EIRP,

Mobile/Portable stations =200mW (23dBm) EIRP

10 MHz:

Base/Fixed stations =10W (40dBm) EIRP,

Mobile/Portable stations =400mW (26dBm) EIRP

20 MHz:

Base/Fixed stations =20W (43 dBm) EIRP,

Mobile/Portable stations =800mW (29dBm) EIRP

Suggested typical value for Base/Fixed stations = 40dBm EIRP

Suggested typical value for Mobile/Portable stations = 20dBm EIRP

### 802.11 [2]

802.11y feature support

RTS/CTS: enabled

11e: HCF - EDCA

CW: 15 > 1023

Maximum frame transmission duration = 4ms

Use of an enabling signal

Link adaptation

Number of MCS – sensitivities and TX powers

CCA parameters

SIFS = 16/32/64 µs

SlotTime = 9/13/21 µs

DIFS = SIFS + 2 x SlotTime

DIFS = 34/58/106 µs

SlotTime = D2 + CCA + M2 + Rx/Tx

D2 (aRxRFDelay + aRxPLCPDelay) = 1/1/1 µs

CCA (Clear Channel Assessment) = 4/8/16 µs

M2 (aMACProcessingDelay) = 2/2/2 µs

Rx/Tx (aRXTXTurnaroundTime) = 2/2/2 µs

CCA-CS threshold

-82dBm (20MHz)

-85dBm (10MHz)

-88dBm (5MHz)

CCA-ED threshold

-72dBm (20MHz)

-75dBm (10MHz)

-78dBm (5MHz)

### 802.16

802.16h feature support

UCP (Uncoordinated Coexistence Protocol) subclause 6.4.2.4 [1]:

DCS (Dynamic Channel Selection) subclause 6.4.2.3.2 [1]

LBT (Listen-Before-Talk) subclause 6.4.3.4 [1]

aEQP (Adaptive Extended Quiet Period) subclauses 6.4.3.2 and 6.4.3.3 [1]

Parameters

Taken from WiMAX Forum Mobile WiMAX System Profile [3].

Frame duration=5ms

47 symbols per frame

- 26 in the downlink (1 for preamble)

- 21 in the uplink

102µs symbol duration

TTG > 50µs

RTG = 60µs

SSTTG = SSRTG = 50µs.

Assumptions about the permutations used.

Link adaptation

Number of MCS – sensitivities and TX powers

LBT (Listen-Before-Talk) parameters

LBT applied to: DL and UL

DL LBT slot location: slot 26

UL LBT slot location: slot 47

DL/UL LBT Threshold:

-72dBm (20MHz)

-75dBm (10MHz)

-78dBm (5MHz)

aEQP (Adaptive Extended Quiet Period) parameters

Initial EQP duty cycle: 10% on.

aEQP threshold 1 (Channel Occupied test):

-77dBm (20MHz)

-80dBm (10MHz)

-83dBm (5MHz)

aEQP threshold 2 (User detected test):

-72dBm (20MHz)

-75dBm (10MHz)

-78dBm (5MHz)

## External driving parameters

Traffic model

Packet inter-arrival time: exponentially distributed.

Packet size: uniformly distributed in the range: 0.1x1500 –0.9x1500 bytes

Traffic transport type: UDP

DL/UL load ratio: 60/40

Fragmentation rules and parameters

## Regulatory requirements

3.65GHz

5, 10, 20MHz channels

ACIR

The FCC rules are documented in 47 CFR 90, Subpart Z – Wireless Broadband Services in the 3650-3700 MHz Band.

The current rule making proposes a Non-Exclusive Registration Use licensing mechanism for the entire 3650 – 3700 MHz band.  Licensees are required to registers their base stations online via FCC’s ULS.  And, they must delete the registrations for unused stations.  License terms are for 10 years.  Interference among base stations of different service providers are expected to be resolved among the providers themselves by ‘mutually satisfactory arrangements’.

The following technical requirements appear in 47 CFR 90 Sub-Part Z:

- 25 Watt EIRP maximum power in 25MHz bandwidth for Base and Fixed stations

- 1 Watt EIRP maximum power in 25MHz bandwidth for Mobile and Portable stations

- 1W / MHz EIRP maximum PSD for Base and Fixed stations

- 40mW / MHz EIRP maximum PSD for Mobile and Portable stations

- Sectorized antenna permitted only if each sector transmits different information

- Beamforming is subject to the 25 Watt EIRP requirement

- 43 + 10 Log(P) OOBE, with the 1% rule included

- Mobile stations may only transmit if they can decode an enabling signal from a base station

- Mobile stations may transmit to one another directly only if they can decode an enabling signal from a base station

- Airborne operation prohibited

- 150 km exclusion zone around FSS stations – unless agreed with the FSS licensee

- 80 km exclusion zones around following federal radiolocation stations

  - St. Inigoes, MD	38° 10’ 0”  N  ,  76° 23’ 0” W

  - Pascagoula, MS	30° 22’ 0”  N  ,  88° 29’ 0”  W

  - Pensacola, FL 	30° 21’ 28” N  ,  87° 16’ 26” W

- Fixed devices must be at least 8 / 56 km away from international borders if the antenna looks within 160° / 200° sector toward the border – unless coordinated with Mexico or Canada.

References:

[1] IEEE P802.16h/D2: Air Interface for Fixed Broadband Wireless Access Systems Improved Coexistence Mechanisms for License-Exempt Operation, Draft Standard.

[2] IEEE P802.11y/D1.1: Draft STANDARD for Information Technology — Telecommunications and information exchange between systems— Local and metropolitan area networks- Specific requirements— Part 11:Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications: 3650-3700 MHz Operation in USA.

[3] WiMAX Forum Mobile System Profile Release 1.0 Approved Specification (Revision 1.2.2: 2006-11-17)