IEEE P802.19
Wireless Coexistence

| Parameters for simulation of Wireless Coexistence in the US and Canada 3.65GHz band |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-06-27 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Paul Piggin | NextWave Broadband | 12670 High Bluff Drive San Diego CA 92130 USA | 1 858 480 3100 | ppiggin @ nextwave.com |

Abstract

This document provides a set of assumptions and simulation parameters for use in modelling wireless coexistence in the 3.65GHz band for US and Canada.

Notice: This document has been prepared to assist IEEE 802.19. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.19.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the TAG of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Shellhammer@ieee.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.19 TAG. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

## Introduction and scenarios

This document is arranged to provide simulation parameters for a number of defined scenarios for use in coexistence analysis between collocated or neighbouring 802.16h and 802.11y. Once described in this section the following sections provide reference to which parameters are applicable to which scenario. It is possible to combine scenarios; it is also possible to combine scenarios where a scenario is specific to a given technology.

The scenarios are described thus:

Scenario A

Outdoor only scenario with fixed subscribers

This scenario considers an outdoor deployment with fixed outdoor subscribers only.

Scenario B

Outdoor only scenario with fixed and mobile subscribers

This scenario considers an outdoor deployment with fixed and mobile outdoor subscribers.

Scenario C

Outdoor to indoor scenario with portable and mobile subscribers

This scenario considers an outdoor deployment with indoor portable and mobile subscribers.

Scenario D

Indoor only scenario with portable and mobile subscribers

This scenario considers an indoor only deployment with indoor portable and mobile subscribers.

Scenario E

A combined scenario

This scenario considers a combination of all the above scenarios.

## Environmental factors

### Large-scale fading

This section describes the model for calculation of pathloss caused by obstructions within the propagation environment. These obstructions can be attributed to terrain and/or building clutter.

General

Carrier frequency = 3675MHz (mid band)

Outdoor-outdoor, and outdoor-indoor model

The pathloss model is based on the Stanford University Interim (SUI) model [4]. This model is for outdoor operation with a correction for building penetration. General pathloss expression:

PathlossTotal = Pathlossoutdoor-outdoor(mean) + SM + BPL					[1]

Specific parameters to calculate Pathlossoutdoor-outdoor(mean) are:

Terrain type B (suburban)

Base Station height: 25m

Subscriber Station height:

Fixed Station: 10m

Portable Station: 2m

Mobile Station: 2m

Cell radius < 10km.

Range ≥ 100m.

Scenarios A, B:

Standard Deviation = 8-10dB, page 4 [4].

Shadow Margin (SM) = 6dB (90% coverage, 75% at the cell edge)

Building Penetration Loss (BPL) = 0dB

Amendment for outdoor to indoor propagation

Scenarios C, E:

Standard Deviation = 10-12dB.

Shadow Margin (SM) = 8dB (90% coverage, 75% at the cell edge)

Building Penetration Loss (BPL) = 6dB (at window), 12dB (elsewhere)

Indoor model

Scenarios D, E:

What is a suitable indoor propagation model?

Interference calculation model between 802.11y and 802.16h

For the shadow fading we need to also consider the CCA sensing link and the interference link

- One approach is to pick the shadow fading value randomly between stations and then keep them fixed for those locations

- On each experiment the shadow fading is fixed between stations are fixed at the beginning of the experiment since during the experiment the station locations do not change

### Small-scale fading

This section describes the model for calculation of fading caused by obstructions and movement of obstructions within the propagation environment.

Outdoor propagation

Scenario A:

Ricean K factor = 12dB (Fade margin = 2dB)

Scenarios B, C, D, E:

Rayleigh fading (Fade margin = ?)

Indoor propagation

Scenarios D, E:

What are suitable indoor propagation values?

## Device parameters

### General

Nominal maximum antenna gain and radiation pattern

Base Station: 10dBi (omni)

Subscriber Station:

Fixed Station: 0dBi

Portable Station: 0dBi

Mobile Station: 0dBi

Cabling loss

Base Station: 3dB (802.16), 3dB (802.11)

Subscriber Station:

Fixed Station: 0dB (802.16), 0dB (802.11)

Portable Station: 0dB (802.16), 0dB (802.11)

Mobile Station: 0dB (802.16), 0dB (802.11)

Noise Figure and Implementation Margin

Base Station: 4dB (802.16), 10dB (802.11)

Subscriber Station:

Fixed Station: 6dB (802.16), 10dB (802.11)

Portable Station: 6dB (802.16), 10dB (802.11)

Mobile Station: 6dB (802.16), 10dB (802.11)

NB Due to selection of sensitivity numbers from the standard [6] then Implementation Margin for 802.11 is 0dB. 10dB Noise Figure for 802.11 is taken from subclause 19.5.1 [6]

EIRP based on maximum limits of Power Spectral Density (47 CFR 90, Subpart Z)

5 MHz:

Base/Fixed stations = 5W (37dBm) EIRP,

Mobile/Portable stations =200mW (23dBm) EIRP

10 MHz:

Base/Fixed stations =10W (40dBm) EIRP,

Mobile/Portable stations =400mW (26dBm) EIRP

20 MHz:

Base/Fixed stations =20W (43dBm) EIRP,

Mobile/Portable stations =800mW (29dBm) EIRP

Suggested typical value for Base/Fixed stations = 40dBm EIRP

Suggested typical value for Mobile/Portable stations = 20dBm EIRP

### 802.11y specific parameters [2]

General

Based on the OFDM PHY for the 5GHz band (Clause 17 [6])

802.11y feature support

Maximum frame transmission duration = 4ms

Use of an enabling signal DSE (subclause 11.14 [2])

RTS/CTS: enabled

Receiver sensitivity and link adaptation

Number of MCS = 8

Figure 1 Receiver sensitivities for OFDM 5GHz PHY [6].

Antenna configuration

SISO configuration?

MIMO configuration?

CCA and medium access parameters

Taken from table 147 [6].

- HCF (Hybrid Coordination Function) is specified in the 802.11e amendment

- HCF consists of EDCA (Enhanced Distributed Channel Access, distribution function) and HCCA (HCF Controlled Channel Access, centralized function)

- WMM (Wi-Fi Multimedia) certifies the EDCA and TXOP (Transmit Opportunity) features

- EDCA and TXOP features enhance the QoS support in 802.11

- EDCA introduces 4 AC (Access Categories) that prioritizes traffic class access to the air interface

- TXOPs are used to provide a station with a time period in which to transmit in a non-contended manner

- Contention Window = 3, 7

Timing values (Format = 20MHz/10MHz/5MHz):

SIFS = 16/32/64 µs

SlotTime = 9/13/21 µs

AIFS[AC] = SIFS + AIFSN[AC] x SlotTime

AIFSN[AC] = 2

AIFS[AC] = 34/58/106 µs

AC (Access Category)= AC_VO (highest priority)

SlotTime = D2 + CCA + M2 + Rx/Tx

D2 (aRxRFDelay + aRxPLCPDelay) = 1/1/1 µs

CCA (Clear Channel Assessment) = 4/8/16 µs

M2 (aMACProcessingDelay) = 2/2/2 µs

Rx/Tx (aRXTXTurnaroundTime) = 2/2/2 µs

CCA-CS threshold

-82dBm (20MHz)

-85dBm (10MHz)

-88dBm (5MHz)

CCA-ED threshold

-72dBm (20MHz)

-75dBm (10MHz)

-78dBm (5MHz)

### 802.16h specific parameters [1]

General

Based on the OFDMA PHY (subclause 8.4 [5]) and profiled in WiMAX Forum Mobile WiMAX System Profile [3].

802.16h feature support

UCP (Uncoordinated Coexistence Protocol) subclause 6.4.2.4 [1]:

DCS (Dynamic Channel Selection) subclause 6.4.2.3.2 [1]

LBT (Listen-Before-Talk) subclause 6.4.3.4 [1]

aEQP (Adaptive Extended Quiet Period) subclauses 6.4.3.2 and 6.4.3.3 [1]

Parameters

Taken from WiMAX Forum Mobile WiMAX System Profile [3].

Frame duration=5ms

47 symbols per frame

- 26 in the downlink (1 for preamble)

- 21 in the uplink

102µs symbol duration

TTG > 50µs (RTD and Timing Advance dependant)

RTG = 60µs

SSTTG = SSRTG = 50µs

Receiver sensitivity and link adaptation

Number of MCS = ?

?

Antenna configuration

SISO configuration?

MIMO configuration?

LBT (Listen-Before-Talk) parameters

LBT applied: DL and UL

DL LBT slot location: slot 26

UL LBT slot location: slot 47

DL/UL LBT Threshold:

-72dBm (20MHz)

-75dBm (10MHz)

-78dBm (5MHz)

aEQP (Adaptive Extended Quiet Period) parameters

Initial EQP duty cycle: 10% on.

aEQP threshold 1 (Channel Occupancy test):

-77dBm (20MHz)

-80dBm (10MHz)

-83dBm (5MHz)

aEQP threshold 2 (User detection test):

-72dBm (20MHz)

-75dBm (10MHz)

-78dBm (5MHz)

## External driving parameters

Traffic model

Packet inter-arrival time: exponentially distributed

Packet size: uniformly distributed in the range: 0.1x1500 –0.9x1500 bytes

Traffic transport type: UDP, (VoIP due to 802.11e assumptions)

DL/UL load ratio (802.16h): 60/40

Fragmentation rules and parameters?

ACK size?

Others

Interference threshold: 10dB greater than the CCA-ED thresholds (bandwidth specific), i.e.

-72dBm (20MHz)

-75dBm (10MHz)

-78dBm (5MHz)

Maximum number of SS per AP/BS = 4.

## Regulatory requirements

3.65GHz

Channel width = 5, 10, 20MHz

The FCC rules are documented in 47 CFR 90, Subpart Z – Wireless Broadband Services in the 3650-3700 MHz Band.

The current rule making proposes a Non-Exclusive Registration Use licensing mechanism for the entire 3650 – 3700 MHz band.  Licensees are required to registers their base stations online via FCC’s ULS.  And, they must delete the registrations for unused stations.  License terms are for 10 years.  Interference among base stations of different service providers are expected to be resolved among the providers themselves by ‘mutually satisfactory arrangements’.

The following technical requirements appear in 47 CFR 90 Sub-Part Z:

- 25 Watt EIRP maximum power in 25MHz bandwidth for Base and Fixed stations

- 1 Watt EIRP maximum power in 25MHz bandwidth for Mobile and Portable stations

- 1W / MHz EIRP maximum PSD for Base and Fixed stations

- 40mW / MHz EIRP maximum PSD for Mobile and Portable stations

- Sectorized antenna permitted only if each sector transmits different information

- Beamforming is subject to the 25 Watt EIRP requirement

- 43 + 10 Log(P) OOBE, with the 1% rule included

- Mobile stations may only transmit if they can decode an enabling signal from a base station

- Mobile stations may transmit to one another directly only if they can decode an enabling signal from a base station

- Airborne operation prohibited

- 150 km exclusion zone around FSS stations – unless agreed with the FSS licensee

- 80 km exclusion zones around following federal radiolocation stations

  - St. Inigoes, MD	38° 10’ 0”  N  ,  76° 23’ 0” W

  - Pascagoula, MS	30° 22’ 0”  N  ,  88° 29’ 0”  W

  - Pensacola, FL 	30° 21’ 28” N  ,  87° 16’ 26” W

- Fixed devices must be at least 8 / 56 km away from international borders if the antenna looks within 160° / 200° sector toward the border – unless coordinated with Mexico or Canada.

References

[1] IEEE P802.16h/D2: Air Interface for Fixed Broadband Wireless Access Systems Improved Coexistence Mechanisms for License-Exempt Operation, Draft Standard.

[2] IEEE P802.11y/D1.1: Draft STANDARD for Information Technology — Telecommunications and information exchange between systems— Local and metropolitan area networks- Specific requirements— Part 11:Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications: 3650-3700 MHz Operation in USA.

[3] WiMAX Forum Mobile System Profile Release 1.0 Approved Specification (Revision 1.2.2: 2006-11-17)

[4] IEEE 802.16.3c-01/29: Channel Models for Fixed Wireless Applications, Erceg et al., IEEE 802.16 Broadband Wireless Access Working Group, January 2001.

[5] IEEE Standard for Local and metropolitan area networks, Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems.

[6] Standard for Information Technology— Telecommunications and information exchange between systems— Local and metropolitan area networks— Specific requirements Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications

Abbreviations

aEQP		Adaptive Extended Quiet Period

BPL		Building Penetration Loss

CCA-CS	Clear Channel Assessment – Carrier Sense

CCA-ED	Clear Channel Assessment – Energy Detect

DCS		Dynamic Channel Selection

EQP		Extended Quiet Period

LBT		Listen Before Talk

UCP		Uncoordinated Coexistence Protocol