IEEE P802.19
Wireless Coexistence

| Parameters for simulation of Wireless Coexistence in the US 3.65GHz band |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-07-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Paul Piggin | NextWave Broadband | 12670 High Bluff Drive San Diego CA 92130 USA | 1 858 480 3100 | ppiggin @ nextwave.com |

Abstract

This document provides a set of assumptions and simulation parameters for use in modelling wireless coexistence in the 3.65GHz band for US.

Notice: This document has been prepared to assist IEEE 802.19. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.19.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the TAG of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Shellhammer@ieee.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.19 TAG. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

## Introduction and scenarios

This document provides simulation parameters for a number of defined scenarios for use in coexistence analysis between collocated or neighbouring 802.16h [1] and 802.11y [2] systems. The scenarios are described in this section with the following sections providing reference to which parameters are applicable to which scenario. It is possible to combine scenarios; it is also possible to combine scenarios where a scenario is specific to a given technology.

The scenarios are described thus:

Scenario A

Outdoor only scenario with fixed subscribers

This scenario considers an outdoor deployment with fixed outdoor subscribers only.

Scenario B

Outdoor only scenario with fixed and mobile subscribers

This scenario considers an outdoor deployment with fixed and mobile outdoor subscribers.

Scenario C

Outdoor to indoor scenario with portable and mobile subscribers

This scenario considers an outdoor deployment with indoor portable and mobile subscribers.

Scenario D

Indoor only scenario with portable and mobile subscribers

This scenario considers an indoor only deployment with indoor portable and mobile subscribers.

Radiation safety limits need to be observed in this scenario. The notional maximum transmitter power of 1W/MHz EIRP maximum PSD for Base stations is expected to require capping.

Scenario E

A combined scenario

This scenario considers a combination of all the above scenarios.

## Environmental factors

### Large-scale fading

This section describes the model for calculation of pathloss caused by obstructions within the propagation path from transmitter to receiver. These obstructions can be attributed to terrain and/or building clutter.

General

Carrier frequency = 3675MHz (mid band)

Outdoor-to-outdoor and outdoor-to-indoor model

The pathloss model is based on the Stanford University Interim (SUI) model [4]. This model is for outdoor operation with a correction for building penetration. The general pathloss expression is:

PathlossTotal = Pathlossoutdoor-outdoor(mean) + SM + BPL					[1]

Specific parameters to calculate Pathlossoutdoor-outdoor(mean) are:

Terrain type B (suburban)

Base Station height: 25m

Subscriber Station height:

Fixed Station: 10m

Portable Station: 2m

Mobile Station: 2m

100m ≤ Radius < 10km.

Scenarios A, B:

Standard Deviation = 8-10dB, page 4 [4].

Shadow Margin (SM) = 6dB (90% coverage, 75% at the cell edge)

Building Penetration Loss (BPL) = 0dB

Amendment for outdoor-to-indoor propagation

Scenarios C, E:

Standard Deviation = 10-12dB.

Shadow Margin (SM) = 8dB (90% coverage, 75% at the cell edge)

Building Penetration Loss (BPL) = 6dB (at window), 12dB (elsewhere)

Indoor model

Scenarios D, E:

The 802.11n indoor propagation model is applicable, clause 2 [7].

Base Station height: 2m

Subscriber Station height:

Portable Station: 2m

Mobile Station: 2m

### Small-scale fading

This section describes the model for calculation of fading caused by obstructions and movement of obstructions within the propagation environment.

Outdoor propagation

Scenario A:

Fixed Station: 10m

Ricean K factor = 12dB (Fade margin = 2dB)

Scenarios B, C:

Portable Station: 2m

Mobile Station: 2m

Rayleigh fading.

Fade margin: 2Tx-2Rx=5dB, 2Tx-1Rx=10dB, 1Tx-1Rx=10-15dB.

Indoor propagation

Scenarios D, E:

The 802.11n indoor propagation model is applicable, clause 2 [7].

## Device parameters

### General

Nominal maximum antenna gain and radiation pattern

Base Station (outdoor): 10dBi (omni)

Base Station (outdoor): 18dBi (sectorized)

Base Station (indoor): 0dBi (omni)

Subscriber Station:

Fixed Station: 8dBi (directional)

Fixed Station: 5dBi (omni)

Portable Station: 2dBi

Mobile Station: 2dBi

Cabling loss

Base Station: 3dB (802.16), 3dB (802.11)

Subscriber Station (connector loss):

Fixed Station: 0.5dB (802.16), 0.5dB (802.11)

Portable Station: 0.5dB (802.16), 0.5dB (802.11)

Mobile Station: 0.5dB (802.16), 0.5dB (802.11)

Noise Figure

Base Station: 4dB (802.16), 0dB (802.11)

Subscriber Station:

Fixed Station: 6dB (802.16), 0dB (802.11)

Portable Station: 6dB (802.16), 0dB (802.11)

Mobile Station: 6dB (802.16), 0dB (802.11)

Implementation Margin

Base Station: 5dB (802.16), 0dB (802.11)

Subscriber Station:

Fixed Station: 5dB (802.16), 0dB (802.11)

Portable Station: 5dB (802.16), 0dB (802.11)

Mobile Station: 5dB (802.16), 0dB (802.11)

NB Due to the way the sensitivity numbers for 802.11 are presented in the standard [6] then Implementation Margins and Noise Figures are here set to 0dB.

EIRP

EIRP values based on maximum limits of Power Spectral Density (47 CFR 90, Subpart Z)

5 MHz:

Base/Fixed stations = 5W (37dBm) EIRP,

Mobile/Portable stations =200mW (23dBm) EIRP

10 MHz:

Base/Fixed stations =10W (40dBm) EIRP,

Mobile/Portable stations =400mW (26dBm) EIRP

20 MHz:

Base/Fixed stations =20W (43dBm) EIRP,

Mobile/Portable stations =800mW (29dBm) EIRP

### 802.11y specific parameters [2]

General

Based on the OFDM PHY for the 5GHz band (clause 17 [6])

802.11y feature support

Maximum frame transmission duration = 4ms (subclause I.1 [2])

Use of an enabling signal DSE (subclause 11.14 [2])

RTS/CTS: enabled

Receiver sensitivity and link adaptation

Number of MCS = 8

Figure 1 Receiver sensitivities for OFDM 5GHz PHY [6].

20MHz data rates: 54, 48, 36, 24, 18, 12, 9, 6 Mbps.

10MHz data rates: 27, 24, 18, 12, 9, 6, 4.5, 3 Mbps (for -3dB on sensitivity for 20MHz).

5MHz data rates: 13.5, 12, 9, 6, 4.5, 3, 2.25, 1.5 Mbps (for -6dB on sensitivity for 20MHz).

Antenna configuration

SISO assumed unless otherwise stated.

CCA and medium access parameters

Taken from table 147 [6].

Figure 2 Time domain representation of medium access for 802.11.

- HCF (Hybrid Coordination Function) is specified in the 802.11e amendment [6].

- HCF consists of EDCA (Enhanced Distributed Channel Access, distribution function) and HCCA (HCF Controlled Channel Access, centralized function).

- WMM (Wi-Fi Multimedia) certifies the EDCA and TXOP (Transmit Opportunity) features.

- EDCA and TXOP features enhance the QoS support in 802.11.

- EDCA introduces 4 AC (Access Categories) that prioritizes traffic class access to the air interface.

- TXOPs are used to provide a station with a time period in which to transmit in a non-contended manner.

Figure 3 Values for EDCA 4 AC parameters [9].

Range dependency on propagation time

Subclause 7.3.2.9 Country information element of [6] states:

The Country information element contains the information required to allow a station to identify the regulatory domain in which the station is located and to configure its PHY for operation in that regulatory domain.

The Coverage Class field of the regulatory triplet specifies the aAirPropagationTime characteristic used in basic service set (BSS) operation, as shown in Table 27. The characteristic aAirPropagationTime describes variations in actual propagation time that are accounted for in a BSS and, together with maximum transmit power level, allow control of BSS diameter.

From Table 27 [6]:

Coverage class value = 0 for aAirPropagationTime ≤1µs

Coverage class value = 1 for aAirPropagationTime 3µs

Coverage class value = 2 for aAirPropagationTime 6µs

…

Coverage class value = 31 for aAirPropagationTime 93µs

Subclause 9.8.4 of [2] states:

Radio waves propagate at 30 m/μs in free space, and, for example, 3 μs would be the ceiling for BSS maximum one way distance of ~450 m (~900m round trip).

Therefore recommended Coverage class value = 7, (21µs), giving a cell radius of 3000m, round trip 6000m. This is set to half the maximum value calculated in Annex 1.

Timing values

Reference 9.2.10 DCF timing relations [6] and Figure 2.

(Legend 20MHz/10MHz/5MHz)

SIFS = 16/32/64 µs

AIFS[AC] = SIFS + AIFSN[AC].aSlotTime

From Figure 170 [6]

aSlotTime = aCCATime + aRTTXTurnaroundTime + aAirPropagationTime + aMACProcessingTime

aCCATime (CCA) = 4/8/16 µs

aMACProcessingDelay (M2) = 2/2/2 µs

aRXTXTurnaroundTime (Rx/Tx) = 2/2/2 µs

aAirPropagationTime (D2) = 	21 µs (Coverage class value = 7, cell radius of 3000m, round trip 6000m) (all bandwidths)

AIFSN[AC] = 2

aSlotTime = 23/27/35 µs

AIFS[AC] = 74/98/146 µs

AC (Access Category) = AC_VO (highest priority) [Contention Window = 3, 7]

CCA-CS threshold

-82dBm (20MHz)

-85dBm (10MHz)

-88dBm (5MHz)

CCA-ED threshold

-72dBm (20MHz)

-75dBm (10MHz)

-78dBm (5MHz)

Probability of detection during sensing time > 90% in all cases.

### 802.16h specific parameters [1]

General

Based on the OFDMA PHY (subclause 8.4 [5]) and profiled in WiMAX Forum Mobile WiMAX System Profile [3].

802.16h feature support

UCP (Uncoordinated Coexistence Protocol) subclause 6.4.2.4 [1]:

DCS (Dynamic Channel Selection) subclause 6.4.2.3.2 [1]

LBT (Listen-Before-Talk) subclause 6.4.3.4 [1]

aEQP (Adaptive Extended Quiet Period) subclauses 6.4.3.2 and 6.4.3.3 [1]

Parameters

Taken from WiMAX Forum Mobile WiMAX System Profile [3].

Frame duration = 5ms

47 symbols per frame

- 26 in the downlink (1 for preamble)

- 21 in the uplink

102µs symbol duration

TTG > 50µs (RTD and Timing Advance dependant)

RTG = 60µs

SSTTG = SSRTG = 50µs

Receiver sensitivity and link adaptation

Number of MCS = 7

Permutation = PUSC

No HARQ

|  |  | Sensitivity (dBm) |  |  | Data rates (kbps) (per subchannel) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 5MHz |  | 10MHz |  | 20MHz |  |
|  | Eb/No (dB) | 5MHz | 10MHz | 20MHz | DL | UL | DL | UL | DL | UL |
| QPSK1/2 | 2.9 | -104.1 | -101.1 | -98.1 | 150.4 | 153.6 | 150.4 | 153.6 | 150.4 | 151.4 |
| QPSK3/4 | 6.3 | -101.8 | -98.8 | -95.8 | 225.6 | 230.4 | 225.6 | 230.4 | 225.6 | 227.1 |
| 16QAM1/2 | 8.6 | -97.7 | -94.7 | -91.7 | 300.8 | 307.2 | 300.8 | 307.2 | 300.8 | 302.8 |
| 16QAM3/4 | 12.7 | -96.6 | -93.6 | -90.6 | 451.2 | 460.8 | 451.2 | 460.8 | 451.2 | 454.2 |
| 64QAM1/2 | 13.8 | -93.5 | -90.5 | -87.5 | 451.2 | 460.8 | 451.2 | 460.8 | 451.2 | 454.2 |
| 64QAM2/3 | 16.9 | -92.4 | -89.4 | -86.4 | 601.6 | 614.4 | 601.6 | 614.4 | 601.6 | 605.6 |
| 64QAM3/4 | 18 | -90.5 | -87.5 | -84.5 | 676.8 | 691.2 | 676.8 | 691.2 | 676.8 | 681.3 |
| Subchannels available: |  |  |  |  | 24 | 17 | 48 | 35 | 96 | 70 |

Figure 4 802.16h sensitivity and data rate assumption taken from WiMAX Forum RCT [10].

Antenna configuration

SISO assumed unless otherwise stated.

LBT (Listen-Before-Talk) parameters

LBT applied: DL and UL

DL LBT slot location: slot 26

UL LBT slot location: slot 47

DL/UL LBT Threshold:

-72dBm (20MHz)

-75dBm (10MHz)

-78dBm (5MHz)

aEQP (Adaptive Extended Quiet Period) parameters

Initial EQP duty cycle: 10% on.

aEQP threshold 1 (Channel Occupancy test):

-77dBm (20MHz)

-80dBm (10MHz)

-83dBm (5MHz)

aEQP threshold 2 (User detection test):

-72dBm (20MHz)

-75dBm (10MHz)

-78dBm (5MHz)

## External driving parameters

VoIP traffic model

A VoIP traffic model is assumed due to assumptions about 802.11e Access Categories.

Figure 5 VoIP traffic model [11].

From [12]:

AMR with Header Compression IPv4(IPv6)

Voice payload:

Active: 33 bytes

Inactive: 7bytes

Protocol header:

2 bytes (4 bytes)

Fragmentation

Maximum transmission period of 4ms between medium sensing for 802.11y.

DL/UL load ratio (802.16h): 60/40

Other external driving parameters

Interference threshold: 10dB greater than the CCA-ED thresholds (bandwidth specific), i.e.

-72dBm (20MHz)

-75dBm (10MHz)

-78dBm (5MHz)

Maximum number of SS per AP/BS = 4 (802.16 and 802.11).

## Regulatory requirements

3.65-3.7GHz

Channel width = 5, 10, 20MHz

The FCC rules are documented in 47 CFR 90, Subpart Z – Wireless Broadband Services in the 3650-3700 MHz Band.

The current rule making proposes a Non-Exclusive Registration Use licensing mechanism for the entire 3650 – 3700 MHz band.  Licensees are required to registers their base stations online via FCC’s ULS.  And, they must delete the registrations for unused stations.  License terms are for 10 years.  Interference among base stations of different service providers are expected to be resolved among the providers themselves by ‘mutually satisfactory arrangements’.

The following technical requirements appear in 47 CFR 90 Sub-Part Z:

- 25 Watt EIRP maximum power in 25MHz bandwidth for Base and Fixed stations

- 1 Watt EIRP maximum power in 25MHz bandwidth for Mobile and Portable stations

- 1W / MHz EIRP maximum PSD for Base and Fixed stations

- 40mW / MHz EIRP maximum PSD for Mobile and Portable stations

- Sectorized antenna permitted only if each sector transmits different information

- Beamforming is subject to the 25 Watt EIRP requirement

- 43 + 10 Log(P) OOBE, with the 1% rule included

- Mobile stations may only transmit if they can decode an enabling signal from a base station

- Mobile stations may transmit to one another directly only if they can decode an enabling signal from a base station

- Airborne operation prohibited

- 150 km exclusion zone around FSS stations – unless agreed with the FSS licensee

- 80 km exclusion zones around following federal radiolocation stations

  - St. Inigoes, MD	38° 10’ 0”  N  ,  76° 23’ 0” W

  - Pascagoula, MS	30° 22’ 0”  N  ,  88° 29’ 0”  W

  - Pensacola, FL 	30° 21’ 28” N  ,  87° 16’ 26” W

- Fixed devices must be at least 8 / 56 km away from international borders if the antenna looks within 160° / 200° sector toward the border – unless coordinated with Mexico or Canada.

References

[1] IEEE P802.16h/D2: Air Interface for Fixed Broadband Wireless Access Systems Improved Coexistence Mechanisms for License-Exempt Operation, Draft Standard.

[2] IEEE P802.11y/D3.0: Draft STANDARD for Information Technology — Telecommunications and information exchange between systems— Local and metropolitan area networks- Specific requirements— Part 11:Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications: 3650-3700 MHz Operation in USA.

[3] WiMAX Forum Mobile System Profile Release 1.0 Approved Specification (Revision 1.2.2: 2006-11-17)

[4] IEEE 802.16.3c-01/29: Channel Models for Fixed Wireless Applications, Erceg et al., IEEE 802.16 Broadband Wireless Access Working Group, January 2001.

[5] IEEE Standard for Local and metropolitan area networks, Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems.

[6] Standard for Information Technology— Telecommunications and information exchange between systems— Local and metropolitan area networks— Specific requirements Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications.

[7] TGn channel models, IEEE 802.11-03/940r4, May 2004.

[8] Broadband Wireless Internet Forum White Paper, BWIF - Bringing Broadband Wireless Access Indoors, Document Number WP-4_TG-1, Version 1.0, September 25, 2001.

[9] Analysis of IEEE 802.11e for QoS Support in Wireless LANs, Mangold, Choi, Hiertz, Klein, Walke, IEEE Wireless Communications, December 2003.

[10] WiMAX Forum Mobile Radio Conformance Tests (MRCT), Release 1.0 Approved Specification (Revision 1.1.0).

[11] WiMAX Forum System Evaluation Methodology, Draft V0.8,10/17/2006.

[12] Mobile WiMAX VoIP Capacity System Level Simulations, Roshni Srinivasan, Tolis Papathanassiou, Shailender Timiri, Wireless Standards and Technology Mobility Group, Intel Corporation March 26, 2007.

Abbreviations

AC		Access Categories

aEQP		Adaptive Extended Quiet Period

AP		Access point

BPL		Building Penetration Loss

BS		Base Station

CCA-CS	Clear Channel Assessment – Carrier Sense

CCA-ED	Clear Channel Assessment – Energy Detect

DCS		Dynamic Channel Selection

EIRP		Effective Isotopic Radiated Power

EQP		Extended Quiet Period

LBT		Listen Before Talk

MCS		Modulation and Coding Schemes

PUSC		Partial Usage of SubChannels

SISO		Single Input Single Output

SS		Subscriber Station

STA		Subscriber STAtion

UCP		Uncoordinated Coexistence Protocol

Definitions

Base Station	A general term referring to both an 802.11 AP and 802.18 BS.

Subscriber Station	A general term referring to both an 802.11 STA and 802.18 SS.

Annex 1

802.11y uplink budget evaluation for aPropagationTime calculation

This analysis is used to calculate the maximum range of an 802.11y cell in the 3.65GHz band. This maximum range is used to calculate the maximum round trip time used in turn to evaluate 802.11y medium access parameters.

Assumptions

- Fixed subscriber (max. output power 10W (40dBm))

- Subscriber cabling/connector loss = 0.5dB

- Base Station antennas gain = 18dBi (Sectorized)

- Excluding: MIMO, diversity gains, HARQ,

| 802.11y |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Max STA EIRP | 5 | W | 5MHz channel |  |  |
|  |  | 37 | dBm |  |  |  |
|  | STA connector loss | 0.5 | dB |  |  |  |
|  | BS cabling loss | 3 | dB |  |  |  |
|  | BS antenna gain | 18 | dBi |  |  |  |
|  | Imp. Loss | 0 | dB |  |  |  |
|  | Rx sens. | -88 | dBm | 1.5Mbps | BPSK 1/2 |  |
|  | Max System Gain | 139.5 | dB |  |  |  |
|  |  |  |  | Pathloss Exponent | 2.5 |  |
|  | Free Space Pathloss | 139.1 | dB | Frequency | 3675 | MHz |
|  |  |  |  | Cell range | 6500 | m |