IEEE P802.19
Wireless Coexistence

| Parameters for simulation of Wireless Coexistence in the US 3.65GHz band |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-11-07 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Paul Piggin | NextWave Broadband | 12670 High Bluff Drive San Diego CA 92130 USA | 1 858 480 3100 | ppiggin @ nextwave.com |

Abstract

This document provides a set of assumptions and simulation parameters for use in modelling wireless coexistence in the 3.65GHz band for the US.

Notice: This document has been prepared to assist IEEE 802.19. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.19.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the TAG of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Shellhammer@ieee.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.19 TAG. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Table of contents

1	Table of contents	2

2	Introduction and document usage	3

3	Scenarios	3

4	Environmental factors	3

4.1	Large-scale fading	3

4.1.1	General	3

4.1.2	Outdoor-to-outdoor and outdoor-to-indoor model	4

4.1.3	Amendment for outdoor-to-indoor propagation model	4

4.1.4	Indoor model	4

4.2	Small-scale fading	4

4.2.1	Outdoor propagation	5

4.2.2	Indoor propagation	5

5	Device parameters	5

5.1	General	5

5.1.1	Nominal maximum antenna gain	5

5.1.2	Horizontal radiation patterns	5

5.1.3	Cabling loss	7

5.1.4	Maximum EIRP	7

5.1.5	Link adaptation	8

5.1.6	Interference thresholds for frame error calculation	8

5.2	802.11y specific parameters	8

5.2.1	General	9

5.2.2	802.11y feature support for 3.65GHz	9

5.2.3	Receiver sensitivity and link adaptation	9

5.2.4	Antenna configuration	9

5.2.5	CCA and medium access parameters	9

5.3	802.16h specific parameters	12

5.3.1	General	12

5.3.2	802.16h feature support for 3.65GHz	12

5.3.3	Parameters	13

5.3.4	Receiver sensitivity and link adaptation	14

5.3.5	Antenna configuration	14

5.3.6	aEQP (Adaptive Extended Quiet Period) parameters	14

6	External driving parameters	14

6.1	Traffic model	15

6.1.1	VoIP traffic model	15

6.1.2	Fragmentation	15

7	Regulatory requirements	15

8	References	17

9	Abbreviations	17

10	Definitions	18

11	Annex 1 - 802.11y uplink budget evaluation	18

12	Annex 2 - 802.11y recommended parameters for CX-CBP	19

12.1	Beacon period	19

12.2	Beacon transmission	19

12.3	Quiet intervals	19

# Introduction and document usage

This document provides simulation parameters for a number of defined scenarios for use in coexistence analysis between collocated or neighbouring 802.16h [1] and 802.11y [2] systems.

It is intended that parameters within the document are mandatory for comparative simulation studies.

Algorithms are optional. However some algorithms are suggested to facilitate comparisons specifically where the algorithm in question is not the main focus of a given study.

# Scenarios

The scenarios are described in this section with the following sections providing reference to which parameters are applicable to which scenario. It is possible to combine scenarios; it is also possible to combine scenarios where a scenario is specific to a given technology.

The scenarios are described thus:

Scenario A

Outdoor only scenario with fixed subscribers

This scenario considers an outdoor deployment with fixed outdoor subscribers only.

Scenario B

Outdoor only scenario with fixed and mobile subscribers

This scenario considers an outdoor deployment with fixed and mobile outdoor subscribers.

Scenario C

Outdoor to indoor scenario with portable and mobile subscribers

This scenario considers an outdoor deployment with indoor portable and mobile subscribers.

Scenario D

Indoor only scenario with portable and mobile subscribers

This scenario considers an indoor only deployment with indoor portable and mobile subscribers.

Radiation safety limits need to be observed in this scenario. The notional maximum transmitter power of 1W/MHz EIRP maximum PSD for Base stations is expected to require capping.

Scenario E

A combined scenario

This scenario considers a combination of all the above scenarios.

# Environmental factors

## Large-scale fading

This section describes the model for calculation of pathloss caused by obstructions within the propagation path from transmitter to receiver. These obstructions can be attributed to terrain and/or building clutter.

### General

Carrier frequency = 3675MHz (mid band)

### Outdoor-to-outdoor and outdoor-to-indoor model

The pathloss model is based on the Stanford University Interim (SUI) model [4]. This model is for outdoor operation with a correction for building penetration. In this document a set of propagation models applicable to the multi-cell architecture is presented. [4] gives as typical parameters for scenario the following:

- Cells are < 10 km in radius, variety of terrain and tree density types

- Under-the-eave/window or rooftop installed directional antennas (2 – 10 m) subscriber devices

- 15-40 m base station antennas

- High cell coverage requirement (80-90%)

The general pathloss expression is:

PathlossTotal = Pathlossoutdoor-outdoor + SM + BPL						[1]

Specific parameters to calculate Pathlossoutdoor-outdoor are:

Terrain type B (suburban)

Base Station height: 25m

Subscriber Station height:

Fixed Station: 10m

Portable Station: 2m

Mobile Station: 2m

Scenarios A, B:

Standard Deviation = 8-10dB, page 4 [4].

Shadow Margin (SM) = 6dB (90% area coverage, 75% at the cell edge)

Building Penetration Loss (BPL) = 0dB

### Amendment for outdoor-to-indoor propagation model

Scenarios C, E:

Standard Deviation = 10-12dB.

Shadow Margin (SM) = 8dB (90% area coverage, 75% at the cell edge)

Building Penetration Loss (BPL) = 6dB (at window), 12dB (elsewhere)

### Indoor model

Scenarios D, E:

The 802.11n indoor propagation model is applicable, clause 2 [7].

Base Station height: 2m

Subscriber Station height:

Portable Station: 2m

Mobile Station: 2m

## Small-scale fading

This section describes the model for calculation of fading caused by obstructions and movement of obstructions within the propagation environment.

### Outdoor propagation

Scenario A:

Fixed Station: 10m

Ricean K factor = 12dB (Fade margin = 2dB)

Scenarios B, C:

Portable Station: 2m

Mobile Station: 2m

Rayleigh fading.

Fade margin: 2Tx-2Rx=5dB, 2Tx-1Rx=10dB, 1Tx-1Rx=10-15dB.

### Indoor propagation

Scenarios D, E:

The 802.11n indoor propagation model is applicable, clause 2 [7].

# Device parameters

## General

### Nominal maximum antenna gain

Base Station (outdoor): 10dBi (omni directional)

Base Station (outdoor): 18dBi (directional)

Base Station (indoor): 0dBi (omni directional)

Subscriber Station:

Mobile Station: 2dBi (omni directional)

Portable Station: 2dBi (omni directional)

Fixed Station: 5dBi (omni directional)

Fixed Station: 8dBi (directional - indoor)

Fixed Station: 18dBi (directional - outdoor)

### Horizontal radiation patterns

This section is divided into the three antenna types specified above, namely:

- Directional base station,

- Directional subscriber station,

- Omni directional.

Each antenna type is described in turn using representative values. Specific scenarios can be addressed using the nominal maximum antenna gains given above.

Directional base station

A reference base station antenna is given in [11] and also [14]. For angle θ (relative to the boresight direction), the antenna has a gain (dB) of:

Gθ=Gmax+max−12θθ3dB2,−GFB						[2]

Where θ3dB is the 3dB beamwidth (70 degrees), θ≤180, Gmax is the maximum antenna gain (boresight direction, ~18 dBi) and GFB is the front-to-back ratio ( ~25 dBi).

This antenna is expected to be deployed in a 3 sector configuration, each sector covering 120 degrees. A diagrammatic representation is given in Figure 2.

Figure 1 An example sector beam pattern for base station (directional - outdoor) antenna [11] and [14].

Directional subscriber station

Maximum antenna gain Gmax = 18dBi.

3dB beamwidth, θ3dB =~ 20 degrees. This is in both azimuth and elevation planes.

Front-to-back ratio, GFB = 25dB.

A diagrammatic representation of an idealised radiation pattern is given in Figure 2.

Figure 2 An example radiation pattern for a directional subscriber station antenna.

Omni directional

Maximum antenna gain Gθ=Gmax.

Front-to-back ratio, GFB = 0dB.

Elevation discrimination ranges from ~6 degrees for higher gain base station antennas to upwards of ~45 degrees for lower gain subscriber antennas.

### Cabling loss

Base Station: 1dB (802.16), 1dB (802.11)

Subscriber Station (connector loss):

Fixed Station: 0.5dB (802.16), 0.5dB (802.11)

Portable Station: 0.5dB (802.16), 0.5dB (802.11)

Mobile Station: 0.5dB (802.16), 0.5dB (802.11)

### Maximum EIRP

Maximum EIRP values are based on maximum limits of Power Spectral Density (47 CFR 90, Subpart Z)

5 MHz:

Base/Fixed stations = 5W (37dBm) EIRP,

Mobile/Portable stations =200mW (23dBm) EIRP

10 MHz:

Base/Fixed stations =10W (40dBm) EIRP,

Mobile/Portable stations =400mW (26dBm) EIRP

20 MHz:

Base/Fixed stations =20W (43dBm) EIRP,

Mobile/Portable stations =800mW (29dBm) EIRP

NB 802.11 devices do not implement TPC so transmit power is at the maximum regulation permitted level and MCS.

### Link adaptation

Schemes for link adaptation are outside the scope of the standards on which this document draws. To facilitate comparisons between different simulations, not necessarily focusing on link adaptation, this section describes a basic fixed MCS selection link adaptation scheme that may be used.

#### Fixed MCS selection

The simplest scheme for link adaptation within a simulation is to provide a link budget assessment (with an appropriate margin) and the selection of the highest permissible MCS for the link based on pathloss and equipment parameters. For 802.16-based systems transmit power can be selected for a given MCS in the range of powers supported for that MCS. It should be noted that TPC is specified for 802.11-based systems but is not expected to be implemented in this simulation. In this case 802.11-based systems will transmit at maximum power for the selected MCS.

A 2dB margin is placed around MCS transition points. That is to say, if during MCS selection the link is within 2dB of the next highest modulation scheme the lower modulation scheme is selected. This ensures a link runs in an optimal state.

### Interference thresholds for frame error calculation

The interference threshold for frame error calculation is a method used to determine if a frame is received in error. The way to determine if a frame is in error is by simulating the wireless channel and the receiver. A simplified method may use the criteria below, which shall be true for the full duration of the received frame:

S(I+N)>SNR+FM									[3]

where:

S is the Signal power,

N is the Noise level,

I is the Interference level,

FM is the Fade margin.

## 802.11y specific parameters

### General

Based on the OFDM PHY for the 5GHz band (clause 17 [6]) specified in [2].

### 802.11y feature support for 3.65GHz

Maximum frame transmission duration between CCA = 4ms (subclause I.1 [2])

Use of an enabling signal DSE (subclause 11.14 [2])

RTS/CTS: enabled

### Receiver sensitivity and link adaptation

- Number of MCS = 8

- AWGN

Figure 3 Receiver sensitivities for OFDM 5GHz PHY [6]; NB an Implementation Margin of 5dB, and a Noise Figure of 10dB is implicitly assumed in these sensitivity values.

20MHz data rates: 54, 48, 36, 24, 18, 12, 9, 6 Mbps.

10MHz data rates: 27, 24, 18, 12, 9, 6, 4.5, 3 Mbps (for -3dB on sensitivity for 20MHz).

5MHz data rates: 13.5, 12, 9, 6, 4.5, 3, 2.25, 1.5 Mbps (for -6dB on sensitivity for 20MHz).

### Antenna configuration

SISO assumed unless otherwise stated.

### CCA and medium access parameters

Taken from table 147 [6].

Figure 4 Time domain representation of medium access for 802.11.

- HCF (Hybrid Coordination Function) is specified in the 802.11e amendment [6].

- HCF consists of EDCA (Enhanced Distributed Channel Access, distribution function) and HCCA (HCF Controlled Channel Access, centralized function).

- WMM (Wi-Fi Multimedia) certifies the EDCA and TXOP (Transmit Opportunity) features.

- EDCA and TXOP features enhance the QoS support in 802.11.

- EDCA introduces 4 AC (Access Categories) that prioritizes traffic class access to the air interface.

- TXOPs are used to provide a station with a time period in which to transmit in a non-contended manner.

Figure 5 Values for EDCA 4 AC parameters [9].

For analysis the highest priority (AC_VO) and one of the lowest priority (AC_BE) ACs is recommended for use.

Range dependency on propagation time

Subclause 7.3.2.9 Country information element of [6] states:

The Country information element contains the information required to allow a station to identify the regulatory domain in which the station is located and to configure its PHY for operation in that regulatory domain.

The Coverage Class field of the regulatory triplet specifies the aAirPropagationTime characteristic used in basic service set (BSS) operation, as shown in Table 27. The characteristic aAirPropagationTime describes variations in actual propagation time that are accounted for in a BSS and, together with maximum transmit power level, allow control of BSS diameter.

From Table 27 [6]:

Coverage class value = 0 for aAirPropagationTime ≤1µs

Coverage class value = 1 for aAirPropagationTime 3µs

Coverage class value = 2 for aAirPropagationTime 6µs

…

Coverage class value = 31 for aAirPropagationTime 93µs

Subclause 9.8.4 of [2] states:

Radio waves propagate at 300 m/μs in free space, and, for example, 3 μs would be the ceiling for BSS maximum one way distance of ~450 m (~900m round trip).

Scenarios A, B, C, E:

Outdoor case: Coverage class value = 6, (18µs), giving a cell radius of 2600m, round trip 5400m. The maximum cell radius as calculated in Annex 1.

Scenarios D, E:

Indoor case: Coverage class value = 0, (≤1µs), giving a cell radius of 150m, round trip 300m.

Timing values

Reference 9.2.10 DCF timing relations [6] and Figure 4.

(Legend 20MHz/10MHz/5MHz)

SIFS = 16/32/64 µs

AIFS[AC] = SIFS + AIFSN[AC].aSlotTime

From Figure 170 [6]

aSlotTime = aCCATime + aRTTXTurnaroundTime + aAirPropagationTime + aMACProcessingTime

aCCATime (CCA) = 4/8/16 µs

aMACProcessingDelay (M2) = 2/2/2 µs

aRXTXTurnaroundTime (Rx/Tx) = 2/2/2 µs

aAirPropagationTime (D2) = 	18 µs (Scenarios A, B, C, E) (all bandwidths)

aAirPropagationTime (D2) = 	1 µs (Scenarios D, E) (all bandwidths)

|  | AC_VO AIFSN[AC_VO] = 2 [Contention Window = 3, 7] | AC_BE AIFSN[AC_BE] = 3 [Contention Window = 15, 1023] |
| --- | --- | --- |
| Outdoor Scenarios A, B, C, E | aSlotTime = 26/30/38µs AIFS[AC_VO] = 68/92/140µs | aSlotTime = 26/30/38µs AIFS[AC_BE] = 94/122/178µs |
| Indoor Scenarios D, E | aSlotTime = 9/13/21µs AIFS[AC_VO] = 34/58/106µs | aSlotTime = 9/13/21µs AIFS[AC_BE] = 43/71/127µs |

Figure 6 aSlotTime and AIFS[AC] durations based on AC and deployment assumptions.

CCA-CS threshold

-82dBm (20MHz)

-85dBm (10MHz)

-88dBm (5MHz)

CCA-ED threshold

-72dBm (20MHz)

-75dBm (10MHz)

-78dBm (5MHz)

These thresholds are intended to be no higher (less sensitive) than the values stated.

These threshold values are referenced in the receiver after the receiving antenna and any associated connector/cabling losses. Probability of detection during sensing time > 90% in all cases.

## 802.16h specific parameters

### General

Based on the OFDMA PHY (subclause 8.4 [5]) and profiled in WiMAX Forum Mobile WiMAX System Profile [3] specified in [1].

### 802.16h feature support for 3.65GHz

[1] specifies a number of features for coexistence, these are described in this section.

#### Uncoordinated Coexistence Protocol (UCP)

Uncoordinated Coexistence Protocol (UCP) [1] is specified as a set of features to provide coexistence in the 3.65GHz band. UCP is specified in the following sub-clauses:

UCP (Uncoordinated Coexistence Protocol) subclause 6.4.2.3 [1]:

DCS (Dynamic Channel Selection) subclause 6.4.2.3.2 [1]

aEQP (Adaptive Extended Quiet Period) subclauses 6.4.3.2.5 and 6.4.3.2.6 [1]

LBT (Listen-Before-Talk) subclause 6.4.2.3.7 [1]

Some suggested parameters for the DMA (Dynamic Medium Acquisition) aspect of LBT 6.4.2.3.7 [1]. This is shown diagrammatically in Figure 7.

MINFRST = TCMA + TFRAME_END_OFFSET

TCMA = 4/8/16μs (20MHz/10MHz/5MHz channels)

TFRAME_END_OFFSET = 50μs (minimum time to switch Rx/Tx and send FRS (Frame Reservation Signal), all bandwidths)

MINFRST = 54/58/66μs (20MHz/10MHz/5MHz channels)

MAXFRST = 4000µs

K = 1

Figure 7 Representation of parameters used in DMA when an attempt is made to reclaimed the medium. DMA operation is shown for two distinct cases Tmedium_quiet > FRSTn and Tmedium_quiet < FRSTn . The timings in the figure are not to scale.

#### Coordinated Contention Based Protocol (CX-CBP)

Coordinated Contention Based Protocol (CX-CBP) [1] is specified as a set of features to provide coexistence in the 3.65GHz band. CX-CBP is specified and supported in the following sub-clauses:

CX-CBP (Coordinated Contention Based Protocol) subclause 15.4.2.1.3 [1]

Procedure flow in WirelessMAN-CX subclause 15.1.3 [1]

Provision for WirelessMAN-CX procedure subclause 15.2 [1]

Interference assessment and basic connectivity creation 15.3 [1]

Annex 2 details behavior, in the form of recommended parameters, required by 802.11 systems when working in a coexistence environment with CX-CBP. Specifically Annex 2 describes recommendations for required 802.11 quiet intervals which respect the 802.16 frame structure. 802.11 systems are also required to maintain beacon transmissions aligned with the 5ms frames of 802.16 systems.

### Parameters

Taken from WiMAX Forum Mobile WiMAX System Profile [3].

Frame duration = 5ms

47 symbols per frame

- 28 in the downlink (1 for preamble)

- 19 in the uplink

- Representing the nearest fit to a 60% downlink, 40% uplink asymmetry

103µs symbol duration

TTG > 50µs (RTD and Timing Advance dependant)

RTG = 60µs

SSTTG = SSRTG = 50µs

### Receiver sensitivity and link adaptation

- Number of MCS = 7

- Permutation = PUSC

- Coding = CTC

- No HARQ

- AWGN

|  |  | Sensitivity (dBm) |  |  | Data rates (kbps) (per subchannel) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 5MHz |  | 10MHz |  | 20MHz |  |
|  | Eb/No (dB) | 5MHz | 10MHz | 20MHz | DL | UL | DL | UL | DL | UL |
| QPSK1/2 | 2.9 | -91.1 | -88.1 | -85.1 | 150.4 | 153.6 | 150.4 | 153.6 | 150.4 | 151.4 |
| QPSK3/4 | 6.3 | -88.8 | -85.8 | -82.8 | 225.6 | 230.4 | 225.6 | 230.4 | 225.6 | 227.1 |
| 16QAM1/2 | 8.6 | -84.7 | -81.7 | -78.7 | 300.8 | 307.2 | 300.8 | 307.2 | 300.8 | 302.8 |
| 16QAM3/4 | 12.7 | -83.6 | -80.6 | -77.6 | 451.2 | 460.8 | 451.2 | 460.8 | 451.2 | 454.2 |
| 64QAM1/2 | 13.8 | -80.5 | -77.5 | -74.5 | 451.2 | 460.8 | 451.2 | 460.8 | 451.2 | 454.2 |
| 64QAM2/3 | 16.9 | -79.4 | -76.4 | -73.4 | 601.6 | 614.4 | 601.6 | 614.4 | 601.6 | 605.6 |
| 64QAM3/4 | 18 | -77.5 | -74.5 | -71.5 | 676.8 | 691.2 | 676.8 | 691.2 | 676.8 | 681.3 |
| Subchannels available: |  |  |  |  | 24 | 17 | 48 | 35 | 96 | 70 |

Figure 8 802.16h sensitivity and data rate assumption taken from WiMAX Forum RCT [10], NB An Implementation Margin of 5dB, and a Noise Figure of 8dB is implicitly assumed in these sensitivity values.

### Antenna configuration

SISO assumed unless otherwise stated.

### aEQP (Adaptive Extended Quiet Period) parameters

Initial EQP duty cycle: 10% on.

aEQP threshold 1 (Channel Occupancy test):

-77dBm (20MHz)

-80dBm (10MHz)

-83dBm (5MHz)

aEQP threshold 2 (User detection test):

-72dBm (20MHz)

-75dBm (10MHz)

-78dBm (5MHz)

These threshold values are referenced in the receiver after the receiving antenna and any associated connector/cabling losses. Probability of detection during sensing time > 90% in all cases.

# External driving parameters

External driving parameters are suggested in this section. Detail of these processes, external to the simulation, require description to enable a meaningful comparison of different simulations.

## Traffic model

This section provides information on traffic models used in simulation. The details herein are not intended to be exhaustive. Further background information is contained in [15].

### VoIP traffic model

A VoIP traffic model is assumed due to assumptions about 802.11e Access Categories.

Figure 9 VoIP traffic model [11].

From [12]:

AMR with Header Compression IPv4(IPv6)

Voice payload:

Active: 33 bytes

Inactive: 7bytes

Protocol header:

2 bytes (4 bytes)

### Fragmentation

Maximum transmission period of 4ms between medium sensing for 802.11y.

DL/UL load ratio (802.16h): 60/40

There are typically 4 to 10 SS per AP/BS (802.16 and 802.11).

# Regulatory requirements

3.65-3.7GHz

Channel width = 5, 10, 20MHz

The FCC rules are documented in 47 CFR 90, Subpart Z – Wireless Broadband Services in the 3650-3700 MHz Band [13].

The current rule making proposes a Non-Exclusive Registration Use licensing mechanism for the entire 3650 – 3700 MHz band.  Licensees are required to registers their base stations online via FCC’s ULS.  And, they must delete the registrations for unused stations.  License terms are for 10 years.  Interference among base stations of different service providers are expected to be resolved among the providers themselves by ‘mutually satisfactory arrangements’.

The following technical requirements appear in 47 CFR 90 Sub-Part Z:

- 25 Watt EIRP maximum power in 25MHz bandwidth for Base and Fixed stations

- 1 Watt EIRP maximum power in 25MHz bandwidth for Mobile and Portable stations

- 1W / MHz EIRP maximum PSD for Base and Fixed stations

- 40mW / MHz EIRP maximum PSD for Mobile and Portable stations

- Sectorized antenna permitted only if each sector transmits different information

- Beamforming is subject to the 25 Watt EIRP requirement

- 43 + 10 Log(P) OOBE, with the 1% rule included

- Mobile stations may only transmit if they can decode an enabling signal from a base station

- Mobile stations may transmit to one another directly only if they can decode an enabling signal from a base station

- Airborne operation prohibited

- 150 km exclusion zone around FSS stations – unless agreed with the FSS licensee

- 80 km exclusion zones around following federal radiolocation stations

  - St. Inigoes, MD	38° 10’ 0”  N  ,  76° 23’ 0” W

  - Pascagoula, MS	30° 22’ 0”  N  ,  88° 29’ 0”  W

  - Pensacola, FL 	30° 21’ 28” N  ,  87° 16’ 26” W

- Fixed devices must be at least 8 / 56 km away from international borders if the antenna looks within 160° / 200° sector toward the border – unless coordinated with Mexico or Canada.

# References

[1] IEEE P802.16h/D3: Air Interface for Fixed Broadband Wireless Access Systems Improved Coexistence Mechanisms for License-Exempt Operation, Draft Standard.

[2] IEEE P802.11y/D5.0: Draft STANDARD for Information Technology — Telecommunications and information exchange between systems— Local and metropolitan area networks- Specific requirements— Part 11:Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications: 3650-3700 MHz Operation in USA.

[3] WiMAX Forum Mobile System Profile Release 1.0 Approved Specification (Revision 1.2.2: 2006-11-17)

[4] IEEE 802.16.3c-01/29r4: Channel Models for Fixed Wireless Applications, Erceg et al., IEEE 802.16 Broadband Wireless Access Working Group, January 2001.

[5] IEEE Standard for Local and metropolitan area networks, Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems.

[6] Standard for Information Technology— Telecommunications and information exchange between systems— Local and metropolitan area networks— Specific requirements Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications.

[7] TGn channel models, IEEE 802.11-03/940r4, May 2004.

[8] Broadband Wireless Internet Forum White Paper, BWIF - Bringing Broadband Wireless Access Indoors, Document Number WP-4_TG-1, Version 1.0, September 25, 2001.

[9] Analysis of IEEE 802.11e for QoS Support in Wireless LANs, Mangold, Choi, Hiertz, Klein, Walke, IEEE Wireless Communications, December 2003.

[10] WiMAX Forum Mobile Radio Conformance Tests (MRCT), Release 1.0 Approved Specification (Revision 1.1.0).

[11] WiMAX Forum System Evaluation Methodology, Draft V1.0, 1/30/2007.

[12] Mobile WiMAX VoIP Capacity System Level Simulations, Roshni Srinivasan, Tolis Papathanassiou, Shailender Timiri, Wireless Standards and Technology Mobility Group, Intel Corporation March 26, 2007.

[13] Title 47 – Telecommunication, Part 90 – Private Land Mobile Radio Services: Subpart Z - Wireless Broadband Services in the 3650-3700 MHz Band, Part 90.1301 - 90.1337, Federal Communications Commission.

[14] 3GPP TR 25.814, 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Physical layer aspects for  evolved Universal Terrestrial Radio Access (UTRA). http://www.3gpp.org/ftp/Specs/html-info/25814.htm

[15] CDMA2000 Evaluation Methodology (C30-20061204-062A-Evaluation_methodology_text_V6), ftp://ftp.3gpp2.org/TSGC/Working/2006/2006-12-Maui/TSG-C-2006-12-Maui/WG3/

# Abbreviations

AC		Access Categories

aEQP		Adaptive Extended Quiet Period

AP		Access point

BPL		Building Penetration Loss

BS		Base Station

CBP		Contention Based Protocol

CCA-CS	Clear Channel Assessment – Carrier Sense

CCA-ED	Clear Channel Assessment – Energy Detect

DCS		Dynamic Channel Selection

EIRP		Effective Isotopic Radiated Power

EQP		Extended Quiet Period

LBT		Listen Before Talk

MCS		Modulation and Coding Schemes

PUSC		Partial Usage of SubChannels

SISO		Single Input Single Output

SS		Subscriber Station

STA		Subscriber STAtion

TPC		Transmit Power Control

UCP		Uncoordinated Coexistence Protocol

# Definitions

Base Station	A general term referring to both an 802.11 AP and 802.18 BS.

Subscriber Station	A general term referring to both an 802.11 STA and 802.18 SS.

TU	Time Unit. A measurement, specific to 802.11 systems, of a time interval equal to 1024 µs.

# Annex 1 - 802.11y uplink budget evaluation

802.11y uplink budget evaluation for aPropagationTime calculation

This analysis is used to calculate the maximum range of an 802.11y cell in the 3.65GHz band. This maximum range is used to calculate the maximum round trip time used in turn to evaluate 802.11y medium access parameters.

Assumptions

- Fixed subscriber (max. output power 5W (37dBm))

- Subscriber cabling/connector loss = 0.5dB

- Base Station antennas gain = 18dBi (Sectorized)

- Excluding: MIMO, diversity gains, HARQ,

| Max STA EIRP | 5 | W | 5MHz channel |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 37 | dBm |  |  |  |
| STA connector loss | 0.5 | dB |  |  |  |
| BS cabling loss | 1 | dB |  |  |  |
| BS antenna gain | 18 | dBi |  |  |  |
| Imp. Loss | 0 | dB |  |  |  |
| Rx sens. | -88 | dBm | 1.5Mbps | BPSK 1/2 |  |
| System Gain | 141.5 | dB |  |  |  |
|  |  |  | SUI parameters | Terrain B (suburban) |  |
|  |  |  | AP height | 25 | m |
|  |  |  | STA height | 10 | m |
| SUI Pathloss | 141.5 | dB | Frequency | 3675 | MHz |
|  |  |  | Cell range | 2600 | m |

# Annex 2 - 802.11y recommended parameters for CX-CBP

802.11y recommended parameters for Coordinated Contention Based Protocol (CX-CBP) [1] operation

## Beacon period

The beacon period should be equal to 117 TUs (Time Units). This is 119.8ms. This interval is selected to be similar to the preferred 100ms beacon period of 802.11 and be similar to the 20ms period of the CX-Frame [1].

## Beacon transmission

To occur at the start of the CXCBI [1] (when 802.16 is not scheduled to transmit). The beacon should appear randomly in one of the CXCBIs.

## Quiet intervals

Quiet intervals are scheduled so as to not overlap with the CXSBI intervals (11.9.2 Quieting channels for testing, and 7.3.2.23 Quiet Element [6]). The Quiet Elements should use the following parameters:

Quiet Element 1

Quiet count = 1 (start during next beacon)

Quiet period = 1 (in every beacon interval)

Quiet offset = 9 (9*1.024= 9.216ms)

Quiet duration = 11 (11*1024 = 11.26ms)

Quiet Element 2

Quiet count = 1

Quiet period = 1

Quiet offset = 29 (29*1.024= 29.7ms)

Quiet duration = 10 (10*1.024 = 10.24ms)

Quiet Element 3

Quiet count = 1

Quiet period = 1

Quiet offset = 48 (48*1.024= 49.152ms)

Quiet duration = 11 (11*1.024 = 11.26ms)

Quiet Element 4

Quiet count = 1

Quiet period = 1

Quiet offset = 68 (68*1.024= 69.63ms)

Quiet duration = 10 (10*1.024 = 10.24ms)

Quiet Element 5

Quiet count = 1

Quiet period = 1

Quiet offset = 87 (87*1024= 89.08ms)

Quiet duration = 11 (11*1.024 = 11.26ms)

Quiet Element 6

Quiet count = 1

Quiet period = 1

Quiet offset = 107 (107*1024= 109.57ms)

Quiet duration = 10 (10*1.024 = 10.24ms).

The quiet intervals related to the CX-Frame are shown in Figure 10.

Figure 10 The relationship between the 802.11 quiet intervals and the CX-Frame [1]