IEEE P802.19
Wireless Coexistence

| Parameters for simulation of Wireless Coexistence in the US 3.65GHz band |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-22-4 |  |  |  |  |
| Author: |  |  |  |  |
| Name | Company | Address | Phone | email |
| Paul Piggin | NextWave Wireless | NextWave Wireless Unit 7 Greenways Business Park Bellinger Close Chippenham, Wiltshire SN15 1BN, UK. | 44 1249 800 100 | ppiggin @ nextwave.com |

Abstract

This document provides a set of assumptions and simulation parameters for use in modelling wireless coexistence in the 3.65GHz band for the US.

Notice: This document has been prepared to assist IEEE 802.19. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.19.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the TAG of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Shellhammer@ieee.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.19 TAG. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

The following people were involved in the production of this document.

Technical Editor

Paul Piggin

Contributing Authors

Mariana Goldhamer

Shahar Hauzner

Eldad Perahia

Paul Piggin

Ahmed Sadek

Aryan Saed

Steve Shellhammer

Participants

Douglas Chan

Amir Ghasemi

Bruce Kramer

Dan Lubar

Shlomo Malka

Nat Natarajan

Ziv Nuss

Kathy Sohrabi

Ken Stanwood

Adrian Stephens

John Sydor

Darcy Swain

Steve Whitesell

Ariton Xhafa

# Table of contents

1	Table of contents	3

2	Document version release notes	4

3	Introduction and document usage	4

4	Scenarios	5

5	Environmental factors	5

5.1	Large-scale fading	5

5.1.1	General description of outdoor propagation model	5

5.1.2	Base Station to Subscriber Station propagation	6

5.1.3	Base Station to Base Station propagation	7

5.1.4	Subscriber Station to Subscriber Station propagation	7

5.1.5	Indoor model	8

5.2	Small-scale fading	8

5.2.1	Outdoor propagation	8

5.2.2	Indoor propagation	8

6	Device parameters	8

6.1	General	8

6.1.1	Channel bandwidth	8

6.1.2	Nominal antenna gains	9

6.1.3	Horizontal antenna radiation patterns	9

6.1.4	Cabling loss	12

6.1.5	Maximum EIRP	13

6.1.6	Typical EIRP	13

6.1.7	Power Control	13

6.1.8	Link adaptation	13

6.1.9	Interference thresholds for frame error calculation	14

6.2	802.11y specific parameters	14

6.2.1	General	14

6.2.2	802.11y feature support for 3.65GHz	14

6.2.3	Receiver sensitivity	14

6.2.4	CCA and medium access parameters	15

6.3	802.16h specific parameters	18

6.3.1	General	18

6.3.2	802.16h feature support for 3.65GHz	18

6.3.3	802.16 parameters	19

6.3.4	Receiver sensitivity	19

7	External driving parameters	20

7.1	Traffic model	20

7.1.1	Unimodal traffic model	20

7.1.2	Multimodal traffic model	21

7.1.3	VoIP traffic model	21

7.1.4	Fragmentation	22

8	Coexistence metrics	22

9	Regulatory requirements	22

10	References	24

11	Abbreviations	24

12	Definitions	25

13	Annex 1	25

14	Annex 2	27

14.1	Beacon period	27

14.2	Beacon transmission	27

14.3	Quiet intervals	27

# Document version release notes

| Date | Author | Action | Notes |
| --- | --- | --- | --- |
| 30 Nov 2007 | Paul Piggin | Creating r11 | Following from 802.19 CC 29 November 2007: > Addition of OFDMA subchannelisation gain in the 802.16h uplink budget. > Modification to the traffic model definition. > Addition of a coexistence metrics section ready to reference a companion coexistence metrics document. |
| 2 Jan 2008 | Paul Piggin | Creating r12 | Following from 802.19 CC 13 December 2007: > Modifications to section 6.1.7 and 6.1.8. > Modifications to section 7.1. > Added reference to Up-link OFDMA subchannel allocation. |
| 4 Jan 2008 | Paul Piggin | Creating r13 | Following from 802.19 CC 3 January 2008: > Add a specific reference for 802.16 in the references section and modified text accordingly. > Added in section 7.1.1. values for offered load based on the packet sizes and packet inter-arrival times. |
| April 2008 | Paul Piggin | Creating r14 | Incorporating resolved comments following Electronic Commenting contained in document 19-08-0007-0x-0000-compiled-comments-on-sim-and-metrics-documents. |

# Introduction and document usage

This document provides simulation parameters for a number of deployment scenarios for use in Coexistence Assurance analysis between 802.16h [1] and 802.11y [2] systems in the 3.65-3.7GHz band in the US.

In order to ensure a reasonably concise and representative set of simulation cases for Coexistence Assurance the scenarios in section 4 are marked as either mandatory or optional. A scenario marked as mandatory is required for inclusion in any Coexistence Assurance analysis, whilst an optional scenario can be used to provide additional supplementary information. Parameters within the document are either associated with scenario (in which case these parameters are implicitly mandatory or optional) or stand alone. The stand alone parameters are explicitly labelled as either mandatory or optional, and where appropriate to which scenarios they apply. In this way it is possible to narrow parameter selection for a given scenario where a number of possible parameters may apply.

All algorithms are optional. However some algorithms are suggested to facilitate comparisons specifically where the algorithm in question is not the main focus of a given study.

# Scenarios

The scenarios are described in this section with the following sections providing reference to which parameters are applicable to which scenario. The following scenarios apply to both 802.11y and 802.16h systems. It is possible to combine scenarios; it is also possible to combine scenarios where a scenario is specific to a given technology.

The scenarios are described thus:

Scenario A – MANDATORY

Outdoor only scenario with fixed subscribers

This scenario considers an outdoor deployment with fixed outdoor subscribers only.

Scenario B - OPTIONAL

Outdoor only scenario with mobile subscribers

This scenario considers an outdoor deployment with mobile outdoor subscribers.

Scenario C - MANDATORY

Outdoor to indoor scenario with mobile subscribers

This scenario considers an outdoor deployment with indoor mobile subscribers.

Scenario D - OPTIONAL

Indoor only scenario with portable and mobile subscribers

This scenario considers an indoor only deployment with indoor portable and mobile subscribers.

Radiation safety limits need to be observed in this scenario. The notional maximum transmitter power of 1W/MHz EIRP maximum PSD for Base stations is expected to require capping.

Scenario E - OPTIONAL

A combined scenario

This scenario considers a combination of all the above scenarios.

# Environmental factors

## Large-scale fading

This section describes the model for calculation of pathloss caused by obstructions within the propagation path from transmitter to receiver. These obstructions can be attributed to terrain and/or building clutter.

### General description of outdoor propagation model

The pathloss model used for studies in the outdoor-to-outdoor and outdoor-to-indoor environments is based on that discussed in [4]. This Unified Pathloss Model provides propagation assessment for the wanted signal from the Base Station to the Subscriber Station and also includes the impact of interfering signals:

- Base Station to Base Station,

- Base Station to Subscriber Station,

- Subscriber Station to Subscriber Station.

The expression to calculate pathloss is:

LHU(d)=69.55+26.16 log( fc)-13.82log( htx)-a(hrx)+(44.9-6.55 log( htx)log (d)	[1]

where fc is the carrier frequency is MHz, htx and hrx are the transmitting and receiving antenna heights in meters, respectively, and d is the separation distance in km. a(htx) denotes the mobile antenna correction factor and for frequencies larger than 300 MHz in large cities this is given by:

a(hrx)=3.2log11.75hrx2−4.97							[2]

For a suburban area the path loss is modified:

LHSU(d)=LHU(d)−2logfc282−5.4						[3]

LHSU(d) shall be used for coexistence studies. MANDATORY (Scenarios A, B, C, E)

Carrier frequency (fc) = 3675MHz (mid band) MANDATORY (Scenarios A - E)

The general pathloss expression is:

PathlossTotal = LHSU(d) + SM + BPL							[4]

Where SM is the applicable Shadow Margin and BPL is the applicable Building Penetration Loss.

### Base Station to Subscriber Station propagation

This section considers propagation between Base Station and Subscriber Station.

#### Outdoor-to-outdoor propagation model

The pathloss model is based that discussed in [4]. This model is for outdoor operation with a correction for building penetration.

Specific parameters to calculate LHSU(d) are:

Scenarios A, B, C (MANDATORY), Scenarios, E (OPTIONAL):

Base Station height (htx): 25m

Subscriber Station height:

Fixed Station (hrx): 10m Scenarios A only

Mobile Station (hrx): 2m Scenarios B, C only

Specific parameters to calculate PathlossTotal are:

Scenarios A (MANDATORY), B, E (OPTIONAL):

Standard Deviation = 8dB.

Shadow Margin (SM) = 6dB (90% area coverage, 75% at the cell edge)

Building Penetration Loss (BPL) = 0dB

#### Amendment for outdoor-to-indoor propagation

LHSU(d) is calculated as per section 5.1.2.1. Specific parameters to calculate PathlossTotal are:

Scenarios C (MANDATORY), E (OPTIONAL):

Standard Deviation = 10dB.

Shadow Margin (SM) = 8dB (90% area coverage, 75% at the cell edge)

Building Penetration Loss (BPL) = 12dB

### Base Station to Base Station propagation

This section considers propagation between Base Stations. Specific parameters to calculate LHSU(d) and PathlossTotal are:

Scenarios A, B, C (MANDATORY), Scenarios E (OPTIONAL):

Base Station height (htx and hrx): 25m

Standard Deviation = 0dB.

Building Penetration Loss (BPL) = 0dB

### Subscriber Station to Subscriber Station propagation

This section considers propagation between Subscriber Stations. Specific parameters to calculate LHSU(d) and PathlossTotal are:

Scenarios A (MANDATORY), Scenarios E (OPTIONAL):

Fixed Station (htx and hrx): 10m

Standard Deviation = 10dB.

Building Penetration Loss (BPL) = 0dB

Scenarios B, C (MANDATORY), Scenarios E (OPTIONAL):

Mobile Station (htx and hrx): 2m

Standard Deviation = 10dB.

Building Penetration Loss (BPL) = 24dB

### Indoor model

Scenarios D (MANDATORY), E (OPTIONAL):

The 802.11n indoor propagation model is applicable, clause 2 [8].

Base Station height: 2m

Subscriber Station height:

Portable Station: 2m

Mobile Station: 2m

## Small-scale fading

This section describes the model for calculation of fading caused by obstructions and movement of obstructions within the propagation environment.

### Outdoor propagation

Scenario A (MANDATORY):

Fixed Station: 10m

Ricean K factor = 12dB (Multipath fade margin = 2dB)

Scenarios B (OPTIONAL), C (MANDATORY):

Mobile Station: 2m

Rayleigh fading.

Multipath fade margin:

802.11 (1Tx/1Rx: Downlink + Uplink) = 13dB [Note: Selection combining derived]

802.16 (2Tx/2Rx: Downlink) = 5dB [Note: Combined array transmitted power not to exceed regulatory requirements]

802.16 (1Tx/2Rx: Uplink) = 10dB

### Indoor propagation

Scenarios D (OPTIONAL), E (OPTIONAL):

The 802.11n indoor propagation model is applicable, clause 2 [8].

# Device parameters

## General

### Channel bandwidth

The following channel bandwidths are supported:

5MHz – MANDATORY (Scenarios A - E)

10MHz – OPTIONAL (Scenarios A - E)

20MHz – OPTIONAL (Scenarios A - E)

[Note: 802.11 5MHz transmission mode is mandated as part of the standard [7]]

### Nominal antenna gains

Nominal bore sight antenna gains; for omni directional antennas this is the bore sight gain in elevation and for directional antennas the bore sight gain in azimuth and elevation.

Base Station:

Base Station (outdoor): 18dBi (directional) – OPTIONAL (Scenarios A, B, C, E)

Base Station (outdoor): 10dBi (omni directional) – MANDATORY (Scenarios A, B, C, E)

Base Station (indoor): 0dBi (omni directional) – MANDATORY (Scenarios D, E)

Subscriber Station:

Mobile Station: 2dBi (omni directional) – MANDATORY (Scenarios B, C, D, E)

Portable Station: 2dBi (omni directional - indoor) – OPTIONAL (Scenarios D, E)

Portable Station: 8dBi (directional - indoor) – MANDATORY (Scenarios D, E)

Fixed Station: 5dBi (omni directional) – OPTIONAL (Scenarios A, D, E)

Fixed Station: 18dBi (directional - outdoor) – MANDATORY (Scenarios A, E)

### Horizontal antenna radiation patterns

This section specifies three antenna types specified above namely:

- Directional base station,

- Directional subscriber station,

- Omni directional.

Each antenna type is described in turn using appropriate values. The equation for describing each antenna’s horizontal radiation pattern is presented in the remainder of this section.

A reference base station antenna is given in [12] and also [15]. For angle θ (relative to the boresight direction), the antenna has a gain (dB) of:

Gθ=Gmax+max−12θθ3dB2,−GFB						[5]

Where θ3dB is the 3dB beamwidth, θ≤180, Gmax is the maximum antenna gain (boresight direction) and GFB is the front-to-back ratio. This expression is used to represent both Base Station and Subscriber Station antennas.

#### Directional Outdoor Base Station antenna

These are the parameters for the Directional Outdoor Base Station antenna – OPTIONAL (Scenarios A, B, C, E).

Maximum antenna gain, Gmax = 18dBi.

3dB beamwidth, θ3dB = 70 degrees.

Front-to-back ratio, GFB = 25dB.

This antenna is expected to be deployed in a 3 sector configuration, each sector covering 120 degrees. A diagrammatic representation is given in Figure 1.

Figure 1 The sector beam pattern for Directional Outdoor Base Station antenna [12] and [15].

#### Directional Outdoor Subscriber Station antenna

These are the parameters for the Directional Outdoor Base Station antenna – MANDATORY (Scenarios A, E).

Maximum antenna gain Gmax = 18dBi.

3dB beamwidth, θ3dB = 40 degrees. This is in both azimuth and elevation planes.

Front-to-back ratio, GFB = 25dB.

A diagrammatic representation is given in Figure 2.

Figure 2 The beam pattern for Directional Outdoor Subscriber Station antenna.

#### Directional Indoor Subscriber Station antenna

These are the parameters for the Directional Outdoor Base Station antenna – MANDATORY (Scenarios D, E).

Maximum antenna gain Gmax = 8dBi.

3dB beamwidth, θ3dB = 40 degrees.

Front-to-back ratio, GFB = 15dB.

A diagrammatic representation is given in Figure 2.

Figure 3 The beam pattern for Directional Indoor Subscriber Station antenna.

#### Omni Directional Base Station and Subscriber Station antennas

These are the parameters for the Omni Directional Base Station and Subscriber Station antennas.

Maximum antenna gain, Gθ=Gmax.

Gmax = 10dBi: Base Station Outdoor – MANDATORY (Scenarios A, B, C, E).

Gmax = 0dBi: Base Station Indoor – MANDATORY (Scenarios D, E).

Gmax = 2dBi: Mobile Station – MANDATORY (Scenarios B, C, D, E).

Gmax = 2dBi: Portable Station Indoor – OPTIONAL (Scenarios D, E).

Gmax = 5dBi: Fixed Station Outdoor – OPTIONAL (Scenarios A, E).

Front-to-back ratio, GFB = 0dB – MANDATORY (Scenarios A - E).

Elevation discrimination ranges from ~6 degrees for higher gain base station antennas to upwards of ~45 degrees for lower gain subscriber station antennas.

### Cabling loss

Base Station: 1dB (802.16), 1dB (802.11) – MANDATORY (Scenarios A - E)

Subscriber Station (connector loss):

Fixed Station: 0.5dB (802.16), 0.5dB (802.11) – MANDATORY (Scenarios A, E)

Portable Station: 0.5dB (802.16), 0.5dB (802.11) – MANDATORY (Scenarios D, E)

Mobile Station: 0.5dB (802.16), 0.5dB (802.11) – MANDATORY (Scenarios B, C, D, E)

### Maximum EIRP

Maximum EIRP values are based on maximum limits of Power Spectral Density (47 CFR 90, Subpart Z)

5 MHz: – MANDATORY (Scenarios A - E)

Base/Fixed stations = 5W (37dBm) EIRP,

Mobile/Portable stations =200mW (23dBm) EIRP

10 MHz: – OPTIONAL (Scenarios A - E)

Base/Fixed stations =10W (40dBm) EIRP,

Mobile/Portable stations =400mW (26dBm) EIRP

20 MHz: – OPTIONAL (Scenarios A - E)

Base/Fixed stations =20W (43dBm) EIRP,

Mobile/Portable stations =800mW (29dBm) EIRP

### Typical EIRP

Typical transmit powers are presented in this section.

5 MHz: – MANDATORY (Scenarios A - E)

802.16 outdoor Base Station transmits at maximum power.

802.11 outdoor Base Station transmits at maximum power.

802.16 and 802.11 Fixed, Portable and Mobile subscribers transmit at 17dBm.

10 MHz: – OPTIONAL (Scenarios A - E)

802.16 outdoor Base Station transmits at maximum power.

802.11 outdoor Base Station transmits at maximum power.

802.16 and 802.11 Fixed, Portable and Mobile subscribers transmit at 20dBm.

20 MHz: – OPTIONAL (Scenarios A - E)

802.16 outdoor Base Station transmits at maximum power.

802.11 outdoor Base Station transmits at maximum power.

802.16 and 802.11 Fixed, Portable and Mobile subscribers transmit at 23dBm.

### Power Control

802.16-based systems shall follow the power control rules as described in clause 8.4.10 of [5] and as amended in [6].

### Link adaptation

Link adaptation is intended to maintain a consistent power density at the Base Station by setting an appropriate modulation and FEC rate at each subscriber station. The Base Station may change the subscriber station’s transmit power through power correction messages.

The simplest scheme for link adaptation within a simulation is to provide a link budget assessment (with an appropriate margin) and the selection of the highest permissible MCS for the link based on pathloss and equipment parameters. For 802.16-based systems transmit power may be selected for a given MCS in the range of powers supported for that MCS. It should be noted that TPC is specified for 802.11-based systems but is not expected to be implemented in this simulation. In this case 802.11-based systems will transmit at maximum power for the selected MCS.

For 802.16-based systems MCS selection shall be implemented as described in [18]. This is part of the uplink OFDMA subchannel allocation scheme.

A 2dB margin (MANDATORY (Scenarios A - E)) shall be imposed at the MCS transition points for both 802.16 and 802.11 systems. That is to say, if during MCS selection the link is within 2dB of the next highest modulation scheme the lower modulation scheme shall be selected. This ensures a link runs in an optimal state.

### Interference thresholds for frame error calculation

The interference threshold for frame error calculation is a method used to determine if a frame is received in error. The way to determine if a frame is in error is by simulating the wireless channel and the receiver. A simplified method may use the criteria below, which shall be true for the full duration of the received frame:

S(I+N)>SNR+FM									[6]

where:

S is the Signal power,

N is the Noise level,

I is the Interference level,

FM is the interference Fade margin. The value is 2dB – MANDATORY (Scenarios A - E)

## 802.11y specific parameters

### General

Based on the OFDM PHY for the 5GHz band (clause 17 [7]) specified in [2].

### 802.11y feature support for 3.65GHz

Maximum frame transmission duration between CCA is 4ms (subclause I.1 [2]). The maximum transmission of 4ms and is either a single frame or multiple frames for the case of EDCA that is covered by a TXOP.

Use of an enabling signal DSE (subclause 11.14 [2])

RTS/CTS: enabled

### Receiver sensitivity

- Number of MCS = 8

- AWGN

Figure 4 Typical receiver sensitivities for OFDM 5GHz PHY [7]; Note: an Implementation Margin of 5dB, and a Noise Figure of 10dB is implicitly assumed in these sensitivity values.

20MHz data rates: 54, 48, 36, 24, 18, 12, 9, 6 Mbps – OPTIONAL (Scenarios A - E)

10MHz data rates: 27, 24, 18, 12, 9, 6, 4.5, 3 Mbps (for -3dB on sensitivity for 20MHz) – OPTIONAL (Scenarios A - E)

5MHz data rates: 13.5, 12, 9, 6, 4.5, 3, 2.25, 1.5 Mbps (for -6dB on sensitivity for 20MHz) – MANDATORY (Scenarios A - E)

### CCA and medium access parameters

Taken from table 147 [7].

Figure 5 Time domain representation of medium access for 802.11.

- HCF (Hybrid Coordination Function) is specified in the 802.11e amendment [7].

- HCF consists of EDCA (Enhanced Distributed Channel Access, distribution function) and HCCA (HCF Controlled Channel Access, centralized function).

- WMM (Wi-Fi Multimedia) certifies the EDCA and TXOP (Transmit Opportunity) features.

- EDCA and TXOP features enhance the QoS support in 802.11.

- EDCA introduces 4 AC (Access Categories) that prioritizes traffic class access to the air interface.

- TXOPs are used to provide a station with a time period in which to transmit in a non-contended manner.

Figure 6 Values for EDCA 4 AC parameters [10].

Access Categories:

AC_VO (Highest priority) OPTIONAL (Scenarios A - E)

AC_VI OPTIONAL (Scenarios A - E)

AC_BE MANDATORY (Scenarios A - E)

AC_BK (Lowest priority) OPTIONAL (Scenarios A - E)

Range dependency on propagation time

Subclause 7.3.2.9 Country information element of [7] states:

The Country information element contains the information required to allow a station to identify the regulatory domain in which the station is located and to configure its PHY for operation in that regulatory domain.

The Coverage Class field of the regulatory triplet specifies the aAirPropagationTime characteristic used in basic service set (BSS) operation, as shown in Table 27. The characteristic aAirPropagationTime describes variations in actual propagation time that are accounted for in a BSS and, together with maximum transmit power level, allow control of BSS diameter.

From Table 27 [7]:

Coverage class value = 0 for aAirPropagationTime ≤1µs

Coverage class value = 1 for aAirPropagationTime 3µs

Coverage class value = 2 for aAirPropagationTime 6µs

…

Coverage class value = 31 for aAirPropagationTime 93µs

Subclause 9.8.4 of [2] states:

Radio waves propagate at 300 m/μs in free space, and, for example, 3μs would be the ceiling for BSS maximum one way distance of ~450 m (~900m round trip).

MANDATORY Scenarios A, B, E:

Outdoor case: Coverage class value = 6, (18µs), giving a cell radius of 2300m, round trip 4600m (15.3µs). The maximum cell radius as calculated in Annex 1.

MANDATORY Scenarios C, E:

Outdoor case: Coverage class value = 1, (3µs), giving a cell radius of 250m, round trip 500m (1.7µs). The maximum cell radius as calculated in Annex 1.

MANDATORY Scenarios D, E:

Indoor case: Coverage class value = 0, (≤1µs), giving a cell radius of 150m, round trip 300m.

Timing values

Reference 9.2.10 DCF timing relations [7] and Figure 5.

(Legend 20MHz (OPTIONAL (Scenarios A - E))/10MHz (OPTIONAL (Scenarios A - E))/5MHz (MANDATORY (Scenarios A - E)))

SIFS = 16/32/64µs

AIFS[AC] = SIFS + AIFSN[AC].aSlotTime

From Figure 170 [7]

aSlotTime = aCCATime + aRTTXTurnaroundTime + aAirPropagationTime + aMACProcessingTime

aCCATime (CCA) = 4/8/16µs

aMACProcessingDelay (M2) = 2/2/2µs

aRXTXTurnaroundTime (Rx/Tx) = 2/2/2µs

aAirPropagationTime (D2) = 	18µs – MANDATORY (Scenarios A, B, E) (all bandwidths)

aAirPropagationTime (D2) = 	3µs – MANDATORY (Scenarios C, E) (all bandwidths)

aAirPropagationTime (D2) = 	1µs – MANDATORY (Scenarios D, E) (all bandwidths)

|  | AC_BE AIFSN[AC_BE] = 3 [Contention Window = 15, 1023] |
| --- | --- |
| Outdoor MANDATORY Scenarios A, B, E | aSlotTime = 26/30/38µs AIFS[AC_BE] = 94/122/178µs |
| Outdoor MANDATORY Scenarios C, E | aSlotTime = 11/15/23µs AIFS[AC_BE] = 49/77/133µs |
| Indoor MANDATORY Scenarios D, E | aSlotTime = 9/13/21µs AIFS[AC_BE] = 43/71/127µs |

Figure 7 aSlotTime and AIFS[AC] durations based on deployment assumptions.

CCA-CS threshold

-82dBm (20MHz) – OPTIONAL (Scenarios A - E)

-85dBm (10MHz) – OPTIONAL (Scenarios A - E)

-88dBm (5MHz) – MANDATORY (Scenarios A - E)

CCA-ED threshold

-72dBm (20MHz) – OPTIONAL (Scenarios A - E)

-75dBm (10MHz) – OPTIONAL (Scenarios A - E)

-78dBm (5MHz) – MANDATORY (Scenarios A - E)

These thresholds are intended to be no higher (less sensitive) than the values stated.

These threshold values are referenced in the receiver after the receiving antenna and any associated connector/cabling losses. Probability of detection during sensing time > 90% in all cases.

## 802.16h specific parameters

### General

Based on the OFDMA PHY (subclause 8.4 [5] and as amended in [6]) and profiled in WiMAX Forum Mobile WiMAX System Profile [3] specified in [1].

### 802.16h feature support for 3.65GHz

[1] specifies a number of features for coexistence, these are described in this section.

#### Uncoordinated Coexistence Protocol (UCP)

Uncoordinated Coexistence Protocol (UCP) [1] is specified as a set of features that provide coexistence in the 3.65GHz band. The detailed specification of UCP (6.4.1.3 [1]) is given in the following sub-clauses:

- Dynamic Channel Selection (DCS) subclause 6.4.1.3.2 (and 6.4.1.3.1) [1]

- Frame structure and frame allocation subclause 6.4.1.3.3 [1]

- Listen-Before-Talk with Dynamic Medium Acquisition (DMA) subclause 6.4.1.3.4 [1]

The UCP feature is captured in the System Profile ProfX_UC 12.5.1.1. [1].

Frame structure and frame allocation subclause 6.4.1.3.3 [1] references the use of the Dynamic Medium Acquisition Discovery Protocol as detailed in subclause 6.4.1.3.4.2 [1]. This Discovery Protocol shall be used in this realization of UCP as a means of providing frame sharing for 802.16-to-802.16 and 802.16-to-802.11.

The Dynamic Medium Acquisition Discovery Protocol (6.4.1.3.4.2 [1]) refines the DMA algorithm for 802.16-to-802.16 coexistence, as described in subclause 6.4.1.3.4.1 [1]. The DMA algorithm, whose responsibility it is to facilitate fair sharing of the medium, is described in general terms in 6.4.1.3.4 [1]. Uplink subframe symbols may be taken, if required in the DMA region (6.4.1.3.4.1 [1]), to facilitate coexistence based on the other systems sharing the band.

Thresholds for detection are given in 6.4.1.3.4.1 [1]. The thresholds are intended to be no higher (less sensitive) than the values stated and are referenced in the receiver after the receiving antenna and any associated connector/cabling losses. Probability of detection during sensing time > 90% in all cases.

#### Coordinated Contention Based Protocol (CX-CBP)

Coordinated Contention Based Protocol (CX-CBP) [1] is specified as a set of features to provide coexistence in the 3.65GHz band. CX-CBP is specified and supported in the following sub-clauses:

CX-CBP (Coordinated Contention Based Protocol) subclause 15.4.1.4 [1]

Provision for WirelessMAN-CX procedure subclause 15.2 [1]

Annex 2 details behavior, in the form of recommended parameters, required by 802.11 systems only when working in a synchronized coexistence environment with CX-CBP. Specifically Annex 2 describes recommendations for required 802.11 quiet intervals which respect the 802.16 frame structure. 802.11 systems are also required to maintain beacon transmissions aligned with the 5ms frames of 802.16 systems.

### 802.16 parameters

MANDATORY (Scenarios A - E) Taken from WiMAX Forum Mobile WiMAX System Profile [3].

Frame duration = 5ms

47 symbols per frame

- 28 in the downlink (1 for preamble)

- 19 in the uplink

- Representing the nearest fit to a 60% downlink, 40% uplink asymmetry

103µs symbol duration

TTG > 50µs (RTD and Timing Advance dependant)

RTG = 60µs

SSTTG = SSRTG = 50µs

### Receiver sensitivity

- Number of MCS = 7

- Permutation = PUSC

- Coding = CTC

- No HARQ

- AWGN

|  |  | Sensitivity (dBm) |  |  | Data rates (kbps) (per subchannel) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 5MHz |  | 10MHz |  | 20MHz |  |
|  | Eb/No (dB) | 5MHz | 10MHz | 20MHz | DL | UL | DL | UL | DL | UL |
| QPSK1/2 | 2.9 | -91.1 | -88.1 | -85.1 | 150.4 | 153.6 | 150.4 | 153.6 | 150.4 | 151.4 |
| QPSK3/4 | 6.3 | -88.8 | -85.8 | -82.8 | 225.6 | 230.4 | 225.6 | 230.4 | 225.6 | 227.1 |
| 16QAM1/2 | 8.6 | -84.7 | -81.7 | -78.7 | 300.8 | 307.2 | 300.8 | 307.2 | 300.8 | 302.8 |
| 16QAM3/4 | 12.7 | -83.6 | -80.6 | -77.6 | 451.2 | 460.8 | 451.2 | 460.8 | 451.2 | 454.2 |
| 64QAM1/2 | 13.8 | -80.5 | -77.5 | -74.5 | 451.2 | 460.8 | 451.2 | 460.8 | 451.2 | 454.2 |
| 64QAM2/3 | 16.9 | -79.4 | -76.4 | -73.4 | 601.6 | 614.4 | 601.6 | 614.4 | 601.6 | 605.6 |
| 64QAM3/4 | 18 | -77.5 | -74.5 | -71.5 | 676.8 | 691.2 | 676.8 | 691.2 | 676.8 | 681.3 |
| Subchannels available: |  |  |  |  | 24 | 17 | 48 | 35 | 96 | 70 |

Figure 8 802.16h sensitivity and data rate typical values taken from WiMAX Forum RCT [11], Note: An Implementation Margin of 5dB, and a Noise Figure of 8dB is implicitly assumed in these sensitivity values.

Note: 20MHz channels – OPTIONAL (Scenarios A - E), 10MHz channels – OPTIONAL (Scenarios A - E), 5MHz channels – MANDATORY (Scenarios A - E).

Uplink OFDMA subchannel allocation for 802.16h systems is described in [18].

# External driving parameters

External driving parameters are suggested in this section. Detail of these processes, external to the simulation, require description to enable a meaningful comparison of different simulation cases. The traffic model is the only way to specify the offered load; the 802.11y and 802.16h systems will try to transmit this offered load according to the 802.11y and 802.16h protocols. No additional traffic shaping will be assumed.

## Traffic model

This section provides information on traffic models used in simulation. The details herein are not intended to be exhaustive. Further background information is contained in [16]. It is assumed throughout that the packet sizes are defined at the network layer 3; appropriate MAC overheads are required where applicable.

802.11y ACK SDU offered to the PHY 14bytes: MANDATORY (Scenarios A - E)

The traffic models included are summarized thus:

Unimodal traffic model (section 7.1.1):

Fixed packet size: MANDATORY (Scenarios A - E)

Packet size derived from a uniformly distribution random variable: OPTIONAL (Scenarios A - E)

Fixed inter arrival time: MANDATORY (Scenarios A - E)

Inter arrival time derived from a uniformly distribution random variable: OPTIONAL (Scenarios A - E)

Multimodal traffic model (section 7.1.2):

OPTIONAL (Scenarios A - E)

VoIP traffic model (section 7.1.3):

OPTIONAL (Scenarios A - E)

Fragmentation (section 7.1.4):

MANDATORY (Scenarios A - E)

The number of subscribers per AP/BS (802.16 and 802.11):

4: MANDATORY (Scenarios A - E)

Otherwise: OPTIONAL (Scenarios A - E)

The traffic models are described in more detail in the following sections.

### Unimodal traffic model

A unimodal traffic model based on a single packet size is defined.

| Probability (%) | Packet size (bytes) |
| --- | --- |
| 100 | 1500 |

Figure 9 Unimodal traffic model based on Internet traffic observations.

Packet inter arrival time varies as required and defines the offered traffic load. Suitable inter arrival times are 100ms, 20ms, 5ms, 1.25ms.

| Inter arrival time (ms) | Offered load (kbps) | MANDATORY (Scenarios A - E) |
| --- | --- | --- |
| 100 | 120 | Yes |
| 20 | 600 | Yes |
| 5 | 2400 | Yes |
| 1.25 | 9600 | Yes |

Figure 10 Packet inter arrival times with corresponding values of offered load.

### Multimodal traffic model

A multimodal traffic model based on aggregated internet traffic observations is defined. The observations are from one day’s CAIDA (www.caida.org) based measurements within a municipal wi-fi mesh network.

| Probability (%) | Packet size (bytes) |
| --- | --- |
| 20 | 1500 |
| 20 | 554 (majority MTU discovery) |
| 60 | 114 |

Figure 11 Multimodal traffic model based on Internet traffic observations.

Packet inter arrival time varies as required and defines the offered traffic load. Suitable inter arrival times are 100ms, 75ms, 50ms, 25ms, 20ms, 10ms, 7.5ms, 5ms, 2.5ms, 2ms, 1ms, 0.75ms, 0.5ms.

### VoIP traffic model

A VoIP traffic model is suitable for the appropriate 802.11e Access Category.

Figure 12 VoIP traffic model [12].

From [13]:

AMR with Header Compression IPv4(IPv6)

Voice payload:

Active: 33 bytes

Inactive: 7bytes

Protocol header:

2 bytes (4 bytes)

### Fragmentation

Maximum transmission period of 4ms between medium sensing for 802.11y.

# Coexistence metrics

Coexistence metrics are discussed in [17]. This section makes reference to this document in which is stated those metrics that shall be MANDATORY and which OPTIONAL for the purpose of Coexistence Assurance analysis.

# Regulatory requirements

3.65-3.7GHz

Channel width = 5, 10, 20MHz

The FCC rules are documented in 47 CFR 90, Subpart Z – Wireless Broadband Services in the 3650-3700 MHz Band [14].

The current rule making proposes a Non-Exclusive Registration Use licensing mechanism for the entire 3650 – 3700 MHz band.  Licensees are required to registers their base stations online via FCC’s ULS.  And, they must delete the registrations for unused stations.  License terms are for 10 years.  Interference among base stations of different service providers are expected to be resolved among the providers themselves by ‘mutually satisfactory arrangements’.

The following technical requirements appear in 47 CFR 90 Sub-Part Z:

- 25 Watt EIRP maximum power in 25MHz bandwidth for Base and Fixed stations

- 1 Watt EIRP maximum power in 25MHz bandwidth for Mobile and Portable stations

- 1W / MHz EIRP maximum PSD for Base and Fixed stations

- 40mW / MHz EIRP maximum PSD for Mobile and Portable stations

- Sectorized antenna permitted only if each sector transmits different information

- Beamforming is subject to the 25 Watt EIRP requirement

- 43 + 10 Log(P) OOBE, with the 1% rule included

- Mobile stations may only transmit if they can decode an enabling signal from a base station

- Mobile stations may transmit to one another directly only if they can decode an enabling signal from a base station

- Airborne operation prohibited

- 150 km exclusion zone around FSS stations – unless agreed with the FSS licensee

- 80 km exclusion zones around following federal radiolocation stations

  - St. Inigoes, MD	38° 10’ 0”  N  ,  76° 23’ 0” W

  - Pascagoula, MS	30° 22’ 0”  N  ,  88° 29’ 0”  W

  - Pensacola, FL 	30° 21’ 28” N  ,  87° 16’ 26” W

- Fixed devices must be at least 8 / 56 km away from international borders if the antenna looks within 160° / 200° sector toward the border – unless coordinated with Mexico or Canada.

# References

[1] IEEE P802.16h/D4: Air Interface for Fixed Broadband Wireless Access Systems Improved Coexistence Mechanisms for License-Exempt Operation, Draft Standard.

[2] IEEE P802.11y/D5.0: Draft STANDARD for Information Technology — Telecommunications and information exchange between systems— Local and metropolitan area networks- Specific requirements— Part 11:Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications: 3650-3700 MHz Operation in USA.

[3] WiMAX Forum Mobile System Profile Release 1.0 Approved Specification (Revision 1.2.2: 2006-11-17)

[4] IEEE P802.19-08/0006, A Unified Path-Loss Model for Coexistence Study, IEEE P802.19 Coexistence TAG, Ahmed Sadek, Stephen J. Shellhammer.

[5] IEEE Standard for Local and metropolitan area networks, Part 16: Air Interface for Fixed Broadband Wireless Access Systems. IEEE Std 802.16-2004; October 2004.

[6] IEEE Standard for Local and metropolitan area networks, Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems. Amendment 2: Physical and Medium Access Control Layers for Combined Fixed and Mobile Operation in Licensed Bands. IEEE Std 802.16e-2005 and IEEE Std 801.6-2004/Cor1-2005; February 2006.

[7] Standard for Information Technology— Telecommunications and information exchange between systems— Local and metropolitan area networks— Specific requirements Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications.

[8] TGn channel models, IEEE 802.11-03/940r4, May 2004.

[9] Broadband Wireless Internet Forum White Paper, BWIF - Bringing Broadband Wireless Access Indoors, Document Number WP-4_TG-1, Version 1.0, September 25, 2001.

[10] Analysis of IEEE 802.11e for QoS Support in Wireless LANs, Mangold, Choi, Hiertz, Klein, Walke, IEEE Wireless Communications, December 2003.

[11] WiMAX Forum Mobile Radio Conformance Tests (MRCT), Release 1.0 Approved Specification (Revision 1.1.0).

[12] WiMAX Forum System Evaluation Methodology, Draft V1.0, 1/30/2007.

[13] Mobile WiMAX VoIP Capacity System Level Simulations, Roshni Srinivasan, Tolis Papathanassiou, Shailender Timiri, Wireless Standards and Technology Mobility Group, Intel Corporation March 26, 2007.

[14] Title 47 – Telecommunication, Part 90 – Private Land Mobile Radio Services: Subpart Z - Wireless Broadband Services in the 3650-3700 MHz Band, Part 90.1301 - 90.1337, Federal Communications Commission.

[15] 3GPP TR 25.814, 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Physical layer aspects for  evolved Universal Terrestrial Radio Access (UTRA). http://www.3gpp.org/ftp/Specs/html-info/25814.htm

[16] CDMA2000 Evaluation Methodology (C30-20061204-062A-Evaluation_methodology_text_V6), ftp://ftp.3gpp2.org/TSGC/Working/2006/2006-12-Maui/TSG-C-2006-12-Maui/WG3/

[17] IEEE P802.19-07/0020, Coexistence Metrics for the 3650 MHz Band, IEEE P802.19 Coexistence TAG, Stephen J. Shellhammer.

[18] IEEE P802.19-07/0027, Up-link OFDMA sub-channel allocation for Wireless Coexistence in US 3.65GHz band, IEEE P802.19 Coexistence TAG, Shahar Hauzner, Mariana Goldhamer.

# Abbreviations

AC		Access Categories

aEQP		Adaptive Extended Quiet Period

AP		Access point

BPL		Building Penetration Loss

BS		Base Station

CBP		Contention Based Protocol

CCA-CS	Clear Channel Assessment – Carrier Sense

CCA-ED	Clear Channel Assessment – Energy Detect

DCS		Dynamic Channel Selection

DMA		Dynamic Medium Acquisition

EIRP		Effective Isotopic Radiated Power

EQP		Extended Quiet Period

FEC		Forward Error Correction

FER		Frame Error Rate

LBT		Listen Before Talk

MCS		Modulation and Coding Schemes

MTU		Maximum Transmission Unit

OFDMA	Orthogonal Frequency Division Multiple Access

PDU		Protocol Data Unit

PUSC		Partial Usage of SubChannels

SDU		Service Data Unit

SISO		Single Input Single Output

SS		Subscriber Station

STA		Subscriber STAtion

TPC		Transmit Power Control

TXOP		Transmit OPportunity

UCP		Uncoordinated Coexistence Protocol

# Definitions

Base Station	A general term referring to both an 802.11 AP and 802.18 BS.

Subscriber Station	A general term referring to both an 802.11 STA and 802.18 SS.

TU	Time Unit. A measurement, specific to 802.11 systems, of a time interval equal to 1024 µs.

# Annex 1

802.16h & 802.11y uplink budget evaluation for Scenarios A and C

This analysis is used to calculate the maximum range of an 802.16h and 802.11y cell in the 3.65GHz band. This maximum range is used to calculate the maximum round trip time used in turn to evaluate 802.11y medium access parameters via the term aPropagationTime; the 802.16h uplink budget is for information and comparison purposes.

Assumptions

- Subscriber (max. output power 50mW (17dBm))

- Subscriber connector loss = 0.5dB

- Base Station cabling loss = 1dB

- Base Station antennas gain = 10dBi (Omni directional)

- Multipath fade margins for Scenario A:

  - 802.11y (1Tx/1Rx: Uplink) = 2dB

  - 802.16h (1Tx/2Rx: Uplink) = 2dB

- Multipath fade margins for Scenario C:

  - 802.11y (1Tx/1Rx: Uplink) = 13dB

  - 802.16h (1Tx/2Rx: Uplink) = 10dB

- The OFDMA PHY of 802.16h provides uplink subchannelization gain. At the cell edge it is assumed that for the 5MHz channel case 1 of the possible 17 subchannels are used. This equates to an effective 10log(17)=12.3dB gain.

- The System Gain is used to calculate an appropriate cell range based on the SUI propagation model [5.1.2.1] and using the parameters defined. In this evaluation Shadow Margin is not applied.

| Parameter | 802.16h | 802.11y |  |  | Note |
| --- | --- | --- | --- | --- | --- |
| Subscriber output power [x] | 17 | 17 | dBm |  | 5MHz channel |
| Subscriber antenna gain [y] | 18 | 18 | dBi |  |  |
| Subscriber cabling loss [z] | 0.5 | 0.5 | dB |  |  |
| Subscriber EIRP [a] | 34.5 | 34.5 | dBm | [x]+[y]-[z] |  |
| BS antenna gain [b] | 10 | 10 | dBi |  |  |
| BS cabling loss [c] | 1 | 1 | dB |  |  |
| Fade margin [d] | 2 | 2 | dB |  | Based on antenna configuration (Section 4.2.1) |
| Effective OFDMA gain [e] | 12.3 | 0 | dB |  | 1 of 17 subchannels are used at the cell edge |
| Receiver sensitivity [f] | -91.1 | -88 | dBm |  | 802.11: 1.5Mbps BPSK 1/2; 802.16: 1.5Mbps QPSK 1/2 |
| Data rate | 0.153 | 1500 | Mbps |  |  |
| System Gain | 144.9 | 129.5 | dB | [a]+[b]-[c]-[d]+[e]-[f] |  |
| SUI parameters | Terrain C (Rural) | Terrain C (Rural) |  |  | See Section 4.1.2 |
| Base Station height | 25 | 25 | m |  |  |
| Subscriber height | 10 | 10 | m |  |  |
| Frequency | 3675 | 3675 | MHz |  |  |
| SUI Pathloss | 145.1 | 129.5 | dB |  | Required to match System Gain with no Shadow Margin |
| Cell range | 5300 | 2300 | m |  |  |

Figure 13 802.16h & 802.11y uplink budget evaluation for Scenario A.

| Parameter | 802.16h | 802.11y |  |  | Note |
| --- | --- | --- | --- | --- | --- |
| Subscriber output power [x] | 17 | 17 | dBm |  | 5MHz channel |
| Subscriber antenna gain [y] | 2 | 2 | dBi |  |  |
| Subscriber cabling loss [z] | 0.5 | 0.5 | dB |  |  |
| Subscriber EIRP [a] | 18.5 | 18.5 | dBm | [x]+[y]-[z] |  |
| BS antenna gain [b] | 10 | 10 | dBi |  |  |
| BS cabling loss [c] | 1 | 1 | dB |  |  |
| Fade margin [d] | 10 | 13 | dB |  | Based on antenna configuration (Section 4.2.1) |
| Effective OFDMA gain [e] | 12.3 | 0 | dB |  | 1 of 17 subchannels are used at the cell edge |
| Receiver sensitivity [f] | -91.1 | -88 | dBm |  | 802.11: 1.5Mbps BPSK 1/2; 802.16: 1.5Mbps QPSK 1/2 |
| Data rate | 0.153 | 1500 | Mbps |  |  |
| System Gain | 120.9 | 102.5 | dB | [a]+[b]-[c]-[d]+[e]-[f] |  |
| SUI parameters | Terrain C (Rural) | Terrain C (Rural) |  |  | See Section 4.1.2 |
| Base Station height | 25 | 25 | m |  |  |
| Subscriber height | 2 | 2 | m |  |  |
| Frequency | 3675 | 3675 | MHz |  |  |
| SUI Pathloss | 120.7 | 102.3 | dB |  | Required to match System Gain with no Shadow Margin |
| Cell range | 675 | 250 | m |  |  |

Figure 14 802.16h & 802.11y uplink budget evaluation for Scenario C.

# Annex 2

802.11y recommended parameters for Coordinated Contention Based Protocol (CX-CBP) [1] operation

## Beacon period

The beacon period should be equal to 117 TUs (Time Units). This is 119.8ms. This interval is selected to be similar to the preferred 100ms beacon period of 802.11 and be similar to the 20ms period of the CX-Frame [1].

## Beacon transmission

To occur at the start of the CXCBI [1] (when 802.16 is not scheduled to transmit). At the beginning of the CXCBI there are two symbols in which 802.16 does not transmit. The 802.11 base station shall check which of the 6 possible beacon transmission slots has minimum interference and use it for beacon transmission. The CXCBI shall be used for the 802.11 base station downlink data transmission first, but at least one slot will be kept vacant for another 802.11 base station beacon transmission.

## Quiet intervals

Quiet intervals are scheduled so as to not overlap with the CXSBI intervals and to leave the start of the CXCBI for 802.11 base station downlink transmission. (11.9.2 Quieting channels for testing, and 7.3.2.23 Quiet Element [7]). The Quiet Elements should use the following parameters:

Quiet Element 1

Quiet count = 1 (start during next beacon)

Quiet period = 1 (in every beacon interval)

Quiet offset = 9 (9*1.024= 9.216ms)

Quiet duration = 11 (11*1024 = 11.26ms)

Protected start of next CXCBI: 1.024*(9+11)-20=240ns

Quiet Element 2

Quiet count = 1

Quiet period = 1

Quiet offset = 29 (29*1.024= 29.7ms)

Quiet duration = 11 (11*1.024 = 11.26ms)

Protected start of next CXCBI: 1.024*(40)-40=240ns

Quiet Element 3

Quiet count = 1

Quiet period = 1

Quiet offset = 48 (48*1.024= 49.152ms)

Quiet duration = 11 (11*1.024 = 11.26ms)

Protected start of next CXCBI: 1.024*(48+11)-60=416ns

Quiet Element 4

Quiet count = 1

Quiet period = 1

Quiet offset = 68 (68*1.024= 69.63ms)

Quiet duration = 11 (11*1.024 = 11.26ms)

Protected start of next CXCBI: 1.024*(69+10)-80=896ns

Quiet Element 5

Quiet count = 1

Quiet period = 1

Quiet offset = 87 (87*1024= 89.08ms)

Quiet duration = 11 (11*1.024 = 11.26ms)

Protected start of next CXCBI: 1.024*(87+11)-100=352ns

Quiet Element 6

Quiet count = 1

Quiet period = 1

Quiet offset = 107 (107*1024= 109.57ms)

Quiet duration = 11 (11*1.024 = 11.26ms)

Protected start of next CXCBI: 1.024*(107+12)-120=830ns

The quiet intervals related to the CX-Frame are shown in Figure 15.

Figure 15 The relationship between the 802.11 quiet intervals and the CX-Frame [1]