IEEE P802.19
Wireless Coexistence

| Up-link System Gain for Wireless Coexistence in the US 3.65GHz band |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-11-28 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Mariana Goldhamer | Alvarion | 21A, Ha Barzel Street, Tel Aviv, Israel | +972 544 225548 | mariana.goldhamer@alvarion.com |

Abstract

This document provides a set of assumptions and simulation parameters for use in modelling wireless coexistence in the 3.65GHz band for the US.

Notice: This document has been prepared to assist IEEE 802.19. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.19.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the TAG of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Shellhammer@ieee.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.19 TAG. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Introduction

This contribution provides:

- the missing text in annex 1 of

(http://ieee802.org/19/pub/2007/19-07-0011-10-0000-Parameters-for-Simulation.doc)

- the correct link budget calculation, as no corrections are needed to 802.11y link budget once the sensitivity can be taken from the standard. It was agreed in the meeting to use for the both systems the numbers as they are indicated in the referenced documents.

# Annex 1

802.16h & 802.11y uplink budget evaluation

This analysis is used to calculate the maximum range of an 802.16h and 802.11y cell in the 3.65GHz band. This maximum range is used to calculate the maximum round trip time used in turn to evaluate 802.11y medium access parameters via the term aPropagationTime; the 802.16h uplink budget is for information and comparison purposes.

Assumptions

- Subscriber (max. output power 50mW (17dBm))

- Subscriber connector loss = 0.5dB

- Base Station cabling loss = 1dB

- Base Station antennas gain = 10dBi (Omni directional)

- Fade margins:

  - 802.11y (1Tx/1Rx: Uplink) = 13dB

  - 802.16h (1Tx/2Rx: Uplink) = 10dB

- The OFDMA PHY of 802.16h provides uplink subchannelization gain.

The 802.16 link-budget is calculated using the up-link OFDMA gain. In general, the up-link OFDMA gain is calculated to compensate the difference between the down-link and up-link system gains, such to have similar data rates for the both directions.

The downlink system gain is increased for 802.16h systems by a difference of 5dB in the fade margin, because in down-link is used the 2*2 antenna configuration requiring a fade margin of 5dB. In up-link is used one Tx antenna and 2Rx antennas, requiring a fade margin of 10dB.

The BS transmit power for a 5MHz channel is limited to 5W (37dBm), for a BS antenna gain of 10dbi. The resulting electrical transmit power is 37-10=27dBm. The difference between the BS and SS transmit powers is 10dB.

The required OFDMA gain to compensate the difference in fade margin and electrical power is 15dB. However, due to the limitation of the available number of up-link channels (17) the OFDMA gain will be limited to 10log(17)= 12.3dB.

- Assumptions aligned with Scenario A.

- The System Gain is used to calculate an appropriate cell range based on the SUI propagation model [Error: Reference source not found] and using the parameters defined. In this evaluation Shadow Margin is not applied.

- The 802.11y and 802.16h systems use the following modulation and coding rates at the cell margin:

  - 802.11y uses BPSK ½

  - 802.16h uses QPSK ½

| Parameter | 802.16h | 802.11y |  |  | Note |
| --- | --- | --- | --- | --- | --- |
| Subscriber output power [x] | 17 | 17 | dBm |  | 5MHz channel |
| Subscriber antenna gain [y] | 18 | 18 | dBi |  |  |
| Subscriber cabling loss [z] | 0.5 | 0.5 | dB |  |  |
| Subscriber EIRP [a] | 34.5 | 34.5 | dBm | [x]+[y]-[z] |  |
| BS antenna gain [b] | 10 | 10 | dBi |  |  |
| BS cabling loss [c] | 1 | 1 | dB |  |  |
| Fade margin [d] | 10 | 13 | dB |  | Based on antenna configuration (Section 4.2.1) |
| Effective OFDM gain [e] | 12.3 | 0 | dB |  |  |
| Receiver sensitivity [g] | -91.1 | -88 | dBm |  | 802.11: 1.5Mbps BPSK 1/2 802.16: 1.5Mbps QPSK 1/2 |
| System Gain | 136.9 | 118.5 | dB | [a]+[b]-[c]-[d]+[e] -[g] |  |
| SUI parameters | Terrain C (Rural) | Terrain C (Rural) |  |  | See Section 4.1.2 |
| Base Station height | 25 | 25 | m |  |  |
| Subscriber height | 10 | 10 | m |  |  |
| Frequency | 3675 | 3675 | MHz |  |  |
| SUI Pathloss | 136.9 | 120.4 | dB |  | Required to match System Gain with no Shadow Margin |
| Cell range |  | 1400 | m |  |  |

Figure 1 802.16h & 802.11y uplink budget evaluation.