IEEE P802.19
Wireless Coexistence

| Up-link OFDMA sub-channel allocation for Wireless Coexistence in US 3.65GHz band |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-12-06 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Shahar Hauzner | Alvarion | 21A, Ha Barzel Street, Tel Aviv, Israel | +972 3 7674150 |  |
| Mariana Goldhamer | Alvarion | 21A, Ha Barzel Street, Tel Aviv, Israel | +972 544 225548 | mariana.goldhamer@alvarion.com |

Abstract

This document provides rules for the up-link sub-channel allocation in 802.16h.

Notice: This document has been prepared to assist IEEE 802.19. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.19.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the TAG of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Shellhammer@ieee.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.19 TAG. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Introduction

This document provides rules for the up-link sub-channel allocation in 802.16h. The algorithm below is a simplified algorithm, however takes into consideration the OFDMA up-link gain, the sub-channel allocation based on the distance from the Base Station and the data traffic.

# Rules for up-link sub-channel allocation

Simple procedure of assigning subchannels to SSs

- Compute the Rx power in the BS for each SS (rx_power_i), according to appropriate channel model. The SS will transmit at its maximum power. Order the SS starting from the lowest received power to the highest and assign an index to each SS following the created order.

- For those SSs for which the Rx power in the BS is less than the sensitivity level required for the minimal QPSK ½ rate (-91.5 dBm), compute the maximum sub-channel number for the i-th SS:
max_subch_no_i = floor(17/10(-91.5-(received power))/10)

- For the other SSs the maximum number of subchannels is max_subch_no_i=17.

- During the simulation run, for every MAC Frame:

  - Assign every SSi transmitting in this MAC  frame one sub-channel

aSSi = 1

  - Repeat using the rule below until allocating all the subchannels to the active SSi:

For SSi:

- Determine if the aSSi is enough for the data to be transmitted; if YES, go to the next SSi

- If the maximum number of possible subchannels is greater then the number already allocated for this SS, allocate another sub-channel for this SS, and otherwise keep the allocated number of subchannels as long as all the sub-channels are allocated:

if max_subch_no_i > aSSi, then aSSi = aSSi+1;

- Continue b. until the sub-channel allocation is finished for every SSi

  - Compute the noise level for SSi:

    - Ni = N - 10log (17/aSSi)

  - Compute the SNR for SSi:

SNRi = rx_power_i – Ni

  - Assign the suitable modulation and coding rate