IEEE P802.19
Wireless Coexistence

| Comparison between SUI and HATA Path-Loss Models |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-03-16 |  |  |  |  |
| Author(s): Shahar Hauzner |  |  |  |  |
| Name | Company | Address | Phone | email |
| Shahar Hauzner | Alvarion | 21A, Ha Barzel Street, Tel Aviv, Israel | +972 3 7674150 | Shahar.hauzner@alvarion.com |

Abstract

This document compares between the SUI and HATA path-loss models, for the parameters used in the wireless coexistence simulation.

Notice: This document has been prepared to assist IEEE 802.19. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.19.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the TAG of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Shellhammer@ieee.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.19 TAG. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Comparison between path-loss models

The following document presents the path loss for SUI (terrain type B), modified HATA (from Ahmed's contribution) and FSPL models, according to the following parameters:

- BS height = 25 m

- MS height = 10 m

- f = 3650 MHz

The SUI path loss was calculated according to:

PLSUI=20log104π⋅100⋅fc+104−0.0065hTx+17.1hTxlog10d100+6log10f2⋅109−10.8log10hRx2

Where fis the carrier frequency in Hz, c is the speed of light, hTx is the transmitter height, hRx is the receiver height, and dis the distance between the transmitter and receiver.

The modified HATA path loss was calculated according to:

PLHATA=69.55+26.16 log10f-13.82log10hTx-3.2log1011.75hRx2+4.97+(44.9-6.55 log10(hTx)log (d)

Where f, c, hTx, hRx, and dare the same as above.

The free space path loss was calculated according to:

FSPL=20log104π⋅d⋅fc

Where f, c, and dare the same as above.

Figure 1 below shows the path loss for these models, per Tx-Rx type:

Figure 1

Figure 2 below focus on distances of less than 500 m (for the same parameters as above):

Figure 2

From Figure 2, the following observations can be made:

- SUI model:

  - BS-MS PL is better the FSPL for distance of less than 180 m (max difference = 5 dB)

  - BS-BS PL is better the FSPL for distance of less than 260 m (max difference = 10 dB)

  - MS-MS PL is better the FSPL for distance of less than 150 m (max difference = 5 dB)

- Modified HATA model:

  - BS-MS PL is always worse than FSPL

  - BS-BS PL is better the FSPL for distance of less than 200 m (max difference = 4.5 dB)

  - MS-MS PL is always worse than FSPL

Figure 3 shows the path loss for the SUI, HATA and FS models for the following parameters:

- BS height = 25 m

- MS height = 2 m

- f = 3650 MHz

Figure 3

Figure 4 below focus on distances of less than 500 m (for the same parameters as above):

Figure 4

From Figure 4, the following observations can be made:

- SUI model:

  - BS-MS PL is better the FSPL for distance of less than 100

  - BS-BS PL is better the FSPL for distance of less than 260 m (max difference = 10 dB)

  - MS-MS PL is better the FSPL for distance of less than 100 m

- Modified HATA model:

  - BS-MS PL is always worse than FSPL

  - BS-BS PL is better the FSPL for distance of less than 200 m (max difference = 4.5 dB)

  - MS-MS PL is always worse than FSPL