IEEE P802.19
Wireless Coexistence

| Simple Packet Error Model for Wireless Coexistence in US 3.65GHz band Simulation |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-04-06 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Shahar Hauzner | Alvarion | 21A, Ha Barzel Street, Tel Aviv, Israel | +972 3 7674150 | Shahar.hauzner@alvarion.com |

Abstract

This document provides packet error model for wireless coexistence simulation.

Notice: This document has been prepared to assist IEEE 802.19. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.19.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the TAG of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Shellhammer@ieee.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.19 TAG. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

1. Introduction

System level simulation requires a performance estimate for each wireless link, but it is typically infeasible to conduct exhaustive link-level emulation within a reasonable amount of time. Therefore, it is critical to have a simple link error probability predictor that accurately models OFDM system performance. The exponential effective SINR mapping (EESM) method [2] is shown to be a very good link error prediction method for multi-carrier systems, which can map the instantaneous channel state sets into a single effective SINR value (a scalar value) that may then be used to find an estimate of the block-error probability from a set of basic AWGN link-level performance. EESM link error prediction model greatly simplifies the link-to-system mapping interface. Since both 802.11y and 802.16h use OFDM\OFDMA PHY, this method is particularly suitable for the simulation.

- 2. EESM Based link prediction for OFDM\OFDMA

The exponential ESM (EESM) is derived from Chernoff Union Bound of the pairwise error probability (PEP) [2], and can be generalized to multi-state channel with blocks of different SNR values of each subcarrier or each symbol transmission time interval (TTI). When only considered the influence of channel frequency selectivity, the equation of EESM is given by [3, 4]

SINRpacket=−βlog1Ni=1Ne−SINRsubβ

Where SINRpacket is the packet's equivalent SINR (in linear scale), SINRsub is the sub-carrier SINR (in linear scale), N is the number of sub-carriers over which the packet is transmitted, and β is a scaling coefficient determined by the selected modulation and code-rate. The packet is received correctly if the  SINRpacket exceeds the AWGN threshold (determined by the link's rate) plus the specified fade margin.

- 3. EESM for the wireless coexistence simulation

Define a packet as a continuous transmission of a block of data, using a single modulation and code-rate.

The received packet can be sectioned into non-overlapping sections, each section with its own received SINR. Since interference can be viewed a kind of flat fading, affecting all the sub-carriers that fall within its duration, the EESM equation can be re-written as:

SINRpacket=−βlog1Nk=1Mi=1Nke−SINRsubβ=−βlogk=1MNkNe−SINRkβ

Where M is the number of different SINR sections the received packet, Nk is the number of sub-carriers that fall in section k, and SINRk is the received SINR during section k. Since the number of sub-carriers un the section is proportional to its duration, the EESM equation can again be re-written as:

SINRpacket=−βlogk=1MTkTpackete−SINRkβ

Where Tk is the section's duration and Tpacket is the packet's duration.

Table 1, taken from [5] and [7], shows the β values for different modulation\coding rates:

| # | Modulation | Code Rate | β |
| --- | --- | --- | --- |
| 1 | BPSK | ½ | 0.91 |
| 2 | BPSK | ¾ | 0.98 |
| 3 | QPSK | ½ | 1.57 |
| 4 | QPSK | ¾ | 1.69 |
| 5 | QAM16 | ½ | 4.56 |
| 6 | QAM16 | ¾ | 7.33 |
| 7 | QAM64 | ½ | 12.5 |
| 8 | QAM64 | 2/3 | 23.3 |
| 9 | QAM64 | ¾ | 27.7 |

Table 1: beta values for different modulation/code rates

The packet is received in error if

SINRpacket<SNRAWGN+FM

Where SNRAWGNis the SNR specified in the Parameters document [6] for the correct reception of the appropriate modulation and code rate of the packet, and FM is the fade margin defined for the simulated scenario. SNRAWGN can be taken directly from Figure 8 in [6] for the 802.16h system, and derived from Figure 3 in [6] for the 802.11y system.

- 4. References

[1] Hao Liu; Liyu Cai; Hongwei Yang; Dong Li, "EESM Based Link Error Prediction for Adaptive MIMO-OFDM System", Vehicular Technology Conference, 2007. VTC2007-Spring. IEEE 65th 22-25 April 2007 Page(s):559 – 563

[2] M. Pauli, U. Wachsmann, S. Tsai, “Quality determination for a wireless communications link,” U.S. Patent 6 822 80, Oct. 9, 2003

[3] Nortel Networks, “Effective SIR computation for OFDM system-level simulations,” 3GPP TSG RAN WG1 #35, R1-031370, 2003. IEEE 802.16m-07/037r2

[4] Draft IEEE 802.16m Evaluation Methodology,

[5] WiMAX System Evaluation Methodology

[6] Paul Piggin, Parameters for simulation of Wireless Coexistence in the US 3.65GHz band, IEEE 802.19-07/11r10, November 2007

[7] A. Alexiou, D. Astély, K. Brüninghaus, P. Coronel, M. Döttling, M. Fuchs, J. Giese, M. Haardt, M. Hennhöfer, T. Jiang, E. Jorswieck, A. Kapur, Z. Li, A. Medles, T. Mousley, M. Olsson, K. Roberts, S. Rouquette-Léveil, W. Schott, A. Sezgin, P. Skillermark, V. Stankovic, P. Svedman, S. Visuri, D. Wang, T. Wild, E. Zimmermann, "Assessment of Advanced Beamforming and MIMO Technologies", IST-2003-507581 WINNER, D2.7 ver 1.0

The  value for BPSK¾ was not found in any document. The value shown was interpolated from the rest of the  values in the table.