IEEE P802.19

Wireless Coexistence

| TVWS Coexistence PAR |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-09-24 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Steve Shellhammer | Qualcomm | 5775 Morehouse Drive San Diego, CA 92121 | (850) 658-1874 | shellhammer@ieee.org |
| Mark Cummings, Ph. D. | SWIM | 348 Camino al Lago Atherton, Ca 94027 | +1650 854 4406 | markcummings@envia.com |
| Ari Ahtiainen | Nokia | P.O. Box 407, F1-00045 Nokia Group, Itamerenkatu 11-13 00180, Helsinki, Finland | +358 (0)7180 36426 | ari.p.ahtianen@nokia.com |
| Mika Kasslin | Nokia | P.O. Box 407, F1-00045 Nokia Group, Itamerenkatu 11-13 00180, Helsinki, Finland | +358 (0)7180 36294 | mika.kasslin@nokia.com |

Draft PAR Confirmation Number

Submittal Email: sshellha@qualcomm.com

Type of Project: PAR a new Standard

1.1 Project Number: P802.19

1.2 Type of Document: Standard for

1.3 Life Cycle: Full

1.4 Is this project in ballot now? No

2.1 Title of Standard: IEEE Standard for Information Technology - Telecommunications and Information Exchange Between Systems - Local and Metropolitan Area Networks - Specific Requirements - Part 19: TV White Space Coexistence Mechanisms

3.1 Name of Working Group: Wireless Coexistence (C/LM/WG802.19) Contact information for Working Group Chair Steve Shellhammer Qualcomm, Inc. 5775 Morehouse Drive San Diego, CA 92121, USA Email: shellhammer@ieee.org Working Group Vice Chair: Ivan Reede Amersys ADDRESS Email: I_reede@amerisys.com

3.2 Sponsoring Society and Committee: IEEE Computer Society/Local and Metropolitan Area Networks(C/LM) Contact information for Sponsor Chair: Paul Nikolich 18 Bishops Lane Lynnfield, MA 01940, USA Email: p.nikolich@ieee.org Contact information for Standards Representative:

4.1 Type of Ballot: Individual

4.2 Expected Date of Submission for Initial Sponsor Ballot: 2012-12

4.3 Projected Completion Date for Submittal to RevCom: 2013-12

5.1 Approximate number of people expected to work on this project: 30

5.2 Scope of Proposed Standard: The standard specifies mechanisms for coexistence among dissimilar or independently operated TV Band Device (TVBD) networks and dissimilar TV Band Devices.

5.3 Is the completion of this standard is dependent upon the completion of another standard: No If yes, please explain:

5.4 Purpose of Proposed Standard: The purpose of the standard is to enable the family of IEEE 802 Wireless Standards to most effectively use TV White Space by providing standard coexistence mechanisms among dissimilar or independently operated TVBD networks and dissimilar TVBDs. This standard addresses coexistence for IEEE 802 networks and devices and will also be useful for non IEEE 802 networks and TVBDs

5.5 Need for the Project: Existing IEEE 802 standards groups are developing standards and amendments, to comply with the regulatory rules for use of TV white space. Other non-IEEE 802 wireless standards for use of the TV white space are also in development. In order for these various dissimilar TVBD networks and devices to effectively coexist in the TVWS spectrum, collaborative use of coexistence mechanisms is needed. This collaboration among dissimilar TVBD networks and devices uses the coexistence mechanisms provided in this standard. In order to enhance utilization of the TV white space bands standardized coexistence mechanisms are needed. Mechanisms such as those discussed in the explanatory notes may be considered.

5.6 Stakeholders for the Standard: Designers of TVWS MAC/PHY standards and implementations.

Intellectual Property 6.1.a. Has the IEEE-SA policy on intellectual property been presented to those responsible for preparing/submitting this PAR prior to the PAR submittal to the IEEE-SA Standards Board? Yes If yes, state date: 2009-09-21 If no, please explain: 6.1.b. Is the Sponsor aware of any copyright permissions needed for this project? No If yes, please explain: 6.1.c. Is the Sponsor aware of possible registration activity related to this project? No If yes, please explain:

7.1 Are there other standards or projects with a similar scope? Yes Explanation: IEEE P1900.4a is a standards project which addresses dynamic spectrum access networks in the white space frequency bands. Sponsor Organization: IEEE Project/Standard Number: P1900.4a Project/Standard Date: 0000-00-00 Project/Standard Title: Standard for Architectural Building Blocks Enabling Network-Device Distributed Decision Making for Optimized Radio Resource Usage in Heterogeneous Wireless Access Networks - Amendment: Architecture and Interfaces for Dynamic Spectrum Access Networks in White Space Frequency Bands Explanation: IEEE P802.22 is a MAC/PHY standards project for operation in the TVWS which includes coexistence mechanisms. Sponsor Organization: IEEE Project/Standard Number: P802.22 Project/Standard Date: 0000-00-00 Project/Standard Title: Draft Standard for Wireless Regional Area Networks Part 22: Cognitive Wireless RAN Medium Access Control (MAC) and Physical Layer (PHY) specifications: Policies and procedures for operation in the TV Bands Explanation: Ecma International TC48-TG1 is a MAC/PHY standards project for operation in the TVWS which includes coexistence mechanisms. Sponsor Organization: Ecma International Project/Standard Number: TC48-TG1 Project/Standard Date: 0000-00-00 Project/Standard Title: Wireless Communications using Television White Spaces (TVWS).

7.2 International Standards Activities a. Adoptions Is there potential for this standard to be adopted by another organization? No Organization: Technical Committee Name: Technical Committee Number: Contact person Name: Contact Phone: Contact Email: b. Joint Development Is it the intent to develop this document jointly with another organization? No Organization: Technical Committee Name: Technical Committee Number: Contact person Name: Contact Phone: Contact Email: c. Harmonization Are you aware of another organization that may be interested in portions of this document in their standardization development efforts? No Organization: Technical Committee Name: Technical Committee Number: Contact person Name: Contact Phone: Contact Email:

8.1 Additional Explanatory Notes: (Item Number and Explanation) Section 5.2 (Scope) The term “dissimilar” used in this document refers to the networks and devices, which use different radio technologies adapted for TV White Space Frequency Bands. Section 5.2 (Scope) The term “independently operated” used in this document refers to networks which may or may not use the same radio technology but are operated by independent entities which do not necessarily have a business relationship for coordinating their use of TV White Space Frequency Bands; furthermore, such similar radio access technologies may not have an otherwise available means of coexistence. For example, this differentiates independently operated from a situation in which a single administrative authority manages the coexistence between dissimilar technologies deployed in a composite network. Sec. 5.5 (Need): The radio technology independent coexistence mechanisms that are standardized may address the following and related areas: Discovery is the process of determining that there are two or more dissimilar and independently operated wireless networks or devices attempting to use the same White Space frequency range in the same location. This can occur in two ways. The two networks or devices may attempt to enter at the same time or one may be present and a second seek to enter. In the second case, a network operating in White Space, must periodically check for new entrants and a new entrant must check before entering. A Connection for Coexistence is necessary for two dissimilar and independently operated wireless networks or devices to exchange information in order to collaboratively share spectrum. A Logical Mechanism for Promoting Coexistence is a mechanism that involves the exchange of information between different dissimilar or independently operated networks or devices and may also involve algorithms seeking to maximize the quality of service for all participants.

Bottom of Form