Proposal for Wireless Coexistence using an IEEE 802.19.1 Geographic Electromagnetic Radiation Domain Control System

Approved <XX MONTH 20XX>

IEEE-SA Standards Board

Copyright © 2010 by AmeriSys, Inc.

4230 St John's Blvd, S231

Montreal, Quebec, Canada,

All rights reserved.

This document is a proposal for an unapproved draft of a proposed IEEE Standard. As such, this document is subject to change. USE AT YOUR OWN RISK! Because this is an unapproved proposal, this document must not be utilized for any conformance/compliance purposes. Permission is hereby granted for IEEE Standards Committee participants to reproduce this document for purposes of international standardization consideration. Prior to adoption of this document, in whole or in part, by another standards development organization, permission must first be obtained from the IEEE Standards Activities Department (stds.ipr@ieee.org). Other entities seeking permission to reproduce this document, in whole or in part, must also obtain permission from the IEEE Standards Activities Department.

IEEE Standards Activities Department

445 Hoes Lane

Piscataway, NJ 08854, USA

Abstract: This document Draft Wireless coexistence using a Geographic Electromagnetic Radiation Domain Control System in the IEEE 802.19.1 Systems

Keywords: IEEE 802.19.1, coexistence, radio, wireless, broadband, TV bands, white spaces

IEEE Standards documents are developed within the IEEE Societies and the Standards Coordinating Committees of the IEEE Standards Association (IEEE-SA) Standards Board. The IEEE develops its standards through a consensus development process, approved by the American National Standards Institute, which brings together volunteers representing varied viewpoints and interests to achieve the final product. Volunteers are not necessarily members of the Institute and serve without compensation. While the IEEE administers the process and establishes rules to promote fairness in the consensus development process, the IEEE does not independently evaluate, test, or verify the accuracy of any of the information or the soundness of any judgments contained in its standards.

Use of an IEEE Standard is wholly voluntary. The IEEE disclaims liability for any personal injury, property or other damage, of any nature whatsoever, whether special, indirect, consequential, or compensatory, directly or indirectly resulting from the publication, use of, or reliance upon this, or any other IEEE Standard document.

The IEEE does not warrant or represent the accuracy or content of the material contained herein, and expressly disclaims any express or implied warranty, including any implied warranty of merchantability or fitness for a specific purpose, or that the use of the material contained herein is free from patent infringement. IEEE Standards documents are supplied “AS IS.”

The existence of an IEEE Standard does not imply that there are no other ways to produce, test, measure, purchase, market, or provide other goods and services related to the scope of the IEEE Standard. Furthermore, the viewpoint expressed at the time a standard is approved and issued is subject to change brought about through developments in the state of the art and comments received from users of the standard. Every IEEE Standard is subjected to review at least every five years for revision or reaffirmation, or every ten years for stabilization. When a document is more than five years old and has not been reaffirmed, or more than ten years old and has not been stabilized, it is reasonable to conclude that its contents, although still of some value, do not wholly reflect the present state of the art. Users are cautioned to check to determine that they have the latest edition of any IEEE Standard.

In publishing and making this document available, the IEEE is not suggesting or rendering professional or other services for, or on behalf of, any person or entity. Nor is the IEEE undertaking to perform any duty owed by any other person or entity to another. Any person utilizing this, and any other IEEE Standards document, should rely upon his or her independent judgment in the exercise of reasonable care in any given circumstances or, as appropriate, seek the advice of a competent professional in determining the appropriateness of a given IEEE standard.

Interpretations: Occasionally questions may arise regarding the meaning of portions of standards as they relate to specific applications. When the need for interpretations is brought to the attention of IEEE, the Institute will initiate action to prepare appropriate responses. Since IEEE Standards represent a consensus of concerned interests, it is important to ensure that any interpretation has also received the concurrence of a balance of interests. For this reason, IEEE and the members of its societies and Standards Coordinating Committees are not able to provide an instant response to interpretation requests except in those cases where the matter has previously received formal consideration. A statement, written or oral, that is not processed in accordance with the IEEE-SA Standards Board Operations Manual shall not be considered the official position of IEEE or any of its committees and shall not be considered to be, nor be relied upon as, a formal interpretation of the IEEE. At lectures, symposia, seminars, or educational courses, an individual presenting information on IEEE standards shall make it clear that his or her views should be considered the personal views of that individual rather than the formal position, explanation, or interpretation of the IEEE.

Comments for revision of IEEE Standards are welcome from any interested party, regardless of membership affiliation with IEEE. Suggestions for changes in documents should be in the form of a proposed change of text, together with appropriate supporting comments. Recommendations to change the status of a stabilized standard should include a rationale as to why a revision or withdrawal is required. Comments and recommendations on standards, and requests for interpretations should be addressed to:

Secretary, IEEE-SA Standards Board

445 Hoes Lane

Piscataway, NJ 08854

USA

Authorization to photocopy portions of any individual standard for internal or personal use is granted by The Institute of Electrical and Electronics Engineers, Inc., provided that the appropriate fee is paid to Copyright Clearance Center. To arrange for payment of licensing fee, please contact Copyright Clearance Center, Customer Service, 222 Rosewood Drive, Danvers, MA 01923 USA; +1 978 750 8400. Permission to photocopy portions of any individual standard for educational classroom use can also be obtained through the Copyright Clearance Center.

Introduction

This standard is a system used to improve the wireless coexistence of IEEE wireless networks.  Adhering to the wireless coexistence specifications provided in this standard enables an IEEE 802.19.1 cognitive network to provide service at the highest broadband rates, to serve the largest area, and to make the most efficient use of the spectrum that is available.

Notice to users

Laws and regulations

Users of these documents should consult all applicable laws and regulations. Compliance with the provisions of this standard does not imply compliance to any applicable regulatory requirements. Implementers of the standard are responsible for observing or referring to the applicable regulatory requirements. IEEE does not, by the publication of its standards, intend to urge action that is not in compliance with applicable laws, and these documents may not be construed as doing so.

Copyrights

This document is copyrighted by the IEEE. It is made available for a wide variety of both public and private uses. These include both use, by reference, in laws and regulations, and use in private self-regulation, standardization, and the promotion of engineering practices and methods. By making this document available for use and adoption by public authorities and private users, the IEEE does not waive any rights in copyright to this document.

Updating of IEEE documents

Users of IEEE standards should be aware that these documents may be superseded at any time by the issuance of new editions or may be amended from time to time through the issuance of amendments, corrigenda, or errata. An official IEEE document at any point in time consists of the current edition of the document together with any amendments, corrigenda, or errata then in effect. In order to determine whether a given document is the current edition and whether it has been amended through the issuance of amendments, corrigenda, or errata, visit the IEEE Standards Association web site at http://ieeexplore.ieee.org/xpl/standards.jsp, or contact the IEEE at the address listed previously.

For more information about the IEEE Standards Association or the IEEE standards development process, visit the IEEE-SA web site at http://standards.ieee.org.

Errata

Errata, if any, for this and all other standards can be accessed at the following URL: 
http://standards.ieee.org/reading/ieee/updates/errata/index.html. Users are encouraged to check this URL for errata periodically.

Interpretations

Current interpretations can be accessed at the following URL: http://standards.ieee.org/reading/ieee/interp/
index.html.

Patents

[If the IEEE has not received letters of assurance prior to the time of publication, the following notice shall appear:]

Attention is called to the possibility that implementation of this recommended practice may require use of subject matter covered by patent rights. By publication of this recommended practice, no position is taken with respect to the existence or validity of any patent rights in connection therewith. The IEEE is not responsible for identifying Essential Patent Claims for which a license may be required, for conducting inquiries into the legal validity or scope of Patents Claims or determining whether any licensing terms or conditions provided in connection with submission of a Letter of Assurance, if any, or in any licensing agreements are reasonable or non-discriminatory. Users of this recommended practice are expressly advised that determination of the validity of any patent rights, and the risk of infringement of such rights, is entirely their own responsibility. Further information may be obtained from the IEEE Standards Association.

[The following notice shall appear when the IEEE receives assurance from a known patent holder or patent applicant prior to the time of publication that a license will be made available to all applicants either without compensation or under reasonable rates, terms, and conditions that are demonstrably free of any unfair discrimination.]

Attention is called to the possibility that implementation of this recommended practice may require use of subject matter covered by patent rights. By publication of this recommended practice, no position is taken with respect to the existence or validity of any patent rights in connection therewith. A patent holder or patent applicant has filed a statement of assurance that it will grant licenses under these rights without compensation or under reasonable rates, with reasonable terms and conditions that are demonstrably free of any unfair discrimination to applicants desiring to obtain such licenses. Other Essential Patent Claims may exist for which a statement of assurance has not been received. The IEEE is not responsible for identifying Essential Patent Claims for which a license may be required, for conducting inquiries into the legal validity or scope of Patents Claims, or determining whether any licensing terms or conditions provided in connection with submission of a Letter of Assurance, if any, or in any licensing agreements are reasonable or non-discriminatory. Users of this recommended practice are expressly advised that determination of the validity of any patent rights, and the risk of infringement of such rights, is entirely their own responsibility. Further information may be obtained from the IEEE Standards Association.

Participants

At the time this draft recommended practice was submitted to the IEEE-SA Standards Board for approval, the IEEE 802.19.1  Working Group had the following membership:

xxxxx, Chair

yyyyy, Vice Chair

Participant1

Participant2

Participant3

Participant4

Participant5

Participant6

Participant7

Participant8

Participant9

The following members of the <individual/entity> balloting committee voted on this recommended practice. Balloters may have voted for approval, disapproval, or abstention.

(to be supplied by IEEE)

Balloter1

Balloter2

Balloter3

Balloter4

Balloter5

Balloter6

Balloter7

Balloter8

Balloter9

When the IEEE-SA Standards Board approved this recommended practice on <XX MONTH 20XX>, it had the following membership:

(to be supplied by IEEE)

<Name>, Chair

<Name>, Vice Chair

<Name>, Past President

<Name>, Secretary

SBMember1

SBMember2

SBMember3

SBMember4

SBMember5

SBMember6

SBMember7

SBMember8

SBMember9

*Member Emeritus

Also included are the following nonvoting IEEE-SA Standards Board liaisons:

<Name>, NRC Representative

<Name>, DOE Representative

<Name>, NIST Representative

<Name>

IEEE Standards Program Manager, Document Development

<Name>

IEEE Standards Program Manager, Technical Program Development

Contents

1. Overview

1.1 Scope

1.2 Purpose

1.3 Sample application areas

1.4 Conformance

2. Normative references

3. Definitions, acronyms and abbreviations

3.1 Definitions

3.2 Acronyms and aabreviations

4. System description

4.1 Architecture

4.2 Logical entities

4.3 Interfaces

5. The IEEE 802.19.1 reference model

5.1 General description

5.2 Service Access Points (SAPs)

5.2.1 Data formats and parameters

6. Procedures and protocols

6.1 Generic procedures for information exchange

6.2 Protocols

7. Coexistence mechanisms and algorithms

Annex A: Use cases

Annex B: Evaluation methodology

Annex C: Coexistence Enabler (CE) software reference design

Proposal for Wireless Coexistence using an IEEE 802.19.1 Geographic Electromagnetic Radiation Domain Control System

IMPORTANT NOTICE: This standard is not intended to ensure safety, security, health, or environmental protection. Implementers of the standard are responsible for determining appropriate safety, security, environmental, and health practices or regulatory requirements.

This IEEE document is made available for use subject to important notices and legal disclaimers. 
These notices and disclaimers appear in all publications containing this document and may 
be found under the heading “Important Notice” or “Important Notices and Disclaimers 
Concerning IEEE Documents.” They can also be obtained on request from IEEE or viewed at http://standards.ieee.org/IPR/disclaimers.html.

- 1. Overview

  - Scope

The standard specifies radio technology independent methods for coexistence among dissimilar or independently operated TV Band Device (TVBD) networks and dissimilar TV Band Devices.

  - Purpose

The purpose of the standard is to enable the family of IEEE 802 Wireless Standards to most effectively use TV White Space by providing standard coexistence methods among dissimilar or independently operated TVBD networks and dissimilar TVBDs. This standard addresses coexistence for IEEE 802 networks and devices and will also be useful for non IEEE 802 networks and TVBDs.

- 1.3  Sample application areas

The sample applications are all 802 wireless networks and potentially non 802 wireless networks wishing to coexist with 802.19.1 compliant networks. The main application area of this standard is in the TV white space. However 802.19.1 compliance may not be limited to TV white space frequency bands and maybe used at large.

- 1.4 Conformance

- 2. Normative references

The following referenced documents are indispensable for the application of this document (i.e., they must be understood and used, so each referenced document is cited in text and its relationship to this document is explained). For dated references, only the edition cited applies. For undated references, the latest edition of the referenced document (including any amendments or corrigenda) applies.

- 3. Definitions, acronyms and abbreviations

- 3.1 Definitions

| Term | Definition |
| --- | --- |
| Acceptable Interference | An interference caused by an interference signal that does not degrade the current selection of modulation and coding at a victim receiver. |
| Aggregated Interference Power | The total power of interference observed at a particular location as received all interferers. |
| Algorithm | An effective method for solving a problem using a finite sequence of well defined instructions. |
| Architecture | A framework for the specification of a system's physical components and their functional organization and configuration, its operational principles and procedures, as well as data formats used in its operation. |
| Authentication | Authentication is a security measure to verify the identity of a subscriber. This verification is usually based on a username and password. In GSM the SIM is used to identify a subscriber for authentication.. |
| Authorization | Authorization is the process of granting or denying a subscriber access to a network or to certain services after the user has been authenticated. |
| Awareness | The capability of knowing the existence of a TVWS network/device. |
| Backhaul | In telecommunications, the intermediate communication link(s) between the end user and the core communications network. For example, for cellular transmissions, the backhaul is the link(s) between the cellular tower and the core communications system. |
| Carrier Sense Multiple Access | Carrier Sense Multiple Access is a media access control scheme in which a transmitter verifies the absence of other traffic before transmitting. CSMA is often combined with a mechanism to detect collisions if two stations happen to send data at the same time. If this mechanism is used, it is called CSMA/CD. |
| Channel Reselection | The process of selecting a new radio frequency channel for operation. |
| Channel Selection | The act of determining one or more channels to access for operation |
| Channel Sharing | The act of using a given physical channel by several entities or networks |
| Channel Switching | The act of leaving one channel and utilizing another channel to continue operation |
| Clear Channel Assessment | A logical function which determines the current state of use of a wireless medium. |
| Coexistence | The act of two or more TVBD networks/devices sharing resources without causing destructive interference to one another |
| Coexistence Enabler | A logical entity in 802.19.1 system usually implemented with in TVBD devices that interacts with other 802.19.1 logical entities in order to enable coexistence solutions |
| Coexistence Mechanism | a method of realizing coexistence among TV white space network/devices |
| Coexistence Metric | A measurable quantity that can be used to indicate how well two or more networks or devices are coexisting |
| Coexistence Service | Functions and solutions as well as methods provide to TV white space networks to achieve coexistence |
| Coexistence Service Provider | An entity which provides services enabling devices or networks better coexist. |
| Coverage Area | An area within which the received signal power is above a certain reception threshold and service is obtained |
| Device Configuration | The setting of operational parameters of a device or network |
| Device Reconfiguration | The act of setting new operational parameters for a device or network |
| Discovery | Determining the presence of TVBD network or device and identifying its attribute such as ID |
| Dynamic Frequency Selection | The ability of a system to switch to a different frequency (physical operating channel) based on channel measurements avoiding interference to regulatory protected devices. |
| Fairness | Principle of non discriminatory, uniform application of rules in allocating or dividing a competitive resource such as spectrum. Simple fairness is based on equal division of resources among all users. Rules for fairness may also consider specific user requirements (user load, QoS, priority, etc), aggregate spectral efficiency, etiquette (first come first served, etc.), and user or network policies. |
| Fixed | A type of devices that is not moved from location to location. |
| Frequency Division Multiple Access | Channel access method used in multiple-access protocols as a channelization protocol. It gives users an individual allocation of one or several frequency bands, or channels. |
| Geolocation | A geographic location on the earth specified in three spherical coordinates, latitude, longitude, and altitude. Geolocation may include uncertainty or error metrics. |
| Harmful Interference | An interference caused by an interference signal that seriously degrades, obstructs, or repeatedly interrupts the radio communication at a victim receiver. |
| Independently Operated | Networks which are operated independent from one another regardless of their underlying air interface similarity (we count find could definition at the moment) |
| Interference | The effect of unwanted energy due to one or a combination of emission(s), radiation(s), or induction(s) on reception in a radio communications system, manifested by any performance degradation, misinterpretation, or loss of information that could be extracted in the absence of such unwanted energy. |
| Interference Area | An area bounded by a contour where the received signal power is above a certain interference threshold |
| Interference Threshold | A power level of unwanted RF signal above which can cause service degradation |
| Internet Protocol | The Internet Protocol is used in interconnected systems of packet-switched computer communication networks, it provides for transmitting blocks of data called datagrams from sources to destinations, where sources and destinations are hosts identified by fixed length addresses; it also provides for fragmentation and reassembly of long datagrams, if necessary, for transmission through “small packet” networks. |
| Listen Before Talk | Listen Before Talk (LBT) is a technique used in radio communications whereby a radio transmitter first senses its radio environment before it starts a transmission. LBT can be used by a radio device to find a network the device is allowed to operate on or to find a free radio channel to operate on. Synonym: Listen Before Transmit |
| Mobile | A type of device that uses network communication while in motion. |
| Policy | A set of coexistence rules or procedures which are (mandatory) followed by all entities in the IEEE 802.19.1 system |
| Portable | A type of device that may be moved from location to location, but that only uses network communications while at a fixed location. |
| Quality of Service | The Quality of Service (QoS) refers to the capability of a network to offer a service with a certain quality. The quality of a service can be related to a number of different parameters. Some commonly used parameters are: Availability of a link, Number of bit errors, Latency (delay in the network), Jitter. The ITU has defined four different categories of services. Each with it's own quality of service associated to it: Conversational, Streaming, Interactive, Background. |
| Radio Technology Independent | Refers to the fact that the implementation is conditioned on the specific radio technology implemented by the platform. |
| Security | A condition that results from the establishment and maintenance of protective measures that ensures a state of inviolability from hostile acts or influences. |
| Sensing | The act of scanning the frequency spectrum in order to detect the presence of primary signals |
| Sensing area | An area within which sensing can be performed with a predetermined performance |
| Sensing Data | Data of which purpose is to facilitate the act of sensing |
| Spectrum Etiquette | Is a set of rules governing how spectrum resources should be shared between different parties in a beneficial manner |
| Time Division Multiple Access | Channel access method for shared medium networks. It allows several users to share the same frequency channel by dividing the signal into different time slots. |
| Transmit Power Control | Transmit Power Control (TPC) or sometimes called Dynamic Power Control (DPC) is a mechanism used in radio communications to reduce the power of a radio transmitter to the minimum necessary to maintain the link with a certain quality. TPC is used to avoid interference into other devices and/or to extend the battery life. |
| TV white space | Broadcast television spectrum which is locally unused by licensed services. |
| TV white space coexistence database | A repository that can collect and provide information necessary for coexistence decision making. |
| TV white space device | An unlicensed radio transmitter that operates in the broadcast television spectrum at locations where that spectrum is not being used by licensed services (this unused TV spectrum is often termed “white spaces”) |
| TV white space network | A wireless network where the interconnections of nodes are established by radio communication in TV white space. |
| TV white space regulatory database or Geo-location Database | A database repository of any type (centralized or decentralized) containing various pieces of information needed to create conformance to the services rules and/or laws governing operation in the TV white space spectrum bands. Note that said data can be used by any number of actors such as regulators, operators, manufacturers, and even spectrum users to effect the best and highest use of the spectrum according to laws governing its use. |

- 3.2 Acronyms and abbreviations

For the purposes of this document, the following terms and definitions apply. The IEEE Standards Dictionary: Glossary of Terms & Definitions should be referenced for terms not defined in this clause.

- 4 System description

- 4.1 Architecture

The 802.19.1 system architecture has three logical entities and eleven logical interfaces. An 802.19.1 logical entity is defined by its functional role(s) and its interfaces with other 802.19.1 logical entities and with external elements.

The four logical entities are:

Coexistence Manager (CM)

Coexistence Enabler (CE)

Coexistence Discovery and Information Server (CDIS)

TVWS Enabler

The eleven logical interfaces are:

Interface A

Interface B1

Interface B2

Interface B3

Interface B4

Interface C

Interface D

Interface E

Interface F

Interface G

Interface H

Additionally, the 802.19.1 system interacts with seven external elements:

- TVWS database

- TVBD network or device

- Operator Management Entity (OME)

- Coexistence Database Server (CDS)

- Coexistence Topographic Server (CTS)

- Coexistence DNS (CDNS)

- Coexistence Radius Server (CRS)

Interface B2

Interface B3

Interface B1

TVWS

Database

Operator

Mamagement

Entity

Interface C

Interface D

Interface B4

Coexistence

Database Server

Coexistence

Topo Server

Coexistence

DNS

Interface H

Interface F

Interface E

802.19.1 Scope

Coexistence

Radius Server

Coexistence

Discovery and

Information Server

TVWS

Enabler

Interface G

Coexistence

Manager

Coexistence

Enabler

TVBD network

or device

Interface A

Coexistence

Manager

Figure 1. 802.19.1 system architecture

- 4.2 Logical entities

### Coexistence Enabler (CE)

The first functional roles of the CE is to request and obtain information, required for coexistence, from TVBD network or device. It's second role is to translate reconfiguration requests/commands and control information received from the CM into TVBD-specific reconfiguration requests/commands and send them to the TVBD network or device.

### Coexistence Manager (CM)

The CM runs under the exclusive supervision and control of the OME of a transmitter or network of transmitters, thereby assisting network operators in management related to TVWS coexistence. It requests secure client-server connections, queries servers, receives responses and notifications. Transmitter operators who want to operate and coexist may, via the OME, use a CM to assess if a channel is cleared for use and available in a given area and a given modulation. The CM in turn queries a Coexistence Database Server (CDS) and TVWS database (where applicable by local regulations) to obtain information about channel availability. To discover other CM's the CDS is queried for a given geographical area. The CDS responds with the appropriate information giving the CM the means to communicate with other CM's in the area.

One of the tasks and responsibilities of the CM is to receive and analyze specific bandwidth allocation requests made by the transmitter operator. It analyzes and resolves local transmitter geographic electromagnetic radiation coexistence issues in a given geographic reception area based on available data, established rules and agreements. The result of this analysis is a matrix of maximum allowable field strength vectors. This time-bound matrix covers the entire geographic area the transmitted field may significantly reach, including direct paths, reflections, etc. This multidimensional matrix has indices of time, polarization and incident arrival angle. Having this information the CM makes coexistence decision and provides corresponding request/commands and control information to CE's. Exchange of support information is required for this task and the CM may dialog with other CMs, the CDS and the Coexistence Topographic Server (CTS) to execute the task. This standard through the passthrough commands of interface B3 allows SCs and CMs to operate in both hierarchical or distributed fashion.

Transmitter and receiver antennas may have complex free space radiation patterns. The CM, as a cognitive system device, knows a-priori about the transmitter's antenna properties (via information stored and provided by the local CE) and the surrounding terrain (where a topographical server is available in a sufficient resolution to add significant value to the radiation pattern) and propagation characteristics. It considers all these factors and determines the maximum allowable EIRP and field strengths emanating from the transmitting antenna in its determination of the maximum output power a given transmitter may emit. The output of the CM is the transmitter's maximum allowable radiated power in dBm over a requested frequency range and operating period. The CM, requesting and maintaining active connections also receives and reacts to pro-active environmental change notifications from other CM's and the CDS.

Spectral Client

A Spectral Client (SC) is an 802.19.1 compliant CM deprived of interface B1. It runs under the exclusive supervision and control of the OME. Transmitter operators who want protection or want to improve coexistence use an authorized SC to announce their operating space.

The SC is used to create, edit and delete registered electromagnetic radiation domain records and to make claims about these domains.

#### Coexistence Database Server

The Coexistence Database Server (CDS) receives secure connection requests from registered 802.19.1 CM's, grants secure connection sessions, stores and forwards information, receives and responds to CM and peer CDS queries, issues update notifications to peer CDS' and validates update requests for a given geographical area or a regulatory domain. The CDS acts as an authoritative information cache and forwarder in the network. Therefore, each CDS in the network answers about a given domain or how to get “closer” to another CDS that can provide authoritative or more authoritative answers about a domain or provides authoritative information it has.

### Coexistence Discovery and Information Server (CDIS)

The functional roles of the CDIS are to support discovery of CMs, CDIS, CTS, CRS, CDNS and the TVWS database in the way that it facilitates to open the interfaces between CMs. Another task is to collect, aggregate, and provide information facilitating coexistence and this includes data storage, data processing, etc.

TVWS Enabler

The functional role of the TVWS enabler is to translate, request, obtain and update information required for coexistence from the TVWS database and translate reconfiguration requests/commands and control information received from the CM into the local regulator's TVWS database specific format and send them to the TVWS database.

4.3 Interfaces

The eleven logical interfaces in 802.19.1 system architecture can be split into three groups:

- Interfaces between 802.19.1 entities

  - Interface B1

  - Interface B2

  - Interface B3

  - Interface B4

- Interface between an 802.19.1 entity and TVBD network/device

  - Interface A

  - Interface E

- Interfaces between 802.19.1 entities and TVWS database or OME or Coexistence Database Server or Coexistence Topographic Server (CTS) or Coexistence Radius Server (CRS)

  - Interface C

  - Interface D

  - Interface F

  - Interface G

  - Interface H

Different interfaces in each group are distinguished by their usage, types of information exchanged, and underlying protocols.

### Interface A resides between the CE and the TVBD network or device. In the direction from TVBD network or device to the CE we find:

  - Information required for coexistence

  - Configuration/information requests for coexistence

  - Configuration/measurement/information responses for coexistence

  - And other information as needed

In the direction from the CE to the TVBD network or device we find:

  - Reconfiguration requests/commands and control information (corresponding to coexistence requests/commands and control information received from CM)

  - Requests/commands related to control of measurements performed by TVBD network or device

  - And other information as needed

### Interface B1 resides between the CE and the CM. In the direction from the CE to the CM:

  - Information required for coexistence (information obtained from TVBD network or device)

  - And other information as needed

### In the direction from the CM to the CE:

  - Coexistence requests/commands and control information

  - And other information as needed

### Interface B2 resides between the CM and the CDIS. In the direction from the CM to the CDIS:

  - Information required for discovery (obtained by this CM)

  - Information required for coexistence (obtained by this CM)

  - And other information as needed

In the direction from the CDIS to the CM:

  - Information required for discovery (obtained by other CMs)

  - Information required for coexistence (obtained by other CMs)

  - And other information as needed

### Interface B3 resides between the CM and the CM. In the direction from the CM to the CM:

  - Information and message required for discovery and coexistence

  - And other information as needed

### Interface B4 resides between the CM and the TVWS enabler. In the direction from  the TVWS enabler to the CM

  - Information required for coexistence (information obtained from TVBD network or device)

  - And other information as needed

In the direction from the CM to the TVWS enabler

  - Information required for coexistence (information obtained from TVWS database

  - And other information as needed

##### Interface C resides between the CM/CDIS and the TVWS database. In the direction from theTVWS database to the TVWS enabler

  - Information required for coexistence (information on available TV channels)

  - And other information as needed

In the direction from the TVWS enabler to the TVWS database

  - Registration information required by regulators about the TVBD

  - And other information as needed

### Interface D resides between the CM and the OME. In the direction from the OME to the CM:

  - Network operator related information e.g. spectrum policy/limitations concerning operator networks

  - And other information as needed

Interface E resides between the Coexistence DNS (CDNS) and the CDIS. In the direction from the CDNS to the CDIS:

  - The CDNS informs the CDIS of other devices and networks for coexistence

  - And other information as needed

In the direction from the CDIS to the CDNS

  - The CDNS receives and store information about networks/device names that are under the format of a conventional DNS. In order to allow the DNS search by area, the URL domain name is prefixed by the latitude and longitude of the device or network

  - And other information as needed

### Interface F resides between the CM and the Coexistence Topographic Server (CTS). In the direction from the CTS to the CM:

  - Topographic data from SRTM2002 database or from an ARCGIS database is sent to the CM for co-channel coexistence. Topography affect propagation conditions whereby it may promote propagation to near free space conditions or provide isolation allowing spectral reuse.

In the direction from the CM to the CTS

  - Request topographic data from SRTM2002 database or from an ARCGIS database in order to perform the above.

### Interface G resides between the CM and the CRS. In the direction from the CM to the CRS:

  - The CM register to the CRS or log in

  - Security items may consist of login names, passwords, etc.

  - And other information as needed

In the direction from the CRS to the CM

  - The CRS give or not the permission to the CM for coexistence.

  - And other information as needed

### Interface H resides between the CM and the CDS. In the direction from the CM to the CDS:

  - The CM send information to be stored for coexistence

  - And other information as needed

In the direction from the CDS to the CM:

  - The database give the information required for the CM to do an analysis about coexistence with his peers and finally to take a decision

  - And other information as needed

- The IEEE 802.19.1 reference model

5.1 General description

### 5.1.1 Overview

The 802.19.1 system consists of interconnected networks and devices. In many ways, the 802.19.1 system  is similar to the well known Internet Domain Name System (DNS) that resolves URLs to IP addresses.

Through the Operator Management Entity (OME) , the system may enhance operator awareness. This may help the operator in resolving coexistence issues amongst unlicensed operators and in the protection of licensed incumbents. The 802.19.1 system is not a coexistence assurance or a licensed incumbent protection dispute resolution system. Assurance that regulatory radiation levels are not exceeded is the exclusive responsibility and remains the exclusive responsibility of transmitter operators.

The 802.19.1 system network receives, validates, conveys and disseminates data pertaining to the maximum electromagnetic radiation levels a license-exempt transmitter (or array of transmitters for Same Frequency Networks) should be allowed to impress on a victim receiver at a given geographic position and time, before such radiation starts to cause significant degradation to the receiver's ability to correctly decode another signal. The 802.19.1 system is designed from the ground up to allow for enhanced coexistence, subjugation of license-exempt services to licensed incumbents and provisions for voluntary coordination between transmitter operators.

The The 802.19.1 system has been designed with concern for confidentiality of private information. Clients may request information from The 802.19.1 system for whatever reason and for entire geographical areas, irrespective of whether they actually have transmitters or receivers in the area and without divulging how many they may have and where they are. As requests cover an area, the request by itself does not divulge quantity or specific location of transmitters or receivers. Information stored in 802.19.1 devices is protected and secure and only transmitted to authorized recients on a need to know basis using well established cryptographic communication protocols.

    - Coverage and Coexistence example

There may be coverage and coexistence issues.

Take, for example, 2 base stations with omnidirectional antennas and a 4W EIRP output.

On the coexistence front, non-communicating networks working on a co-channel basis will interfere with each other well beyond their intended coverage area as they will be too far to communicate over the air to each other but will cause severe noise degradation in the adjoining areas.

For example, a 900 Mhz WRAN with a -92 dBm sensitivity that can barely operate at 6 Mbps total BW. However, it will hamper communications to a -102 dBm level, which means an interference  radius 3 times larger than the over the air communication range.

Interference range

100Km

QPSK range

30Km

QAM-16 range

10Km

QAM-64 range

3Km

Example Sensitivity

Modulation	Speed		sensitivity

900 MHz	2400 MHz

Mbps		T-dBm	U-dBm

801.11b		1	DBPSK	-92	-97

801.11b		2	DQPSK	-89	-96

801.11b		5.5	CCK	-86	-95

801.11b		11	CCK	-83	-92

801.11g		6	BPSK	-87	-94

802.11g		9	BPSK		-93

80211g		12	QPSK		-91

802.11g		18	QPSK		-90

801.11g		24	QAM-16	-78	-86

801.11g		36	QAM-16	-74	-83

801.11g		48	QAM-64	-69	-77

801.11g		54	QAM-64	-67	-74

- 5.1.3 System design

This section provides requirements for 802.19.1 system. System requirements that can be grouped into four different categories: General, Discovery, Communication and Algorithm. The requirements can be also grouped from the view point of whether they relate to the system’s internal operation or interactions with world outside the system. Both these aspects are illustrated in the Figure 1 below that provides an overview of the system requirements.

Figure 2. Overview of system requirements

5.1.3.1 Requirement R1

802.19.1 system contains a Coexistence Discovery and Information Server (CDIS)  enabling the discovery of 802.19.1 compliant networks and devices. 802.19.1 systems need to identify potential 802.19.1 compliant networks or devices that need to coexist as a crucial step in order to achieve coexistence. The CDIS enables discovery of 802.19.1 compliant networks and devices via queries and responses of one or more specialized DNS services. This specialized DNS service has in it's URL the latitude and longitude of all 802.19.1 compliant systems it knows as well as the URL of peers specialized DNS services. Queries made to this DNS include an area of interest and the response contains all 802.19.1 networks and devices known to this DNS as well as the IP addresses of any other DNS known by this DNS to have additional information about devices in the requested area. Via iterative requests and responses the entire contents of a geographical area may be obtained. DNS' also registered to higher level DNS' as is done in the normal domain naming system.

5.1.3.2 Requirement R2

802.19.1 system shall obtain and update information required for TVWS coexistence via interface C. The format and content of the queries to and the updates of the TVWS database and the responses from the TVWS database are determined by local regulations and 802.19.1 systems shall have a driver tailored to the requirements of local regulators where operation is provided. If such driver is not available 802.19.1 systems shall respond to queries with “TVWS database not available”. Updates will be periodically refreshed and will comply to local regulatory requirements.

Updates of the records of the specialized DNS services shall be updated according to their “time to live”. The TTL of the DNS record is a standard item defined by the IETF and is beyond the scope of this standard. It ranges from 1 to 65535 second(s). These records may be updated more frequently, for example, for devices which are subject to location changes or which may autonomously changes their spectrum utilization.

5.1.3.3 Requirement R3

The coexistence manager shall exchange obtained information. The information to be exchanged between networks and devices may be centralized or distributed in various database servers. For example there may be a TVWS server, a Geographic/topographic resolver, a specialized DNS server and a coexistence database server (CDS server). The coexistence manager dialogs via procedures and protocols defined in section 6 with these various servers using SSH links.

5.1.3.4 Requirement R4

The coexistence manager shall be able to provide reconfiguration requests and/or commands as well as corresponding control information to 802.19.1 compliant TVBD networks and devices to implement TVWS coexistence decisions via interface B1. Examples of reconfiguration requests/commands by  the coexistence manager are: to change center frequency, to adjust transmit power, affect time scheduling.

The coexistence manager may request another coexistence manager to take actions to improve coexistence between peers via interface B3. It may also send requests, receive notifications and confirmations from the coexistence enabler via interface B1.

The coexistence manager may also send and receive reconfiguration requests and/or commands and corresponding control information from another coexistence manager via a SAP on interface B1 and/or interface B3. This allows the coexistence manager to control radio network nodes, e.g. base stations, access points, stations, customer premises equipment (CPE).

5.1.3.5 Requirement R5

The wealth of information collected by the networks and devices needs to be analyzed by the 802.19.1 system. The analysis may be done in the CM. However to minimize the cost of a device the non real time analysis may be performed by an external centralized or distributed service tied to the coexistence database server.

5.1.3.6 Requirement R6

After analysis the 802.19.1 system may make TVWS coexistence decisions. These decisions will be sent by the coexistence manager through coexistence enabler via interface B1. Furthermore these decisions may be communicated to the coexistence database server or other coexistence managers via interface B3.

For this version of the draft, we propose that only similar networks (self-coexistence) should be allowed to operate co-channel and that co-channel operation of dissimilar networks should be kept for future revision of the standard and therefore out of scope of this proposal. Self-coexistence is well documented and provided by nature in CSMA/CD networks as well as some TDMA networks.

5.1.3.7 Requirement R7

The proposed topology for decision making is scalable. At one extreme, using a single server, it allows for both a centralized model. If additional servers are added it allows a distributed architecture. The physical implementation is flexible allowing for direct autonomous implementation in devices or networks whilst permitting lower cost or legacy devices to use the services of a dedicated or shared external CM.

Attention has been taken to ensure that operation is possible in dynamically addressed realms which are normally located behind NAT firewalls and not directly accessible via a fixed public IP address on a well known TCP/IP port.

Figure 3 is an example that depicts client 5 directly on the internet, a cluster of clients (1,2,3,4) behind a NAT firewall, some of which are in a mesh configuration (1,2,3) and finally a client 3 that can only be accessed via two routes (NAT-4-2-3 or NAT-4-1-2-3) depending on whether link 2-4 is operational.

Link 1-2

Link 1-4

Link 2-3

Link 2-4

Device 5

SU

Device 1

SU

Internet

Optional

NAT

Device 4

SU

Device 2

SU

Device 3

SU

Link 4-N

Link N-I

Link 5-I

Figure 3.  Example of various client network topology

CDS

Server

CE 2

Device 5

SU

CE 4

Human

Interface

Device

CE H

Network A

Network B

CE 1

CE 4

Device 1

SU

CE 3

Internet

Optional

NAT

Device 4

SU

Device 2

SU

Device 3

SU

network

Link type legend

private CE-CDS

private CDS-ext

TVWS

Server

CDS

Server

SQL

Server

RADUIS

Server

Topo

Server

GERDCS

DNS Server

private SU-CE

Figure 4.  Example network topology with some servers behind NAT firewall

CDS

Server

CE 2

Device 5

SU

CE 4

Human

Interface

Device

CE H

Network A

Network B

CE 1

CE 4

Device 1

SU

CE 3

Internet

Optional

NAT

Device 4

SU

Device 2

SU

Device 3

SU

network

Link type legend

private CE-CDS

private CDS-ext

TVWS

Server

CDS

Server

SQL

Server

RADUIS

Server

Topo

Server

GERDCS

DNS Server

private SU-CE

Figure 5. Example network topology with all servers on the internet

5.1.3.8 Requirement R8

802.19.1 system shall support appropriate security mechanisms. This shall include user/device authentication, integrity and confidentiality of open exchanges, and data privacy and policy correctness attestation and enforcement.

This proposal recommends that standard and well established security be used for all exposed communications. The allowable protocols shall be SSH and TLS for machine to machine communication and HTTPS for human to machine communications. 802.19.1 system shall be able to authenticate, provide integrity and/or confidentiality to all entities involved in 802.19.1 data exchange. If a peer device does not behave in compliance with this standard the 802.19.1 system shall enable enforcement of coexistence policies for the 802.19.1 compliant TVBD networks or devices by tagging this device in the coexistence database as non-compliant and taking any secure means to detect and/or to remediate the compromised device/behavior.

802.19.1 system shall support privacy of sensitive data, and secure means to store and process such data while it resides in 802.19.1 entities. For stored data this proposal also purports to use well established security mechanisms to protect sensitive information (see section 6).

5.1.3.9 Requirement R9

As described in section 7, a set of mechanisms to achieve coexistence of TVBD networks and devices.

5.2 Service Access Points (SAPs)

SAPs are found within the 802.19.1 Scope area in figure 1  at each interface termination.

Coexistence enabler Interface A SAP

This interface is beyond the scope of the standard. It shall comply to specific  requirements of the manufacturers of the TVBD devices.

CM Interface B1 SAP

The CM shall support SNMP passthrough.

CM B1 SAP may send the following commands

SNMP_Query:SMNP_query_string

CE Interface B1 SAP

The CE shall support SNMP passthrough.

The CE may send the following responses to the CM

SNMP_NOT_SUPPORTED_BY_DEV - the CE or Device does not support SNMP

SNMP_RESPONSE:response_string

The CE may send the following notifications to the CM

TBD

CM Interface B2 SAP

CM may issue commands to CDIS

DISCOVER_IP_SERVICES(North, West, South, East,LowFrequency, HighFrequency)

CDIS interface B2 SAP

DISCOVER_IP_SERVICES(North, West, South, East,LowFrequency, HighFrequency) - upon reception of this command, the CDIS will immediately iteratively query the CDNS servers it knows and respond with a report listing the most authoritative IP services discovered and any security items that may accompany them such as security items returned by a Coexistence Radius Server. Security items may consist of login names, passwords, cryptographic keys, shared secrets, etc.

CDIS may send notifications to CM

The CDIS will periodically refresh its discovered IP service list and notify CM whenever a change occurs for the specified area and frequency range.

CM interface B3

The local CM shall support CE remote passthrough requests. This allows a remote CM to send commands to a local CE passing through the local CM

CM_CE:CE_B1_SAP_string

The local CM may issue CE passthrough requests. This allows the local CM to send commands to a remote CE passing through the remote CM

CM_CE:CE_B1_SAP_string

The local CM shall support CDIS passthrough requests. This allows a remote CM to send commands to a local CDIS passing through the local CM

CM_CDIS:CDIS_B2_SAP_string

The local CM may issue CDIS passthrough requests. This allows the local CM to send commands to a remote CDIS passing through the remote CM

CM_CDIS:CDIS_B2_SAP_string

The local CM shall support TVWS enabler passthrough requests. This allows a remote CM to send commands to a local TVWS enabler passing through the local CM

CM_TE:TE_B4_SAP_string

The local CM may issue TVWS enabler passthrough requests. This allows the local CM to send commands to a remote TVWS enabler passing through the remote CM

CM_TE:TE_B4_SAP_string

CDIS and CM Interface B4 SAP

TBD

TVWS Enabler Interface B4 SAP

TBD

TVWS Enabler Interface C SAP

This interface is beyond the scope of the standard. It shall comply to local regulation requirements where such requirements exist.

CM Interface D SAP

CM interface D SAP is identical to CM interface B3

Commands on this interface shall be carried out over SSH or HTTPS channels. All Interface B3 commands, responses and notifications shall be supported over the SSH channel. This effectively make Interface D a superset of interface B3.

CDIS Interface E SAP

CDIS may issue queries to CDNS

refer to RFC1034 and RFC1035

URL=North.West.South.East.LowFrequency.HighFrequency.GERDCS.NET)

CM Interface F SAP

Commands and resonses on this interface shall comply to SRTM2002 or ARCGIS over SSH or HTTPS protocol

CM Interface G SAP

Optional, TBD

CM Interface H SAP (editors note: documentation in progress)

The CDS contains tables and fields as follows.

##### Domain_table

A domain is a named collection of bubbles with an author, claims made by the author, root GS URL and Time to Live (TTL). Each bubble is a construct with up to 6 dimensions describing particulars of the claim. The bubble dimensions are constituted of a three dimension geographical area (bound by altitude, latitude and longitudes at its physical borders), a one dimension frequency span (bound by a lower and upper frequency at its bandwidth borders) and a 2 dimension range of direction of arrivals (bound by azimuth and elevation, in degrees).

###### Domain_table.name_field

The domain name (DN) is a 32 character human friendly metafore that identifies a given domain. The domain name “*” is a reserved word. A query with the “*” domain is then based strictly on a specific geographic area and frequency span.

###### Domain_table.code_field

The domain code (DC) is a machine to machine 4 character code (32 bits) used as a shorthand for the domain name. As all bubbles refer to the domain, this shorthand version save disk space, transmission time and CPU cycles in the processing of the data.

###### Domain_table.claims_field

Claims describe the authority of the author over the domain. Typical values are “license-exempt” or “primary incumbent license#......”. A blank or null claim field is interpreted as “unlicensed”.

###### Domain_table.Time_To_Live_field

Time To Live (TTL) is a domain data validation specification used to force periodic updates to the database. It is an int32 integer of seconds. A TTL value of 0xFFFFFFFF indicates that the requested data is not available. Other negative values indicate the time since the TTL has expired. Positive values in the range of 0x00000000 (0 seconds) to 0x00093A79 (decimal 604799 seconds or 1 week) indicate the remaining TTL before a refresh is required. A positive value of 0x7FFFFFFF is reserved for spectral reservation cancellations. Positive values outside this range are reserved for future use.

###### NumberOfBubbles

The Number of Bubbles (NOB) indicates how many bubbles form the domain or in the case of a response to a geographically limited query, the number of bubbles that will be returned.

#### Spectral Limit Record

The Spectral Limit Record (SLR) record contains all the data describing a specific bubble with a signal amplitude level (in microvolts/meter) in a given direction of arrival (in degrees from true north) that can not be exceeded without causing harm to the domain author's service.

###### Latitude and Longitude

The earth's radius is ~6380km. A degree of longitude at the equator is equivalent to 111.35 km. 802.19.1  uses a resolution of .00001 degrees and therefore can operate to approximately one meter resolution of latitude and longitude. Decimal degrees with five digits to the right of the decimal point are used to communicate all latitudes and longitudes. The optional decimal point is ignored by the GS as all longitude and latitude data values are int32 values.

###### Altitude

802.19.1 represents altitude over sea level in meters with a resolution of one meter. Altitude is an int32 value. The top of the bubble is represented by the “Altitude max” value. The bottom of the bubble is represented by the “Altitude min” value. These value have a range of:

+2,000,000,000 > Altitude max > Altitude min > -2,000,000,000

###### Arrival Angle

Azimuth reference frame:	0 degrees = signal arrives from true north

90 degrees = signal arrives from true west

resolution: 2 degrees

type: integer

Azimuth range min	range:   -181 < H/V offset < Azimuth range max

Azimuth range max	range:   Azimuth range min < H/V offset <+181

Elevation 		0 degrees = signal arrives ground

90 degrees = signal arrives from logical horizon

180 degrees = signal arrives from zenith

resolution: 1 degree

type: integer

Elevation range min	range:   -91 < H/V offset < Elevation range max

Elevation range max	range:   Elevation range min < H/V offset < +91

###### Polarization

Polarization is represented by the ratio of Horizontal to Vertical polarization and the phase offset between the two. The “H/V ratio” is expressed in decibels and has a range of:  -121 < H/V ratio < +121. It is of int8 type. The “H/V phase offset” is the phase offset of the vertical component in relation to the horizontal component. It is of int8 type, each integer value representing 2 degrees of offset. Its range is: -182 < H/V offset <182

###### Frequency Range

Min and max frequencies are of unsigned int32 format in KHz with a range of 0-4 Penta Hz.

Range: 0 < Frequency Min < Frequency Max < 2^32 KHz

###### Amplitude

This is the maximum value of the modulus of the amplitude (in dBμv/m) of a signal arriving at the specified arrival angle and polarization before it causes significantly harmful interference to a receiver located in this bubble.

Create CDS database

//

Create table domains(

name varchar(32),

id varchar(4) not null, (unique)

claims varchar(128),

ttl int(11)

owner_user_id varchar(4)

max_hop_count smallint(4)

);

CREATE TABLE IF NOT EXISTS `tblBubbles` (

`domain_id` varchar(4) character set utf8 NOT NULL,

`bubble_id` smallint(4) NOT NULL default '0',

`description` varchar(200) character set utf8 NOT NULL,

`shape` varchar(12) character set utf8 NOT NULL default 'CIRCLE',

`fill_color` varchar(6) character set utf8 NOT NULL default 'FF0000',

`zorder` smallint(6) NOT NULL default '1',

`azim_min` smallint(4) default NULL,

`azim_max` smallint(4) default NULL,

`elev_min` tinyint(4) default NULL,

`elev_max` tinyint(4) default NULL,

`hv_ratio` tinyint(4) default NULL,

`hv_offset` tinyint(4) default NULL,

`freq_min` int(11) default NULL,

`freq_max` int(11) default NULL,

`amplitude` tinyint(4) default NULL,

`radius` int(11) NOT NULL default '0',

`hat_center` int(11) NOT NULL default '0',

`hat_receivers` int(11) NOT NULL default '0',

PRIMARY KEY  (`domain_id`,`bubble_id`)

) ENGINE=MyISAM DEFAULT CHARSET=latin1;

/******

the following table is used by the CDS to keep track of who is registered (based on timestamp age) for each domain. This is used by the server to issue notifications to all registered users of a domain when a change happens to the domain data set.

******/

Create table registered_users(

user_id int,

domain_code varchar(4),

last_reg_req tmiestamp(), // last timestamp when GeolocationRegistrationRequest(* or domain_code); was received

);

/******

the following table  is used by the CDS to hold information about registered users, the scope of what they can access and make absolutely sure only registered users have access to CDS data. This is important as it controls AAA, etc.

******/

Create table users(

user_id int primary_key,

registrant_name varchar(128),

registrant_url varchar(128),

password varchar(32),

lat_min int, lat_max int, lon_min int, lon_max int, // scope limits

upload_count int, download_count int, // accounting info to bill registered user activity

// other AAA stuff TBD

);

5.2.1 Data formats and parameters

#### Geolocation Registration Requests

A Geolocation Registration Request (GRR) requests the establishment and maintenance of a virtual connection between a CM and a CDS. It is re-issued at least every minute to indicate continued presence of a CM to a CDS and its current interest in up to date information.

// request registration to a specific domain

GeolocationRegistrationRequest('CONNECT',DomainName);

// request termination of registration to a specifi domain

GeolocationRegistrationRequest'TERMINATE',DomainName);

// request maintenance of all alctive registrations

GeolocationRegistrationRequest('MAINTAIN');

// request termination of all alctive registrations

GeolocationRegistrationRequest('BYE');

#### Spectral Limit Queries

A Spectral Limit Querry (SLQ) is issued by a CM and received by a CDS. This querry specifes a domain, a geograpical area delimited by top left (North-West) and bottom right (South-East) lattutde and longitude and a frequency range in Hertz (LowFrequenecy <= range < HighFrequency). Examples of such a querry are:

// request for all domains in a geagraphic area and frequency range

SpectralLimitQuerry(”*”, North, West, South, East, LowFrequency, HighFrequency);

// request for a specific domain, namely “WXYZ”

SpectralLimitQuerry(”WXYZ”,”*”,”*”,”*”,”*”,”*”,”*”);

// request for a domain entire dataset in a geagraphic area

SpectralLimitQuerry(”WXYZ”, North, West, South, East, “*”,”*”);

#### //Revision Version

// Protocol: Request for a specific domain

SpectralLimitQuerry('DomainName', North, South, West, East, Alt_Min, Atl_Max, Freq_Min, Freq_Max, Azim_Min, Azim_Max, Elev_Min, Elev_Max, HV_Ratio, HV_Offset, Amplitude);

#### Spectral Limit Record Transmission

When a CDS receives an SLQ, it may transmit an array of Spectrum Limit Records (SLR). Furthermore, it may respond with the URL of another authoritive CDS caching data on the requested area. If the CDS responds with a URL, the CM should re-issue the same SLQ to this URL.

SpectralLimitRecord(DomainName, Claim, HopCount, AuthorURL, TTL, QuantityOfBubbles);

SpectralLimitRecord(DomainName[BubbleID], North, West, South, East, LowFrequency, HighFrequency, Amplitude);

EX:

SpectralLimitQuerry('Domain01', '*', '*', '*', '*', '*', '*', '*', '*', '*', '*', '*', '*', '*', '*', '*','*');

SpectralLimitQuerry('Domain01', '*', '*', '*', '*', '*', '*', '*', '*', '*', '*', '*', '*', '*', '*', '*','100');

SpectralLimitQuerry('Domain01', '*', '*', '*', '*', '*', '*', '*', '*', '*', '*', '*', '*', '*', '*', '*','10');

SpectralLimitQuerry('*', '*', '*', '*', '*', '*', '*', '*', '*', '*', '*', '*', '*', '*', '*', '*','100');

/Revision Version

SpectralLimitRecord('DomainName[BubbleID]', North, South, West, East, Alt_Min, Atl_Max, Freq_Min, Freq_Max, Azim_Min, Azim_Max, Elev_Min, Elev_Max, HV_Ratio, HV_Offset, Amplitude);

Unsoliscitated SLR may also be sent by any CDS to any registered CM at any time. The CM may update its record if the TTL of the record it receives from a CDS is greater than the TTL of its cuurent record for the specified area. Furthermore, the CM shall acknowledge the update of its SLR or NAK with the reason for refusal. A CDS may delete an SLR who's TTL is 0x7FFFFFFF as soon as it is assured that all registered CM and subordinate OME have acknowledged reception of the updated SLR. A CM may delete a SLR who's TTL has expired after requesting a refresh to an authoritative CDS for the area and obtaining either a TTL=0x7FFFFFFF or a TTL=0xFFFFFFFF record as response. BubbleID is an array index in the SLR array. BubbleID zero is reserved for the Domain Name Definition.

#### Spectral Client Protocol

The OME has the ability to request additions, deletions and edits of the CDS' SLR's via a SC. After validation, the CDS may acknowledge the SC of the SLR update or NAK the SC and refuse to update the SLR based on the SC request and the CDS' internal policies. These policies are beyond the scope of this proposal and could, for example, include comparison of a territory request to license grants of the regulator of the area and international treaties. The NAK shall be accompanied with a human readable text stating the reason for update refusal.

A SC may also define a domain name. The domain definition is a list of DefineSLR statements defining the maximum amplitude for given domain bubbles.

// old Version

DefineSLR(DomainName, MaxHopCount, TTL);

DefineSLR(DomainName[BubbleID], North, West, South, East, LowFrequency, HighFrequency,  Amplitude);

DefineSLR(DomainName, North, West, South, East, LowFrequency, HighFrequency,  Amplitude);

...

DefineSLR(”WXYZ[n]”, North, West, South, East, LowFrequency, HighFreq, Amplitude);

// Revison Version:

DefineSLR(DomainName, MaxHopCount, TTL, Claims);

EX:

DefineSLR('Domain06', '10', '15', 'test claims');

DefineSLR(”WXYZ[n]” , North, South, West, East , Alt_Min, Atl_Max, Freq_Min, Freq_Max

, Azim_Min, Azim_Max, Elev_Min, Elev_Max, HV_Ratio, HV_Offset

, Amplitude);

EX:

DefineSLR('Domain06[200]', 12, 14, 35, 12, -12, 12, 5, 50, 4, 5, 6, 7, 8, 9, 10);

Note:

DefineSLR

Long_Min = Ouest

Long_Max = Est

Lat_Min = Sud

Lat_Max = Nord

Using domain name definitions, a SC may define irregularly shaped bubble areas, define image frequency and adjacent channel protection level bubbles, etc.

An OME, network access point or base station (collectively hereon referred to as “Base Station” or “BS”) queries the CM with the range of date & time, longitude, latitude, altitude and expected angle of arrival of a signal at a reception point. The CM having consulted known authoritative CDS for the point of interest responds with the maximum allowable radiation level (in microvolts per meter) corresponding to this range of parameters. Typically, a transmitter operator OME will send many such queries to a SC, mapping each and every point in the space around the transmitter and capping the transmit power according to its transmission antenna radiation pattern, terrain topography surrounding the transmitter, propagation models and conditions at the time of desired transmission. In turn, if the operator/transmitter controls and allows subjugate transmitters to operate, the operator bears the responsibility of collecting the GERDCS information for each of these transmitters and capping their transmit power accordingly, taking into account their antenna radiation pattern, gain and orientation.

Responsibility for respecting regulator rules and protecting incumbents solely remains with the transmitter's operator. It is the sole responsibility of the network operator to operate the CM and ensure that the CM obtains its information from authoritative CDS. In doubt or if any claims are made by a licensed incumbent, it is the network operator's responsibility to immediately turn down any potentially offending transmitter and then to immediately advise CDS operators of any errors or omissions in the CDS data.

The CDS stores and documents an audit trail of the originator identity and time stamp of any CM requests.

6. Procedures and protocols

6.1 Generic procedures for information exchange

6.2 Protocols

7. Coexistence mechanisms and algorithms

Annex A: Use cases

Annex B: Evaluation methodology

Annex C: Coexistence Enabler (CE) software reference design

The IEEE Standards Dictionary: Glossary of Terms & Definitions is available at http://shop.ieee.org/.