IEEE P802.19.1
Wireless Coexistence Task Group

| Coexistence System proposal |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-11-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Joe Kwak | InterDigital | POB 93, Hawkesbuty, ON Canada K6A2R4 | 630-739-4159 | joekwak@sbcglobal.net |
| Juan Carlos Zuniga | InterDigital |  |  | JuanCarlos.Zuniga @interdigital.com |
| Alex Reznik | InterDigital |  |  | Alex.Reznik @interdigital.com |

Abstract

This document is a response to the call for proposals (19-10/0057r2) of 802.19 Task Group 1 (TG1). This document contains a text proposal for a coexistence system specification. The proposal has been prepared using terms and a draft outline that is a different from the one in the system design document (19-10/0055r3) of 802.19 TG1. The modified architecture entity names and terms used in this draft better reflect the anticipated applications for a Coexistence System than the terms and names used in the system design document. The draft outline is submitted to the task group for consideration to facilitate development of the 802.19.1 draft specification.

Service definition and service bundling have been developed in more detail than was presented at the September meeting.  Coexistence Basic Service is provided to any registered CE. Basic services are  limited to network/environmnet discovery and disclosure of available channels in CE location.  Coexistence Information Services and Coexistence Management Services are optional.  A CE may subscribe to these optional services after the CE identity has been authenticated by the Coexistence System.

Certain areas of this proposal are incomplete and are so marked with a TBD.  Further revisions to this proposal will be made prior to presentation to the TG.

Notice: This document has been prepared to assist IEEE 802.19. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

1. Overview

[TBD]

1.1 Scope

[TBD]

1.2 Purpose

[TBD]

1.3 Sample application areas

[TBD]

1.4 Conformance

[TBD]

2. Normative references

[TBD]

3. Definitions, acronyms and abbreviations

3.1 Definitions

[TBD]

3.2 Acronyms and abbreviations

[TBD]

4. Coexistence Services System description

This standard defines a Coexistence Services System (CSS) to collect,  provide and share information about the local TVWS radio environment and provides coexistence solutions to improve the coexistence among TVWS users and networks who subscribe to these services. These services are provided on the internet  in a media independent fashion by using a Coexistence Client function which resides in the subscriber and provides convergence and translation functions for the particular link-layer technologies, e.g. 802.11, in use at the current time.

The following items are not within the scope of this standard:

—	Regulatory body  (FCC and others)  rules and regulations governing use of the TVWS

—	Registration and access to TVWS incumbent protection database, as define by regulatory body.

—	Enhancements specific to particular link-layer technologies that are required to support this standard (they will be carried out by those respective link-layer technology standards)

—	Higher layer (layer 3 and above) enhancements that are required to support this standard

4.1 Architecture

The CSS architecture has three logical entities and six logical interfaces, as shown in Figure 1. A CSS logical entity is defined by its functional role(s) and its interfaces with external entities and other CSS entities.

The CSS logical entities are:

- Media Independent Support Server (MISS),

- Media Independent Coexistence Manager (MICM),

- Coexistence Client (CC).

The above CSS entities interact with three external entities:

- Subscribing TVBD network or device (the CSS user),

- TVWS database,

- Operator Management Entity (OME).

The CSS logical external interfaces are:

- Interface A, between the CC and the subscribing TVBD network or device,

- Interface C, between the MICM or MICS and the regulatory TVWS database,

- Interface D, between the MICM and an Operator Management Entity

The CSS logical internal interfaces are:

- Interface B1, between the CC and the MICM,

- Interface B2, between the MICM and MICS,

- Interface B3, between the local MICM and any remote peer MICMs.

CSS

Media Independent Coexistence Manager (MICM)

Coexistence Client

(CC)

Interface B3

TVBD network or device

Interface A

Media Independent Support  Server (MISS)

TVWS Database

Interface B2

Interface C

Interface C

Operator Management Entity

Interface D

: Boundary of CSS,

entities inside are part of this CSS,

others are external to this CSS.

Remote Peer MICM in

neighborCSS

Interface B1

Figure 1: 802.19.1 Coexistence Services System (CSS)

The implementation details of external interfaces C and D are defined elsewhere by each Operator and Database Service Provider.  However, this standard provides a set of service access primitives which may be used by Operators and Database Service Providers to better understand the functions and operation of the CSS and define the information exchanges required on interfaces C and D.   Further information about interface C and D may be found in the Annexes.

The implementation details of external interface A are defined by the subscriber equipment manufacturer and are also out of scope for this standard.  However, this standard provides a set of service access primitives which may be used by manufacturers to better understand the functions and operation of the CSS and define the information exchanges required on interface A.

The internal interfaces are defined in detail in this standard to promote interoperability of CSS entities which may be produced by different manufacturers.  In addition, these internal interfaces may be viewed as external interfaces when the CSS entities are not collocated so that external transport, e.g.  over IP, is needed to implement the interface.

4.2 Logical entities

The six CSS logical entities are defined by their functions, operations and interfaces.  The major functions of each CSS entity are listed here.

4.2.1 MISS functions

The major functions of the MISS are

- To support discovery of available CSS resources and CSS entity location and addressing,

- To facilitate interface establishment between CMs

- To collect, aggregate, and provide information facilitating coexistence for subscribing CSS users.  This includes updated TVWS channel availability information, geo location data storage, data processing, etc.

4.2.2 MICM

The major functions of the MICM are

- To assist discovery of other MICMs

- To identify and solve coexistence problems and to provide coexistence solutions for subscribing TVBD networks and devices

- To implement coexistence decision making which includes generating and providing corresponding coexistence requests/commands and control information to CC(s)

- To support exchange of information required for CSS services among MICMs. This may include data exchange concerning current radio environment, information describing the MICM domain and geographical domain boundaries,  and information to assist hierarchical and/or peer-to-peer decision making capabilities for MICM deployments

- To provide updated TVWS channel availability information to connected CC(s)

- To assist network operators to configure and manage the CSS services for their networks.

4.2.3  CC

The major functions of the CC are

- To request and obtain sensing, configuration, channel state and other information required for coexistence from TVBD network or device

- To translate reconfiguration requests/commands and other information received from the MICM in media independent formats  into TVBD-specific formats and send them to the TVBD network or device.

- To translate reconfiguration requests/commands and other information received from the TVBD network or device into media independent formats and send them to the MICM.

    - TVBD-C

The TVBD-C is a logical CSS user entity representing CSS client functions embedded in a user TVBD device as shown in Figure 2.  The TVBD-C contains a Coexistence client (CC). The CC interfaces to the TVBD device by means of two SAPs, the CSS_Net_SAP and the Coex_Link_SAP.  Additional SAP details are provided in clause 5.2.  Note that the TVBD-C may also be a PoA such as an AP or BS and may connect to many other CSS users and non-CSS TVBDs.

Medium: 802.11, 802.22

or other

PHY

MAC

.

LLC

L3 and

Higher Layers

LLC_SAP

PHY_SAP

CSS

CSS_Net_SAP

Coex_Link_SAP

Interface A

Interface B1

Coexistence Client

(CC)

(Media Independent Transport)

(Media Dependent:

-Media Access functions

-Sensing functions

-Configuration functions)

Figure 2: TVBD-C is a CSS user  and a CSS Client

    - TVBD-M

The TVBD-M is a logical CSS user entity representing CSS manager functions embedded in a user TVBD device as shown in Figure 3..  In most applications, the TVBD-M will be a PoA for other TVBD devices such an AP or BS.  As a PoA the TVBD-M may connect to many other CSS users and non-CSS TVBDs. The TVBD-M contains a Coexistence Client (CC) and a Media Independent Coexistence Manager (MICM). The CC interfaces to the TVBD device by means of the Coex_Link_SAP  The MICM interfaces to the TVBD device by means of the CSS_Net_SAP.  Additional SAP details are provided in clause 5.2.

Medium: 802.11, 802.22

or other

PHY

MAC

.

LLC

L3 and

Higher Layers

LLC_SAP

PHY_SAP

CSS

CSS_Net_SAP

Coex_Link_SAP

Interface A

Interface B2

Coexistence Client

(CC)

(Media Independent Transport)

(Media Dependent:

-Media Access functions

-Sensing functions

-Configuration functions)

Media Independent Coexistence Manager

(MICM)

Interface B1

(internal)

Figure 3:TVBD-M is a CSS user who is also a CSS Manager

4.2.6	CServ

The CServ is a logical entity representing the Coexistence Server functions normally located on the  network as shown in Figure 4 .  The CServ contains a Media Independent Support Server (MISS)  and a Media Independent Coexistence Manager (MICM). The CServ interfaces to other CSS entities and non-CSS entities using internet transport and protocols.  The CServ interfaces are defined by detailed message formats (IEs) defined elsewhere in this standard.

TVWS Database

Operator Management Entity

Interface B3

Interface D

Interface C

(defined elsewhere)

(defined elsewhere)

(for synchronization

and coordination

with other MICM(s) )

Interface B1

(to TVBD-C)

CServ

Media Independent Coexistence Manager

(MICM)

Media Independent Support  Server

(MISS)

Interface B2

(internal)

Interface B2

(to TVBD-M)

Figure 4:CSS Server (CServ) is located on the Network

4.3 Interfaces

The CSS employs six different logical interfaces to interconnect the CSS entities within the CSS and also to interconnect the CSS entities with external entities such as the TVWS database.

4.3.1  Interface A

Interface A is the interface between CC and TVBD network or device

Information transferred from the TVBD network or device to the CC includes:

- Configuration information including MAC Address, operating mode, serving channel, TX power, enabled device capabilities, services in use, etc.,

- Channel state information including sensing and measurement data,

- Link state information including QOS, UL and DL traffic load, PoA MAC Address, etc.,

- Requests for coexistence information,

- Information responses to requests for  coexistence information,

- And other information as needed.

Information transferred from CC to TVBD network or device

- Reconfiguration requests/commands and control information (corresponding to coexistence requests/commands and control information received from CM)

- Requests/commands related to control of measurements and sensing performed by TVBD network or device

- Requests for coexistence information,

- Information responses to requests for  coexistence information,

- And other information as needed

4.3.2  Interface B1

Interface B1 is the interface between CC and MICM

Information transferred from CC to MICM

- Information required for coexistence (information obtained from TVBD network or device),

- Requests for coexistence information,

- Information responses to requests for  coexistence information,

- And other information as needed.

Information transferred from MICM to CC

- Coexistence requests/commands and control information

- Requests for coexistence information,

- Information responses to requests for  coexistence information,

- And other information as needed

4.3.3  Interface B2

Interface B2 is the interface between MICM and MIIS

Information transferred from MICM to MIIS

- Information required for discovery (obtained by this MICM)

- Information required for coexistence (obtained by this MICM)

- Requests for coexistence information,

- Information responses to requests for coexistence information,

- And other information as needed

Information transferred from MIIS to MICM

- Information required for discovery (obtained from remote MICMs)

- Information required for coexistence (obtained from remote MICMs)

- Requests for coexistence information,

- Information responses to requests for coexistence information,

- And other information as needed

4.3.4  Interface B3

Interface B3 is the interface between MICM and remote MICM

Information transferred from MICM to MICM

- Information and messages required for discovery and coexistence,

- Information needed to synchronize local CSS database with remote CSS database.

- Requests for coexistence information,

- Information responses to requests for coexistence information,

- And other information as needed

4.3.5  Interface C

Interface C is the interface between MICM/MIIS and TVWS database

Information transferred from MICM/MIIS to TVWS database

- TVBD device or network user information required for registration to access TVWS database,

- Information requests for coexistence ,e.g. information on available TV channels,

- And other information as needed

Information transferred from TVWS database to MICM/MIIS

- Information responses for coexistence, e.g. information on available TV channels,

- And other information as needed

4.3.6  Interface D

Interface A is the interface between CM and OME

Information transferred from OME to CM

- Network operator related information, e.g. spectrum policy/limitations concerning operator networks and resource sharing.

- Neighbor network information, e.g. information and resource sharing agreements with neighboring networks which use CSS,

- CSS configuration information for use of CSS services in the network,

- And other information as needed.

5. CSS Reference Model

5. 1 General Description

This standard defines services offered by the CSS; these services prevent or decrease interference among TVWS users and improve coexistence in the TV band. All CSS services are optional in that a new CSS user registering with the CSS must subscribe to any service offered by the CSS in that area. There are three Coexistence services defined.

- Basic/Support Services

  - This service provides the basic functionality to configure and manage elements, inter-element communication and service usage

  - Comprises of a set of individual services from which some need to be supported by all the entities and some are entity dependent

- Coexistence Information Services

  - This service permits the exchange of information among CSS entities

  - A CC subscribes to this service when the TVBD associated to it wants to obtain information about its environment which may be available in the CSS.

  - A CC which subscribes to the Information Service agrees to provide and share information it has available with other CSS entities.

- Coexistence Management Services

  - Provided by the MICM to the CC

  - A CC subscribes to this service when the TVBD associated to it wants the coexistence service system to make decisions on the operational parameters of the TVBD or network.

  - A CC which may also be a network controller (e.g. 802.11 Access Point) agrees to apply any CSS provided operational parameters to all of its network users.

The CSS provides services through well-defined SAPs for link layers and transport. In the case of a system with multiple underlying radio interfaces of arbitrary type, the CSS users use the event service, command service, and information service provided by CSS to evaluate and select the best medium for operation and to operate efficiently when operating in the dynamic TV band radio environment. These services provided by CSS help the CSS users to effectively coexist with all TVWS users, to minimize interference,  and to maintain service continuity by adapting to varying TVWS radio environments.  In a system containing heterogeneous radio technologies, IEEE802 radios and other radios, the CSS information sharing and control functions helps the CSS users to implement effective coexistence across these heterogeneous network interfaces.

5.1.1 Coexistence information service

The coexistence information service (CIS) provides a framework and corresponding mechanisms by which a CSS entity can discover and obtain network information existing within a geographical area to facilitate operation and coexistence in the TV band.

The neighboring network information discovered and obtained by this framework and mechanisms can also be used in conjunction with user and network operator policies for optimum channel and network selection/reselection and for resource sharing among networks..

During network selection, the CSS user and the CSS need to exchange information about available candidate networks and channels and select the best network/channel combination. In addition the CIS may provide information about the locations and channels in use by other networks which are not candidate networks and use different radio technologies. The network selection/reselection mechanism can select a different network than the current one. The channel selection/reselection mechanism can select a different channel based on channel utilization and the interference environment of that channel. When a new network has been selected, mobility management protocols handle packet routing aspects such as address update and transfer of packet delivery to the new network.

CIS primarily provides a set of information elements (IEs), the information structure and its representation, and a query/response type of mechanism (pull mode) for information transfer. The information can also include spectrum and channel sharing policies and other operator policies. The definition of such policies is outside the scope of this standard. CIS also supports a push mode wherein the information can be pushed to the TVBD network or device.

The information can be present in a TVWS database server or in an operator management entity which is not a part of the CSS. The definitions of the TVWS database server and operator management entity are outside the scope of this standard.

CIS typically provides static link-layer parameters such as channel information, the MAC address and security information of a point of attachment (PoA). Information about available higher layer services in a network can also help in more effective coexistence decision making for the CSS user.

The information provided by CIS conforms to the structure and semantics specified within this standard. CIS specifies a common (or media independent) way of representing this information across different technologies by using a standardized format such as extensible mark-up language (XML) or binary encoding. A structure of information is defined as a schema.

CIS provides the ability to access information about all networks in a geographical area from any single L2 network, depending on how the IEEE 802.19.1 service is implemented. CIS either relies on existing access media specific transports and security mechanisms or relies on L3 transport and L3 security mechanisms to provide access to the information. How this information is developed and deployed in a given network is outside the scope of the standard. Typically, in a heterogeneous network composed of multiple media types, the CIS will collect information from different media types and assemble a consolidated view to facilitate coexistence among the heterogeneous networks in use.

Some networks such as the 802.11 networks already have an existing means of detecting a list of neighborhood base stations within the vicinity of an area via neighbor report messages. Some IEEE standards define similar means and support CSS users in detecting a list of neighborhood access points within the vicinity of an area via either beaconing or via the broadcast of MAC management messages. CIS defines a unified mechanism to the higher layer entities to provide channel and network utilization information in a heterogeneous network environment by a given geographical location. In the larger view, the objective is to help the higher layer channel selection/reselection protocol to acquire a global view of the heterogeneous networks to effect seamless network reconfigurations across these networks.

5.1.2  Coexistence management service

5.1.2.1 General

The command service enables the MICM to control functions of the CSS user to perform and report sensing and measurements, to report and reconfigure operating parameters of the physical, data link, and logical link layers (also known as “lower layers”) in use by the CSS user. The CSS controls the reconfiguration or selection of an appropriate channel by using the coexistence mechanisms and algorithms described in clause 9. If a CSS user subscribes to the command service, all CSS commands are mandatory in nature. When an CSS receives a command, it is always expected to execute the command.

5.1.2.2 Command origination

Commands are invoked by the MICM, as shown in Figure TBD.

5.1.2.3 Command destination

The destination for a command is the CSS user, either the lower layers or a higher layer function for certain commands. Each MICS primitive has a single recipient and is routed by the CSS user to the intended recipient of each command within the CSS user’s  protocol stack.

5.1.2.4 Command service flow

In the case of local commands, messages often propagate from the local MICM to the CC and then from CC to lower layers. In the case of remote commands, messages propagate from Remote MICM in one protocol stack to the CC in a peer protocol stack (with the use of the CSS Protocol). One of the protocol stacks can be present in an CSS user while the other can be present in a fixed network entity. This network entity is either a point of attachment or any node not directly connected to the other protocol stack.

5.1.2.5 Command service use cases and functions

The commands generally carry the upper layer decisions to the lower layers on the local device entity or at the remote entity. For example the command service can be used by the CSS in the network to request that a CSS user share the serving channel using TDMA with another network using a different radio access technology.  Or the command service can be used to request that a specific measurement be performed by a particular TVBD and that the results be reported to the requesting CSS entity.

This CSS accommodates both CSS initiated (by the MICS) and CSS user initiated reconfigurations. Reconfigurations are initiated by changes in the TV band radio environment that leads to the reselection of a channel or a network that supports the CSS user.  When a CSS user intitiates a reconfiguration, the CIS or MIES is used to update the CSS database IEs for the new configuration of that CSS user.

This standard supports a set of media independent commands that help with channel and network selection under different and dynamic interference conditions. These commands allow the MICM to initiate reconfigurations and exchange information about available channels/networks and negotiate the best available channel/network under different conditions. Please refer clause 9 for more information. These commands do not affect packet routing aspects and can be used in conjunction with other mobility management protocols such as MIP and SIP.

5.1.3 CSS reference framework

5.1.3.1 General

The following clause describes the key points with regards to communication between different CSS entities in the CSS user and the network. The reference points in this subclause (5.1.4) are for illustration only. This subclause does not define any specific deployed network system architecture.

5.1.3.2 CSS communication model

CSS Functions communicate with each other for various purposes. The CSS user exchanges CSS information with its CSS point of service (PoS). The CSS in any Network Entity becomes an CSS PoS when it communicates directly with a MICM or CC within a  CSS user. When an CSS in a Network Entity does not have a direct connection to the CSS user, it does not act as an CSS PoS for that particular CSS user. However the same CSS Network Entity can still act as CSS PoS for a different CSS user.

A CSS user can have multiple L2 interfaces. However, CSS communication need not take place on all L2 interfaces of an CSS-capable CSS user. As an example, on an CSS-capable CSS user with three L2 interfaces, namely IEEE 802.11, IEEE 802.22, and IEEE 802.3, the IEEE 802.3 interface might be used only for system administration and maintenance operations, while the IEEE 802.11 and IEEE 802.22 interfaces might engage in the provisioning of CSS services. The CSS user can use L2 transport for exchanging CSS information with an CSS PoS that resides in the same Network Entity as its Network PoA. The CSS user can use L3 transport for exchanging CSS information with an CSS PoS that does not reside in the same Network Entity as its Network PoA. The framework supports use of either L2 or L3 mechanisms for communication among CSS network entities.

Figure 5 shows the CSS communication model. The model shows CSSs in different roles and the communication relationships among them. The communication relationship shown in Figure 5 applies only to CSSs. It is important to note that each of the communication relationships in the communication model does not imply a particular transport mechanism. Rather, a communication relationship only intends to show that passing CSS-related information is possible between the two different CSSs. Moreover, each communication relationship shown in the diagram encompasses different types of interfaces, different transport mechanisms used (e.g., L2, L3), and different CSS service related content being passed (e.g., CIS, MICS, or MIES).

in TVBD-C

or TVBD-M

RP1

RP3

PoA

RP2 & RP3

RP1

RP3

CSS PoS in

TVBD-M

RP1 or

RP2

Other PoA

non-PoA

Network

Entity

CSS user

CSS PoS

in CServ

non-PoA

Network

Entities

RP2

non-PoS CSS in

TVBD-M

RP3

RP3

Client Side

Network Side

non-PoS

CServ

Figure 5—CSS communication model

The communication model assigns different roles to the CSS depending on its position in the system.

- CSS user  in the TVBD-C or TVBD-M

- CSS PoS in the TVBD-M that includes the serving PoA of the CSS user

- CSS PoS in the Network Entity that does not include the serving PoA for the CSS user

- CSS non-PoS in a Network Entity that includes a non-serving PoA

- CSS non-PoS in a Network Entity that does not include a non-serving PoA

The communication model also identifies the following reference points between different instances of

CSSs (see Table 1).

- Reference point RP1: Reference point RP1 refers to CSS procedures between the CSS user and the CSS PoS in a TVBD-M on the Network Entity of its serving PoA. RP1 encompasses communication interfaces over both L2 and L3 and above. CSS content passed over RP1 is related to CIS, MIES, or MICS.

- Reference point RP2:Reference point RP2 refers to CSS procedures between the CSS user and the CSS PoS on a non-PoA Network Entity o. RP2 encompasses communication interfaces over both L2 and L3 and above. CSS content passed over RP2 is related to CIS, MIES, or MICS.

- Reference point RP3: Reference point RP3 refers to CSS procedures between  the CSS PoS and the CSS on a non-PoA, non-PoS  Network Entity. RP3 encompasses communication interfaces over L3 and above and possibly L2 transport protocols like Ethernet bridging, or multi-protocol label switching (MPLS). CSS content passed over RP3 is related to CIS, MIES, or MICS.

Table 1—Summary of reference points

All reference point definitions are within the scope of this standard. Annex D provides a mapping of various CSS messages to the reference points.

5.1.3.3 Deployment Examples for the CSS Services

A network model using a centralized architecture for  CSS services is shown in Figure 6 to better illustrate the flexibility of CSS Service deployment. A single network based CServ is shown managing multiple networks with multiple TVBD-C CSS users.  Note that some or all of the TVBD-C CSS users may be network APs or BSs and provide PoA for other non-CSS TVBDs

TVWS DB

OME

TVBD-C

CC

TVBD-C

CC

TVBD network A

TVBD network B

CServ

Media Independent Coexistence Manager

(MICM)

Media Independent Support  Server

(MISS)

TVBD-C

CC

TVBD-C

CC

Figure 6:CSS Deployment Example with Centralized Management

A network model using a distributed architecture for  CSS services is shown in Figure 7 to better illustrate the flexibility of CSS Service deployment. A single network based CServ is shown supporting multiple networks with multiple TVBD-M CSS users.  In this example, the different TVBD-Ms are sharing the management of different networks in distributed fashion. These TVBD-Ms are usually PoAs for other CSS and non-CSS devices. Note that some or all of the TVBD-C CSS users may be network APs or BSs and provide PoA for other non-CSS TVBDs

TVBD-M

MICM

CC

TVWS Database

TVBD-C

CC

TVBD-C

CC

TVBD network A

TVBD network B

CServ

Media Independent Coexistence Manager

(MICM)

TVBD-C

CC

TVBD-C

CC

TVBD-M

MICM

CC

OME

Interface D

Interface D

Interface A

Interface A

Interface A

Interface A

Interface B2

Interface B2

Interface C

Media Independent Support  Server

(MISS)

TVBD-M

MICM

CC

Operator

Management Entity

Figure 7:CSS Deployment Example with Distributed Management

5.1.4 CSS reference models for link-layer technologies

The CSS provides services through well-defined service access points for CSS users. The following subclauses (5.1.5.1 through 5.1.5.4) describe the reference models for various link layer technologies with CSS functionality.

5.1.4.1 IEEE 802 architectural considerations

The CSS reference models for different IEEE 802 technologies and the general CSS framework is designed to be consistent with the IEEE 802 Architecture for different link-layer technologies. The CSS Function is a management entity that obtains link-layer information from lower layers of different protocol stacks and also from other remote nodes. The CSS Function coordinates handover decision making with other peer CSS Functions in the network.

The CSS Protocol provides the capability for transferring CSS messages between peer CSS Function entities at L2 or at L3. These messages transfer information about different available networks and also provide network switching and handover capability across different networks. The CSS protocol encompasses IEEE 802 technologies such as IEEE 802.11 and IEEE 802.22 and also other non-IEEE 802 technologies such as those specified by 3GPP and 3GPP2 standards. In this sense, the CSS Protocol has different scope and functionality than the Link Layer Discovery Protocol (LLDP) as specified by IEEE Std 802.1ABTM [TBD].

5.1.4.2 General CSS reference model and SAPs

Figure 8 illustrates the position of the CSS in a protocol stack and the interaction of the CSS with other elements of the system. All exchanges between the CSS and other functional entities occur through service primitives grouped into service access points (SAPs).

Medium

PHY

MAC

.

LLC

L3 and Higher Layers

LLC_SAP

PHY_SAP

CSS:

-Information Service

-Event Service

-Command Service

CSS_Net_SAP

Coex_Link_SAP

CSS User: TVBD

Info Svc

Info Svc

Event Svc

Event Svc

Cmnd Svc

Cmnd Svc

Coex_Link_SAP connects to

local MAC/PHY.

CSS_Net_SAP is for L2 or L3

Transport to entities external to TVBD.

Figure 8—General CSS reference model and SAPs

The media agnostic general CSS reference model includes the following SAPs:

- CSS_LINK_SAP: Abstract media dependent interface of CSS with the lower layers of the media specific protocol stacks. Clause TBD provides a description of the primitives used by the CSS_LINK_SAP.

- CSS_NET_SAP: Abstract media dependent interface of CSS that provides transport services over the data plane on the local node, supporting the exchange of CSS information and messages with remote CSS entities. For all transport services over L2, the CSS_NET_SAP uses the primitives specified by the CSS_LINK_SAP.  All other transport services are provided by L3 and higher layers.

The technology-specific instantiation of the CSS_LINK_SAP assumes media-specific names for SAPs and sets of primitives, often reusing names and primitives that already exist in the respective media-specific existing lower-layer SAPs. Primitives defined in CSS_LINK_SAP result in amendments to media-specific SAPs due to additional functionality being defined for interfacing with the CSS. All communications of the CSS with the lower layers of mediaspecific protocol stacks take place through media-specific instantiations of CSS_LINK_SAP.

The message exchanges between peer CSS instances, in particular the type of transport that they use, are sensitive to several factors, such as the nature of the network nodes that contain the peer CSS instances (whether or not one of the two is an CSS user or a PoA), the nature of the access network (whether IEEE 802 or 3G cellular), and the availability of CSS capabilities at the PoA.

Figure 9 presents a summary of the types of relationships that can exist between the CSS and other functional components in the same network node.

IEEE 802.11 TVWS

IEEE 802.22

Other

Media IndependentCoexistence

Manager (MICM)

Interface A=

Coex_LINK_SAP

Coexistence Client (CC)

IEEE 802.11 TVWS

IEEE 802.22

Other

Coexistence Client (CC)

Media Independent Support Server (MISS)

CSS

Interfaces B2&B3=CSS_NET_SAP (L3 Transport)

Interface B2

Interface B1=

MIH_NET_SAP (L2 Transport)

TVBD-C device

Media Independent Coexistence

Manager (MICM)

Interface A=

Coex_LINK_SAP

Interface B1

CSS

CSS

CSS Server (CServ)

Coexistence Server

Interface B3

Interface B2

TVBD-M device in AP or BS

Figure 9—Types of CSS relationship

The general CSS reference model in Figure 8 enables a simple representation of the broad variety of CSS relationships shown in Figure 9. In the model, a mobility-management protocol stack is logically identified within each network node that includes an CSS instance. The provided abstraction makes it easy to isolate and represent the CSS relationships with all pre-existing functional entities within the same network node. Such relationships are both internal (with functional entities that, just like the CSS, share the logical inclusion in the CSS protocols) and external (with functional entities that belong to other planes).

Figure 9 shows how an CSS-enabled CSS user communicates with an CSS-enabled network. The gray arrows show the CSS signaling over the network, whereas the black arrows show local interactions between the CSS and lower and higher layers in the same network or node block. For a more detailed view of local interactions, please refer to technology-specific reference models and service access point in 5.1.5.3 through 5.1.5.7.

When connected to an IEEE 802 network, an CSS user directly uses L2 for exchanging CSS signaling, as the peer CSS can be embedded in a PoA. The CSS user may do this (discuss with TG?) for certain IEEE 802 networks even before being authenticated with the network. However, the CSS user can also use L3 for exchanging CSS signaling, for example in cases where the peer CSS is not located in the PoA, but deeper in the network.

5.1.4.3 CSS reference model for IEEE 802.3

The CSS reference model for IEEE 802.3 is illustrated in Figure 10. The transport of CSS services is supported over the data plane by use of existing primitives defined by the logical link control service access point (LSAP). There are no amendments specified in IEEE Std 802.3 to support any link services defined over the CSS_LINK_SAP in this specification.

Figure 10: CSS reference model for IEEE 802.3  [NEED UPDATED FIG HERE]

5.1.4.4 CSS reference model for IEEE 802.11

Figure 11 shows the CSS reference model for IEEE 802.11. The payload of CSS services over IEEE 802.11 is carried either in the data frames by using existing primitives defined by the LSAP or by using primitives defined by the MAC State Generic Convergence Function (MSGCF) service access point (SAP) (MSGCF_SAP). The MSGCF has access to all management primitives and provides services to higher layers.

It should be noted that sending CSS payload over the LSAP is allowed only after successful authentication and association of the station to the access point (AP). Moreover, before the station has authenticated and associated with the AP, only CSS Information Service and CSS capability discovery messages can be transported over the MSGCF_SAP.

Figure 11: CSS reference model for IEEE 802.11 [NEED UPDATED FIG HERE]

5.2 Service Access Points (SAPs)

5.2.1 General

The CSS interfaces with other layers and functional planes using service access points (SAPs). Each SAP consists of a set of service primitives that specify the interactions between the service user and provider.

The specification of the CSS includes the definition of SAPs that are media independent and recommendations to define or extend other SAPs that are media dependent. Media independent SAPs allow the CSS entities to communicate to coordinate and provide CSS services. The Coex_Link_SAP and associated primitives provide the interface from the CSS to the upper layers of the CSS user protocol stack. Upper layers need to subscribe with the CSS to receive CSS generated events and also for link-layer events that originate at layers below the CSS but are passed on to CSS.

Media dependent SAPs allow the CSS  users to use services from the lower layers of their management planes. All inputs (including the events) from the lower layers of the protocol stack into the CSS  are provided through existing media-specific SAPs such as MAC SAPs, PHY SAPs, and logical link control (LLC) SAPs. Link Commands generated by the CSS to control the PHY and MAC layers during the handover are part of the media-specific MAC/PHY SAPs and are already defined elsewhere.

Figure 12 shows the key CSS-related SAPs for different networks, which are as follows:

- The CSS _LINK_SAP specifies an abstract media dependent interface between the CSS and lower layers media-specific protocol stacks of technologies such as IEEE 802.3, IEEE 802.11, IEEE 802.22. For different link-layer technologies, media-specific SAPs provide the functionality of CSS _LINK_SAP. Amendments are suggested to the respective media-specific SAPs to provide all the functionality as described by CSS _LINK_SAP. CSS users directly send events to the local CSS using the service primitives of the Coex_Link_SAP. The CSS user upper layers need to subscribe with the CSS to receive CSS-generated events.

- The CSS_NET_SAP specifies an abstract media dependent interface of the CSS  user that provides transport services over the data plane on the local node, supporting the exchange of CSS information and messages with remote CSS entities.

CSS

CSS

CSS_NET_SAP

Figure 12: Relationship between different CSS SAPs[NEED UPDATED FIG HERE]

All the SAPs described in this clause 5.2 are media dependant SAPs. Each link-layer technology specifies its own technology-dependent SAPs.

5.2.2.2 CSS_LINK_SAP

This SAP defines the abstract media dependent interface between CSS and different link-layer technologies. Amendments are suggested for different layer technology-specific SAPs based on the definition of this particular SAP.

5.2.2.3 CSS_NET_SAP

CSS_NET_SAP defines the abstract media dependent interface of the CSS that provides transport services over the data plane on the local node, supporting the exchange of CSS information and messages with remote CSS entities. For L2, this SAP uses the primitives provided by CSS_LINK_SAP.

5.2.2.4 MLME_SAP

This SAP defines the interface between the CSS and the management plane of an IEEE 802.11 network. This SAP is used for sending CSS messages between the CSS and local link-layer entities, as well as between peer CSS entities.

5.2.2.7 MSGCF_SAP

This SAP, defined in IEEE P802.11u/D3.0, provides services to CSS based on the IEEE 802.11 MAC state machines and interactions between the IEEE 802.11 sublayers.

5.2.2.9 LSAP

The logical link control service access point (LSAP), defined in IEEE Std 802.2, provides the interface between the CSS and the Logical Link Control sublayer in IEEE 802.3 and IEEE 802.11 networks. This SAP is used for local CSS exchanges between the CSS and the lower layers and for the L2 transport of CSS messages across IEEE 802 access links.

5.3 Service Primitives and Parameters

[TBD]

6. CSS Services

The CSS services described here include basic service, information service and management service. The services are described primarily by listing in an abbreviated fashion the  interface B1 messages used to implement the services. The detailed message description are included in clause 8 of this specification.

# 6.1 Basic/Support Service

These services are bidirectional (CC-MICM) services that are available without separate subscription. These services are not for providing coexistence services. As an example, a CC needs these services for being able to subscribe coexistence information or management services, and making decisions on whether and which service to subscribe.

In addition to the messaging defined here, the basic service set includes an authentication exchange.   The authentication exchange relies on well-known authentication techniques, but involves a number of messages, which are not discussed here.  It should be noted that some messages in the basic service set can be exchanged before or without an authentication exchange ever taking place, while others requires successful authentication.

### 6.1.1  CC Regulatory environment

CC may use this request to discover from coexistence system in which regulatory environment TVBD is in.  This message DOES NOT require successful authentication.

From CC: CC_regulatory_environment_req

- Network/device ID

- Geo-location

From MICM: CC_regulatory_environment_rsp

- Regulatory environment ID/parameters

### 6.1.2 CC registration

CC registers to a MICM.

Note: Registration is the first step to be taken by a CC when joining the coexistence system. Once the CC has successfully registered to a MICM it can subscribe to the services and the MICM can start serving the CC. On the other hand successful registration to a MICM obligates the CC to provide some information to the MICM.  This message DOES NOT require successful authentication.

From CC: CC_register_req

- Network/device ID

- Geo-location

- Max TX power/ TX power class

From MICM: CC_register_rsp

- Accept / reject

- CC_ID (valid CC_ID if accept)

- Reject_reason (if reject)

Note: Before sending the response, the MICM validates the CC, and that it has capacity to serve the CC.

### 6.1.3 CC deregistration

A CC deregisters from the MICM, or a MICM which is no longer able to serve the CC, deregisters the CC.  This message DOES NOT require successful authentication.

CC deregisters itself:

From CC: CC_deregister_req

- Reason (optional)

From MICM: CC_deregister_rsp

Note: Before sending rsp, MICM removes MICM from the CDIS and from neighbor MICMs.

MICM deregisters CC:

From MICM: CC_deregister_command

- Reason

From CC: CC_deregister_command_rsp

- Acknowledges message receipt

### 6.1.4 CC Regulatory Database Query

A registered CC makes a regulatory database Query which may be followed by a MICM query to the database to find the available channel list for the CC location.  This is the only service provided to registered but unauthenticated CCs.  This message DOES NOT require successful authentication. This message exchange does not relieve the CC of meeting regulatory requirements prior to transmission in the TVWS.

From CC: CC_reg_db_req

- Network/device ID

- Request ID

- Geo-location

- Max TX power/ TX power class

- Time stamp

From MICM: CC_reg_db_rsp

- Network/device ID

- Request ID

- Time stamp

- Regulatory response (likely list of available channels, but depends on each regulatory environment)

### 6.1.5 CC service subscription

A CC subscribes information or management service after it has registered to the MICM. With these messages the CC requests and changes the subscribed service. This message REQUIRES successful authentication.  Alternatively, service subscription can be made part of the authentication exchange.

From CC:  CC_subsribe_service_req

- Requested service: Information service / management service

From MICM: CC_subsribe_service_rsp

- Accept / reject

- Reason (if reject)

# 6.2  Coexistence Information Service

Coexistence Information service enables TVBD to receive information which it can use for selecting resources for its network. A CC which subscribes to the information services is obligated to provide information on TVBDs resource use to the coexistence system. The coexistence system provides information related to the TVBD’s spectrum environment.

### 6.2.1 CC TVBD information

With these messages the CC provides information on the TVBD capabilities and characteristics to the coexistence system, after it has registered (or subscribed) to the MICM. The MICM may request parameters and also configure automatic updates for receiving parameters when they change. This message REQUIRES successful authentication.  If the CC device is a network manager or controller, it provides network information and device information for its entire network.  If the CC device is not a manager or constroller (e.g. an 802.11 STA) it provides device information and any known network information.

From MICM: CC_information_req

- List of requested information parameters, e.g.

  - Network ID

  - Network type (RAT, mode,etc)

  - List of devices in the network, for each device:

    - Device ID

    - Network role , PtoP (for STAs), PtoMP (for APs)

    - Device antenna location (point location with accuracy)

    - Device TX power range

    - Device TX power current setting

    - Device RX sensitivity range  (depends on MCS capabilities)

    - Device RX sens current setting

    - Antenna description, Indoor/outdoor, AZ, El, gain, etc

    - …

  - Geo-location, aggregate for network

  - Possible operation frequencies, for network

  - Possible channel bandwidths, for network

  - Coexistance capabilities

  - Channel sharing capabilities (FDMA, TDMA. CSMA)

  - Coexistence value (CV, fairness)

  - …

From CC: CC_information_rsp

- Parameter values for requested information

- If requested info is not provided, a reason code is provided:

  - Information not defined for this device

  - Information defined but not available

  - Information available but sharing not authorized

  - Others…

From MICM: CC_information_update_config_req

- Parameter

- Trigger for automatic update

From CC: CC_information_update_config_rsp

- Current value of the parameter

From CC: CC_information_update

- New value of the updated parameter

Note: CC sends the parameters automatically when the trigger threshold is met.

From MICM: CC_information_update_rsp

- Acknowledges message receipt

### 6.2.2 CC resource use update

The CC informs the coexistence system that the TVBD’s resource use has changed. The MICM may request the resource use information and may configure the CC to send automatic u

From MICM: CC_resource_use_req

From CC: CC_resource_use_rsp

- Bandwidth (start channel + bandwidth)

- Utilization (used time-% in the bandwidth)

- Max operation power

- Antenna configuration

From: CC_resource_use_update_config_req

- Parameter

- Trigger

From CC: CC_ resource_use_update_config_rsp

- Current values of the requested parameters.

From CC: CC_ resource_use_update

- New value of the updated parameter

Note: CC sends CC_resource_use_update after the trigger threshold is met.

From MICM: CC_ resource_use_update_rsp

- Acknowledges message receipt

### 6.2.3 CC spectrum map update

The MICM calculates the spectrum environment information for the TVBD. It is formed from the information received from the TVWS database and the neighbors of the TVBD,and the measurement results from the CC.

From MICM: CC_spectrum_map_update

- States of each channel at the location of the TVBD associated to the CC.

  - Disallowed/Allowed (by regulatory)

  - Occupancy, e.g.

    - utilization

    - RAT(s)

    - interference types

    - interference level (high, medium, low, unknown)

From CC: CC_spectrum_map_update_rsp

- Acknowledges message receipt

# 6.3  Coexistence Management Service

Coexistence management service provides resource allocations to the TVBD. The CC which subscribes to the management services may request resources from the coexistence system, and it is obligated to obey the received resource allocations.  Unless otherwise indicated below, these management requests are issued to a network controller and not to network devices directly.  The network controller uses its control facilities to ensure that all network devices comply with the  management requests .

### 6.3.1  CC Resource Management

### 6.3.1.1 CC resource request

The CC estimates the resource need of the TVBD. When TVBD needs more or less resources the CC sends resource request to the CM. The CC may also use this request to indicate that the resource need has temporarily changed.

From CC: CC_resource_req

- Resource need: utilization on bandwidth.

- Estimate of new resource need duration: temporary change/long term change.

From MICM: CC_resource_rsp

- Accept / Reject

- Reason (if reject)

Comment: May also be only acknowledgement. However if the MICM is in able to immediately estimate whether the request is not valid/it is not able to perform new resource allocation, reject could indicate that.

### 6.3.1.2. CC resource allocation

The MICM sends new resource allocation to the CC, either after the MICM has performed resource allocation itself, or the MICM has received new resource allocation from the MICM of a neighbor TVBD network. Note that this resource allocation may be used by the MICM for channel switching to quiet a channel for measurements.

From MICM: CC_resource_allocation_command

- Bandwidth (Allocated bandwidth: start channel+bandwidth)

- Utilization (Allocated time in the given bandwidth)

- Max operation power

- Antenna configuration

From CC: CC resource_allocation_command_rsp

- Acknowledges message receipt

### 6.3.2 CC measurement  management

With these messages the CC provides measurement information to the coexistence system, after it has registered (or subscribed) to the MICM. The MICM may request measurements and configure automatic measurement updates. In the measurement requests the MICM may give guidance to the measurement configuration (e.g. interval, duration). Note that all TVBDs may not be able to perform measurements exactly as guided. NOTE: Management requests for measurements are issued to network devices and network controllers as individual TVBDs

From MICM: CC_measurement_req

- Channel/set of channels

- Measurement configuration proposal (optional), e.g.

  - measurement interval

  - measurement duration

  - measurement type (RSSI/PER, etc.)

  - measured system/network

From CC: CC_measurement_rsp

- Channel State Vector, including interference information on the channel, e.g.:

  - RSSI

  - Interference utilization

  - Interference interval,

  - Interference type (e.g. originating system)

- Measurement configuration (optional)

  - Measurement interval

  - Measurement duration

  - Measurement type

From MICM: CC_measurement_update_config_req

- Update ID

  - Comment: this is needed if MICM may configure multiple automatic measurements simultaneously with different configurations

- Channel/set of channels

- Trigger for automatic update

- Measurement configuration proposal (optional), e.g.

  - measurement schedule

  - measurement interval

  - measurement duration

  - measurement type (RSSI, PER, etc.)

  - measured system/network

From CC: CC_ measurement_update_config_rsp

- Update ID

- Accept / Reject

- Reason (if reject)

From CC: CC_measurement_update

- Update ID

- Channel State Vector, including interference information on the channel:

  - RSSI

  - Interference utilization

  - Interference interval,

  - Interference type (e.g. originating system)

- Measurement configuration (optional), e.g.

  - Measurement time

  - Measurement interval

  - Measurement duration

Note: CC sends the CC_measurements_update after the trigger threshold is met.

From MICM: CC_measurement_update_rsp

- Acknowledges message receipt

### 6.3.3 CC quieting management

This set of messages enables the MICM to schedule a quieting interval for the network managed by the CC.  The quiet periods may be used to support various measurement needs of the 802.19.1 CSS.

From MICM: CC_quiet_schedule_req

- Schedule ID

- Schedule information (can be periodic, specific), for example

  - start time or list of specific measurement times

  - measurement frequency

  - end time (with an option to continue until revoked)

From CC: CC_ quiet_schedule_rsp

- Schedule ID

- Accept / Reject

- Reason (if reject)

From MICM: CC_quiet_schedule_rvk_req

- Schedule ID

- Revocation time

From CC: CC_ quiet_schedule_rvk_rsp

- Acknowledges message receipt

7. CSS Protocols and Procedures

[TBD]

7.1 Generic procedures for information exchange

[TBD]

7.2 CSS Protocols

[TBD]

8. CSS Message Formats and Information Elements

[TBD]

9. CSS Coexistence Mechanisms and Algorithms

[TBD]

Annex A: Use Cases

[TBD]

Annex B: Evaluation methodology

[TBD]

Annex C: Coexistence Client (CC) Software Reference Design

[TBD]

Annex D: Additional information describing Interface C to the TVWS Database

[TBD]

Annex E: Additional information describing Interface D to the Operator Management Entity

[TBD]