IEEE P802.19
Wireless Coexistence

| Procedures and protocols, and coexistence mechanisms and algorithms proposal |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-10-31 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Hyunduk Kang | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea | +82-42-860-1074 | henry@etri.re.kr |
| Donghun Lee | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea | +82-42-860-0865 | mmdang@etri.re.kr |
| Heonjin Hong | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea | +82-42-860-4860 | hjhong@etri.re.kr |
| Changjoo Kim | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea | +82-42-860-6160 | cskim@etri.re.kr |

Abstract

This document is a procedures and protocols, and coexistence mechanisms and algorithms proposal of 802.19 Task Group 1. The document contains generic procedures for information exchange, protocols, coexistence mechanisms and algorithms of the draft of 802.19.1 standard.

1. Overview

1.1 Scope

The standard specifies radio technology independent methods for coexistence among dissimilar or independently operated TV Band Device (TVBD) networks and dissimilar TV Band Devices.

1.2 Purpose

The purpose of the standard is to enable the family of IEEE 802 Wireless Standards to most effectively use TV White Space by providing standard coexistence methods among dissimilar or independently operated TVBD networks and dissimilar TVBDs. This standard addresses coexistence for IEEE 802 networks and devices and will also be useful for non IEEE 802 networks and TVBDs.

1.3 Sample application areas

TBD

1.4 Conformance

TBD

2. Normative references

TBD

3. Definitions, acronyms and abbreviations

3.1 Definitions

TBD

3.2 Acronyms and abbreviations

TBD

6. Procedures and Protocols

6.1 General

After the interface setup as defined in section 4.4 has been made between two entities, each entity starts its operating stage. Each entity at operating stage is described by a number of designated procedures and events that triggers them. There are mainly three entities needed to be considered:

- Coexistence manger (CM)

- Coexistence enabler (CE)

- Coexistence discovery and information server (CDIS)

The CM at operating stage exchanges information with CE, CDIS, and TVWS DB. The CE at operating stage exchanges information with CM and TVBD. The CDIS at operating stage exchanges information with CM and TVWS DB.

Protocols are described by message exchange sequences between two entities, and a number of processes that handle messages.

6.2 Generic procedures

6.2.1 Coexistence manager operating procedures

Figure 1 describes coexistence manger operating procedures that define a specific event and designated procedure triggered by it. At first the CM shall conduct channel classification to prepare channel allocation to its registered CEs. After that, the CM shall perform channel allocation based on channel classification and other aspects such as TVWS DB update, registered CE discovery update, neighbor CM discovery update, and registered CE’s channel move request and so on. Main operating procedures of the CM consist of two parts: Channel classification and channel allocation. Including these, operating procedures of coexistence manager are as follows:

- CM_Channel_Classification

- CM_Channel_Allocation

- CM_TVWS_Channel_Update

- CM_Registered_CE_Discovery

- CM_Registered_CE_Discovery_Update

- CM_Neighbor_CM_Discovery

- CM_Neighbor_CM_Discovery_Update

- CM_Initiate_Registered_CE_Channel_Move

- CM_Initiate_Channel_Reallocation

- CM_Channel_Identification

- CM_Inform_Event

The procedure CM_Channel_Classification is triggered if flag ‘Initiate_Channel_Classification’ is set to be 1. This flag is set to be 1 if the CM has been initialized after power on or TVWS channels from TVWS DB have been updated. As depicted in figure 2, the CM firstly executes the procedure CM_Registered_CE_Discovery in order to find out context information of registered CEs belong to the CM. After getting context information from registered CEs, the CM accesses TVWS DB to get the list of allowed channels such as the available channel and the restricted channel that can be used by TVBD networks or devices. To do that, the CM might send FCC identifier of each TVBD as required by FCC regulation. In case, TVWS DB is not available within a certain time limit, the CM requests each registered CE to send disconnection request to the CM.

After getting available or restricted channel lists from TVWS DB, the CM executes the procedure CM_Neighbor_CM_Discovery to get context information of neighbor CMs from the CDIS.

Once neighbor CM discovery is accomplished, the CM executes the procedure CM_Channel_Identification as depicted in figure 3. Through this procedure, the CM shall identify the available channel, the restricted channel, the operating channel already taken by the registered CE of the neighbor CM among allowed channels from TVWS DB.

The CM finally sets flag ‘Initiate_Channel_Allocation’ to be 1, in order to trigger the procedure CM_Channel_Allocation.

Figure 1 ─ Coexistence manager operating procedures

Figure 2 ─ Procedure CM_Channel_Classification

Figure 3 ─ Procedure CM_Channel_Identification

The procedure CM_Channel_Allocation is triggered if flag ‘Initiate_Channel_Allocation’ is set to be 1. This flag is set to be 1 if the following occurs:

- If the CM has done the procedure CM_Channel_Classification

- If the neighbor CM discovery has been updated

- If the registered CE discovery has been updated

- If the registered CE requests channel move and there are no available channels or restricted channels to allocate

As shown in figure 4, the CM firstly checks if Timer TRefresh_TVWS_DB is expired. If it is expired, the CM executes the procedure CM_TVWS_Channel_Update. If not, the CM enters the channel allocation process and checks current channel classification.

Based on current channel classification, the CM shall decide if individual channel assignment is possible for all registered CEs considering its neighbor CM. If possible, i.e., in individual channel assignment mode, the CM allocates an exclusive operating channel to each registered CE and sends reconfiguration request to registered CEs. If a registered CE does not accept the CM’s reconfiguration request, the CM discards that registered CEs. The CM updates channel classification again to reflect channel allocation, and sets flag ‘Initiate_Registered_CM_Channel_ Classification_Discovery’ to be 1, Finally the CM sends context Information to CDIS.

If individual channel assignment mode is not possible, the CM enters co-channel sharing mode and shall apply a proper operating channel selection algorithm and/or a co-channel sharing mechanism to each registered CE. Negotiation might be needed if a negotiation with its neighbor CM is needed.

The procedure CM_TVWS_Channel_Update is periodically executed during the CM operation. If TVWS DB is updated, the CM executes the procedure CM_Initiate_Channel_Reallocation notifying channel shutdown to all registered CEs, and goes back to the procedure CM_Channel_Classification. In case, TVWS DB is not available within a certain time limit, the CM notifies shutdown of all operating channels to all registered CEs, and requests each registered CE to send disconnection request to the CM.

The procedure CM_Registered_CE_Discovery_Update is triggered if registered CE list of the CM has been changed. Through this procedure the CM executes the procedure CM_Registered_CE_Discovery and the procedure CM_Channel_Allocation one by one.

The procedure CM_Neighbor_CM_Discovery_Update is triggered if neighbor CM list has been changed. The CM executes the procedure CM_Neighbor_CM_Discovery and the procedure CM_Channel_Allocation by turns.

The procedure CM_Initiate_Registered_CE_Channel_Move is triggered if a registered CE of the CM requests channel move due to failure of required quality of service (QoS) with allocated operating channel from the CM. As depicted in figure 5 the CM shall allocate a new operating channel to the CE requesting channel move if there are available channels or restricted channels. After that the CM shall update channel classification, and send context information to the CDIS. If there are no available channels or restricted channels to allocate, the CM shall set flag ‘Initiate_Channel_Allocation’ to be 1 and execute the procedure CM_Channel_Allocation.

The procedure CM_Inform_Event is triggered if the CM received an event request from the CDIS or its neighbor CMs.

Figure 4 ─ Procedure CM_Channel_Allocation

Figure 5 ─ Procedure CM_Initiate_Registered_CE_Channel_Move

6.2.2 Coexistence enabler operating procedures

Figure 6 describes coexistence enabler operating procedures that define a specific event and designated procedure triggered by it. Operating procedures of coexistence manager are as follows:

- CE_Inform_Context_Information

- CE_Inform_Event

- CE_TVBD_Reconfiguration

- CE_Request_Channel_Move

The procedure CE_Inform_Context_Information is triggered if the CE receives context information request from the CM.

The procedure CE_TVBD_Reconfiguration is triggered if the CE receives reconfiguration request from the CM. Through this procedure the CE shall reconfigure TVBD as the CM requested. The CE shall check the validity of allocated operating channel from the CM by asking TVWS DB.

The procedure CE_Request_Channel_Move is triggered if the CE detects failure of required quality of service (QoS) with allocated operating channel from the CM. The CE shall request a new operating channel to the CM.

The procedure CE_Inform_Event is triggered if the CM received an event request from the CM.

Figure 6 ─ Coexistence enabler operating procedures

6.2.3 Coexistence discovery and information server operating procedures

Figure 7 describes coexistence enabler operating procedures that define a specific event and designated procedure triggered by it. Operating procedures of coexistence manager are as follows:

- CDIS_TVWS_Channel_Update

- CDIS_Neighbor_CM_Discovery

- CDIS_Registered_CM_Channel_Classification_Discovery

- CDIS_Inform_Event

The procedure CDIS_TVWS_Channel_Update is periodically executed during the CDIS operation.

The procedure CDIS_Neighbor_CM_Discovery is triggered if flag ‘Initiate_Neighbor_CM_Discovery’ is set to be 1. This flag is set to be 1 if the registered CM list or context information of the registered CM has been updated. This procedure shall find neighbor CMs and its corresponding CEs that might cause harmful co-channel interference between them.

The procedure CDIS_Registered_CM_Channel_Classification_Discovery is triggered if flag ‘Initiate_CDIS_ Registered_CM_Channel_Classification_Discovery’ is set to be 1. This flag is set to be 1 if channel classification of the registered CM has been updated. Through this procedure the CDIS shall gather information on channel classification of each registered CM.

The procedure CDIS_Inform_Event is triggered if the CM received an event request from the registered CM.

Figure 7 ─ Coexistence discovery and information server operating procedures

6.3 Protocols

6.3.1 General

The logical entities in the 802.19.1 system communicate with each other using the transmission control protocol/internet protocol (TCP/IP) over internet. This clause specifically defines a protocol message format and protocol messages for data transfers between 802.19.1 entities, or between 802.19.1 entity and TVWS DB.

6.3.2 Protocol message format

TBD

6.3.3 Protocol messages

The following sub-clauses specify different protocol messages and contents for interface between entities in the 802.19.1 system. All messages convey a source identifier followed by a destination identifier as the first two of the protocol payload part of the message.

Any message received that does not contain the source and destination identifiers, or has an unrecognizable or invalid message shall be discarded without sending any indication to the source entity.

The message used in the 802.19.1 entity is defined by three kinds of messages; context information message, event message and reconfiguration message.

Context information message conveys fundamental information of the corresponding entity for decision making on radio resource usage optimization within this standard to be possible. The context information message in the 802.19.1 system is defined for interface between CM-CE, CM-CDIS, TVWS DB-CM and TVWS DB-CDIS respectively.

Event message is used for a means of notifying a change of the entity status that triggers specific procedures for maintaining system performance. The event message in the 802.19.1 system is defined for interface between CM-CE and CM-CDIS respectively.

Reconfiguration message provides the information for setting hardware and/or software of TVBD in order to properly change its operating parameters in the physical and MAC layers, such as operating channel, transmitting power and communicating coverage. The reconfiguration message in the 802.19.1 system is defined for interface between CM and CE.

Brief descriptions of all messages defined in the 802.19.1 system are given in table 1.

Table 1 ─ Summary of messages

| Category | Name | Description |
| --- | --- | --- |
| Context information message | TVBD-CI-REQ | Request context information of TVBD used by CM |
|  | TVBD-CI-RSP | Send context information of TVBD to CM used by CE |
|  | CDIS-CI-REQ | Request context information of CDIS used by CM |
|  | CDIS-CI-RSP | Send context information of CDIS to CM |
|  | CM-CI-REQ | Request context information of CM used by CDIS |
|  | CM-CI-RSP | Send context information of CM to CDIS |
|  | TVWSDB-CI-REQ | Request context information of TVWS DB used by CM |
|  | TVWSDB-CI-RSP | Send context information of TVWS DB to CM |
|  | TVWS-DB-REQ | Request context information of TVWS DB used by CDIS |
|  | TVWS-DB-RSP | Send context information of TVWS DB to CDIS |
| Event message | TVBD-EV-REQ | Request event notification of TVBD used by CM |
|  | TVBD-EV-IND | Confirm receipt of event request message used by CE |
|  | TVBD-EV-RSP | Notify the detected event of TVBD to CM |
|  | CDIS-EV-REQ | Request event notification of CDIS used by CM |
|  | CDIS-EV-IND | Confirm receipt of event request message |
|  | CDIS-EV-RSP | Notify the detected event of CDIS to CM |
|  | CM-EV-REQ | Request event notification of CM used by CDIS |
|  | CM-EV-IND | Confirm receipt of event request message |
|  | CM-EV-RSP | Notify the detected event of CM to CDIS |
| Reconfiguration message | TVBD-RC-REQ | Request reconfiguration of TVBD used by CM |
|  | TVBD-RC-RSP | Send reconfiguration results of TVBD to CM |

More specific description of each message for interface between entities in the 802.19.1 system is given in following sub-clause

6.3.3.1 Message lists and contents between CM and CE

Figure 8 shows the message sequence chart (MSC) between CM and CE where the message is categorized into three kinds of messages; context information, event and reconfiguration messages. In addition, P in figure 8 stands for the specific procedure executed in the corresponding entity and employs specification and description language (SDL). When the 802.19.1 system is started or initialized, the entity of CM sends the request message of context information, i.e., TVBD-CI-REQ to all of CEs in the 802.19.1 system. Then, each CE in the system collects the context information from the corresponding TVBD as shown in procedure 1 (P1), and sends the response message with context information, i.e., TVBD-CI-RSP to CM.

Based on the context information from TVBDs, CM has made decision of channel assignment which has two kinds of modes; individual channel assignment and co-channel sharing, and has sent the reconfiguration message of TVBD-RC-REQ to CE. After receiving reconfiguration message, CE informs a reconfiguration command to the corresponding TVBD as shown in procedure 2 (P2), and sends the reconfiguration results of the corresponding TVBD through the message of TVBD-RC-RSP to CM.

After completing the channel assignment, CM sends the event request message of TVBD-EV-REQ for system performance maintenance. As a receipt, CE gives the acknowledgement of the corresponding events, i.e., TVBD-EV-IND as shown in procedure 3 (P3). Then, CE periodically checks the events requested by CM as shown in procedure 4 (P4), and notifies the detected event through the message of TVBD-EV-RSP to CM.

More specific description of each message for interface between CM and CE is given in following sub-clause

Figure 8 ─ Message sequence chart (MSC) between CM and CE

6.3.3.1.1 TVBD-CI-REQ

This message used by CM is transmitted to CE to request context information of the corresponding TVBD. This message is generated on CM before accessing TVWS DB for discovery of all registered TVBDs in system. The contents of this message are given as follows

| Name | Contents | Description |
| --- | --- | --- |
| TVBD-CI-REQ | SourceCMIdentifier | Identifier of CM |
|  | DestinationTVBDIdentifier | Identifier of TVBD |
|  | CIList | Flag array of reported context information selected by set of the corresponding flag, ordered as follows: - FCC identifier of TVBD - Type of TVBD - Radio interface of TVBD - Antenna gain of TVBD - Antenna height of TVBD - Geolocation of TVBD - Coverage of TVBD - Required bandwidth of TVBD - Measurements of TVBD - Reconfiguration option of TVBD |

6.3.3.1.2 TVBD-CI-RSP

This message used by CE is transmitted to CM to send the selected context information of the corresponding TVBD. This message is generated on CE after collecting context information from the corresponding TVDB. The contents of this message are given as follows

| Name | Contents | Description |
| --- | --- | --- |
| TVBD-CI-RSP | SourceTVBDIdentifier | Identifier of TVBD |
|  | DestinationCMIdentifier | Identifier of CM |
|  | TVBDFCCIdentifier | FCC identifier of TVBD |
|  | TVBDType | Type of TVBD - Fixed device type - Personal/portable mode I type - Personal/portable mode II type |
|  | TVBDRadioInterface | Radio interface of TVBD |
|  | TVBDAntennaGain | Antenna gain of TVBD |
|  | TVBDAntennaHeight | Antenna height of TVBD |
|  | TVBDGeolocation | Geolocation of TVBD: - Latitude - Longitude - Altitude |
|  | TVBDCoverage | Communication coverage of TVBD |
|  | TVBDRequiredBandwidth | Required bandwidth of TVBD |
|  | TVBDMeasurements- TVBDQoS, Reserved | Measurements of TVBD parameters: - QoS of TVBD - Reserved |
|  | TVBDReconfigurationOption | Any reconfiguration option of TVBD |

6.3.3.1.3 TVBD-EV-REQ

This message used by CM is transmitted to CE to request the event detection of the corresponding TVBD. This message is generated on CM after completing the channel assignment for system performance maintenance. The contents of this message are given as follows

| Name | Contents | Description |
| --- | --- | --- |
| TVBD-EV-REQ | SourceCMIdentifier | Identifier of CM |
|  | DestinationTVBDIdentifier | Identifier of TVBD |
|  | EventList | Flag array of the reported event selected by set of the corresponding flag, ordered as follows: - Geolocation change of TVBD - QoS degradation of TVBD under required performance - Coverage change of TVBD |

6.3.3.1.4 TVBD-EV-IND

This message used by CE is transmitted to CM to give the acknowledgement of the corresponding event. This message is generated on CE after receiving TVBD-EV-REQ message as a receipt. The contents of this message are given as follows

| Name | Contents | Description |
| --- | --- | --- |
| TVBD-EV-IND | SourceTVBDIdentifier | Identifier of TVBD |
|  | DestinationCMIdentifier | Identifier of CM |
|  | Status | Flag array for acknowledgement of the selected event, ordered as follows: - Geolocation change of TVBD - QoS degradation of TVBD - Coverage change of TVBD |

6.3.3.1.5 TVBD-EV-RSP

This message used by CE is transmitted to CM to notify the detected event of the corresponding TVBD. This message is generated on CE after detecting event of the corresponding TVBD. The contents of this message are given as follows

| Name | Contents | Description |
| --- | --- | --- |
| TVBD-EV-RSP | SourceTVBDIdentifier | Identifier of TVBD |
|  | DestinationCMIdentifier | Identifier of CM |
|  | FlagTVBDGeolocationChange | Flag to specify if geolocation of TVBD is changed |
|  | FlagTVBDQoSChange | Flag to specify if QoS of TVBD is degraded under the required performance |
|  | FlagTVBDCoverageChange | Flag to specify if coverage of TVBD is changed |

6.3.3.1.6 TVBD-RC-REQ

This message used by CM is transmitted to CE to request reconfiguration of the corresponding TVBD. This message is generated on CM after making decision of channel assignment for spectrum resource sharing. The contents of this message are given as follows

| Name | Contents | Description |
| --- | --- | --- |
| TVBD-RC-REQ | SourceCMIdentifier | Identifier of CM |
|  | DestinationTVBDIdentifier | Identifier of TVBD |
|  | CoexistenceMode-IndividualChannelAssignmentMode, CoChannelSharingMode | Flag to specify used coexistence Mode: - Individual channel assignment mode - Co-channel sharing mode |
|  | TVBDOperatingChannel | Operating channel of TVBD |
|  | TVBDCoverage- FixedType, ModeIType, ModeIIType | Coverage of TVBD : - Fixed device type - Personal/portable mode I type - Personal/portable mode II type |
|  | CoChannelSharing | Additional information for co-channel sharing |

6.3.3.1.7 TVBD-RC-RSP

This message used by CE is transmitted to CM to inform the reconfiguration results of the correspond TVBD. This message is generated on CE after finishing TVBD reconfiguration based on command from CM. The contents of this message are given as follows

| Name | Contents | Description |
| --- | --- | --- |
| TVBD-RC-RSP | SourceTVBDIdentifier | Identifier of TVBD |
|  | DestinationCMIdentifier | Identifier of CM |
|  | Status- CoexistenceMode, TVBDOperatingChannel, TVBDCoverage, Reserved | Flag array of reported reconfiguration parameters, ordered as follows - Coexistence mode of TVBD - Operating channel of TVBD - Coverage of TVBD - Reserved |

6.3.3.2 Message lists and contents between CM and CDIS

Figure 9 ─ Message sequence chart (MSC) between CM and CDIS

Figure 9 shows the message sequence chart (MSC) between CM and CDIS where the messages used in this MSC are context information and event messages. In addition, P in figure 9 stands for the specific procedure executed in the corresponding entity and employs SDL.  For channel classification, the entity of CM requests context information of CDIS through CDIS-CI-REQ message. Then, CDIS collects the context information of the registered CMs including neighbor CMs as shown in procedure 1 (P1), and sends the response message with context information, i.e., CDIS-CI-RSP to CM.

After completing channel assignment, CM sends the event request message of CDIS-EV-REQ for system performance maintenance. As a receipt, CDIS gives the acknowledgement of the corresponding events, i.e., CDIS-EV-IND as shown in procedure 2 (P2). Then, CDIS periodically checks the events requested by CM as shown in procedure 3 (P3), and notifies the detected event through the message of CDIS-EV-RSP to CM.

Similarly to CM, the CDIS request the context information and event detection of CM through CM-CI-REQ and CM-EV-REQ messages, respectively. After receiving these messages, CM sends the context information, or notifies the detected event with acknowledgement through the messages of CM-CI-RSP, CM-EV-IND and CM-EV-RSP, individually.

More specific description of each message for interface between CM and CDIS is given in following sub-clause

6.3.3.2.1 CDIS-CI-REQ

This message used by CM is transmitted to CDIS to request context information of the registered CM or neighbor CM. This message is generated on CM for channel classification of TVWS channel. The contents of this message are given as follows

| Name | Contents | Description |
| --- | --- | --- |
| CDIS-CI-REQ | SourceCMIdentifier | Identifier of CM |
|  | DestinationCDISIdentifier | Identifier of CDIS |
|  | CIList | Flag array of reported context information selected by set of the corresponding flag, ordered as follows: - Channel list of TVWS - Constraint of channel use - Identifier of registered CM - TVBD type of registered CE of registered CM - Geolocation of registered CE of registered CM - Coverage of registered CE of registered CM - Channel classification list of registered CM - Negotiation option of registered CM - Identifier of neighbour CM - Corresponding CE list of neighbour CM - TVBD type of corresponding CE of neighbor CM - Geolocation of corresponding CE of neighbour CM - Coverage of corresponding CE of neighbor CM - Channel classification list of neighbour CM - Negotiation option of neighbor CM |

6.3.3.2.2 CDIS-CI-RSP

This message used by CDIS is transmitted to CM to send context information of the registered CM or neighbor CM. This message is generated on CDIS after receiving context information request message from CM. The contents of this message are given as follows

| Name | Contents | Description |
| --- | --- | --- |
| CDIS-CI-RSP | SourceCDISIdentifier | Identifier of CDIS |
|  | DestinationCMIdentifier | Identifier of CM |
|  | TVWSChannelList | Channel list of TVWS |
|  | ChannelUseConstraint | Constraint of channel use |
|  | RegisteredCMID | Identifier of registered CM |
|  | RegisteredCMRegisteredCETVBDType | TVBD type of registered CE of registered CM - Fixed device type - Personal/portable mode I type - Personal/portable mode II type |
|  | RegisteredCMRegisteredCEGeolocation | Geolocation of registered CE of registered CM - Latitude - Longitude - Altitude |
|  | RegisteredCMRegisteredCECoverage | Coverage of registered CE of registered CM |
|  | RegisteredCMChannelClassificationList | Channel classification list of registered CM |
|  | RegisteredCMNegotiationOption | Negotiation option of registered CM |
|  | NeighborCMID | Identifier of neighbour CM |
|  | NeighborCMCorrespondingCEList | Corresponding CE list of neighbour CM |
|  | NeighborCMCorrespondingCETVBDType | TVBD type of corresponding CE of neighbor CM - Fixed device type - Personal/portable mode I type - Personal/portable mode II type |
|  | NeighborCMCorrespondingCEGeolocation | Geolocation of corresponding CE of neighbour CM - Latitude - Longitude - Altitude |
|  | NeighborCMCorrespondingCECoverage | Coverage of corresponding CE of neighbor CM |
|  | NeighborCMChannelClassificationList | Channel classification list of neighbour CM |
|  | NeighborCMNegotiationOption | Negotiation option of neighbor CM |

6.3.3.2.3 CDIS-EV-REQ

This message used by CM is transmitted to CDIS to request the event detection of the neighbor CM. This message is generated on CM for performance maintenance of the 802.19.1 system. The contents of this message are given as follows

| Name | Contents | Description |
| --- | --- | --- |
| CDIS-EV-REQ | SourceCMIdentifier | Identifier of CM |
|  | DestinationCDISIdentifier | Identifier of CDIS |
|  | EventList | Flag array of the reported event selected by set of the corresponding flag, ordered as follows: - TVWS channel list change - Channel use constraint change - Neighbor CM change - Corresponding CE list change of neighbor CM - Channel classification list change of neighbor CM - Negotiation option change of neighbor CM |

6.3.3.2.4 CDIS-EV-IND

This message used by CDIS is transmitted to CM to give the acknowledgement of the corresponding event. This message is generated on CDIS after receiving the event request message from CM.  The contents of this message are given as follows

| Name | Content | Description |
| --- | --- | --- |
| CDIS-EV-IND | SourceCDISIdentifier | Identifier of CDIS |
|  | DestinationCMIdentifier | Identifier of CM |
|  | Status | Flag array for acknowledgement of the selected event, ordered as follows: - TVWS channel list change - Channel use constraint change - Neighbor CM change - Corresponding CE list change of neighbor CM - Channel classification list change of neighbor CM - Negotiation option change of neighbor CM |

6.3.3.2.5 CDIS-EV-RSP

This message used by CDIS is transmitted to CM to notify the detected event of CDIS. This message is generated on CDIS after detecting corresponding event of CDIS. The contents of this message are given as follows

| Name | Contents | Description |
| --- | --- | --- |
| CDIS-EV-RSP | SourceCDISIdentifier | Identifier of CDIS |
|  | DestinationCMIdentifier | Identifier of CM |
|  | FlagTVWSChannelListChange | Flag to specify if TVWS channel list is changed |
|  | FlagChannelUseConstraintChange | Flag to specify if channel use constraint is changed |
|  | FlagNeighborCMChange | Flag to specify if neighbor CM is changed |
|  | FlagNeighborCMCorrespondingCEListChange | Flag to specify if corresponding CE list of neighbor CM is changed |
|  | FlagNeighborCMChannelClassificationListChange | Flag to specify if channel classification list of neighbor CM is changed |
|  | FlagNeighborCMNegotiationOptionChange | Flag to specify if negotiation option of neighbour CM is changed |

6.3.3.2.6 CM-CI-REQ

This message used by CDIS is transmitted to the registered CM to request context information of the registered CM. This message is generated on CDIS for discovery of registered CMs in the 802.19.1 system. The contents of this message are given as follows

| Name | Contents | Description |
| --- | --- | --- |
| CM-CI-REQ | SourceCDISIdentifier | Identifier of CDIS |
|  | DestinationCMIdentifier | Identifier of CM |
|  | CIList | Flag array of reported context information selected by set of the corresponding flag, ordered as follows: - TVBD type of registered CE of registered CM - Geolocation of registered CE of registered CM - Coverage of registered CE of registered CM - Channel classification list of registered CM - Negotiation option of registered CM |

6.3.3.2.7 CM-CI-RSP

This message used by the registered CM is transmitted to CDIS to send context information of the registered CM. This message is generated on the registered CM after receiving the context information request message from CDIS. The contents of this message are given as follows

| Name | Contents | Description |
| --- | --- | --- |
| CM-CI-RSP | SourceCMIdentifier | Identifier of CM |
|  | DestinationCDISIdentifier | Identifier of CDIS |
|  | RegisteredCMRegisteredCETVBDType | TVBD type of registered CE of registered CM - Fixed device type - Personal/portable mode I type - Personal/portable mode II type |
|  | RegisteredCMRegisteredCEGeolocation | Geolocation of registered CE of registered CM - Latitude - Longitude - Altitude |
|  | RegisteredCMRegisteredCECoverage | Coverage of registered CE of registered CM |
|  | RegisteredCMChannelClassificationList | Channel classification list of registered CM |
|  | RegisteredCMNegotiationOption | Negotiation option of registered CM |

6.3.3.2.8 CM-EV-REQ

This message used by CDIS is transmitted to CM to request the event detection of CM. This message is generated on CDIS for performance maintenance of the 802.19.1 system.  The contents of this message are given as follows

| Name | Contents | Description |
| --- | --- | --- |
| CM-EV-REQ | SourceCDISIdentifier | Identifier of CDIS |
|  | DestinationCMIdentifier | Identifier of CM |
|  | EventList | Flag array of the reported event selected by set of the corresponding flag, ordered as follows: - Registered CE list change of CM - Geolocation change of registered CE of CM - Coverage change of registered CE of CM - Channel classification list change of CM - Negotiation option change of CM |

6.3.3.2.9 CM-EV-IND

This message used by CM is transmitted to CDIS to give the acknowledgement of the corresponding event. This message is generated on CM after receiving the event request message from CDIS.  The contents of this message are given as follows

| Name | Content | Description |
| --- | --- | --- |
| CM-EV-IND | SourceCMIdentifier | Identifier of CM |
|  | DestinationCDISIdentifier | Identifier of CDIS |
|  | Status | Flag array for acknowledgement of the selected event, ordered as follows: - Registered CE list change of CM - Geolocation change of registered CE of CM - Coverage change of registered CE of CM - Channel classification list change of CM - Negotiation option change of CM |

6.3.3.2.10 CM-EV-RSP

This message used by CM is transmitted to CDIS to notify the detected event of CM. This message is generated on CDIS after detecting corresponding event of CM. The contents of this message are given as follows

| Name | Contents | Description |
| --- | --- | --- |
| CM-EV-RSP | SourceCMIdentifier | Identifier of CM |
|  | DestinationCDISIdentifier | Identifier of CDIS |
|  | FlagCMRegisteredCEListChange | Flag to specify if registered CD list of CM is changed |
|  | FlagCMRegisteredCEGeolocationChange | Flag to specify if registered CE geolocation of CM is changed |
|  | FlagCMRegisteredCECoverageChange | Flag to specify if registered CE coverage of CM is changed |
|  | FlagCMChannelClassificationListChange | Flag to specify if channel classification list of CM is changed |
|  | FlagCMNegotiationOptionChange | Flag to specify if negotiation option of CM is changed |

6.3.3.3 Message lists and contents between TVWS DB and CM/CDIS

Figure 10 shows the message sequence chart (MSC) between TVWS DB and CM/CDIS where the message used in this MSC is context information message. For obtaining information of usable spectrum channel, the entities of CM and CDIS request context information of TVWS DB through TVWSDB-CI-REQ and TVWS-DB-REQ messages, respectively. Then, TVWS DB sends the response message with context information of usable spectrum channel to the corresponding entity through the message of TVWSDB-CI-RSP for CM and message of TVWS-DB-RSP for CDIS.

More specific description of each message for interface between TVWS DB and CM/CDIS is given in following sub-clause

Figure 10 ─ Message sequence chart (MSC) between TVWS DB and CM/CDIS

6.3.3.3.1 TVWSDB-CI-REQ

This message used by CM is transmitted to TVWS DB to request context information of usable spectrum channel. This message is generated on CM for the channel classification of TVWS channel. The contents of this message are given as follows

| Name | Contents | Description |
| --- | --- | --- |
| TVWSDB-CI-REQ | SourceCMIdentifier | Identifier of CM |
|  | DestinationTVWSDBIdentifier | Identifier of TVWS DB |
|  | TVBDFCCIdentifierList | A list of TVBD FCC identifier |

6.3.3.3.2 TVWSDB-CI-RSP

This message used by TVWS DB is transmitted to CM to send context information of usable spectrum channel. This message is generated on TVWS DB after receiving the context information request message from CM.  The contents of this message are given as follows

| Name | Contents | Description |
| --- | --- | --- |
| TVWSDB-CI-RSP | SourceTVWSDBdentifier | Identifier of TVWS DB |
|  | DestinationCMIdentifier | Identifier of CM |
|  | TVWSChannelList | TVWS channel list |
|  | ChannelUseConstraint | Channel use constraint |

6.3.3.3.3 TVWS-DB-REQ

This message used by CDIS is transmitted to TVWS DB to request context information of usable spectrum channel. This message is generated on CDIS for the channel classification of TVWS channel. The contents of this message are given as follows

| Name | Contents | Description |
| --- | --- | --- |
| TVWS-DB-REQ | SourceCDISIdentifier | Identifier of CDIS |
|  | DestinationTVWSDBIdentifier | Identifier of TVWS DB |
|  | TVBDFCCIdentifierList | A list of TVBD FCC identifier |

6.3.3.3.4 TVWS-DB-RSP

This message used by TVWS DB is transmitted to CDIS to send context information of usable spectrum channel. This message is generated on TVWS DB after receiving the context information request message from CDIS. The contents of this message are given as follows

| Name | Contents | Description |
| --- | --- | --- |
| TVWS-DB-RSP | SourceTVWSDBdentifier | Identifier of TVWS DB |
|  | DestinationCDISIdentifier | Identifier of CDIS |
|  | TVWSChannelList | TVWS channel list |
|  | ChannelUseConstraint | Channel use constraint |

7. Coexistence mechanisms and algorithms

7.1 General

Coexistence mechanisms and algorithms shall mitigate coexistence among dissimilar or independently operated TVBD networks and dissimilar TVBDs. There might be two different coexistence problems to be solved related to channel use. One is how to allocate a proper operating channel to each TVBD network (or device) regarding its neighbor TVBD network (or device). The other is how to share the same channel if two or more TVBD networks (or devices) have the same operating channel.

7.2 Channel assignment algorithms

TBD

7.3 Co-channel sharing mechanisms

TBD

Annex A: Use cases

TBD

Annex B: Evaluation methodology

TBD

Annex C: Coexistence Enabler (CE) Software reference design

TBD