IEEE P802.19
TV White Space Coexistence Methods

| ITU-R Contribution on IEEE 802 Standardization for TVWS Operation |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-05-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Steve Shellhammer, | Qualcomm |  |  | sshellha@qualcomm.com |
| Joe Kwak | InterDigital Communications |  |  | joekwak@SBCGLOBAL.NET |
| Junyi Wang | NICT |  |  | junyi.wang@nict.go.jp |
| Tuncer Baykas | NICT |  |  | tuncerbaykas@nict.go.jp |

| Working Party: ITU-R WP-1B, 5A, 6A | Document No: IEEE-SA WP1B/xx |
| --- | --- |
| Reference: Document CPM11-2/TEMP/31-E, WRC-12 Agenda Item 1.19 | Date: May 11, 2011 |
| Document Title: Information Paper on developments of Cognitive Radio Standards in IEEE 802 |  |
| Author: | Contact info: |
| Purpose/Objective: To provide background on developments of IEEE-SA Standards relying on cognitive radio capabilities such as using geolocation capability and accessing a database providing available frequencies and/or maximum allowable EIRP in the television bands, and using RF sensing to detect presence of incumbent radiocommunication systems to adaptively change operating frequency to avoid interference. Abstract: This document describes activities in the various Working Groups of the IEEE 802 towards the development of wireless telecommunication standards that include cognitive radio capabilities for opportunistic operation in the white space of the television broadcasting bands. |  |

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair Apurva Mody <apurva.mody@ieee.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Document IEEE-SA WP1B/xx

IEEE-SA WP5A/xy

IEEE-SA WP6A/yy

9 May 2011

English only

Wireless telecommunication standards under development within IEEE 802
utilizing cognitive radio capabilities for operation in the TV White Space

Information Paper

- Introduction

This document describes IEEE 802 standards projects for wireless networks operating in the TV White Space.  A number of cognitive radio techniques are included within  these standards to enable operation according to the various regulatory rules that have been, or will be, established by different Administrations, for operation in the TV white space.

- 802.22 Wireless Regional Area Networks (WRAN)

- 802.19 TV White Space Coexistence Methods

There are several standards projects for wireless networks in the limited TV white space.  In 2009 the IEEE identified potential coexistence issues and as a result initiated a standards project for coexistence of wireless networks in the TV white space. The IEEE 802.19.1 standards project on “TV White Space Coexistence Methods” was approved by the IEEE standards board in December 2009.

The scope and purpose of the project are provided below:

Project Scope: The standard specifies radio technology independent methods for coexistence among dissimilar or independently operated TV Band Device (TVBD) networks and dissimilar TV Band Devices.

Project Purpose: The purpose of the standard is to enable the family of IEEE 802 Wireless Standards to most effectively use TV White Space by providing standard coexistence methods among dissimilar or independently operated TVBD networks and dissimilar TVBDs. This standard addresses coexistence for IEEE 802 networks and devices and will also be useful for non IEEE 802 networks and TVBDs.

The standard will utilize the location awareness and adaptive abilities of the TVBDs, which are capabilities that will be common in the TVBDs.

As part of the standard development a system design document [8] was developed.  The system design document includes a coexistence system architecture that is shown in Figure 1.  The architecture consists of three logical entities and five logical interfaces.

The logical entities are:

- Coexistence Manager (CM)

- Coexistence Enabler (CE)

- Coexistence Discovery and Information Server (CDIS).

- Coexistence system architecture

The coexistence system design will provide radio environment awareness to a diverse range of unlicensed TVBDs using any radio access technology by implementing a distributed, local, geographic database of unlicensed TVBDs. In addition the coexistence system will offer management services to subscribing TVBDs to implement a standardized spectrum resource allocation among the unlicensed users of the TVWS.

The IEEE 802.19 working group is currently preparing a draft standard. The first draft standard is expected in 2012.  The plan is to begin sponsor ballot in early 2013 and for the standard to be published in late 2013.

.

- 802.15

- 802.11af

- Conclusion

- References

[1]	US FCC, ET Docket 02-380: Notice of Inquiry, In the Matter of “Additional Spectrum for Unlicensed Devices Below 900MHz and in the 3GHz Band”, Released December 20, 2002

[2]	US FCC, ET Docket 04-186: Notice of Proposed Rule Making, In the Matter of “Unlicensed Operation in the TV Broadcast Bands”, Released May 25, 2004

[3]	IEEE Std 802.22.1TM-2009, “Standard for Information Technology — Telecommunications and Information Exchange Between Systems — Local and Metropolitan Area Networks – Specific Requirements — Part 22.1: Enhanced Protection for Low-Power, Licensed Devices Operating in Television Broadcast Bands,” November 2009.

[4]	US FCC, ET Docket 04-186: First Report and Order and Further Notice of Proposed Rule Making, In the Matter of “Unlicensed Operation in the TV Broadcast Bands”, Released October 18, 2006.

[5]	U.S. FCC, ET Docket 08-260, “Second Report and Order and Memorandum Opinion and Order in the Matter of Unlicensed Operation in the TV Broadcast Bands,” November 14, 2008.

[6]	IEEE Std 802.22TM-2011, “Standard for Information Technology — Telecommunications and Information Exchange Between Systems — Local and Metropolitan Area Networks – Specific Requirements — Part 22: Wireless Regional Area Networks, Cognitive Wireless RAN Medium Access Control (MAC) and Physical Layer (PHY) specifications: Policies and procedures for operation in the TV Bands,” July 2011.

[8] “System Design Document”, IEEE 802.19-10/55r3, March 2010, available at: https://mentor.ieee.org/802.19/dcn/10/19-10-0055-03-0001-system-design-document.doc

____________________________