IEEE P802.11
Wireless LANs

| Comment Resolution regarding to CID 53 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-11-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Hyunduk Kang | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea | +82-42-860-1074 | henry@etri.re.kr |
| Donghoon Lee | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea | +82-42-860-0865 | mmdang@etri.re.kr |
| Byung-Jang Jeong | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea | +82-42-860-6765 | bjjeong@etri.re.kr |

Abstract

This document provides proposed resolution for CID 53 received in the 2nd TG review of Draft P802.19.1/DF1.

| CID | Page | Sub-clause | Line # | Comment | Proposed Change | Response |
| --- | --- | --- | --- | --- | --- | --- |
| 53 | 45 | 5.2.4 |  | It is difficult to how the obtaining neighbor information procedure works with the current description. More explanation will be helpful to understand. | Revise the paragraph. | Revise the sentence as proposed below |

Propose

Change the original normative text of sub-clause 5.2.4.1 obtaining neighbor list procedure

from:

This procedure, illustrated in Figure 20, is performed when CM wants to obtain the neighbor list from the CDIS.

to:

This procedure, illustrated in Figure 20, is performed when CM wants to obtain the neighbor list from the CDIS. Based on the CM’s subscription to the CDIS, CM shall get neighbor list information for intra-CM neighbors only or both intra-CM and inter-CM neighbors.

Change the original normative text of sub-clause 5.2.4.2 obtaining neighbor report procedure

from:

This procedure, illustrated in Figure 21, is performed when TVBD network or device wants to obtain neighbor report from CM.

to:

This procedure, illustrated in Figure 21, is performed when TVBD network or device wants to obtain neighbor report from CM. The CM shall provide neighbor list information for all neighbors; both intra-CM and inter-CM neighbors.

Change the original normative text of sub-clause 5.2.4.3 providing updated neighbor list procedure

from:

This procedure, illustrated in Figure 22, is performed when neighbor information is changed for one or several TVBD networks or devices of a CM.

to:

This procedure, illustrated in Figure 22, is performed when neighbor information is changed for one or several TVBD networks or devices of a CM. Only one CM and one CDIS are illustrated in Figure 22. The CDIS announces updated neighbor list to multiple CMs if necessary.

Change the original normative text of sub-clause 5.2.4.4 providing updated neighbor report procedure

from:

This procedure, illustrated in Figure 23, is performed when neighbor information is changed for one or several TVBD networks or devices subscribed to the information service. Only one CE and TVBD network or device illustrated in Figure 23.

to:

This procedure, illustrated in Figure 23, is performed when neighbor information is changed for one or several TVBD networks or devices subscribed to the information service. Only one CE and TVBD network or device illustrated in Figure 23. The CM announces updated neighbor list to multiple CEs if necessary.