IEEE P802.11
Wireless LANs

| Comment Resolution regarding to CID 54 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-11-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Hyunduk Kang | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea | +82-42-860-1074 | henry@etri.re.kr |
| Donghoon Lee | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea | +82-42-860-0865 | mmdang@etri.re.kr |
| Byung-Jang Jeong | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea | +82-42-860-6765 | bjjeong@etri.re.kr |

Abstract

This document provides proposed resolution for CID 54 received in the 2nd TG review of Draft P802.19.1/DF1.

| CID | Page | Sub-clause | Line # | Comment | Proposed Change | Response |
| --- | --- | --- | --- | --- | --- | --- |
| 54 | 48 | 5.2.6 |  | It is difficult to how the channel classification information procedure works with simple description. More explanation will be helpful to understand. | Revise the paragraph. | Revise the sentence as proposed below |

Propose

Change the original normative text of sub-clause 5.2.6.1 Obtaining channel classification information by CM procedure

from:

This procedure, illustrated in Figure 28, is used by a CM to obtain channel classification information from another CM.

to:

This procedure, illustrated in Figure 28, is used by a CM to obtain channel classification information from another CM. Channel classification information includes classified channel lists based on channel status; available, restricted, protected, unclassified, operating, and coexistent.

Change the original normative text of sub-clause 5.2.6.3 Announcing channel classification information update to CM procedure

from:

This procedure, illustrated in Figure 30, is used by a CM to announce channel classification information update to another CM.

to:

This procedure, illustrated in Figure 30, is used by a CM to announce channel classification information update to another CM. Only one CM pair is illustrated in Figure 30. One CM announces updated channel classification information to multiple CMs if necessary.

Change the original normative text of sub-clause 5.2.6.4 Announcing channel classification information update to CE procedure

from:

This procedure, illustrated in Figure 31, is by a CM to announce channel classification information update to a CE.

to:

This procedure, illustrated in Figure 31, is by a CM to announce channel classification information update to a CE. Only one CM and one CE are illustrated in Figure 31. One CM announces updated channel classification information to multiple CEs if necessary.