IEEE P802.19
TV Whitespace Coexistence Methods

| Comment Resolutions regarding to CID 75 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-11-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Donghoon Lee | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea | +82-42-860-0865 | mmdang@etri.re.kr |
| Hyunduk Kang | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea | +82-42-860-1074 | henry@etri.re.kr |
| Byung-Jang Jeong | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea | +82-42-860-6765 | bjjeong@etri.re.kr |

Abstract

This document provides proposed resolutions for comments 75 received in the 2nd TG review of Draft P802.19.1/DF1.

| CID | Clause | Subclause | Page | Line | Type | Comment | Proposed change |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 75 | 5 | 5.4 | 80 |  | T | CM needs subscribed service information (i.e. information service or management service) of neighbor TVBD network of device which is registered to another CM | Data type of subscribed service of ReqInfoDescr and ReqInfoValue might be added in section 5.4 |

Discussion:

CID 75 points out that a CM needs the type of subscribed service information (i.e., information service or management service) of neighbor TVBD network or device registered to another CM. For example, this information is used when a CM classified the neighbor TVBDs which need negotiation or not. This is because a CM considers negotiation of neighbor TVBD networks or devices which subscribe management service. Then, we propose adding the subscribed service information at the data type of obtaining information procedures message.

Proposal (Modification is marked by blue color)

- ReqInfoDescr :: = sequence of enumeration{

- SINR,

- desiredBandwidth,

- desiredQoS,

- desiredCoverage,

- channelNumber,

- subscribedService,

- …

- }

- ReqInfoValue ::= sequence of sequence {

- reqInfoDescr		ReqInfoDescr,

- reqInfoValue		choice {SINRvalue	real,

- desiredBandwidthValue	real,

- desiredOccupancyValue	real,

- desiredQoSValue		real,

- desiredCoverageValue	real,

- channelNumberalue…	real,

- otherValue	Any}

- subscribedSerive	SubscribedService

- }

## Editing instructions:

TG1 Editor:

- Add blue colored content in data types of ReqInfoDescr and ReqInfoValue