IEEE P802.19
Wireless Coexistence

| Measurements and measurement reporting |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-04-24 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Päivi Ruuska | Nokia Research Center | Visiokatu 1, 33720 Tampere, Finland | +358-718035433 | paivi.m.ruuska@nokia.com |
| Jari Junell | Nokia Research Center | Itämerenkatu 9, 00180 Helsinki, Finland | +358-718036575 | jari.junell@nokia.com |
| Mika Kasslin | Nokia Research Center | Itämerenkatu 9, 00180 Helsinki, Finland | +358-718036294 | mika.kasslin@nokia.com |

Abstract

This document contains text proposal for section 9.6 “Measurements by WSOs”. Additionally, new message descriptions and data type definitions are proposed for sections 4.3.2, 5.3 and 5.4 to reflect the changes in the section 9.6.

Notice: This document has been prepared to assist IEEE 802.19. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Editorial instructions: Replace section 9.6 in the DF2.08 with the following section and update section numbering as required.

## 9.6 Measurements and measurement reporting

A CM has means to obtain measurement results from all the CEs it serves. The CM shall be able to configure the measurement reporting. The CM may request the CE to provide one-time measurements or scheduled measurements by sending a measurement request to the CE (as defined in 5.2.8).

A WSO performs measurements according to its capabilities. A WSO provides measurement results to the CE and the CE shall form the IEEE 802.19.1 measurement reports out from the measurement results. In the CE registration the CE shall indicate the CM which of the measurement reports it supports with the WSO. The measurement reports are defined in section 5.3.9.

The following sub-clauses define the measurement types and the way how the measurement report fields are filled with each measurement type.

### 9.6.1 SINR measurement

The signal to interference plus noise ratio (SINR) measurement indicates the ratio of the received signal power (S-IN) to the average interference plus noise power (IN):

SINR= (S-IN) / IN,	where

S = Received signal strength (RSS) measured at the reference point of the measuring device during reception of own network transmissions

IN = RSS measured at the reference point of the measuring device during periods when the measuring device considers the channel to be idle

The SINR is indicated in dB and it is scaled in steps of 0.5 dB to obtain the SINR values, which cover the range from –10 dB (value = 0) to + 117 dB (value = 254). The value 255 indicates that SINR is not available.

### 9.6.2 FER measurement

The frame error rate (FER) measurement indicates the ratio of the frames that the measuring device received with errors to the total number of frames received by the measuring device. The measurement is carried over all the wireless links the measuring device has during the measurement period.

The FER is indicated as the percentage, linearly scaled with 255 representing that all frames have errors (FER=100%). The percentage shall be computed using the following formula:

FER = Integer ((number of frames with errors / number of measured frames) × 255)

### 9.6.3 Interference plus noise floor measurement

The interference plus noise floor (IPNF) measurement is an indication of the average interference plus noise power measured at the reference point of the measuring device during periods when the measuring device considers the channel to be idle.

The IPNF is indicated in dBm and is scaled in steps of 0.5 dBm to obtain values, which cover the range from –134 dBm (value = 0) to – 7 dBm (value = 254). The value 255 indicates that IPNF is not available.

### 9.6.4 Signal distribution measurement

The signal distribution measurement indicates the ratio of time that received signal is within each defined received signal level range. The received signal level is measured at the reference point of the measuring device and in this measurement type any detected signal is considered.

The signal distribution measurement report contains a set of signal level ranges and indications of proportion of detected signal within the ranges. The signal level ranges are indicated with means of level thresholds that determine signal level ranges that are all equal in size. The proportion of detected signal within each signal level range is indicated as the percentage of time the measuring device has detected signal within the signal level range. The percentage is computed using the following formula:

Signal distribution within a signal level range = Integer ((time that the received signal is between the thresholds defined for the signal level range / measurement duration) × 255)

Figure 1 shows an example of thresholds and the proportion of detected signal within each signal level range.

Figure 1: An example of signal distributions

### 9.6.5 Spectrum measurement

The spectrum measurement provides the frequency response as perceived by the measuring device. The measurement is performed at the reference point of the measuring device.

The spectrum measurement report contains the number of sub-channels, the received signal strength for each sub-channel, and the measurement bandwidth.

The number of sub-channels indicates into how many sub-channels the measured and reported bandwidth is divided. The number of sub-channels value is indicated in the range from 1 to 4095. This field also indicates the number of the fields indicating the received signal strength for the sub-channel in the frame.

The received signal strength for the sub-channel is defined as described for the received signal strength for the measured bandwidth. The received signal strength for the sub-channels is indicated in dBm and is scaled in steps of 0.5 dBm to obtain values, which cover the range from –134 dBm (value = 0) to – 7 dBm (value = 254). The frame contains a field for each sub-channel.

The measurement bandwidth is indicated in kHz.

Figure 2: An example of frequency response, measured bandwidth and sub-channel

### 9.6.6 Channel load measurements

The channel load measurement provides a channel utilization measure as seen by the measuring device. Two different channel load measurement types are defined: Channel load of the network in which the measuring device operates and Total channel load.

9.7.6.1 Own network channel load

Own network channel load is defined as the percentage of time that the measuring device assesses the channel to be utilized by its own network.

The own network channel load is indicated as the percentage of time, linearly scaled with 255 representing that the channel is 100% utilized by the own network. This percentage shall be computed using the following formula:

Own network channel load = Integer((time that the channel is occupied by the own network / measurement duration) × 255)

9.7.6.2 Total channel load

Total channel load is defined as the percentage of time that the measuring device assesses the channel to be utilized.

The total channel load is indicated as the percentage of time, linearly scaled with 255 representing that the channel is assessed to be utilized 100% of the measurement time. This percentage shall be computed using the following formula:

Total channel load = Integer((channel utilized time / measurement duration) × 255)

### 9.6.7 Measurements detecting other spectrum users

These measurements enable a measuring device to report operation of other wireless transmitters, which it has detected on the TV band spectrum. A CE that supports these measurement reports is connected to a WSO that has capabilities to recognize the type of the other wireless transmitter that it reports. As an example the measuring device may have spectrum sensor with feature detector capable of detecting specific radio access technology, or it may recognize the operation of other wireless transmitters because it supports the same radio access technology.

RAT of the detected wireless transmitter is indicated with technology type field.

### 4.3.2 COEX_MEDIA_SAP

Editorial instructions: Replace the measurement related data types in section 4.3.2 of the DF2.08 from line 6 through line 46 on draft page 37 with the following.

MeasurementSchedule ::= SEQUENCE {

measStartTime	                        REAL,

numberOfMeasurements         INTEGER,

timeBetweenMeasurements    REAL

}

FrequencyIndication ::= SEQUENCE {

startFreq		INTEGER,

stopFreq		INTEGER

}

ChannelIndication ::= SEQUENCE OF INTEGER

MeasurementFreq ::= CHOICE {

startStopFreq		FrequencyIndication,

channelNumber	ChannelIndication

}

MeasurementType ::= ENUMERATED {

sinr,

fer,

ipnf,

signalDistribution,

spectrum,

ownNetworkChannelLoad,

totalChannelLoad,

otherUsers

}

MeasurementDescription ::= SEQUENCE {

measType 		MeasurementType,

measSchedule 		MeasurementSchedule,

measFreq		MeasurementFreq

}

SinrReport ::= SEQUENCE {

measBW	INTEGER,

sinr 		REAL

}

FerReport ::= SEQUENCE {

fer		REAL

}

IpnfReport ::= SEQUENCE {

measBW	INTEGER,

ipnf		REAL

}

SignalDistributionReport ::= SEQUENCE {

measBW			INTEGER,

lowEndOfSignalLevelRange	INTEGER,

numberOfSignalLevelRange	INTEGER,

widthOfSignalLevelRange	INTEGER,

signalProportionPerRange	SEQUENCE OF REAL

}

SpectrumReport ::= SEQUENCE {

measBW			INTEGER,

lowEndOfReportedBandwidth	INTEGER,

numberOfSubchannels		INTEGER,

widthOfSubchannel		INTEGER,

signalStrengthPerSubchannel	SEQUENCE OF REAL

}

ChannelLoadReport ::= SEQUENCE {

load		REAL

}

OtherUsersReport ::= SEQUENCE {

technologyType	NetworkTechnology

}

MeasurementReport ::= CHOICE {

sinrReport 				SinrReport,

ferReport 				FerReport,

ipnfReport 				IpnfReport,

signalDistributionReport		SignalDistributionReport,

spectrumReport			SpectrumReport,

ownNetworkChannelLoadReport		ChannelLoadReport,

totalChannelLoadReport		ChannelLoadReport,

otherUsersReport			OtherUsersReport

}

MeasurementResult ::= SEQUENCE OF SEQUENCE {

measDescription	MeasurementDescription,

measReport		MeasurementReport,

}

## 5.3 Messages

Editorial instructions: Replace the measurement related messages in section 5.3 of the DF2.08 from line 27 through line 35 on draft page 65 with the following.

-- Requesting and obtaining measurement procedure messages

MeasurementRequest ::= SEQUENCE {

-- Measurement description

measDescription	MeasurementDescription

}

MeasurementResponse ::= SEQUENCE OF SEQUENCE {

-- Measurement result in form of measurement reports

measDescription	MeasurementDescription,

measReport 		MeasurementReport,

}

## 5.4 Data types

Editorial instructions: Replace the measurement related data types in section 5.4 of the DF2.08 from line 36 on draft page 72 through line 15 on draft page 73 with the following.

MeasurementSchedule ::= SEQUENCE {

measStartTime	                        REAL,

numberOfMeasurements         INTEGER,

timeBetweenMeasurements    REAL

}

FrequencyIndication ::= SEQUENCE {

startFreq		INTEGER,

stopFreq		INTEGER

}

ChannelIndication ::= SEQUENCE OF INTEGER

MeasurementFreq ::= CHOICE {

startStopFreq		FrequencyIndication,

channelNumber	ChannelIndication

}

MeasurementType ::= ENUMERATED {

sinr,

fer,

ipnf,

signalDistribution,

spectrum,

ownNetworkChannelLoad,

totalChannelLoad,

otherUsers

}

MeasurementDescription ::= SEQUENCE {

measType 		MeasurementType,

measSchedule 		MeasurementSchedule,

measFreq		MeasurementFreq

}

SinrReport ::= SEQUENCE {

measBW	INTEGER,

sinr 		REAL

}

FerReport ::= SEQUENCE {

fer		REAL

}

IpnfReport ::= SEQUENCE {

measBW	INTEGER,

ipnf		REAL

}

SignalDistributionReport ::= SEQUENCE {

measBW			INTEGER,

lowEndOfSignalLevelRange	INTEGER,

numberOfSignalLevelRange	INTEGER,

widthOfSignalLevelRange	INTEGER,

signalProportionPerRange	SEQUENCE OF REAL

}

SpectrumReport ::= SEQUENCE {

measBW			INTEGER,

lowEndOfReportedBandwidth	INTEGER,

numberOfSubchannels		INTEGER,

widthOfSubchannel		INTEGER,

signalStrengthPerSubchannel	SEQUENCE OF REAL

}

ChannelLoadReport ::= SEQUENCE {

load		REAL

}

OtherUsersReport ::= SEQUENCE {

technologyType	NetworkTechnology

}

MeasurementReport ::= CHOICE {

sinrReport 				SinrReport,

ferReport 				FerReport,

ipnfReport 				IpnfReport,

signalDistributionReport		SignalDistributionReport,

spectrumReport			SpectrumReport,

ownNetworkChannelLoadReport		ChannelLoadReport,

totalChannelLoadReport		ChannelLoadReport,

otherUsersReport			OtherUsersReport

}