IEEE P802.19
TV Whitespace Coexistence Methods

| Comment Resolutions regarding to Messages and Data types |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-03-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Donghun Lee | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea | +82-42-860-0865 | mmdang@etri.re.kr |
| Hyunduk Kang | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea | +82-42-860-1074 | henry@etri.re.kr |
| Byung-Jang Jeong | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea | +82-42-860-6765 | bjjeong@etri.re.kr |

Abstract

This document provides comment resolutions on messages and data types of Draft P802.19.1/DF2.02

- Discussion:

Required slot time and window time in the data type of RequiredResource are necessary for negotiation. Then, we propose the revised paragraph in 4.3 and 5.4, respectively.

Proposal (Adding parameters marked by blue color)

  - Data type definition

RequiredResourceElement ::= SEQUENCE {

requiredBandwidth	REAL,

expectedLoad            	REAL,

requiredSlotTime	REAL,

requiredWindowTime	REAL,

…

}

  - Data types

RequiredResource ::= SEQUENCE OF SEQUENCE{

requiredBandwidth	REAL,

expectedLoad            	REAL,

requiredSlotTime	REAL,

requriedWindowTime	REAL,

…

}

- Discussion:

- The data type of CxStatus of 5.3 sub-clause in the revised draft of DF2.01 is newly added to some messages which are enumerated multiple options such as noError, rejected, invalidEntityStatus and etc.

- The data type of the draft of DF1, however, is Boolean in messages of

  - AuthenticationResponse,

  - SubscriptionResponse,

  - SubscriptionChangeResponse,

  - CMMasterCMConfirm,

  - MasterCMConfirm,

  - ReconfigurationResponse,

  - CMReconfigrationResponse.

- In addition, some messages have no payload in the draft of DF1, but the revised draft of DF2.01 added the data type of CxStatus such as

  - CoexistenceSetInformationResponse,

  - CoexistenceReportResponse,

  - InfoAquiringResponse,

  - CoexistenceSetInformationConfirm,

  - MeasurementResponse,

  - MeasurementConfirm

  - CMMasterSlaveCMConfigurationConfirm

Proposal

- It shall provide the specific reason  why the data type of the draft 2.01 is revised as indicated in above.

- Discussion:

CMChannelClassificaitonRequest of 27th line and ChannelClassificationRequest of 37th line at 64 page in the revised draft of DF2.01 are typos.

Proposal

- CMChannelClassificaitonRequest of 27th line at 64 page is revised into CMChannelClassificaitonResponse.

- ChannelClassificationRequest of 37th line at 64 page is revised into ChannelClassificationResponse.