IEEE P802.19
Wireless Coexistence

| Comment Resolution on the Monitoring Scheme of Session Aliveness among Coexistence System Entities |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-05-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Jinho Kim | LGE | LG R&D Complex 533, Hogye-1dong, Dongan-Gu, Anyang-Shi, Kyungki-Do, 431-749, Korea | +82-31-450-1867 | boiler4.kim@lge.com |
| Jinyoung Chun | LGE | LG R&D Complex 533, Hogye-1dong, Dongan-Gu, Anyang-Shi, Kyungki-Do, 431-749, Korea | +82-31-450-1856 | jiny.chun@lge.com |
| Suhwook Kim | LGE | LG R&D Complex 533, Hogye-1dong, Dongan-Gu, Anyang-Shi, Kyungki-Do, 431-749, Korea |  |  |
| Wookbong Lee | LGE |  |  |  |
| Jinsam Kwak | LGE |  |  |  |

Abstract

This document provides normative text related to a connection (aliveness of a session) monitoring scheme among Coexistence System entities such as CE, CM, and CDIS.

| CID | Clause | Subclause | Page | Line | Type | Comment | Suggested Remedy | Response |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69 |  | 5.2.1.1 |  |  |  | In the procedure of WSO authentication, what would be happen if the CM is powered off suddenly without any notification? Furthermore, before sending "Authentication Request", how does the CE expect that the CM is powered on and will work normally ? | In order to keep real-time robustness of the system, we suggest an keepalive message exchange and timeout rule. |  |

[Note- Underlined blue text is new text as below.]

Discussion

Problem

Question 1: In the procedure of WSO de-authentication (Fig. 9 in the section 5.2.1.2 ), what would be happen if the CM is powered off suddenly without any notification?

- Answer : Because the CE doesnot know that the CM is gone, CE just sends any request to the CM and waits for its response for some time.  (This is wasting time I think.)

Question 2: Furthermore, before sending "Deauthentication Request", how does the CE expect that the CM is still powered on and will work normally?

- Answer: When the CM is powered off suddenly in any reason, this fact should be notice to the related CEs and the CDIS as soon as possible in order to avoid meaningless requests and responses. So we suggest KeepAlive Request and Response messages between CDIS and CM, and between CM and CE.  These messages will make the system more robust.

Propose

Editor’s note: Insert the following text into the end of 5.2.13 Coexistence Set Element Reconfiguration Procedure.

5.2.14 Session Aliveness Monitoring Procedure

5.2.14.1. Session Aliveness Monitoring between a CDIS and a CM

This procedure, illustrated in Figure xx, is started when a CM is registered to the CDIS.

Figure xx. Periodic session monitoring between a CDIS and CMs

5.2.14.2. Session Aliveness Monitoring between a CM and a CE

This procedure, illustrated in Figure xx, is started when a CE is registered to the CM.

Figure xx. Periodic session monitoring between a CM and CEs

Propose

Editor’s note: Add four parameters into CxPayload message in the section 5.3  as below:

CxPayload ::= CHOICE {

authenticationRequest    AuthenticationRequest,

…

…

eventConfirm   EventConfirm,

cm_KeepAliveRequest    CM_KeepAliveRequest,

cm_KeepAliveResponse   CM_KeepAliveResponse,

cdis_KeepAliveRequest    CDIS_KeepAliveRequest,

cdis_KeepAliveResponse  CDIS_KeepAliveResponse

}

Propose

Editor’s note: Insert the following text in the end of section 5.3

--Sending KeepAliveRequest from CM to CE

CM_KeepAliveRequest ::= SEQUENCE {

Period	INTEGER

NumberOfRetry INTERGER OPTIONAL

}

--Sending KeepAliveResponse from CE to CM

CM_KeepAliveResponse ::= SEQUENCE {

Status	INTERGER

}

--Sending KeepAliveRequest from CDIS to CM

CDIS_KeepAliveRequest ::= SEQUENCE {

Period	INTEGER

NumberOfRetry INTERGER OPTIONAL

}

--Sending KeepAliveRequest from CM to CDIS

CDIS_KeepAliveRequest ::= SEQUENCE {

Status	INTERGER

}