IEEE P802.19
TV Whitespace Coexistence Methods

| Comment Resolutions regarding to final index of 10 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-05-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Donghun Lee | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea | +82-42-860-0865 | mmdang@etri.re.kr |
| Hyunduk Kang | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea | +82-42-860-1074 | henry@etri.re.kr |
| Byung-Jang Jeong | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea | +82-42-860-6765 | bjjeong@etri.re.kr |

Abstract

This document provides proposed resolutions for comment 10 received in the 802.19.1 LB.

| Final index | Clause | Subclause | Page | Line | Type | Comment | Proposed change |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 4 | 4.2.1 | 15,16 | 9,7 | T | The payload and data type for transportPref in CP_PACKET_SEND.request and in CP_PACKET_SEND.confirm are wrong | Replace transportPref and TransportPref with TransportType and TransportType, respectively |

Discussion:

Final index 10 pointed out that the payload and data type for transportPref in CP_PACKET_SEND.request and in CP_PACKET_SEND.confirm are wrong. Then, our proposal is given as follows

Proposal (Revision part is marked by blue color)

- 4.2.1.1 CP_PACKET_SEND.request

Table 3 –CP_PACKET_SEND.request primitive parameters

| - Name | - Data type | - Description |
| --- | --- | --- |
| - transportType | - TransportType | - Transport protocol preference |
| - sourceID | - Octet string | - Address of the entity sending a coexistence protocol data unit |
| - destinationID | - Octet string | - Address of the entity to receive a coexistence protocol data unit |
| - coexProtocolPDU | - Octet string | - Coexistence protocol data unit to be transported |

- 4.2.1.2 CP_PACKET_SEND.confirm

- CP_PACKET_SEND.confirm (

- transportType,

- sourceID,

- destinationID,

- transportStatus

- )

Table 4 –CP_PACKET_SEND.confirm primitive parameters

| - Name | - Data type | - Description |
| --- | --- | --- |
| - transportType | - TransportType | - Transport protocol preference |
| - sourceID | - Octet string | - Address of the entity sending a coexistence protocol data unit |
| - destinationID | - Octet string | - Address of the entity to receive a coexistence protocol data unit |
| - transportStatus | - Boolean | - Indicates whether the transfer of a coexistence protocol data unit was successful or not |

- 4.2.1.3 CP_PACKET_SEND.indication

- CP_PACKET_SEND.indication (

- transportType,

- sourceID,

- coexProtocolPDU

- )

Table 5 –CP_PACKET_RECEIVE.indication primitive parameters

| - Name | - Data type | - Description |
| --- | --- | --- |
| - transportType | - TransportType | - Transport protocol preference |
| - sourceID | - Octet string | - Address of the entity from which a coexistence protocol data unit was received |
| - coexProtocolPDU | - Octet string | - The received protocol data unit |