IEEE P802.19
TV Whitespace Coexistence Methods

| Comment Resolutions regarding to final index of 122 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-05-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Donghun Lee | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea | +82-42-860-0865 | mmdang@etri.re.kr |
| Hyunduk Kang | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea | +82-42-860-1074 | henry@etri.re.kr |
| Byung-Jang Jeong | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea | +82-42-860-6765 | bjjeong@etri.re.kr |

Abstract

This document provides proposed resolutions for comment 122 received in the 802.19.1 LB.

| Final index | Clause | Subclause | Page | Line | Type | Comment | Proposed change |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 122 | 5 | 5.3 | 63 | 23,42 |  | In DF1, the payloads of listOfAvailableChannel in AvailableChannelsResponse and AvailableChannelAnnouncement message are optinonal. However, in DF2.08 the optional term is removed | Need the explanation why the optional terms are removed |

Discussion:

Final index 122 pointed out that the payloads of listOfAvailableChannel in AvailableChannelsResponse and AvailableChannelAnnouncement message shall be optional. Then, our proposal is given as follows

Proposal (Revision is marked by blue color)

- 5.3 Messages

AvailableChannelsResponse ::= SEQUENCE {

-- Allowed TVWS channel number list

listOfAllowedTVWSChNumber List OfAllowedTVWSChNumber OPTIONAL,

-- Channel user constraint

constOfChUses ConstOfChUses OPTIONAL,

-- List of available channels

listOfAvailableChannel SEQUENCE OF SEQUENCE {

-- Frequency range

frequencyRange FrequencyRange,

-- Transmission power limit

txPowerLimit REAL OPTIONAL,

-- Start time of availability

availableStartTime GeneralizedTime,

-- Duration of availability

availableDuration REAL,

-- Aggregated interference control parameters

aggregatedInterferenceControlParameters AggregatedInterferenceControlParameters

} OPTIONAL,

AvailableChannelsAnnouncement ::= SEQUENCE {

-- Allowed TVWS channel number list

listOfAllowedTVWSChNumber List OfAllowedTVWSChNumber OPTIONAL,

-- Channel user constraint

constOfChUses ConstOfChUses OPTIONAL,

-- List of available channels

listOfAvailableChannel SEQUENCE OF SEQUENCE {

-- FCC identifier of WSO

deviceFCCID OCTET STRING,

-- Serial number of WSO

deviceSN OCTET STRING,

-- Netowrk identifier of WSO

networkID OCTET STRING,

-- Frequency range

frequencyRange FrequencyRange,

-- Transmission power limit

txPowerLimit REAL OPTIONAL,

-- Start time of availability

availableStartTime GeneralizedTime,

-- Duration of availability

availableDuration REAL,

-- Aggregated interference control parameters

aggregatedInterferenceControlParameters AggregatedInterferenceControlParameters

} OPTIONAL,

}