IEEE P802.19
Wireless Coexistence

| New procedure for procedure configuration |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-07-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Mika Kasslin | Nokia | Itämerenkatu 9, 00180 Helsinki, Finland | +358-718036294 | mika.kasslin@nokia.com |
| Jari Junell | Nokia | Itämerenkatu 9, 00180 Helsinki, Finland | +358-718036575 | jari.junell@nokia.com |

Abstract

This document contains description of a procedure with which an entity can configure operations of another entity with respect to selected procedures.

Notice: This document has been prepared to assist IEEE 802.19. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Editorial instructions: Have the following section with sub-sections incorporated into the draft in section 5.2 with proper section numbering.

## 5.2.14 Procedure Configuration Procedure

This procedure, illustrate in Figure x, is used by an entity (CE, CM or CDIS) to configure operations of another entity with respect to selected procedures.

Figure x – Requesting procedure configuration from another entity

The requesting entity may be a CE, a CM or a CDIS. A CE may transmit a request to a CM. A CM may transmit a request to a CE, to another CM and to a CDIS. A CDIS may transmit a request to a CM.

The initiating entity transmits a Procedure_Configuration_Request message to another entity to configure operations of the other entity with respect to a selected procedure. In the message the initiating entity shall identify the procedure to which the configuration applies and the requested configuration.

A CE may use the procedure to configure CM operations with respect to the following procedures:

- Providing coexistence report (5.2.4.4)

A CM may use the procedure to configure CE operations with respect to the following procedures:

- Obtaining coexistence report (5.2.4.2)

### 5.2.14.1 Configuring coexistence report provisioning

A CE may configure a CM with respect to how often the CM may transmit unsolicited coexistence reports. A CE requests configuration by transmitting a Procedure_Configuration_Request message in which it shall indicate the procedure in question to be the Providing coexistence report procedure and minimum time (timeBetweenMessages) as an integer multiple of 100 ms between two consecutive unsolicited coexistence reports from the CM. The timeBetweenMessages parameter shall be set to ‘0’ to request the CM to transmit unsolicited coexistence reports whenever there is a change in the report. The parameter shall be set to ‘65535’ to request the CM to disable transmission of unsolicited coexistence reports.

Upon receiving from a CE a Procedure_Configuration_Request message which applies to the Providing coexistence report procedure a CM shall configure its coexistence report provisioning for the CE as per the configuration in the message. Once configured the CM shall transmit unsolicited coexistence reports only when the report has been changed since the previous transmission of the report and when at least the minimum time requested by the CE has been elapsed since transmission of the previous unsolicited coexistence report to the CE. If the minimum time parameter was set to ‘0’ (default), the CM should transmit unsolicited coexistence reports whenever there is a change in the report. If the minimum time parameter was set to ‘65535’, the CM shall disable transmission of unsolicited coexistence reports.

### 5.2.14.2 Configuring coexistence report requests

A CM may configure a CE with respect to how often the CE may transmit requests for coexistence reports. A CM requests configuration by transmitting a Procedure_Configuration_Request message in which it shall indicate the procedure in question to be the Obtaining coexistence report procedure and minimum time as an integer multiple of 100 ms between two consecutive coexistence report requests from the CE. If the timeBetweenMessages parameter was set to ‘0’ (default), the CE is allowed to transmit a coexistence report at any time.

Upon receiving from a CM a Procedure_Configuration_Request message which applies to the Obtaining coexistence report procedure a CE shall configure its coexistence report requesting for the CM as per the configuration in the message. Once configured the CE may transmit coexistence report requests only when at least the minimum time requested by the CM has been elapsed since transmission of the previous coexistence report request to the CM.

Editorial instructions: Have the following new message and data type definitions added to sections 5.3 and 5.4 respectively.

## 5.3 Messages

ProdedureConfigurationRequest ::= SEQUENCE {

procedureToConfigure		Procedure,

configuration			ProcedureConfiguration

}

## 5.4 Data types

Procedure ::= ENUMERATED {

ProvidingCoexistenceReport,

ObtainingCoexistenceReport

}

ProcedureConfiguration ::= CHOICE {

timeBetweenMessages		INTEGER (0..65535)

}