IEEE P802.19
Wireless Coexistence

| Comment Resolution regarding to CID 3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-07-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Hyunduk Kang | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | henry@etri.re.kr |
| Donghun Lee | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | mmdang@etri.re.kr |
| Byung-Jang Jeong | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | bjjeong@etri.re.kr |

Abstract

This document provides proposed resolution for CID 3 received in the 1st WG letter ballot of Draft P802.19.1/DF2.08.

| CID | Page | Sub-clause | Line # | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 3 | 1 | 2.1 | 5 | There is no definition of interference which is important concept of the standard. | Add definition |

Discussion

Definitions of interference from other standards are as follows;

1900.1-2008 & IEEE 802.15.2-2003

In a communication system, interference is the extraneous power entering or induced in a channel from natural or man-made sources that might interfere with reception of desired signals or the disturbance caused by the undesired power.

1900.1-2008 & ITU-R Radio Regulations

Interference: The effect of unwanted energy due to one or a combination of emissions, radiations, or inductions upon reception in a radiocommunication system, manifested by any performance degradation, misinterpretation, or loss of information that could be extracted in the absence of such unwanted energy.

Harmful interference: Interference that endangers the functioning of a radionavigation service or of other safety services or seriously degrades, obstructs, or repeatedly interrupts a radiocommunication service operating in accordance with these Regulations (i.e., ITU Radio Regulations)

802.16h-2010

Harmful interference: An interference caused by an interference signal that seriously degrades,

obstructs, or repeatedly interrupts the radio communication.

Destructive interference: Interference that disables a particular PHY receiver from receiving using any combination of its modulation and coding methods.

Acceptable interference: An interference caused by an interference signal that does not degrade the current selection of modulation and coding at a receiver.

802.22-2011

Harmful interference: Any emission, radiation or induction that endangers the functioning of a radio navigation service or of other safety services or seriously degrades, obstructs, or repeatedly interrupts a radiocommunications service operating in accordance with the ITU and local Regulations. [See Article 1 of the ITU Radio Regulations, #1.169 and the FCC 47 CFR 15.3(m).]

Propose

Add definition of harmful interference in sub-clause 2.1 Definition

Harmful interference: Any emission, radiation or induction that endangers the functioning of a radio navigation service or of other safety services or seriously degrades, obstructs, or repeatedly interrupts a radiocommunications service operating in accordance with the ITU and local Regulations. [See Article 1 of the ITU Radio Regulations, #1.169 and the FCC 47 CFR 15.3(m).]