IEEE P802.19
TV Whitespace Coexistence Method

| Comment Resolutions regarding to 5.2.5.3 obtaining available channel list from TVWS database procedure |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-09-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Donghun Lee | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | mmdang@etri.re.kr |
| Hyunduk Kang | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | henry@etri.re.kr |
| Byung-Jang Jeong | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | bjjeong@etri.re.kr |

Abstract

This document provides proposed resolutions regarding to obtaining available channel list from TVWS database procedure in the Draft P802.19.1/DF2.08.

Discussion:

Obtaining available channel list from TVWS database is not well described. Then, we propose the revised paragraph in 5.2.5.3.

Proposal

      - Obtaining available channel list from TVWS database procedure

A CM shall perform the obtaining available channel list from TVWS database procedure when a CM wants to obtain a list of available channels from a TVWS database. An illustrative example of this procedure is shown in Figure 25. CM_AvailableChannels_Request and AvailableChannels_Response messages are defined in clause 5.3 Messages.

- Figure 25 – Obtaining available channel list from TVWS database procedure

When a CM wants to obtain a list of available channels from a TVWS database, it shall generate a CM_AvailableChannels_Request message and shall send this CM_AvailableChannels_Request message to TVWS database. The message includes WSO information as defined in clause of 5.4 Data Types. CM operation related to generating and sending a CM_AvailableChannels_Request message is specified in clause 7 CM Operation.

After TVWS database has received CM_AvailableChannels_Request message from the CM, the TVWS database shall generate an AvailableChannels_Response message, and shall send the AvailableChannels_Response message to the CM. The message includes available channel information as defined in clause of 5.4 Data Types.

## Editing instructions:

TG1 Editor:

- Add blued paragraph in sub-clause 5.2.5.3