IEEE P802.19
TV Whitespace Coexistence Method

| Comment Resolutions regarding to 5.2.7.1 obtaining information from WSO procedure |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-09-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Donghun Lee | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | mmdang@etri.re.kr |
| Hyunduk Kang | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | henry@etri.re.kr |
| Byung-Jang Jeong | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | bjjeong@etri.re.kr |

Abstract

This document provides proposed resolutions regarding to obtaining information from WSO procedure in the Draft P802.19.1/DF2.08.

Discussion:

Obtaining information from WSO procedure is not well described. Then, we propose the revised paragraph in 5.2.7.1.

Proposal

      - Obtaining information from WSO procedure

A CM shall perform the obtaining information from WSO procedure when a CM wants to obtain information from a WSO. An illustrative example of this procedure is shown in Figure 31. InfoAcquiring_Request and InfoAcquiring_Response messages are defined in clause 5.3 Messages.

- Figure 31 – Obtaining information from WSO procedure

When a CM wants to obtain information from a WSO, it shall generate an InfoAcquiring_Request message and shall send this InfoAcquiring_Request message to CE. The message includes list of required information description as defined in clause of 5.4 Data Types. CM operation related to generating and sending an InfoAcquiring_Request message is specified in clause 7 CM Operation.

After the CE has received the InfoAcquiring_Request message from the CM, the CE shall generate GetInfo.request primitive and shall send it to the WSO. CE operation related to generating and sending the GetInfo.request primitive is specified in clause 6 CE Operation.

After the CE has received an GetInfo.response primitive from a WSO, the CE shall generate an InfoAcquiring_Response message and shall send this InfoAcquiring_response message to CM. The message includes required information value as defined in clause of 5.4 Data Types. CE operation related to generating and sending a InfoAcquiring_response message is specified in clause 6 CE Operation.

After the CM has received the InfoAcquiring_response message from the CE, the CM shall process the InfoAcquiring_response message. CM operation related to processing the InfoAcquiring_response message is specified in clause 7 CM Operation.

## Editing instructions:

TG1 Editor:

- Substitute blued paragraph in sub-clause 5.2.7.1