IEEE P802.19
TV Whitespace Coexistence Method

| Comment Resolutions regarding to 5.2.7.2 obtaining information from another CM procedure |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-09-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Donghun Lee | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | mmdang@etri.re.kr |
| Hyunduk Kang | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | henry@etri.re.kr |
| Byung-Jang Jeong | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | bjjeong@etri.re.kr |

Abstract

This document provides proposed resolutions regarding to obtaining information from another CM procedure in the Draft P802.19.1/DF2.08.

Discussion:

Obtaining information from another CM procedure is not well described. Then, we propose the revised paragraph in 5.2.7.2.

Proposal

      - Obtaining information from another CM procedure

A CM shall perform the obtaining information from another CM procedure when a CM wants to obtain information from another CM. An illustrative example of this procedure is shown in Figure 32. InfoAcquiring_Request and InfoAcquiring_Response messages are defined in clause 5.3 Messages.

- Figure 32 – Obtaining information from another CM procedure

When a CM wants to obtain information from another CM, it shall generate an InfoAcquiring_Request message and shall send this InfoAcquiring_Request message to another CM. The message includes list of required information description as defined in clause of 5.4 Data Types. CM operation related to generating and sending an InfoAcquiring_Request message is specified in clause 7 CM Operation.

After another CM has received InfoAcquiring_Request message from the CM, another CM shall generate an InfoAcquiring_Response message, and shall send the InfoAcquiring_Response message to the CM. The message includes required information value as defined in clause of 5.4 Data Types. CM operation related to generating and sending a InfoAcquiring_response message is specified in clause 7 CM Operation.

After the CM has received the InfoAcquiring_response message from another CM, the CM shall process the InfoAcquiring_response message. CM operation related to processing the InfoAcquiring_response message is specified in clause 7 CM Operation.

## Editing instructions:

TG1 Editor:

- Substitute blued paragraph in sub-clause 5.2.7.2