IEEE P802.19
TV Whitespace Coexistence Method

| Comment Resolutions regarding to 5.2.11.2 sending reconfiguration request from CM to another CM |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-09-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Donghun Lee | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | mmdang@etri.re.kr |
| Hyunduk Kang | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | henry@etri.re.kr |
| Byung-Jang Jeong | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | bjjeong@etri.re.kr |

Abstract

This document provides proposed resolutions regarding to sending reconfiguration request from CM to another CM in the Draft P802.19.1/DF2.08.

Discussion:

Sending reconfiguration request from CM to another CM procedure is not well described. Then, we propose the revised paragraph in 5.2.11.2.

Proposal

      - Sending reconfiguration request from CM to another CM procedure

A CM shall perform the sending reconfiguration request from CM another CM procedure when a CM has made a coexistence decision that requires reconfiguration of the WSO registered in another CM. An illustrative example of this procedure is shown in Figure 43. CM_Reconfiguration_Request and CM_Reconfiguration_Response messages are defined in clause 5.3 Messages.

- Figure 43 - Sending reconfiguration request from CM to another CM

After making a coexistence decision of the WSO registered to another CM, the CM shall generate a CM_Reconfiguration_Request message and shall send this CM_Reconfiguration_Request message to another CM. The message includes reconfiguration parameters and channel classification information, as defined in clause of 5.4 Data Types. CM operation related to generating and sending a CM_Reconfiguration_Request message is specified in clause 7 CM Operation.

After another CM has received CM_Reconfiguration_Request message from the CM, another CM shall perform WSO reconfiguration using the reconfiguration information, and shall send the CM_Reconfiguration_Response message to the CM. The message includes status and failed parameters, as defined in clause of 5.4 Data Types. CM operation related to generating and sending a CM_Reconfiguration_Response message is specified in clause 7 CM Operation.

## Editing instructions:

TG1 Editor:

- Add blued paragraph in sub-clause 5.2.11.2