IEEE P802.19
TV Whitespace Coexistence Method

| Comment Resolutions regarding to 5.2.12.2 sending event indication from CM to another CM |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-09-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Donghun Lee | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | mmdang@etri.re.kr |
| Hyunduk Kang | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | henry@etri.re.kr |
| Byung-Jang Jeong | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | bjjeong@etri.re.kr |

Abstract

This document provides proposed resolutions regarding to sending event indication from CM to another CM in the Draft P802.19.1/DF2.08.

Discussion:

Sending event indication from CM to another CM procedure is not well described. Then, we propose the revised paragraph in 5.2.12.2.

Proposal

      - Sending event indication from CM to another CM procedure

A CM shall perform the sending event indication from CM another CM procedure when a CM wants to send an event indication to another CM. An illustrative example of this procedure is shown in Figure 46. Event_Indication and Event_Confirm messages are defined in clause 5.3 Messages.

- Figure 46 - Sending event indication from CM to another CM

When a CM wants to send an event indication to another CM, it shall generate an Event_Indication message and shall send this Event_Indication message to another CM. The message includes event parameters as defined in clause of 5.4 Data Types. CM operation related to generating and sending an Event_Indication message is specified in clause 7 CM Operation.

After another CM has received Event_Indication message from the CM, another CM shall send the Event_Confirm message to the CM as an acknowledgement. The message does not include any parameters. CM operation related to generating and sending an Event_Confirm message is specified in clause 7 CM Operation.

After the CM has received the Event_Confirm message from another CM, the CM shall process the Event_Confirm message. CM operation related to processing the Event_Confirm message is specified in clause 7 CM Operation.

## Editing instructions:

TG1 Editor:

- Substituteblued paragraph in sub-clause 5.2.12.2