IEEE P802.19
Wireless Coexistence

| Comment Resolution regarding to 5.2.2.3 CM subscription procedure |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-09-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Hyunduk Kang | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | henry@etri.re.kr |
| Donghun Lee | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | mmdang@etri.re.kr |
| Byung-Jang Jeong | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | bjjeong@etri.re.kr |

Abstract

This document provides proposed resolutions regarding to 5.2.2.3 CM subscription procedure in the Draft P802.19.1/DF2.08.

Comment

CM subscription procedure in DF2.08 needs more description.

Propose

Replace the current sub-clause 5.2.2.3 CM subscription procedure in DF2.08 with the followings:

- 5.2.2.3 CM subscription procedure

A CM shall perform this procedure in order to subscribe to discovery service provided by the CDIS. The CM shall subscribe to either inter-CM coexistence discovery or full coexistence discovery. An illustrative example of this procedure is shown in Figure 1. SubscrptionRequest and SubscrptionResponse messages are defined in clause 5.3 Messages.

- Figure 1 ・ CM subscription procedure

After generating the CM subscription information, the CM shall send the SubscrptionRequest message to the CDIS. CM operation related to generating and sending the SubscrptionRequest message is specified in clause 7 CM Operation.

After the CDIS receives the SubscrptionRequest message from the CM, the CDIS shall perform CM subscription, and shall generate the SubscriptionResponse message.

After the CDIS generates the SubscriptionResponse message, the CDIS shall send the SubscriptionResponse message to the CM. CDIS operation related to generating and sending SubscriptionResponse message is specified in clause 8 CDIS Operation.