IEEE P802.19
Wireless Coexistence

| Comment Resolution regarding to 5.2.2.5 WSO subscription change procedure |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-09-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Hyunduk Kang | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | henry@etri.re.kr |
| Donghun Lee | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | mmdang@etri.re.kr |
| Byung-Jang Jeong | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | bjjeong@etri.re.kr |

Abstract

This document provides proposed resolutions regarding to 5.2.2.5 WSO subscription change procedure in the Draft P802.19.1/DF2.08.

Comment

CM subscription procedure in DF2.08 needs more description.

Propose

Replace the current sub-clause 5.2.2.5 WSO subscription change procedure in DF2.08 with the followings:

- 5.2.2.5 WSO subscription change procedure

This procedure, illustrated in Figure 1, is performed when CM wants to ask a WSO to change the type of coexistence service provided by the CM. An illustrative example of this procedure is shown in Figure 1. ChangeSubscrptionRequest and ChangeSubscrptionResponse messages are defined in clause 5.3 Messages.

- Figure 1 ・ WSO subscription change procedure

After the CE receives the ChangeSubscrptionRequest message from the CM, the CE shall generate ChangeSubscrption.request primitive and shall send it to the WSO. CE operation related to generating and sending the ChangeSubscrption.request primitive is specified in clause 6 CE Operation.

After the WSO receives the ChangeSubscrption.request primitive from the CE, the WSO shall determine either acceptance or rejection of the request to change its subscription to the coexistence service by the CM, shall generate the ChangeSubscrption.response primitive, and shall send it to the CE.

After the CE receive the ChangeSubscrption.response primitive from the WSO, the CE shall generate the ChangeSubscrptionResponse message and shall send it to the CM. CM operation related to generating and sending the SubscrptionRequest message is specified in clause 7 CM Operation.