IEEE P802.19
Wireless Coexistence

| Comment Resolution regarding to 5.2.4.1 obtaining coexistence set information procedure |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-09-xx |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Hyunduk Kang | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | henry@etri.re.kr |
| Donghun Lee | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | mmdang@etri.re.kr |
| Byung-Jang Jeong | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | bjjeong@etri.re.kr |

Abstract

This document provides proposed resolutions regarding to 5.2.4.1 obtaining coexistence set information procedure in the Draft P802.19.1/DF2.08.

Comment

Obtaining coexistence set information procedure in DF2.08 needs more description.

Propose

Replace the current sub-clause 5.2.4.1 Obtaining coexistence set information procedure in DF2.08 with the followings:

- 5.2.4.1 Obtaining coexistence set information

A CM shall perform this procedure when the CM needs to obtain coexistence set information from the CDIS. Based on the CM's subscription to the CDIS, CM shall get the information for inter-CM coexistence set elements only or for all coexitence set elements including both intra-CM and inter-CM ones. An illustrative example of this procedure is shown in Figure 1. CoexistenceSetInformationRequest and CoexistenceSetInformationResponse messages are defined in clause 5.3 Messages.

- Figure 1 ・Obtaining coexistence set information

CM operation related to generating and sending the CoexistenceSetInformationRequest message is specified in clause 7 CM Operation.

After a CDIS receives the CoexistenceSetInformationRequest message from the CM, the CDIS shall obtain coexistence set information, and shall send the CoexistenceSetInformationResponse message to the CM. CDIS operation related to generating and sending CoexistenceSetInformationResponse message is specified in clause 8 CDIS Operation.

After the CM has received the CoexistenceSetInformationResponse message from the CDIS, the CM shall process the CoexistenceSetInformationResponse message. CM operation related to processing the CoexistenceSetInformationResponse message is specified in clause 7 CM Operation.