IEEE P802.19
Wireless Coexistence

| Comment Resolution regarding to 5.2.6 obtaining channel classification information procedure |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-09-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Hyunduk Kang | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | henry@etri.re.kr |
| Donghun Lee | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | mmdang@etri.re.kr |
| Byung-Jang Jeong | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | bjjeong@etri.re.kr |

Abstract

This document provides proposed resolutions regarding to 5.2.6 obtaining channel classification information procedure in the Draft P802.19.1/DF2.08.

Comment

Obtaining channel classification information procedure in DF2.08 needs more description.

Propose

Replace the current sub-clause 5.2.6 Obtaining channel classification information procedure (NOT including sub-clause from 5.2.6.1 to 5.2.6.4) in DF2.08 with the followings:

    - Obtaing channel classification information procedure

A CM or CE shall perform this procedure to maintain channel classification information. All TV channels shall be classified in the following sets:

- disallowed channels,

- allowed channels,

- available channels,

- protected channels,

- restricted channels,

- unclassified channels,

- operating channels, and

- coexistent channels.

The disallowed set shall be a set of channels disallowed for any WSOs by regulation, or by the request of incumbents. It is provided by TVWS database and will be updated if necessary. For example, in the U.S., the TV channels 3, 4, and 37 are disallowed by regulation. Also a TV channel registered in the TVWS database by a licensed wireless microphone is disallowed for any WSOs.

The allowed set shall be a set of channels allowed for WSOs. It is provided by TVWS database and is updated as necessary. All WSOs first obtain a list of the allowed channels before their operating over TV channels.

The available set shall be a set of free channels available for WSOs.

The restricted set shall be a set of channels restricted to use with limitations due to regulation. It is be used by WSOs under limited conditions predefined by regulation. For example, in the U.S. a portable/personal TVBD can use “the first adjacent channel of the incumbent activating channel” with limited transmit power (less than 40 mW EIRP) by FCC regulation.

The protected set shall be a set of channels to be protected due to incumbent activity. It cannot be used by any WSOs.

The unclassified set shall be a set of channels has not been classified as one of disallowerd set, available set, or protected set.

The operating set shall be a set of operating channels being used by each WSO. If each WSO has a different operating channel, then spectrum etiquette, i.e., FDM (frequency division multiplexing), among WSOs is achievable.

The coexistent set shall be a set of channels being shared by two or more WSOs as an operating channel. WSOs might need a coexistence mechanism to resolve co-channel interference among them.

shows Venn diagram of TV channels.

- Figure 1  Venn diagram of TV channels

Channel classification information for this procedure shall include classified channel lists based on channel status: available, restricted, protected, unclassified, operating, and coexistent.