IEEE P802.19
Wireless Coexistence

| Comment Resolution regarding to 5.2.6.1 obtaining channel classification information by CM procedure |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-09-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Hyunduk Kang | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | henry@etri.re.kr |
| Donghun Lee | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | mmdang@etri.re.kr |
| Byung-Jang Jeong | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | bjjeong@etri.re.kr |

Abstract

This document provides proposed resolutions regarding to 5.2.6.1 obtaining channel classification information by CM procedure in the Draft P802.19.1/DF2.08.

Comment

Obtaining channel classification information by CM procedure in DF2.08 needs more description.

Propose

Replace the current sub-clause 5.2.6.1 obtaining channel classification information by CM procedure in DF2.08 with the followings:

      - Obtaing channel classification information by CM procedure

A CM shall perform this procedure when it needs to obtain channel classification information from another CM. Channel classification information shall include classified channel lists based on channel status: available, restricted, protected, unclassified, operating, and coexistent. Definition on each channel specified in clause 3.6.2.2. An illustrative example of this procedure is shown in . CMChannelClassificationRequest and CMChannelClassificationResponse messages are defined in clause 5.3 Messages.

After a CM sends the CMChannelClassificationRequest message to another CM. Another CM shall obtain channel classification information based on its registred WSOs, and shall send the CMChannelClassificationResponse message to the CM. CM operation related to generating and sending CMChannelClassificationRequest and CMChannelClassificationResponse messages are specified in clause 7 CM Operation.

After the CM has received the CMChannelClassificationResponse message from another CM, the CM shall process the CMChannelClassificationResponse message. CM operation related to processing the CMChannelClassificationResponse message is specified in clause 7 CM Operation.

- Figure 2・Obtaining channel classification information by CM procedure