IEEE P802.19
Wireless Coexistence

| Comment Resolution regarding to 5.2.6.2 obtaining channel classification information by CE procedure |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-09-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Hyunduk Kang | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | henry@etri.re.kr |
| Donghun Lee | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | mmdang@etri.re.kr |
| Byung-Jang Jeong | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | bjjeong@etri.re.kr |

Abstract

This document provides proposed resolutions regarding to 5.2.6.2 obtaining channel classification information by CE procedure in the Draft P802.19.1/DF2.08.

Comment

Obtaining channel classification information by CE procedure in DF2.08 needs more description.

Propose

Replace the current sub-clause 5.2.6.2 obtaining channel classification information by CE procedure in DF2.08 with the followings:

      - Obtaing channel classification information by CE procedure

A CE shall perform this procedure when the WSO needs to obtain channel classification information from a CM via a CE. An illustrative example of this procedure is shown in Figure 3. ChannelClassificationRequest and ChannelClassificationResponse messages are defined in clause 5.3 Messages.

After the CE receives the ChannelClassification.request primitive from the WSO, the CE shall send the ChannelClassificationRequest message to the CM.

After the CM receives the ChannelClassificationRequest message from the CM, the CM shall obtain channel classification information and shall send the ChannelClassificationResponse message to the CE. CM operation related to generating and sending ChannelClassificationResponse message is specified in clause 7 CM Operation.

After the CE receives the ChannelClassificationResponse message from the CM, the CE shall send the ChannelClassification.response prmitive to the WSO.

- Figure 3・Obtaining channel classification information by CE procedure