IEEE P802.19
Wireless Coexistence

| Comment Resolution regarding to 5.2.6.4 announcing channel classification information update to CE procedure |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-09-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Hyunduk Kang | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | henry@etri.re.kr |
| Donghun Lee | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | mmdang@etri.re.kr |
| Byung-Jang Jeong | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | bjjeong@etri.re.kr |

Abstract

This document provides proposed resolutions regarding to 5.2.6.4 announcing channel classification information update to CE procedure in the Draft P802.19.1/DF2.08.

Comment

Announcing channel classification information update to CE procedure in DF2.08 needs more description.

Propose

Replace the current sub-clause 5.2.6.4 announcing channel classification information update to CE procedure in DF2.08 with the followings:

      - Announcing channel classification information update to CE procedure

A CM shall perform this procedure when channel classification information has been changed for the WSO. The CM shall send updated channel classification information to the WSO via the CE. An illustrative example of this procedure is shown in Figure 5. The ChannelClassificationAnnouncement message is defined in clause 5.3 Messages.

After channel classification information has been changed for the WSO, the CM shall generate the ChannelClassificationAnnouncement message, and shall send the ChannelClassificationAnnouncement message to the CE. CM operation related to generating and sending ChannelClassificationAnnouncement message is specified in clause 7 CM Operation.

After the CE has received the ChannelClassificationAnnouncement message from the CM, the CE shall generate ChannelClassification.indication primitive and shall send it to the WSO. CE operation related to processing the ChannelClassification.indication primitive is specified in clause 6 CE Operation.

- Figure 5・Announcing channel classification information update to CE procedure