IEEE P802.19
TV Whitespace Coexistence Methods

| Comment Resolutions regarding to CIDs of 61, 62, 63, 64, 65, 69, 130 and 131 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-11-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Donghun Lee | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | mmdang@etri.re.kr |
| Hyunduk Kang | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | henry@etri.re.kr |
| Byung-Jang Jeong | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | bjjeong@etri.re.kr |

Abstract

This document provides proposed resolutions for comment 61, 62, 63, 64, 65, 69, 130 and 131 received in the 802.19.1 LB.

| CID | Comment | Category | Page Number | Subclasue | Line Number | Proposed change |
| --- | --- | --- | --- | --- | --- | --- |
| 61 | The payload of transportPref and its data type in CP_PACKET_RECEIVE.indication needs consistency with other primitives | Technical | 17 | 5.2.1.3 | 11 | Replace transportPref and TransportPref with transportPreference and TransportType, respectively |

Discussion:

CID 61 pointed out that the payload of transportPref and its data type in CP_PACKET_RECEIVE.indication needs consistency with other primitives such as CP_PACKET_SEND.request and CP_PACKET_SEND.confirm. Then, our proposal is given as follows

Proposal (Revision is marked by blue color)

- 5.2.1.3 CP_PACKET_RECEIVE.indication

- CP_PACKET_SEND.indication (

- transportPreference,

- sourceID,

- coexProtocolPDU

- )

Table 5 –CP_PACKET_RECEIVE.indication primitive parameters

| - Name | - Data type | - Description |
| --- | --- | --- |
| - transportPreference | - TransportType | - Transport protocol used |
| - … |  |  |

## Editing instructions:

TG1 Editor:

- Substitute blued color

| CID | Comment | Category | Page Number | Subclasue | Line Number | Proposed change |
| --- | --- | --- | --- | --- | --- | --- |
| 62 | Since TV channel mode needs the current occupancy and current Tx power, these data types should be added at datatype of listOfOperatingChNumber in GetRegInfo.response | Technical | 24 | 5.2.2.3.2 | 1 | Revise it as commented |
| 63 | Since TV channel mode needs the current occupancy and current Tx power, these data types should be added at datatype of listOfOperatingChNumber in NewRegInfo.indication | Technical | 26 | 5.2.2.3.4 | 1 | Revise it as commented |

Discussion:

CID 62 and 63 pointed out that the data type of listOfOperatingChNumber in NewRegInfo.indication primitive is missing current Tx power and occupancy. Since this information is also useful in TV channel mode as in operating frequency mode, our proposal is given as follows

Proposal (Revision is marked by blue color)

- Table 15-GetRegInfo.response primitive parameters

| Name | Data type | Description |
| --- | --- | --- |
| … |  |  |
| listOfOperatingResources | CHOICE{ listOfOperatingFrequencies ListOfOperatingFrequencies, listOfOperatingChNumber ListOfOPeratingChNumber | ... |

- Table 17-NewRegInfo.indication primitive parameters

| Name | Data type | Description |
| --- | --- | --- |
| … |  |  |
| listOfOperatingResources | CHOICE{ listOfOperatingFrequencies ListOfOperatingFrequencies, listOfOperatingChNumber ListOfOPeratingChNumber | ... |

- 5.3.2 COEX_MEDIA_SAP

- OperatingChNumber :: = SEQUENCE {

- operatingChNumber	INTEGER,

- occupancy		REAL,

- currentTxPower		REAL

- }

- ListOfOPeratingChNumber ::=SEQUENCE OF OperatingChNumber

Editing instructions:

TG1 Editor:

- Substitute blued color

| CID | Comment | Category | Page Number | Subclasue | Line Number | Proposed change |
| --- | --- | --- | --- | --- | --- | --- |
| 64 | The data type of listOfNetworkID is wrong | Technical | 30 | 5.2.2.4.7 | 1 | Replace it with SEQUENCE OF OCTET STRING |

Discussion:

CID 64 pointed out that the data type of listOfNetworkID in ChannelClassification.request primitive is wrong. This is because NetworkID has been already defined in OCTET STRING, Then our proposal is given as follows

Proposal (Revision is marked by blue color)

- Table 23-ChannelClassification.request primitive parameter

| Name | Data type | Description |
| --- | --- | --- |
| listOfNetworkID | SEQUENCE OF OCTET STRING | Network ID list |

## Editing instructions:

TG1 Editor:

- Substitute blued color

| CID | Comment | Category | Page Number | Subclasue | Line Number | Proposed change |
| --- | --- | --- | --- | --- | --- | --- |
| 65 | The datatype of MeasurementResult is not defined | Technical | 32 | 5.2.2.5.2 | 12 | Add the datatype of MeasurementResult |

Discussion:

CID 65 pointed out that the datatype of MeasurementResult is not defined. Then our proposal for the datatype of MeasurementResult is given as follows

Proposal (Revision is marked by blue color)

- 5.3.2 COEX_MEDIA_SAP

- MeasurementResult :: == SEQUENCE {

- measurementDescription		MeasurementDescription,

- measurementReport			MeasurementReport

- }

## Editing instructions:

TG1 Editor:

- Substitute blued color

| CID | Comment | Category | Page Number | Subclasue | Line Number | Proposed change |
| --- | --- | --- | --- | --- | --- | --- |
| 69 | Since the datatype of ConstOfChUses is defined as sequence, SEQUENCE OF ConstOfChUses is wrong | Technical | 41 | 5.3.2 | 37 | Repliace listOfconstOfChUses and SEQUENCE OF ConstOfChUses with constOfChUses and ConstOfChUses, respectively |
| 130 | Since the datatype of ConstOfChUses is defined as sequence, SEQUENCE OF ConstOfChUses is wrong in AvailableChannelAnnouncement | Technical | 85 | 6.4 | 18 | Repliace listOfconstOfChUses and SEQUENCE OF ConstOfChUses with constOfChUses and ConstOfChUses, respectively |
| 131 | Since the datatype of ConstOfChUses is defined as sequence, SEQUENCE OF ConstOfChUses is wrong in AvailableChannelResponse | Technical | 84 | 6.4 | 44 | Repliace listOfconstOfChUses and SEQUENCE OF ConstOfChUses with constOfChUses and ConstOfChUses, respectively |

Discussion:

CID 69, 130 and 131 pointed out that since the datatype of ConstOfChUses is defined as sequence, SEQUENCE OF ConstOfChUses is wrong. Then our proposal for the datatype of ConstOfChUses is given as follows

Proposal (Revision is marked by blue color)

- 5.3.2 COEX_MEDIA_SAP

- listOfAvailableResources ::=CHOICE {

- …

- chNumbers	SEQUENCE{

- …

- listOfconstOfChUses	ConstOfChUses

- …

- }

- 6.4 Messages

- AvailableChannelsResponse ::=SEQUENCE {

- …

- listOfAvailableResources CHOICE {

- …

- listOfconstOfChUses	ConstOfChUses

- …

- }

- }

- AvailableChannelsAnnouncement ::=SEQUENCE {

- …

- listOfAvailableResources CHOICE {

- …

- listOfconstOfChUses	ConstOfChUses

- …

- }

- }

## Editing instructions:

TG1 Editor:

- Substitute blued color