IEEE P802.19
TV Whitespace Coexistence Method

| CE Operation for profile E |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-03-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Hyunduk Kang | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | henry@etri.re.kr |
| Donghun Lee | ETRI |  |  | mmdang@etri.re.kr |
| Byung-Jang Jeong | ETRI |  |  | bjjeong@etri.re.kr |
| Myung-Sun Song | ETRI |  |  | mssong@etri.re.kr |
| Jae-Ick Choi | ETRI |  |  | jichoi@etri.re.kr |

Abstract

This document provides proposed CE operation for profile E on entities operation.

Discussion:

This document incudes a proposal of CE operation for Profile E. Related procedures, messages, primitives, and data types are also represented.

Proposal

# Entities Operation

# CE Operation for Profile E

# General description

The entity operation for the CE shall consist of the following 5 states:

- Inactive

- Active

- Waiting engagement

- Engaged

- Request sent

- Request received

Each state shall represent as followings:

- Inactive: the state where it is impossible to set up any interfaces with other 802.19.1 entities. The inactive state goes to the active state when initialization process is finished.

- Active: the state where it is possible to set up interface with other 802.19.1 entities. The Active state goes to the inactive state when shutdown process is finished.

- Waiting engagement: the state where one entity has requested for interface setup to the other entity, and is waiting for response from it.

- Engaged: the state where two entities are ready to communicate with each other by exchanging messages or primitives.

- Request sent: the state where the request has been sent from home entity to remote entity, and the home entity is waiting for response from the remote entity.

- Request received: the state where the request has been received from remote entity to home entity, and home entity is waiting for the completion of the request.

State transition shall be occurred as follows:

- The active state goes to the waiting engagement state if CE sends connection request to CM.

- The waiting engagement state goes to the engaged state if CE receives connection response from CM within time limit. If not, it goes back to the active state.

- The engaged state goes to the request received state if CE receives request from WSO/CM. After CE sends corresponding response to WSO/CM it goes back to the engaged state.

- CE sends corresponding event response to CM at the engaged state without receiving event request from CM if CM received the event request from CM in advance.

- The engaged state goes to the request sent state if CE sends request to WSO/CM. After CE receives corresponding response from WSO/CM it goes back to the engaged state.

- CE receives corresponding event response from WSO at the engaged state without sending event request to WSO if CE sent the event request to WSO in advance.

- CE stays the engaged state if it periodically sends being-engagement request to CM.

- The engaged state goes to the active state if CM sends disconnection request to CE.

Flow chart of CE operation is depicted the following figure.

After receiving a request to start operation, a CE shall perform WSO authentication, subscription, and registration procedure in sequence.

After successfully done that, the CE shall switch to the engaged state, where it shall check for incoming messages or primitives or for request to stop operation.

If the CE received event indication from WSO, CE shall perform sending event indication from CE to CM procedure and back to the engaged state.

If the WSO subscribed for information service and the CE received coexistence report request from the WSO, the CE shall perform obtaining coexistence report procedure and back to the engaged state.

If the WSO subscribed for management service and the CE received reconfiguration request form the CM, CE shall perform sending reconfiguration request from CM to CE procedure and back to the engaged state.

If the CE received updated registration information from WSO, CE shall perform WSO registration update procedure and back to the engaged state.

If the CE received channel classification request from WSO, CE shall perform obtaining channel classification information by CE procedure and back to the engaged state.

If the CE received updated channel classification of WSO, CE shall perform announcing channel classification information update to CE procedure and back to the engaged state.

If the CE received available channel list request from CM, CE shall perform obtaining available channel list from WSO procedure and back to the engaged state.

If the CE received measurement request from CM, CE shall perform requesting measurement procedure and back to the engaged state.

If the CE received measurement result from WSO and the CM requested one-time measurement, CE shall perform obtaining one-time measurement procedure and back to the engaged state.

If the CE received measurement result from WSO and the CM requested scheduled measurement, CE shall perform obtaining scheduled measurement procedure and back to the engaged state.

If the CE received subscription update request from WSO, CE shall perform WSO subscription update procedure and back to the engaged state.

If the CE received subscription change request from CM, CE shall perform WSO subscription change procedure and back to the engaged state.

If the CE received a request to stop operation, a CE shall perform WSO registration, subscription, and deauthentication procedure in sequence.

After successfully done that, the CE shall switch to the active state.

Related procedures, messages, primitives, and data types are listed in the following subsections.

# WSO Authentication

To authenticate WSO to CM, a CE shall perform the WSO authentication procedure as shown in reference.

First the CE shall generate and send the GetAuthInfoRequest primitive to the served WSO and shall wait for the GetGetAuthInfoResponse primitive from the WSOs.

Table below shows the parameters in the GetAuthInfoResponse primitive.

| Parameter | Data type | Description |
| --- | --- | --- |
| clientID | IA5String | This parameter contains Client ID. |
| clientPassword | IA5String | This parameter contains Client Password. |
| cmID | CxID | CM ID is the identifier of the CM to which the CE gets connected. |
| serverID | IA5String | Server ID to be used by the CM to authenticate the CE |
| serverPassword | IA5String | Server password to be used by the CM to authenticate the CE |
| cmIPAddress | OCTET STRING | Server IP address for communication with CM server |
| cmPortNumber | INTEGER | Server port number for communication with CM server |
| status | CxMediaStatus | Result code |

After the CE has received the GetAuthInfo Response primitive from the WSO, the CE shall generate and send the AuthenticationRequest message to CM and wait for the AuthenticationResponse message from the CM.

When generating the AuthenticationRequest message, the CE shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | authenticationRequest |

Table below shows the parameters in the authenticationRequest payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| clientID | IA5String | The clientID parameter received from the GetAuthInfo primitive. |
| clientPassword | IA5String | The clientPassword parameter received from the GetAuthInfo primitive. |

Table below shows the parameters of the CxMessage in the AuthenticationResponse message.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | authenticationResponse |

Table below shows the parameters in the authenticationResponse payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| serverID | IA5String | Server ID to be used by the CM to authenticate the CE |
| serverPassword | IA5String | Server password to be used by the CM to authenticate the CE |
| status | CxMediaStatus | status |

After the CE has received the AuthenticationResponse message from the CM, the CE shall generate and send the GetAuthInfoConfirm primitive to the WSOs.

The CE shall set the parameters of the GetAuthInfoConfirm primitive as shown in the table below.

| Parameter | Data type | Description |
| --- | --- | --- |
| status | CxMediaStatus | This parameter shows whether the authentication was successful or not. |

# WSO Subscription

To subscribe coexistence service, a CE shall perform the WSO subscription procedure as shown in reference.

First the CE shall generate and send the GetServiceSubscriptionRequest primitive to the served WSO and shall wait for the GetServiceSubscriptionResponse primitive from the WSOs.

Table below shows the parameters in the GetServiceSubscriptionResponse primitive.

| Parameter | Data type | Description |
| --- | --- | --- |
| subscribedService | SubscribedService | WSO subscription to coexistence service (management or information). |
| status | CxMediaStatus | status |

After the CE has received the GetServiceSubscriptionResponse primitive from the WSO, the CE shall generate and send the SubscriptionRequest message to CM and wait for the SubscriptionResponse message from the CM.

When generating the SubscriptionRequest message, the CE shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | subscriptionRequest |

Table below shows the parameters in the subscriptionRequest payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| subscribedService | SubscribedService | The subscribedService parameter received from the GetServiceSubscriptionResponse primitive. |

Table below shows the parameters of the CxMessage in the SubscriptionResponse message.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | subscriptionResponse |

Table below shows the parameters in the subscriptionResponse payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| status | CxMediaStatus | status |

After the CE has received the SubscriptionResponse message from the CM, the CE shall generate and send the GetServiceSubscriptionConfirm primitive to the WSO.

The CE shall set the parameters of the GetServiceSubscriptionConfirm primitive as shown in the table below.

| Parameter | Data type | Description |
| --- | --- | --- |
| status | CxMediaStatus | This parameter shows whether the subscription was successful or not. |

# WSO Subscription Update

To be subscription update, a CE shall perform the WSO subscription update procedure as shown in reference.

After the CE has received the NewServiceSubscriptionIndication primitive from the WSOs, the CE shall generate and send the SubscriptionRequest message to the CM that serves this CE and wait for the SubscriptionResponse message from the CM.

Table below shows the parameters in the NewServiceSubscriptionIndication primitive.

| Parameter | Data type | Description |
| --- | --- | --- |
| subscribedService | SubscribedService | WSO subscription to coexistence service (management or information). |

When generating the SubscriptionRequest message, the CE shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | subscriptionRequest |

Table below shows the parameters in the subscriptionRequest payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| subscribedService | SubscribedService | The subscribedService parameter received from the NewServiceSubscriptionIndication primitive. |

Table below shows the parameters of the CxMessage in the SubscriptionResponse message.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | subscriptionResponse |

Table below shows the parameters in the subscriptionResponse payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| status | CxMediaStatus | status |

After the CE has received the SubscriptionResponse message from the CM, the CE shall generate and send the GetServiceSubscriptionConfirm primitive to the WSO.

The CE shall set the parameters of the GetServiceSubscriptionConfirm primitive as shown in the table below.

| Parameter | Data type | Description |
| --- | --- | --- |
| status | CxMediaStatus | This parameter shows whether the subscription was successful or not. |

# WSO Subscription Change

To be subscription change, a CE shall perform the WSO subscription change procedure as shown in reference.

After the CE has received the SubscriptionChangeRequest message from CM, the CE shall generate and send the ChangeSubscriptionRequest primitive to a WSO and wait for the ChangeSubscriptionResponse primitive from the WSO.

Table below shows the parameters of the CxMessage in  the SubscriptionChangeRequest message.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | subscriptionChangeRequest |

Table below shows the parameters in the subscriptionChangeRequest payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| newSubscribedService | SubscribedService | Proposed new subscribed coexistence service (management or information). |

The CE shall set the parameters of the ChangeSubscriptionRequest primitive as shown in the table below.

| Parameter | Data type | Description |
| --- | --- | --- |
| newSubscribedService | SubscribedService | The subscribedService parameter received from the SubscriptionChangeRequest message. |

After the CE has received the ChangeSubscriptionResponse primitive from the WSO, the CE shall generate and send the SubscriptionChangeResponse message to the CM.

Table below shows the parameters in the ChangeSubscriptionResponse primitive.

| Parameter | Data type | Description |
| --- | --- | --- |
| status | CxMediaStatus | This parameter shows whether the CE accepted the subscription change request. |

When generating the SubscriptionChangeResponse message, the CE shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | subscriptionChangeResponse |

Table below shows the parameters in the subscriptionChangeResponse payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| status | CxMediaStatus | status |

# WSO Registration

To register WSO to CM, a CE shall perform the WSO registration procedure as shown in reference.

First the CE shall generate and send the GetRegInfoRequest primitive to the served WSO and shall wait for the GetRegInfoResponse primitive from the WSO.

Table below shows the parameters in the GetRegInfoResponse primitive.

| Parameter | Data type | Description |
| --- | --- | --- |
| networkID | OCTET STRING | Identifier of the network which the WSO represents. As an example, in case of IEEE 802.11, this parameter contains the BSSID used by the WSO. |
| networkTechnology | NetworkTechnology | Indicates the radio access technology which the WSO uses. |
| networkType | NetworkType | Indicates network type as specified in regulations. |
| discoveryInformation | DiscoveryInformation | Discovery information of the WSO. |
| txScheduleSupported | BOOLEAN | Indicates whether scheduled transmission is supported or not |
| listOfAvailableChNumbers | ListOfAvailableChNumbers | Information about available white space channels (see table below). |
| listOfSupportedChNumbers | SEQUENCE OF INTEGER | List of supported channel number. |
| listOfOperatingChNumbers | ListOfOperatingChNumbers | The WSO operating channel numbers (see table below). |
| requiredResource | RequiredResource | Resource required for the WSO operation (see table below). |
| measurementCapability | MeasurementCapability | Measurement capability of the WSO (Energy detection or feature detection) |

Table below shows the data type of the each element of the sequence in the listOfAvailableChNumbers parameter.

| Parameter | Data type | Description |
| --- | --- | --- |
| channelNumber | INTEGER | Available channel number |
| availableStartTime | GeneralizedTime | Available start time of the available channel number if applicable. |
| availableDuration | REAL | Available duration of the available channel number if applicable. |
| constOfChUses | ConstOfChUses | If constraint is not known, this parameter is not used. If they are know, this parameter is used. |

Table below shows the data type of the each element of the sequence in the listOfOperatingChNumbers parameter.

| Parameter | Data type | Description |
| --- | --- | --- |
| ChannelNumber | INTEGER | operating channel number |
| occupancy | REAL | If occupancy is not known, this parameter is not used. If it is known, this parameter is set to its value. Range is from 0 to 1. |

Table below shows the data type of the requiredResource parameter.

| Parameter | Data type | Description |
| --- | --- | --- |
| requiredBandwidth | REAL | Required bandwidth for the WSO operation. |
| occupancy | REAL | If expected occupancy is not known, this parameter is not used. If it is known, this parameter is set to its value. Range is from 0 to 1. |

After the CE has received the GetRegInfoResponse primitive from the WSO, the CE shall generate and send the CERegistrationRequest message to the CM that serves this CE and wait for the RegistrationResponse message from the CM.

When generating the CERegistrationRequest message, the CE shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | ceRegistrationRequest |

The CE shall set the parameters in the ceRegistrationRequest payload as shown in the table below.

| Parameter | Data type | Description |
| --- | --- | --- |
| operationCode | OperationCode | Operation code (new, modify, remove) |
| networkID | OCTET STRING | Identifier of the network which the WSO represents. As an example, in case of IEEE 802.11, this parameter contains the BSSID used by the WSO. |
| networkTechnology | NetworkTechnology | Indicates the radio access technology which the WSO uses. |
| networkType | NetworkType | Indicates network type as specified in regulations. |
| discoveryInformation | DiscoveryInformation | Discovery information of the WSO. |
| txScheduleSupported | BOOLEAN | Indicates whether scheduled transmission is supported or not |
| listOfAvailableChNumbers | ListOfAvailableChNumbers | Information about available white space channels (see table below). |
| listOfSupportedChNumbers | SEQUENCE OF INTEGER | List of supported channel number. |
| listOfOperatingChNumbers | ListOfOperatingChNumbers | The WSO operating channel numbers (see table below). |
| requiredResource | RequiredResource | Resource required for the WSO operation (see table below). |
| measurementCapability | MeasurementCapability | Measurement capability of the WSO (Energy detection or feature detection) |

Table below shows the parameters of the CxMessage in the RegistrationResponse message.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | registrationResponse |

After the CE has received the RegistrationResponse message from the CM, the CE shall generate and send the GetRegInfoConfirm primitive to the WSO.

The CE shall set the parameters of the GetRegInfoConfirm primitive as shown in the table below.

| Parameter | Data type | Description |
| --- | --- | --- |
| status | CxMediaStatus | This parameter shows whether the registration was successful or not. |

# WSO Registration Update

To be registration update, a CE shall perform the WSO registration update procedure as shown in reference.

After the CE has received the NewRegInfoIndication primitive from the WSOs, the CE shall generate and send the CERegistrationRequest message to the CM that serves this CE and wait for the RegistrationResponse message from the CM.

Table below shows the parameters in the NewRegInfoIndication primitive.

| Parameter | Data type | Description |
| --- | --- | --- |
| operationCode | OperationCode | Operation code (new, modify, remove) |
| networkID | OCTET STRING | Identifier of the network which the WSO represents. As an example, in case of IEEE 802.11, this parameter contains the BSSID used by the WSO. |
| networkTechnology | NetworkTechnology | Indicates the radio access technology which the WSO uses. |
| networkType | NetworkType | Indicates network type as specified in regulations. |
| discoveryInformation | DiscoveryInformation | Discovery information of the WSO. |
| txScheduleSupported | BOOLEAN | Indicates whether scheduled transmission is supported or not |
| listOfAvailableChNumbers | ListOfAvailableChNumbers | Information about available white space channels (see table below). |
| listOfSupportedChNumbers | SEQUENCE OF INTEGER | List of supported channel number. |
| listOfOperatingChNumbers | ListOfOperatingChNumbers | The WSO operating channel numbers (see table below). |
| requiredResource | RequiredResource | Resource required for the WSO operation (see table below). |
| measurementCapability | MeasurementCapability | Measurement capability of the WSO (Energy detection or feature detection) |

Table below shows the parameters of the CxMessage in the RegistrationResponse message.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | RegistrationResponse |

After the CE has received the RegistrationResponse message from the CM, the CE shall generate and send the NewRegInfoConfirm primitive to the WSO.

The CE shall set the parameters of the NewRegInfoConfirm primitive as shown in the table below.

| Parameter | Data type | Description |
| --- | --- | --- |
| status | CxMediaStatus | This parameter shows whether the registration was successful or not. |

# Coexistence Report for Information Service

To be coexistence report, a CE shall perform the obtaining coexistence report procedure as shown in reference.

After the CE has received the CoexistenceReportRequest primitive from the WSO, the CE shall generate and send the CoexistenceReportRequest message to the CM that serves this CE and wait for the CoexistenceReportResponse message from the CM.

When generating the CoexistenceReportRequest message, the CE shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | coexistenceReportRequest |

Table below shows the parameters of the CxMessage in the CoexistenceReportResponse message.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | coexistenceReportResponse |

Table below shows the parameters in the coexistenceReportResponse payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| coexistenceReport | CoexistenceReport | Information about coexistence set elements of the WSO (see below) |
| channelPriority | ChannelPriority | Channel priority (see below) |

Table below shows the data type of the each element of the sequence in the CoexistenceReport parameter.

| Parameter | Data type | Description |
| --- | --- | --- |
| networkID | OCTET STRING | Neighbor network ID. |
| networkTechnology | NetworkTechnology | This parameter shall be set to the neighbor WSO network technology. |
| listOfOperatingChNumbers | ListOfOperatingChNumbers | The neighbor operating channel numbers. |

Table below shows the data type of the each element of the sequence in the ChannelPriority parameter.

| Parameter | Data type | Description |
| --- | --- | --- |
| channelNumber | INTEGER | Channel number. |
| priority | INTEGER | Channel priority order. |

After the CE has received the CoexistenceReportResponse message from the CM, the CE shall generate and send the CoexistenceReportResponse primitive to the WSO.

The CE shall set the parameters of the CoexistenceReportResponse primitive as shown in the table below.

| Parameter | Data type | Description |
| --- | --- | --- |
| coexistenceReport | CoexistenceReport | Coexistence report information received from the CoexistenceReportResponse message. |
| channelPriority | ChannelPriority | Channel priority received from the CoexistenceReportResponse message.. |

# WSO Reconfiguration for Management Service

To reconfigure WSO, a CE shall perform the WSO reconfiguration procedure as shown in reference.

After the CE has received the ReconfigurationRequest message from the CM, the CE shall generate and send the PerformReconfigurationRequest primitive to the WSO and shall wait for the PerformReconfigurationResponse primitive from the WSO.

Table below shows the parameters of the CxMessage in the ReconfigurationRequest message.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | reconfigurationRequest |

Table below shows the parameters in the reconfigurationRequest payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| OperatingChNumbers | SEQUENCE OF INTEGER | Operating channel list for the WSO. |
| txPowerLimit | REAL | Transmission power limitation |
| channelIsShared | BOOLEAN | TRUE, if the frequency will be shared with other WSOs, FALSE otherwise. |
| txSchedule | TxSchedule | Transmission schedule |
| chClassInfo | ChClassInfo | Channel classification information (see below) |

Table below shows the data type of the chClassInfo parameter.

| Parameter | Data type | Description |
| --- | --- | --- |
| availableChannelList | SEQUENCE OF INTEGER | Available channel list |
| restrictedChannelList | SEQUENCE OF INTEGER | Restricted channel list |
| protectedChannelList | SEQUENCE OF INTEGER | Protected channel list |
| unclassifiedChannelList | SEQUENCE OF INTEGER | Unclassified channel list |
| operatingChannelList | SEQUENCE OF OperatingChannelInfo | Operating channel list |
| coexistenceChannelList | SEQUENCE OF OperatingChannelInfo | Coexistence channel list |

The CE shall set the parameters of the PerformReconfigurationRequest primitive as shown in the table below.

| Parameter | Data type | Description |
| --- | --- | --- |
| listOfOperatingChNumber | SEQUENCE OF INTEGER | listOfOperatingChNumber parameter received from the ReconfigurationRequest message. |
| txPowerLimit | REAL | txPowerLimit parameter received from the ReconfigurationRequest message. |
| channelIsShared | BOOLEAN | channelIsShared parameter received from the ReconfigurationRequest message. |
| txSchedule | TxSchedule | txSchedule parameter received from the ReconfigurationRequest message. |

After the CE has received the PerformReconfigurationResponse primitive from the WSO, the CE shall generate and send the ReconfigurationResponse message to its serving CM.

Table below shows the parameters of the PerformReconfigurationResponse primitive.

| Parameter | Data type | Description |
| --- | --- | --- |
| status | BOOLEAN | Reconfiguration parameter. |
| failedParameters | FailedParameters | Failed parameters. |

The CE shall set the parameters of the each element of the sequence in the ReconfigurationResponse message as shown in the table below.

When generating the ReconfigurationResponse message, the CE shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | reconfigurationResponse |

Table below shows the parameters in the reconfigurationResponse payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| status | BOOLEAN | Reconfiguration parameter received from the PerformReconfigurationResponse primitive. |
| failedParameters | FailedParameters | Failed parameters received from the PerformReconfigurationResponse primitive. |

# Channel Classification Requst by CE

To provide channel classification for WSO, a CE shall perform the obtaining channel classification information by CE procedure as shown in reference.

After the CE has received the ChannelClassificationRequest primitive from the WSOs, the CE shall generate and send the ChannelClassificationRequest message to the CM that serves this CE and wait for the ChannelClassificationResponse message from the CM.

Table below shows the parameters of the ChannelClassificationRequest primitive.

| Parameter | Data type | Description |
| --- | --- | --- |
| listOfNetworkID | SEQUENCE OF OCTET STRING | - List of network ID. |

When generating the ChannelClassificationRequest message, the CE shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | channelClassificationRequest |

Table below shows the parameters of the channelClassificationRequest payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| listOfNetworkID | SEQUENCE OF OCTET STRING | - listOfNetworkID parameter received - from the ChannelClassificationRequest - primitive. |

Table below shows the parameters of the CxMessage in the ChannelClassificationResponse message.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | channelClassificationResponse |

Table below shows the parameters of the each element of the sequence in the channelClassificationResponse payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| networkID | OCTET STRING | Network ID. |
| chClassInfo | ChClassInfo | Channel classification information |

After the CE has received the ChannelClassificationResponse message from the CM, the CE shall generate and send the ChannelClassificationResponse primitive to the WSO.

The CE shall set the parameters in the ChannelClassificationResponse primitive as shown in the table below.

| Parameter | Data type | Description |
| --- | --- | --- |
| chClassInfoList | ChClassInfoList | Channel classification information received from the ChannelClassificationResponse message. (see below) |

Table below shows the data type of the each element of the sequence in the chClassInfoList parameter.

| Parameter | Data type | Description |
| --- | --- | --- |
| networkID | OCTET STRING | Network ID |
| chClassInfo | ChClassInfo | Channel classification information |

# Channel Classification Update

To update channel classification, the CM shall perform the announcing channel classification information update to CE procedure, as shown in reference.

After the CE has received the ChannelClassificationAnnouncement message from the CM, the CE shall generate and send the ChannelClassificationIndication primitive to the WSO.

Table below shows the parameters of the CxMessage in the channelClassificationAnnouncement message.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | channelClassificationAnnouncement |

Table below shows the parameters of the each element of the sequence in the channelClassificationAnnouncement payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| networkID | OCTET STRING | Network ID. |
| chClassInfo | ChClassInfo | Channel classification information |

The CE shall set the parameters in the ChannelClassificationIndication primitive as shown in the table below.

| Parameter | Data type | Description |
| --- | --- | --- |
| chClassInfoList | ChClassInfoList | Channel classification information received from the ChannelClassificationAnnouncement message. |

# Available Channel List from WSO

To get available channel list, the CE shall perform the Obtaining available channel list from WSO procedure as shown in reference.

After the CE has received the AvailableChannelsRequest message from its serving CM, the CE shall generate and send the AvailableChannelListRequest primitive to the WSO and wait for the AvailableChannelListResponse primitive from the WSO.

Table below shows the parameters in the AvailableChannelListResponse primitive.

| Parameter | Data type | Description |
| --- | --- | --- |
| listOfAvailableChNumbers | ListOfAvailableChNumbers | List of available channel number from TVWS DB |

After the CE has received the AvailableChannelListResponse primitive from the WSO, the CE shall generate and send the AvailableChannelsResponse message to its serving CM.

When generating the AvailableChannelsResponse message, the CE shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | availableChannelsResponse |

Table below shows the parameters in the availableChannelsResponse payload as shown in the table below.

| Parameter | Data type | Description |
| --- | --- | --- |
| listOfAvailableChNumbers | ListOfAvailableChNumbers | List of available channel number received from the AvailableChannelListResponse primitive. |

# Event Indication

To indicate event, the CE shall perform the sending event indication from CE to CM procedure, as shown in reference.

After the CE has received the EventIndication primitive from the WSO, the CE shall generate and send the EventIndication message to the CM that serves this CE and wait for the EventConfirm message from the CM.

Table below shows the parameters of the EventIndication primitive.

| Parameter | Data type | Description |
| --- | --- | --- |
| eventParams | EventParams | This parameter contains a list of event parameters. |

When generating the EventIndication message, the CE shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | eventIndication |

Table below shows the parameters of the eventIndication payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| eventParams | EventParams | This parameter contains a list of event parameters received from the EventIndication primitive. (SINR threshold reached or QoS degradation) |

After the CM has received the eventIndication message from the CE, the CM shall generate and send the EventConfirm message to the CE.

Table below shows the parameters of the CxMessage in the EventConfirm message.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | eventConfirm |

# Measurement Request

To request measurement, the CE shall perform the requesting measurements procedure, as shown in reference.

After the CE has received the MeasurementRequest message from CM, the CE shall generate and send the MeasurementConfirm message to the CM that serves this CE and also send the GetMeasurementRequest primitive to the WSO.

Table below shows the parameters of the CxMessage in the MeasurementRequest message.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | measurementRequest |

Table below shows the parameters in the measurementRequest payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| measurementDescription | MeasurementDescription | Measurement description. |

The CE shall set the parameters in the GetMeasurementRequest primitive as shown in the table below.

| Parameter | Data type | Description |
| --- | --- | --- |
| measurementDescription | MeasurementDescription | Measurement description received from the MeasurementRequest message. |

# One-Time Measurement

To report measurement results, the CE shall perform the obtaining one-time measurement procedure, as shown in reference.

After the CE has received the GetMeasurementResponse primitive from the WSO, the CE shall generate and send the MeasurementResponse message to the CM that serves this CE and wait for the MeasurementConfirm message from the CM.

Table below shows the parameters of the each element of the sequence in the GetMeasurementResponse primitive.

| Parameter | Data type | Description |
| --- | --- | --- |
| measurementResult | MeasurementResult | Measurement result. |

When generating the MeasurementResponse message, the CE shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | measurementResponse |

Table below shows the parameters of the each element of the sequence in the measurementResponse payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| measurementResult | MeasurementResult | Measurement results received from the MeasurementResult primitive. |

After the CM has received the MeasurementResponse message from the CE, the CM shall generate and send the MeasurementConfirm message to the CE.

Table below shows the parameters of the CxMessage in the MeasurementConfirm message.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | measurementConfirm |

# Scheduled Measurement

To report scheduled measurement results, the CE shall perform the obtaining scheduled measurement procedure, as shown in reference.

After the CE has received the GetMeasurementResponse primitive from the WSOs, the CE shall generate and send the MeasurementResponse message to the CM that serves this CE and wait for the MeasurementConfirm message from the CM.

Table below shows the parameters of the each element of the sequence in the GetMeasurementResponse primitive.

| Parameter | Data type | Description |
| --- | --- | --- |
| measurementResult | MeasurementResult | Measurement result. |

When generating the MeasurementResponse message, the CE shall set the parameters of the each element of the sequence in the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | measurementResponse |

Table below shows the parameters of the each element of the sequence in the measurementResponse payload as shown in the table below.

| Parameter | Data type | Description |
| --- | --- | --- |
| measurementResult | MeasurementResult | Measurement results received from the received MeasurementResult primitive. |

After the CM has received the MeasurementResponse message from the CE, the CM shall generate and send the MeasurementConfirm message to the CE.

# WSO Deauthentication

To deauthenticate WSO, the CE shall perform the WSO deauthentication procedure, as shown in reference.

After the CE has received stop operation request, the CE shall generate and send the DeauthenticationRequest message to the CM that serves this CE and wait the DeauthenticationResponse message from the CM.

When generating the DeauthenticationRequest message, the CE shall set the parameters of the CxMessage as shown in the table below.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | deauthenticationRequest |

Table below shows the parameters of the DeauthenticationRequest message.

| Parameter | Data type | Description |
| --- | --- | --- |
| clientID | IA5String | This parameter contains Client ID |
| clientPassword | IA5String | This parameter contains Client Password |

Table below shows the parameters of the DeauthenticationResponse message.

| Parameter | Data type | Value |
| --- | --- | --- |
| header | CxHeader | requestID |
| payload | CxPayload | deauthenticationResponse |

Table below shows the parameters of the deauthenticationResponse payload.

| Parameter | Data type | Description |
| --- | --- | --- |
| serverID | IA5String | Server ID to be used by the CM to deauthenticate the CE |
| serverPassword | IA5String | Server password to be used by the CM to deauthenticate the CE |
| status | CxMediaStatus | status |

After the CE has received the DeauthenticationResponse message from the CM, the CE shall generate and send the DeauthenticationConfirm primitive to the WSO.

The CE shall set the parameters in the DeauthenticationConfirm primitive as shown in the table below.

| Parameter | Data type | Description |
| --- | --- | --- |
| status | CxMediaStatus | This parameter shows whether the deauthentication was successful or not. |