IEEE P802.19
TV Whitespace Coexistence Method

| Update to messages |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-03-20 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Hyunduk Kang | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | henry@etri.re.kr |
| Donghun Lee | ETRI |  |  | mmdang@etri.re.kr |
| Byung-Jang Jeong | ETRI |  |  | bjjeong@etri.re.kr |
| Myung-Sun Song | ETRI |  |  | mssong@etri.re.kr |
| Jae-Ick Choi | ETRI |  |  | jichoi@etri.re.kr |

Abstract

This document provides update to messages for the Draft P802.19.1/DF3.02.

# Discussion:

This document provides update to messages to add normative annex.

Proposal

Normative Annex Messages Definition

IEEE802191Message DEFINITIONS AUTOMATIC TAGS ::= BEGIN

-----------------------------------------------------------

--Imported data types

-----------------------------------------------------------

--Imported data types

IMPORTS

--Coexistence report

CoexistenceReport,

--Channel priority

ChannelPriority,

--Network technology

NetworkTechnology,

--Network type

NetworkType,

--Discovery information

DiscoveryInformation,

--List of available channel numbers

ListOfAvailableChNumbers,

--List of operating channel numbers

ListOfOperatingChNumbers,

--Required resource

RequiredResource,

--Measurement capability

MeasurementCapability,

--List of neighbor CM

ListOfNeighborCM

-- Operation code

OperationCode,

--Transmission schedule

TxSchedule,

--Failed parameters

FailedParameters,

--Subscribed service

SubscribedService,

--Status

CxMediaStatus,

--Coexistence protocol entity ID

CxID,

--Channel classification information

ChClassInfo,

--Event parameters

EventParams,

--Measurement description

MeasurementDescription,

--Measurement result

MeasurementResult

FROM IEEE802191DataType;

-----------------------------------------------------------

--Message structure, header structure, and payload types

-----------------------------------------------------------

--Message structure

CxMessage ::= SEQUENCE {

-- Message header

header CxHeader,

-- Message payload

payload CxPayload

}

--Header

CxHeader ::= CHOICE {

--For announcement

none NULL,

--For request or single response

requestID INTEGER (0..2147483647),

--For multiple responses

multipleResponse SEQUENCE {

--Original request ID

requestID INTEGER (0..2147483647),

--Response number

sequenceNumber INTEGER (0..2147483647),

--True is the response is last

isLastResponse BOOLEAN}

}

--Payload types

CxPayload ::= CHOICE {

--Authentication request

authenticationRequest    AuthenticationRequest

--Authentication response

authenticationResponse    AuthenticationResponse

--WSO subscription request

subscriptionRequest    SubscriptionRequest,

--WSO subscription response

subscriptionResponse    SubscriptionResponse,

--WSO subscription change request

subscriptionChangeRequest    SubscriptionChangeRequest,

--WSO subscription change response

subscriptionChangeResponse    SubscriptionChangeResponse,

--CE registration request

ceRegistrationRequest    CERegistrationRequest,

--Registration response

registrationResponse    RegistrationResponse,

--CM registration request

cmRegistrationRequest    CMRegistrationRequest,

--Coexistence set information request

coexistenceSetInformationRequest     CoexistenceSetInformationRequest,

--Coexistence set information response

coexistenceSetInformationResponse    CoexistenceSetInformationResponse,

--Coexistence report request

coexistenceReportRequest    CoexistenceReportRequest,

--Coexistence report response

coexistenceReportResponse    CoexistenceReportResponse,

--Reconfiguration request

reconfigurationRequest    ReconfigurationRequest,

--Reconfiguration response

reconfigurationResponse    ReconfigurationResponse,

--WSO channel classification request

channelClassificationRequest     ChannelClassificationRequest,

-- WSO channel classification response

channelClassificationResponse   ChannelClassificationResponse,

-- WSO channel classification update

channelClassificationAnnouncement     ChannelClassificationAnnouncement,

--Available channel list request from WSO

availableChannelsRequest      AvailableChannelsRequest,

-- Available channel list response from WSO

availableChannelsResponse     AvailableChannelsResponse,

-- Event indication

eventIndication     EventIndication,

-- Event confirm

eventConfirm     EventConfirm,

--WSO measurement request

measurementRequest      MeasurementRequest,

--WSO measurement response

measurementResponse      MeasurementResponse,

--WSO measurement confirm

measurementConfirm      MeasurementConfirm,

--WSO deauthentication request

deauthenticationRequest    DeauthenticationRequest,

--WSO deauthentication response

deauthenticationResponse    DeauthenticationResponse,

}

-----------------------------------------------------------

--WSO & CM authentication

-----------------------------------------------------------

--Request for authentication

AuthenticationRequest :: = SEQUENCE {

--WSO subscription ID

clientID    IA5String,

--WSO subscription password

clientPassword    IA5String

}

--Response for authentication

AuthenticationResponse :: = SEQUENCE {

--CM subscription ID

serverID    IA5String,

--CM subscription password

serverPassword    IA5String

--Status

status   CxMediaStatus

}

-----------------------------------------------------------

--WSO & CM subscription  & update

-----------------------------------------------------------

--Subscription request

SubscriptionRequest ::= SEQUENCE {

--Coexistence service to which WSO is subscribed

subscribedService   SubscribedService

}

-- Subscription response

SubscriptionResponse ::= SEQUENCE {

--Status

status   CxMediaStatus

}

-----------------------------------------------------------

--WSO subscription change

-----------------------------------------------------------

--Request to change subscription

SubscriptionChangeRequest ::= SEQUENCE {

--Coexistence service to which WSO is subscribed

subscribedService   SubscribedService

}

--Response for subscription change

SubscriptionChangeResponse :: = SEQUENCE {

--Status

status   CxMediaStatus

}

-----------------------------------------------------------

--WSO registration & update

-----------------------------------------------------------

--CE registration request

CERegistrationRequest ::= SEQUENCE {

--Operationg code

operationCode    OperationCode,

--Network ID

networkID    OCTET STRING,

--Network technology

networkTechnology    NetworkTechnology,

--Network type

networkType   NetworkType

--Discovery information

discoveryInformation   DiscoveryInformation,

--Transmission schedule is supported or not

txScheduleSupported    BOOLEAN,

--List of available channel number

listOfAvailableChNumbers      ListOfAvailableChNumbers,

--List of supported channel number

listOfSupportedChNumbers     SEQUENCE OF INTEGER,

--List of operating channel number

listOfOperatingChNumbers     ListOfOperatingChNumbers,

--Required resource

requiredResource    RequiredResource

--Measurement capability

measurementCapability      MeasurementCapability

}

--CM registration request

CMRegistrationRequest ::= SEQUENCE {

--Operationg code

operationCode    OperationCode,

--CE ID

ceID    CxID,

--Network ID

networkID    OCTET STRING,

--Network technology

networkTechnology    NetworkTechnology,

--Network type

networkType   NetworkType

--Discovery information

discoveryInformation   DiscoveryInformation,

--List of supported channel number

listOfSupportedChNumbers     SEQUENCE OF INTEGER,

}

-- Registration response

RegistrationResponse :: = SEQUENCE {}

-----------------------------------------------------------

--Coexistence set information

-----------------------------------------------------------

--Request for coexistence set information

CoexistenceSetInformationRequest ::= SEQUENCE {

listOfNetworkID      SEQUENCE OF OCTET STRING

}

--Response for coexistence set information

CoexistenceSetInformationResponse ::= SEQUENCE {

--Network ID

coexistenceReport     CoexistenceReport,

--List of neighbor CM

listOfneighborCM      ListOfneighborCM

}

-----------------------------------------------------------

--Coexistence report

-----------------------------------------------------------

--Request for coexistence report

CoexistenceReportRequest ::= SEQUENCE {}

--Response for coexistence report

CoexistenceReportResponse ::= SEQUENCE {

--Coexistence report information

coexistenceReport     CoexistenceReport,

--Channel priority information

channelPriority     ChannelPriority

}

-----------------------------------------------------------

--WSO reconfiguration

-----------------------------------------------------------

--Reconfiguration request

ReconfigurationRequest ::= SEQUENCE {

--List of operating channel number

listOfOperatingChNumber    SEQUENCE OF INTEGER,

--Transmission power limitation

txPowerLimit    REAL,

--Indication whether the channel is shared

channelIsShared    BOOLEAN,

--Transmission schedule

txSchedule    TxSchedule ,

-- Channel classification information

chClassInfo    ChClassInfo

}

--Reconfiguration response

ReconfigurationResponse ::= SEQUENCE {

--Status

status   BOOLEAN

--Failed parameters

failedParameters    FailedParameters

}

-----------------------------------------------------------

--Channel classification

-----------------------------------------------------------

--Channel classification request

ChannelClassificationRequest ::= SEQUENCE {

-- List of network ID

listOfNetworkID     SEQUENCE OF OCTET STRING

}

-- Channel classification response

ChannelClassificationResponse ::= SEQUENCE  OF SEQUENCE{

-- Network ID

networkID    OCTET STRING

--Channel classification information

chClassInfo      ChClassInfo

}

-----------------------------------------------------------

--Channel classification update

-----------------------------------------------------------

--Channel classification update

ChannelClassificationAnnouncement ::= SEQUENCE OF SEQUENCE {

-- Network ID

networkID    OCTET STRING

--Channel classification information

chClassInfo      ChClassInfo

}

-----------------------------------------------------------

--Available channel list from WSO

-----------------------------------------------------------

-- Available channel request

AvailableChannelsRequest ::= SEQUENCE {}

-- Available channel response

AvailableChannelsResponse ::= SEQUENCE {

--Available channel list information

listOfAvailableChNumbers      ListOfAvailableChNumbers

}

-----------------------------------------------------------

--Event indication

-----------------------------------------------------------

-- Event indication

EventIndication ::= SEQUENCE {

-- Event indication information

eventParams	EventParams

}

-- Event confirm

EventConfirm ::= SEQUENCE {}

-----------------------------------------------------------

--Measurement Request

-----------------------------------------------------------

-- Measurement request

MeasurementRequest ::= SEQUENCE {

-- Measurement request information

measurementDescription	MeasurementDescription

}

-----------------------------------------------------------

-- Measurement results

-----------------------------------------------------------

-- Measurement response

MeasurementResponse ::= SEQUENCE OF SEQUENCE{

-- Measurement results

measurementResult	MeasurementResult

}

-----------------------------------------------------------

--WSO & CM deauthentication

-----------------------------------------------------------

-- WSO deauthentication  request

DeauthenticationRequest ::= SEQUENCE OF {

--WSO subscription ID

clientID    IA5String,

--WSO subscription password

clientPassword    IA5String

}

-- WSO deauthentication  response

DeauthenticationResponse :: = SEQUENCE {

--CM subscription ID

serverID    IA5String,

--CM subscription password

serverPassword    IA5String

--Status

status   CxMediaStatus

}

END