IEEE P802.19
TV Whitespace Coexistence Method

| Update to procedures for profile E |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-05-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Hyunduk Kang | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | henry@etri.re.kr |
| Gwangzeen Ko | ETRI |  |  |  |
| Myung-Sun Song | ETRI |  |  |  |
| Jae-Ick Choi | ETRI |  |  |  |

Abstract

This document provides update to procedures for profile E for the Draft P802.19.1/DF3.03.

Proposal

It is proposed to add the following clause after clause 6.3.2.2

- 6.3.2.3  WSO Deregistration procedure

A CM shall perform this procedure when the CM needs to deregister WSO from the CM. An illustrative example of this procedure is shown in Figure 1. DeregistrationRequest, DeregistrationResponse, CMRegistraionRequest, and RegistrationResponse messages are defined in clause TBD.

- Figure 1 ・ WSO Deregistration

CM operation related to generating and sending the DeregistrationRequest message is specified in clause TBD.

After the CE receives the DeregistrationRequest message from the CM, the CE shall generate and send the Perform Deregistration.request primitive to the CE. CE operation related to generating and sending the Perform Deregistration.request primitive is specified in clause TBD.

After the CE receives the PerformDeregistration.response primitive from the WSO, the CE shall generate and send the DeregistrationResponse message to the CM. CE operation related to generating and sending the PerformDeregistration.request primitive is specified in clause TBD.

After the CM receives the DeregistrationResponse message from the CE, the CM shall generate and send the CMRegistrationRequest message to the CDIS. CM operation related to generating and sending CMRegistrationRequest message is specified in clause TBD.

After the CDIS receives the CMRegistrationRequest message from the CM, the CDIS shall generate and send the RegistrationResponse message to the CM. CDIS operation related to generating and sending RegistrationResponse message is specified in clause TBD.

Proposal

It is proposed to replace the following two clauses for clause from 6.3.9.1 to 6.3.9.4

- 6.3.9.1  Master/Slave CM selection procedure

A CM shall perform this procedure when the CM needs to select for master and slave CM for centralized decision making. An illustrative example of this procedure is shown in Figure 2. MasterCMRequest and MasterCMResponse messages are defined in clause TBD.

- Figure 2 ・ Master/Slave CM selection

CM operation related to generating and sending the MasterCMRequest message is specified in clause TBD.

After another CM receives the MasterCMRequest message from the CM, another CM shall generate and send the MasterCMResponse message to the CE. CM operation related to generating and sending the Perform MasterCMResponse message is specified in clause TBD.

- 6.3.9.2   Master/Slave CM configuration procedure

A CM shall perform this procedure when the CM needs to configure for master and slave CM for centralized decision making. An illustrative example of this procedure is shown in Figure 3. MasterSlaveCMConfigurationRequest and MasterSlaveCMConfigurationResponse messages are defined in clause TBD.

CM operation related to generating and sending the MasterCMRequest message is specified in clause TBD.

After another CM receives the MasterCMRequest message from the CM, another CM shall generate and send the MasterCMResponse message to the CE. CM operation related to generating and sending the Perform MasterCMResponse message is specified in clause TBD.

- Figure 3 ・ Master/Slave CM configuration