IEEE P802.19
TV Whitespace Coexistence Method

| Data types for profile E |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-05-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Hyunduk Kang | ETRI | 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, South Korea |  | henry@etri.re.kr |
| Gwangzeen Ko | ETRI |  |  |  |
| Myung-Sun Song | ETRI |  |  |  |
| Jae-Ick Choi | ETRI |  |  |  |

Abstract

This document provides data types for profile E for the Draft P802.19.1/DF3.03.

Proposal

The text below is proposed for normative annex A.1. based on profile E

Normative Annex Data Types Definition

IEEE802191DataType DEFINITIONS AUTOMATIC TAGS ::= BEGIN

-----------------------------------------------------------

--Exported data types

-----------------------------------------------------------

--Exported data types

EXPORTS

--Coexistence report

CoexistenceReport,

--Channel priority

ChannelPriority,

--Network technology

NetworkTechnology,

--Network type

NetworkType,

--Discovery information

DiscoveryInformation,

--List of available channel numbers

ListOfAvailableChNumbers,

--List of operating channel numbers

ListOfOperatingChNumbers,

--Required resource

RequiredResource,

--Measurement capability

MeasurementCapability,

--List of neighbor CM

ListOfNeighborCM

-- Operation code

OperationCode,

--Transmission schedule

TxSchedule,

--Failed parameters

FailedParameters,

--Subscribed service

SubscribedService,

--Status

CxMediaStatus,

--Coexistence protocol entity ID

CxID,

--Channel classification information

ChClassInfo,

--Channel classification information list

ChClassInfoList,

--Required information description

ReqInfoDescr,

-- Requested information value

ReqInfoValue,

--Event parameters

EventParams,

-- Negotiation status

NegotiationStatus,

-- Negotiation information

NegotiationInformation

-- Winner CM ID list

ListOfWinnerCMID,

-- Slot time position list

ListOfSlotTimePosition

--Measurement description

MeasurementDescription,

--Measurement result

MeasurementResult;

-----------------------------------------------------------

--Coexistence protocol entity ID

-----------------------------------------------------------

CxType ::= ENUMERATED {

ce,

cm,

cdis

}

CxID ::= SEQUENCE {

type    CxType,

id    OCTET STRING}

-----------------------------------------------------------

--Status

-----------------------------------------------------------

--Status

CxMediaStatus ::= ENUMERATED {

noErrorAccepted,

noErrorRejected,

errorInvalidEntityStatus,

errorInvalidArgument,

errorProcessFailure,

errorNetworkFailure,

errorUnknown

}

-----------------------------------------------------------

--Subscribed serviced

-----------------------------------------------------------

SubscribedService::= ENUMERATED {

information,

management,

interCMCoexistenceSetElementsNeighbors,

allCoexistenceSetElementsNeighbors

}

-----------------------------------------------------------

--Network technology

-----------------------------------------------------------

NetworkTechnology ::= ENUMERATED {

ieee802dot11af,

ieee802dot22,

ecma392,

oneSeg

…

}

-----------------------------------------------------------

--Network type

-----------------------------------------------------------

NetworkType ::= ENUMERATED {

fixed,

mode1,

mode2,

…

}

-----------------------------------------------------------

--Discovery information

-----------------------------------------------------------

DiscoveryInformation ::= SEQUENCE {

coordinateX	 		REAL,

coordinateY 			REAL,

coordinateZ 			REAL,

maxTxPower			REAL,

rxSensitivity			REAL,

antennaGain			REAL,

minReqSNR			REAL,

antennaHeight			REAL,

…

}

-----------------------------------------------------------

--Available channel numbers

-----------------------------------------------------------

ConstOfChUseID :: = ENUMERATED {

regulationMaxTxPower,

regulationMaxAntGain,

regulationMaxAntHeight,

regulationTVDBUpdateTime,

outOfBandEmissionLimit,

…

}

ConstOfChUseValue ::= CHOICE {

regulationMaxTxPower 	REAL,

regulationMaxAntMaxGain 	REAL,

regulationAntMaxHeight 	REAL,

regulationTVDBUpdateTime 	REAL,

outOfBandEmissionLimit 	REAL,

…

}

ConstOfChUses ::= SEQUENCE OF SEQUENCE {

constOfChUseID 	ConstOfChUseID,

constOfChUseValue 	ConstOfChUseValue

}

ListOfAvailableChNumbers ::= SEQUENCE OF SEQUENCE {

chNumber 		INTEGER,

availableStartTime 	GeneralizedTime,

availableDuration 	REAL,

constOfChUses 	ConstOfChUses

}

-----------------------------------------------------------

--Operating channel numbers

-----------------------------------------------------------

ListOfOperatingChNumbers ::= SEQUENCE OF SEQUENCE {

chNumber 		INTEGER,

occupancy	 	REAL

}

-----------------------------------------------------------

--Required resource

-----------------------------------------------------------

RequiredResource ::= SEQUENCE {

requiredBandwidth	REAL,

occupancy	 	REAL

}

-----------------------------------------------------------

--Measurement capability

-----------------------------------------------------------

MeasurementCapability ::= ENUMERATED {

energyDetection,

featureDetection

...

}

-----------------------------------------------------------

--Operation code

-----------------------------------------------------------

OperationCode ::= ENUMERATED {

New,

Modify,

Remove

}

-----------------------------------------------------------

--Coexistence report

-----------------------------------------------------------

CoexistenceReport ::= SEQUENCE OF SEQUENCE {

networkID			OCTET STRING,

networkTechnology		NetworkTechnology,

listOfOperatingChNumbers	ListOfOperatingChNumbers

}

ChannelPriority ::= SEQUENCE OF SEQUENCE {

channelNumber		INTEGER,

priority			INTEGER

}

-----------------------------------------------------------

--Coexistence set information

-----------------------------------------------------------

ListOfCoexSetElement ::= SEQUENCE OF SEQUENCE {

networkID			OCTET STRING,

networkTechnology		NetworkTechnology

}

ListOfNeighborCM ::= SEQUENCE OF SEQUENCE {

neighborCMID			CxID,

listOfCoexSetElement		ListOfCoexSetElement

}

-----------------------------------------------------------

--Transmission schedule

-----------------------------------------------------------

TxSchedule ::= SEQUENCE OF SEQUENCE{

scheduleStartTime 			GeneralizedTime,

scheduleDuration 			REAL,

numberOfScheduleRepetitions 		INTEGER,

transmissionStartTime 			REAL,

transmissionDuration 			REAL

}

-----------------------------------------------------------

--Channel classification

-----------------------------------------------------------

OperatingChannelInfo ::= SEQUENCE {

operatingChannelNumber 	INTEGER,

listOfNetworkID 		SEQUENCE OF OCTET STRING,

…

}

ChClassInfo ::= SEQUENCE {

availableChannelList 		SEQUENCE OF INTEGER,

restrictedChannelList 		SEQUENCE OF INTEGER,

protectedChannelList 		SEQUENCE OF INTEGER,

unclassifiedChannelList 	SEQUENCE OF INTEGER,

operatingChannelList 		SEQUENCE OF OperatingChannelInfo,

coexistenceChannelList 	SEQUENCE OF OperatingChannelInfo,

…

}

ChClassInfoList ::= SEQUENCE OF SEQUENCE {

networkID 		OCTET STRING,

chClassInfo 		ChClassInfo

}

-----------------------------------------------------------

--Failed parameters

-----------------------------------------------------------

FailedParameterID ::= ENUMERATED {

listOfoperatingChNumber,

txPowerLimit,

channelIsShared,

txSchedule,

}

FailedParameterValue ::= CHOICE {

listOfoperatingChNumber 		SEQUENCE OF INTEGER,

txPowerLimit 				REAL,

channelIsShared 			BOOLEAN,

txSchedule 				TxSchedule

}

FailedParameters ::= SEQUENCE OF SEQUENCE {

failedParameterID 		FailedParameterID,

failedParameterValue 		FailedParameterValue

}

-----------------------------------------------------------

--Event indication

-----------------------------------------------------------

EventDescr ::= ENUMERATED{

sinrThresholdReached,

qosDegradation,

…

}

EventParams ::= SEQUENCE {

eventDescr 	EventDescr

}

-----------------------------------------------------------

--Information Acquiring

-----------------------------------------------------------

ReqInfoDescr ::= SEQUENCE OF ENUMERATED {

sinr,

desiredBandwidth,

desiredOccupancy,

desiredQoS,

desiredCoverage,

channelNumber,

subscribedService,

interferenceLevel,

fairness,

threshold,

...

}

ReqInfoValue ::= SEQUENCE OF SEQUENCE {

reqInfoDescr	ReqInfoDescr,

reqInfoValue	CHOICE {

sinrValue 			REAL,

desiredBandwidthValue 		REAL,

desiredOccupancyValue 		REAL,

desiredQoSValue 			REAL,

desiredCoverageValue 		REAL,

channelNumberValue 		REAL,

subscribedServiceValue 		SubscribedService

interferenceLevelValue		REAL,

fairnessValue			REAL,

thresholdValue			REAL,

otherValue 			ANY

}

-----------------------------------------------------------

--Negotiation

-----------------------------------------------------------

NegotiationStatus :: = SEQUENCE {

negotiationSuccess    BOOLEAN,

negotiationFailure      BOOLEAN,

underNegotiation       BOOLEAN,

…

}

StartEndTime :: = SEQUENCE {

startTime    REAL,

endTime      REAL

}

TimeSharingUnitInfo ::= SEQUENCE {

referenceTime    REAL,

windowTime      StartEndTime,

slotTime             StartEndTime,

…

}

NegotiationInformation :: = SEQUENCE {

mode                                        BOOLEAN,

listOfChNumber                      SEQUENCE OF INTEGER

timeSharingUnitInfo               TimeSharingUnitInfo,

slotTimePosition                      StartEndTime,

numberOfSlots	                        INTEGER

disallowedSlotTimePosition    StartEndTime,

listOfContentionNumbers        SEQUENCE OF REAL,

…

}

ListOfWinnerCMID ::= SEQUENCE OF CxID

ListOfSlotTimePosition ::= SEQUENCE OF REAL

-----------------------------------------------------------

--Measurement

-----------------------------------------------------------

MeasurementSchedule ::= SEQUENCE {

measStartTime 		REAL,

numberOfMeasurements	INTEGER,

timeBetweenMeasurements 	REAL

}

MeasurementFreq ::= SEQUENCE OF INTEGER

MeasurementType ::= ENUMERATED {

interferenceLevel

}

MeasurementDescription ::= SEQUENCE {

measType 		MeasurementType,

measSchedule 		MeasurementSchedule,

measFreq 		MeasurementFreq

}

MeasurementReport ::= CHOICE {

interferenceLevelValue		REAL,

…

}

MeasurementResult ::= SEQUENCE OF SEQUENCE {

measurementDescription 	MeasurementDescription,

measurementReport	 	MeasurementReport,

}

END